Information for Software Development on the Amstrad 0 PDA600

What you will require:

1	One PenPad

2	The Software Developer's Manual for the PDA600

3	A development machine (preferably MS-DOS compatible).

4	A PCMCIA static RAM card

5	A PCMCIA card drive attached to the development machine

6	A suitable cross assembler/compiler

7	Optionally, other tools such as debuggers, simulators etc.

Amstrad will only be supplying the first two of these items as a combined package for £255+VAT. Alternatively, the manual will be available separately for £49. It is left to developers to get the other items from a third party vendor. More of this later.

Method

The PenPad is based on a static variant of the Zilog Z180 processor which is code compatible with the Z80. Therefore, the cross assembler or cross compiler you use must produce Z80 code. The Software Developer's manual describes the Application Program Interface (API) that the PenPad firmware makes available to third party software writers. This includes about 120 routines necessary for memory allocation, text and graphics output (including high level windows functions), pen input, serial port access, floating point maths, etc. etc.

Developers should read and agree the licence to use the Eden Group API and return the registration card (supplied with the Developer's manual) before considering development of software on the PenPad.

Normally PenPad programs would be written in assembler or C on the PC making use of the API routines. This is assembled/compiled and then linked to produce a runnable Z80 binary image with a special header that the PenPad will recognise as a runnable program (as described in the Software Developer's manual).

The resultant binary image is written onto a PCMCIA static RAM card which is then inserted into the PenPad. The user would then touch the card icon (D) on the desktop (4) to execute the program which could either be a "type 1" windowed application or a "type 2" full screen application.

While the use of an SRAM card during development makes the card writing/software testing cycle as short as a couple of seconds, it is envisaged that finished programs would be written (on PCs) to either OTPROM (One Time Programmable ROM) cards or FLASH EPROM cards. If quantity was very large, masked ROM cards could be used.

The PenPad only has a single PCMCIA version 1 slot and, as it is probable that an application may require quite a lot of RAM to run (although it can share some of the internal RAM of the machine), we would expect that most software developers would consider delivering their application on a card that had a mix of both SRAM and mask/OTP/Flash. Most card manufacturers are willing to make combination cards to order if quantities are sufficient.

Combined cards

Developers who want to market their own applications (see Distribution & Marketing below) may be interested to know that the following manufacturers of PCMCIA cards have confirmed the possibility of constructing combination cards:

Mitsubishi	New Media Corporation	Smart Modular Technologies

Sources of equipment

PCMCIA SRAM cards are starting to become widely available everywhere. Both Amstrad and Amsoft PDA Peripherals sell these items if they cannot be found elsewhere.

PCMCIA card drives to attach to a PC are also available from a number of sources: We have experience of using a couple of these (DIP and Thincard). The only requirement is that they should be able to write a raw binary image to a card (it is possible they may insist on treating it as a DOS FAT compatible drive - this is no good).

DIP sell a "Pico card drive" for about £150. This is a PCMCIA card reader/writer that attaches to the PC's parallel port. Its only slight drawback is that it isn't supplied with routines that will just write a raw binary image to a card. We have written simple routines that allow this to be done and can make these available to anyone who requires them.

We have also used the Databook Thincard drive which is a PCMCIA card reader/writer that mounts internally in a PC drive bay. It is quite expensive (about £300) but does include comprehensive software support including routines to read/write raw binary images.

Mitsubishi have recently announced a developers package that consists of both a PC based PCMCIA card drive (from Elan) which is also being sold with a Mitubishi 1MB PCMCIA SRAM card. The complete package is £299. The package does not include routines to write binary images or to program Flash/OTP/Mask.

A company called Chase Advanced Technologies have recently announced the forthcoming availability (June/July) of two different PCMCIA drives. One is external and connects to the PC's parallel port while the other is an internally mounted drive. Pricing has not been finalised but initial indications are that it will be very competitive.

An alternative to using a PCMCIA SRAM card and a PCMCIA card drive is available from Ranger Computers who produce a "double headed" PCMCIA card which, to the target machine just looks like a PCMCIA memory card but on the other side it maps into the PC memory so the moment that compilation/assembly is finished the resultant code is available to the target machine.

Software tools

In theory it would be possible to use any Z80 language compiler or assembler to write programs for the PenPad but the experience of our own developers, Amsoft, suggests that this may not always be the case. The operating system of PenPad has been written in a combination of Z80 assembler and C. The C compiler used was the IAR ICCZ80 v3.10 compiler. This had to be adapted by Amsoft in certain areas. The modified compiler/libraries are available from Amsoft as detailed below.

Probably the best known source for software developer's tools is Grey Matter. Some typical pricing for their products is as follows:

MS-DOS based Z80 cross assemblers

2500AD Z80 Cross assembler (series 4 - 4.05a)			£190

Avocet AvmacZ80							£320

IAR AZ80 v1.96							£185

Prospero Z80 Cross asm						£145

METAi-01 v5.0 (cross assemble for 50 CPUs)			£445

MS-DOS based C cross compilers

2500AD Z80 C Cross Compiler Series 4				£485

Avocet AVXCZ80 (incl. AvmacZ80 cross assembler)		£605

Aztec C80/ROM 4.10B						£315

Hi-Tech C Z80 7.11							£299

IAR ICCZ80 v3.10							£1,080

Lattice ES-80 v3.04							£375

MS-DOS based Pascal cross compiler

Prospero Pascal Z80 Cross Compiler				£299

In addition to these commercially available language products there are several shareware products that could be used. In particular there is a cross asembler known as Table ASseMbler (TASM (not the Borland Turbo Assembler!)) which is available from many PD software libraries and bulletin boards.

Developers support and complete development kit

Amsoft PDA Peripherals, a division of Eden Group Ltd, is a company that has been specifically set up to market peripherals for the PenPad. It has been formed by the company who wrote the PenPad operating system and the Software Developer's manual. As such, Amsoft is better able to give guidance to developers than anyone else.They have put together two kits containing all the equipment needed to develop programs for the PenPad. This includes the Software and Hardware Technical Reference manual which contains explanation of programming methods and many worked examples of code.

Not only do these kits contain all the tools necessary for PenPad development at less than the cost of the constituent parts, but perhaps the most valuable item in these packages is the on-line support that is available from the experts who actually wrote the PenPad operating system.

IDE Kit

Software and Hardware Technical Reference manual for PDA600

Memory card development kit (Drive and 512KB SRAM card)

including software to write SRAM, OTP, Flash and mixed cards

Integrated Development Environment to run on a PC

including IAR compiler, debugger, library routines, library manager, editor, linker, graphics and native code generator for PDA600.

Full on line support

£850 + VAT

Cross Assembler kit

Software and Hardware Technical Reference manual for PDA600

Memory card development kit (Drive and 512KB SRAM card)

including software to write SRAM, OTP, Flash and mixed cards

Cross Assembler to run on a PC

Full on line support

£750 + VAT

Distribution & Marketing

Obviously any developer is quite free to distribute and market their resultant program in whatever way they choose but Amsoft have been specially set up to cater for the Amstrad PDA market and you may wish to discuss the possibilities of marketing your application through them. By marketing a final program via Amsoft you will benefit from the fact that Amsoft are able to bulk buy PCMCIA combination cards and program Flash cards on an as-needed basis. They already have a mail order business set up for direct sales to the public and, perhaps most importantly, they have their sales brochure inserted in every PenPad box and are potentially in contact with everyone who purchases a PenPad.

Contacts

Amsoft PDA Peripherals	Amstrad Spares Dept.

PO Box 111	169 Kings Road

Macclesfield	Brentwood

Cheshire	Essex, CM14 4EG

Tel: 0625 572779	Tel: 0277 209509

Fax: 0625 572894	Fax: 0277 209559

Smart Modular Technology	Mitsubishi Electric UK Limited

45531 Northport Loop West	Electronic Products Group

Fremont	Travellers Lane

CA 94538	Hatfield

USA	Herts.,    AL10 8XB

Tel: 010 1 510 623 1231	Tel: 0707 276100

Fax: 010 1 510 623 1434	Fax: 0707 278625

New Media Corp.	Chase Advanced Technologies

15375 Barranca Parkway	Salt's Mill

Building B101	Victoria Road

Irvine	Shipley

CA 92718	West Yorkshire

USA	BD18 3JD

Tel: 010 1 714 453 0100	Tel: 0274 583888

Fax: 010 1 714 453 0114	Fax: 0274 581633

DIP Systems Ltd.	Ranger Computers Ltd.	Grey Matter Ltd

32 Frederick Sanger Road	2 Meeting Lane	Prigg Meadow

Surrey Research Park	Duston	Ashburton

Guildford	Northampton	Devon

GU2 5XN	NN5 6JG	TQ13 7DF

Tel: 0483 301555	Tel: 0604 589200	Tel: 0364 654100

Fax: 0483 578072	Fax: 0604 589505	Fax: 0364 654200