IBM PC SERVER 500 SYSTEM/390

QUICK PRODUCT GUIDE

IBM                    IBM PC SERVER 500 SYSTEM/390

QUICK PRODUCT GUIDE

ii  Quick Reference

PREFACE

_______

Welcome to the QUICK PRODUCT GUIDE AND REFERENCE.  for the PC SERVER

_________

S/390.

_____

TRADEMARKS

The following trademarks and (*) registered trademarks of the IBM Corporation

may appear in this document.

DB2*                               OfficeVision/VM

DB2/2                              OS/2*

EtherStreamer                      PS/2*

GDDM*                              RACF

IBM*                               S/370

IBMLink                            S/390*

IMS                                Streamer

LAN Distance                       VisualAge

LANStreamer                        VisualGen

Micro Channel*                     VM/ESA*

MVS/ESA                            VM/XA

NetFinity                          VTAM*

OfficeVision*                      WIN-OS/2

This  guide  can  be  used  to  help  you determine a correct and

reasonable configuration for a PC Server S/390.    It  should  be

used  in  conjunction  with  the ICPA configurator tool to ensure

reasonable and valid configurations.

This guide containes the following sections:

Product / Component Lists:

Lists of the machines and components likely to be ordered as a PC

Server S/390 system.  These lists are organized in the  following

groups:

1.  System Units

2.  System Unit Memory (PC Server & S/390)

3.  PC Server S/390 specific adapters

4.  LAN Adapters

o   Token Ring

o   Ethernet

5.  Disk Storage

o   Internal disk systems

o   External enclosure configurations

o   Disk drives

(C) Copyright IBM Corp. 1995                                              iii

6.  Other

o   Tapes

o   Displays

o   UPS

o   Other / Misc.

USEFUL INFORMATION:

1.  Miscellaneous hints and tips (things to watch out for)

2.  Configuring for dial-in access to the PC Server S/390

3.  Getting  updates  for PC Server S/390 software, configurators

(and this document).

4.  A list of Certified PC Server S/390 resellers

SAMPLE/TYPICAL  CONFIGURATIONS:      Some   sample   or   typical

configurations for several various uses of the PC Server S/390.

Change Log

Changes implemented in each version are noted here.

06/07/95  Added  information  on  tapes  and Wide Area Connector.

Corrected DASD planning aid.

06/08/95  Updated  sample  hardware/software  configurations   to

reflect  current  ICPA  output  (with prices) and minor

adjustments to  configurations  to  better  demonstrate

configuration options.  Numerous typo, minor fixes.

08/25/95  Added  new  'P  Series' displays (655x) and removed old

series.   Old displays  (952x  series)  are  no  longer

available.   Note: did not change sample configs to use

new displays.

09/15/95  Corrected part numbers for several adapters, added more

3516  information,  added  more  Wide  Area   Connector

information,   minor   updates   to   sample   hardware

configurations.   Changed  sample  configs  to  reflect

different monitors.

9/25/95   Configurations  updated  to  reflect reduced prices for

2.25 GB disk drives.

10/18/95  Replaced  'Value'  series  displays  (V15,  etc.)  with

'General Purpose' (G14, G15, G17) displays.

iv  Quick Reference

10/27/95  Updated  sample  configs  with  current monitors, added

more   info   about   slot/bay/bank   terminology   and

configuration, minor change to DASD configurator tables

in checklist.

12/12/95  Added  information about new 8641-MYC model, with power

supply and backplanes pre-installed.  Also noted change

to FCC Class A.

01/09/96  Converted to 'Bookmaster' format and created PostScript

version.  Added sections on  getting  software  updates

and  a  list  of  certified resellers. Added section on

configuring  'Dial-in'  capability.    Updated   VM/ESA

configs  to  Version  2 and dropped old versions of VSE

and MVS.

01/23/96  Updated sample configurations to current prices (as  of

1/19/96).    Added  table  on APC UPS capacity.   Added

'common  problems'  section  and  various  other  minor

changes.

03/25/96  Updates    to   include   information   about   3/12/96

announcements,  additional  external   expansion   unit

information  and  updated  sample configurations to use

most current options.

04/08/96  Multiple   small   editing   changes    for    improved

readability.    Removed 'Common Problems' section since

it is  now  available  electronically  in  the  READ-ME

document.

05/10/96  Added   "Pre-Configured  CD-ROM"  Product/Feature  code

table.  Updated Reseller list.

Preface  v

vi  Quick Reference

CONTENTS

________

PREFACE  . . . . . . . . . . . . . . . . . . . . . . . . . .  III

Trademarks   . . . . . . . . . . . . . . . . . . . . . . .  iii

CHAPTER 1.  PC SERVER S/390 ENHANCEMENTS   . . . . . . . . . .  1

Highlights   . . . . . . . . . . . . . . . . . . . . . . . . .  1

Description  . . . . . . . . . . . . . . . . . . . . . . . . .  2

CHAPTER 2.  SYSTEM UNITS   . . . . . . . . . . . . . . . . . .  5

Specific models  . . . . . . . . . . . . . . . . . . . . . .  5

Other Notes  . . . . . . . . . . . . . . . . . . . . . . . .  6

Memory Options   . . . . . . . . . . . . . . . . . . . . . . .  6

Wide Area Network (WAN) Adapters   . . . . . . . . . . . . . .  7

LAN Adapters   . . . . . . . . . . . . . . . . . . . . . . . .  9

CHAPTER 3.  DASD AND ADAPTERS  . . . . . . . . . . . . . . .   11

Terminology  . . . . . . . . . . . . . . . . . . . . . . .   11

Internal   . . . . . . . . . . . . . . . . . . . . . . . .   12

Hard drives  . . . . . . . . . . . . . . . . . . . . . . .   12

External DASD Enclosures   . . . . . . . . . . . . . . . . .   12

CHAPTER 4.  OTHER COMPONENTS   . . . . . . . . . . . . . . .   19

Tapes  . . . . . . . . . . . . . . . . . . . . . . . . . .   19

Displays   . . . . . . . . . . . . . . . . . . . . . . . .   20

Uninterruptible Power Supplies (UPS)   . . . . . . . . . .   21

Miscellaneous Options  . . . . . . . . . . . . . . . . . .   22

CHAPTER 5.  DIAL-IN ACCESS   . . . . . . . . . . . . . . . .   23

LAN Distance   . . . . . . . . . . . . . . . . . . . . . .   23

8235 'Dial In Access to Lans' (DIALs)  . . . . . . . . . .   25

In addition...   . . . . . . . . . . . . . . . . . . . . .   26

CHAPTER 6.  OBTAINING SOFTWARE AND CONFIGURATOR UPDATES  . .   27

'Sites' of interest  . . . . . . . . . . . . . . . . . . .   27

Configurator Updates   . . . . . . . . . . . . . . . . . .   28

Other Files  . . . . . . . . . . . . . . . . . . . . . . .   28

CHAPTER 7.  CERTIFIED PC SERVER S/390 RESELLERS  . . . . . .   31

CHAPTER 8.  SAMPLE CONFIGURATIONS  . . . . . . . . . . . . .   33

Base Software Prices   . . . . . . . . . . . . . . . . . .   39

Typical Configuration Costs  . . . . . . . . . . . . . . . .   44

Preconfigured Software Stacks  . . . . . . . . . . . . . .   46

CHAPTER 9.  PC SERVER S/390 ORDER CHECKLIST  . . . . . . . .   47

Disk Configuration Aid   . . . . . . . . . . . . . . . . .   49

(C) Copyright IBM Corp. 1995                                              vii

viii  Quick Reference

CHAPTER 1.  PC SERVER S/390 ENHANCEMENTS

________________________________________

New and enhanced functions for the IBM PC Server 500

System/390(R) (PC Server S/390(R)) family of models are

now available.  The new functions are delivered as a code

update for the S/390 Microprocessor Complex.  This update

consists of both Licensed Internal Code and non-licensed

code.

The enhancements include Channel Program support for

Extended Count Key Data (ECKD), 3390/9345 device support

for application developers and commercial accounts and

3350/3330 DASD support for migrating from  older levels

of S/370(TM) hardware.  In addition, the enhanced

functions address existing S/370 customers that have

specific communication requirements such as Synchronous

Data Link Control (SDLC) and Binary Synchronous Control

(BSC) support and TCP/IP terminal emulation to support

3270 data streams.  Support for multiple SCSI tape drives

and improved support for PC printers are included.

Several performance enhancements have been made to the

execution of decimal instructions, support of I/O assist

in VM/ESA, and some I/O path lengths have been shortened

in ESA mode.

The enhanced update is available at no additional charge

or additional license fees to the existing and future

customers of the IBM PC Server 500 S/390 systems.

PLANNED AVAILABILITY DATES: 3/15/95

HIGHLIGHTS

__________

Enhanced code update is offered at no additional charge

or additional license fee.

o   New DASD Support

-   ECKD Channel Program support

-   3390, 3380 Triple Dense, 9345 and 3350/3330

device support

o   Enhanced Communications

-   SDLC - Portmaster Adapter/A  (VM/VSE)

-   SDLC - WAC   (VM/VSE)

-   BiSync - Portmaster Adapter/A  (VM/VSE)

-   BiSync - WAC  (VM/VSE)

-   3172 SDLC Gateway   (MVS)

o   Printer

-   Enhance support for PC printers

o   Tape Support

(C) Copyright IBM Corp. 1995                                                1

-   Support of multiple SCSI tapes

DESCRIPTION

___________

The following functions are either enhanced or added with

the new IBM PC Server 500 S/390 code being made

available.

o   New DASD Support

-   ECKD

-   3390

-   3380 Triple Dense

-   9345

-   3350 / 3330

o   Communications

-   SDLC - Portmaster Adapter/A  (VM/VSE)

-   Up to 2 adapters - 16 lines

-   8 Port Synchronous

-   Switched/non-switched RS-232

-   Autodial V.25 Bis

-   up to 19.2 Kbps

-   Supports VM/VSE environments as ICA

-   SDLC - WAC   (VM/VSE)

-   Up to 4 adapters - 8 lines

-   Switched/non-switched RS-232 and V.35

-   Non-Switched X.21

-   Autodial V.25 Bis

-   up to 2.048 Mbps

-   Supports VM/VSE environments as ICA

-   BiSync - Portmaster Adapter/A  (VM/VSE)

-   Up to 2 adapters - 16 lines

-   8 Port Synchronous

-   Switched/non-switched RS-232

-   up to 19.2Kbps

-   Supports VM/VSE environments as ICA

-   BiSync - WAC  (VM/VSE)

-   Up to 4 adapters - 8 lines

-   Switched/non-switched RS-232 and V.35

-   Non-Switched X.21

-   Autodial V.25 Bis

-   up to 2.048 Mbps

-   Supports VM/VSE environments as ICA

-   3172 SDLC Gateway   (MVS)

-   WAC/Artic-Portmaster Adapter/A

-   SDLC communications

-   Supports MVS environments

o   Printer

-   Enhance support for PC printers

o   Tape Support

-   Support multiple SCSI tapes

The details of the affected device managers follows:

2  Quick Reference

o   AWS3172

The current AWS3172 module will be replaced by three

modules, LAN3172, MGR3172 and WAN3172 to look like

the real 3172 mod 3 in terms of its three base

components.

-   LAN3172

Provides the base Link Services Architecture

(LSA) Peripheral and SubArea LAN 802.2

Interconnect function through VTAM.  Multiple

SNA/LSA Subchannel addresses can be specified and

multiple LAN adapters will be supported through a

single SNA/LSA address.

-   MGR3172

Provides the NetView Box Manager function for use

with NetView in VM and MVS environments.

-   WAN3172

Provides 3172 SNA Interconnect capability to the

VTAM XCA major node by mapping LSA LAN adapters

to SDLC communications using either WAC or

ARTIC/Portmaster Adapter/A lines in a MVS

environment.  The intent is to provide SDLC

connections to MVS VTAM which would otherwise

require a 37x5 NCP/PEP.  SDLC will support leased

point to point, dial in point to point or

multidropped.  Only dialed in connections are

supported.

o   AWSCKD

Provides extended count key data DASD emulation for

3390/9345/ECKD which includes 3380 triple dense

connected to a 3880 model 3 control unit.  In

addition DASD emulation for 3330/3350 devices will be

provided including track overflow. CKD read only

support will be supported for various DASD devices.

o   AWSPBS

Provides support for both SDLC and BiSYNC protocol on

PC Server S/390 utilizing the ARTIC/Portmaster/A

adapter as an Integrated Communication Adapter (ICA).

Both SDLC and BiSYNC can reside simultaneously on the

same adapter card.  Synchronous modes may be

individually selected for each of the eight ports.

Any of the 8 ports can be configured for either SDLC

or BiSYNC at data rates from 1200 to 19,200 bps.  All

ports are independently capable of maximum data

rates.  In addition, Autodial V.25 bis will be

supported.  SDLC is used to transmit EBCDIC code in a

SNA network whereas BiSYNC can transmit both EBCDIC

or ASCII code.  Transmission exchanges are supported

in half-duplex mode only.  Both SDLC and BiSYNC will

support point-to-point switched, point-to-point

non-switched, or multidropped.

Chapter 1.  PC Server S/390 Enhancements  3

o   AWSICA

Provides support for either SDLC or BiSYNC function

over a wide area network using the Wide Area

Connector (WAC) adapter.  Both switched and

non-switched RS-232 and V.35 communication lines and

non-switched X.21 are supported along with Autodial

V.25 bis.  The maximum data rate supported will be

2.048 Mbps.  The WAC adapter will provide up to 2

ports per card with maximum of 4 cards per system.

o   AWS2821

The device manager will include the use of a control

file.  The control file is a simple OS/2 ASCII file,

created with any OS/2 editor which will contain basic

control lines.  The user will have the ability to

send control characters to the PC printer, before

printing any output.  This will allow the printer to

receive controls such as rotating output, selecting

character set and being able to print in duplex mode.

4  Quick Reference

CHAPTER 2.  SYSTEM UNITS

________________________

There are four  primary Models of the PC Server S/390

They all have the following common components:

o   90MHz Pentium Processor

o   32MB ECC Std Memory 70ns  (For Pentium)

o   256KB L2 Write Back Cache Memo

o   S/390 MicroProcessor Complex

o   32MB ECC memory for S/390

o   P/390  LIC

o   2.25 GB F/W SCSI-2 Disks (number varies by model)

o   2.88MB Diskette

o   4X CD-ROM Drive & ServerGuide Rel 2.21 or later

o   4/10GB 4mm DAT tape drive

o   Certified FCC 'Class A'

o   Parallel & Serial(2) ports

o   Mouse port and mouse

o   IBM Keyboard

o   RAID or SCSI-2 Adapter

o   SVGA-NI Graphics Board (1024/768 max)

o   3 Year On-Site Warranty

o   ServerGuide, Netfinity

o   Reference & Diagnostic Diskettes

o   PC Server Reference manuals

o   PC Server S/390 specific Reference Manuals

o   OS/2 Warp, Comm Mgr/2

SPECIFIC MODELS

8641-NYV  NON-ARRAY, 4.5 GB:  Non-Array model with two

2.25 GB SCSI-2 Fast & Wide hard drives.

8641-MYS  ARRAY,  6.75 GB:  Array model with three 2.25

GB Fast & Wide hard drives.  Provides 4.5GB useable

capacity in RAID-5 configuration.

8641-MYF  ARRAY, 11.25 GB:  Array model with five  2.25

GB Fast & Wide hard drives.  Provides 9.0GB useable

capacity in RAID-5 configuration.

8641-MYC  ARRAY, 13.50 GB:  Array model with six  2.25 GB

Fast & Wide hard drives.  Provides 11.25GB useable

capacity in RAID-5 configuration.  Also has the optional

power supply and both optional backplanes preinstalled.

(C) Copyright IBM Corp. 1995                                                5

OTHER NOTES

FCC Classification

All models of the PC Server S/390 are certified as

meeting FCC CLASS A requirements.  "Class A" devices are

intended for use in a "Business Environment", rather than

in a "Residential Environment".

RAID Adapter Cache

Array models come with the IBM Streaming RAID Adapter

which includes 4 MB of RAM, of which more than 3M is

available for Caching.  Non-Array model does not have

cache on the SCSI adapter.

MEMORY OPTIONS

______________

PC Server / Pentium Memory:

In general, any PC Server S/390 being used primarily or

exclusively for S/390 processing (no significant OS/2

workload) will not require any additional PC memory.  If

the PC Server S/390 is to do any significant OS/2 based

work (such as a LAN Server), additional PC memory may be

required to support that workload.

+---  NOTE: --------------------------------------------+

|                                                       |

| All Pentium/Server memory must be installed in        |

| matched pairs of SIMMS.  For 32M total expansion, use |

| 2 16M SIMMs rather than 1 32M SIMM.                   |

|                                                       |

+-------------------------------------------------------+

The PC Server S/390 has a total of 8 memory sockets, with

two of these occupied by the standard 32M of memory (2 x

16MB).  SIMM sizes can be mixed as long as each 'pair' of

SIMMs is the same size.

92G7209   16MB 70ns ECC memory kit (use sets of 2)

92G7210   32MB 70ns ECC memory kit (use sets of 2)

6  Quick Reference

S/390 Processor Memory

We recommend configuring PC Server S/390 systems with

more S/390 memory than might normally be expected because

of the I/O characteristics.  Large S/390 memory sizes

allow the use of "data in memory" techniques and reduces

paging activity, both of which will usually improve

system performance significantly.

Additional S/390 memory comes on a 'daughter card' that

attaches to the S/390 Microprocessor complex card.  The

96M daughter card occupies an adjacent Micro Channel slot

(for power only)

26H2931   32MB ECC memory Expansion for S/390

26H2978   96MB ECC memory Expansion for S/390 (occupies

adjacent MC slot) This memory card is included

and preinstalled on the -MYF and -MYC models.

WIDE AREA NETWORK (WAN) ADAPTERS

________________________________

The following adapters and parts may be used to provide

WAN services via SDLC and BSC communication facilities.

92G7516   MultiProtocol Adapter/A:  Provides SDLC port

(up to 19,200 bps), useful for remote SNA

connection.  Supported on VM and VSE as an ICA

(Integrated Communications Adapter) port.  Not

supported on MVS systems.  (Comm adapter cable

for MultiProtocol:  1502067) (Before using the

MultiProtocol Adapter card, consider the WAC

(below) for improved performance and

expandability.)

33G846X   Wide Area Connector:  One card with multiple

'personalities' to support up to two

communications lines at up to 2.048M bps.  Max

of 4 WAC cards per system.  Can support:

RS-232, V.35, X.21, interfaces and can connect

to various services such as DDS, SW-56, ISDN

Basic, T-1, E-1, and Fractional T-1.  (Part

numbers vary depending on type of interface

required:

o   33G8465 = EIA RS232  (up to  19,200 bps)

o   33G8466 = V.35       ( typically 56Kb )

o   33G8467 = X.21       (packet switched

protocol)

The base adapter (part numbers just above)

provides one communications port or line.  An

Chapter 2.  System Units  7

additional interface and connecting cable is

required to provide the second line.  The part

numbers for this interface/connector are:

o   33G8461 = EIA RS232  Interface & connector

o   33G8462 = V.35       Interface & connector

o   33G8463 = X.21       Interface & connector

The base adapter and expansion interface can be

mixed types.  For example, you can use a

33G6466 (V.35) base adapter with a 33G8461

(RS-232) expansion interface.

+---  NOTE: ----------------------------------+

|                                             |

| * Please note that the PC Server S/390 only |

| provides support for RS-232 and V.35        |

| (switched or non-switched) and X.21         |

| (non-switched only).                        |

|                                             |

+---------------------------------------------+

The WAC adapter with AWSICA provides either

SDLC of BSC support for VM and VSE systems.

Lines which run at 19.2 Kbs or less normally

use the RS-232 interface while lines running at

56Kbs or greater normally use the V.35

interface. Since the WAC plus AWSICA emulate an

ICA, this combination cannot be used on MVS

systems.   For MVS systems, the WAN3172 device

manager can be used to support the WAC adapter

as an SNA VTAM XCA Port.

PORTMASTER/A The "IBM Realtime Interface Co-Processor

Portmaster Adapter/A" (Portmaster) provides up

to 8 SDLC or BSC lines per adapter.  For VM and

VSE systems it is supported by either the

AWSPBS manager or the WAN3172 manager.  For MVS

systems, the WAN3172 manager must be used.  In

any of these systems, each Portmaster adapter

can support up to 8 lines (each line up to 19.2

Kbps) with a maximum of 2 Portmaster adapters

per system.  The AWSPBS manager (VM and VSE)

supports either SDLC or BSC, or a mixture

(WAN3172 only supports SDLC).  Multiple parts

are required for each Portmaster installation.

The following is a typical configuration for

the PC Server S/390

8  Quick Reference

+----------------------------------------------------+

| Table 1.   Typical Portmaster Configuration:       |

|          AWSPBS, WAN3172                           |

+----------+----------------------+-------+----------+

|  PART    |  DESCRIPTION         |       |  PRICE   |

| NUMBER   |                      | QUANTI|Y         |

+----------+----------------------+-------+----------+

| 53F2607  | ARTIC Portmaster/2   |   1   |  $       |

|          | coprocessor adapter  |       | 1,699.   |

|          | for Mch:   2MB       |       |          |

+----------+----------------------+-------+----------+

| 53F2610  | 8 Port RS-232        |   1   |  $       |

|          | interface            |       | 295.     |

+----------+----------------------+-------+----------+

| 53F2619  | 8 connection cable   |   1   |  $       |

|          |                      |       | 246.     |

+----------+----------------------+-------+----------+

|          |    Total             |       |  $ 2,240 |

|          |                      |       |          |

+----------+----------------------+-------+----------+

LAN ADAPTERS

____________

There are many LAN Adapters types offered by IBM, and

even more from other manufacturers.

+---  GENERAL GUIDELINE --------------------------------+

|                                                       |

| If you are not sure what kind of LAN adapter to get,  |

| choose one of the ...Streamer MC 32 adapters.  They   |

| are good performance ones appropriate for use on the  |

| PC Server S/390                                       |

|                                                       |

+-------------------------------------------------------+

Token Ring Adapters

60G1592   Auto LANStreamer MC 32 Adapter:  A good,

all-around, high performance Token Ring

adapter. 32 Bit, Streaming (40 MB/sec)

BusMaster.

73G7137   Dual LanStreamer MC 32:  Essentially two LAN

Streamers on a single adapter.  Just the thing

for 'heavy duty' LAN traffic or for connecting

to multiple rings.

Chapter 2.  System Units  9

Ethernet LAN Adapters

74G0850   EtherStreamer MC 32 Adapter:  A good,

all-around, high performance Ethernet adapter.

32 Bit, Streaming (40 MB/sec) BusMaster.

Supports 10Base2, 10BaseT, and remote IPL.

73G7136   Dual EtherStreamer MC 32: Essentially two

EtherStreamers on a single adapter.  Just the

thing for 'moderate' Ethernet traffic or for

connecting to two segments. Two 10BaseT ports.

(10Base-2 not avail.)

OTHERS    Other LAN Adapters for special environments:

06H5184: Quad-BT PeerMaster Adapter: For

Heavy-duty Ethernet networks, or

interconnecting/bridging multiple segments.

Can even use multiple PeerMaster adapters in

one system. "BT" is for 10Base-T (unshielded

twisted pair) media.

06H6041: Quad-B2 PeerMaster Adapter: Same as

the "BT", except that the "B2" is for 10Base-2

media (thin coax).

48G7171: A non-streaming Ethernet adapter for

'light-duty' networking.

10  Quick Reference

CHAPTER 3.  DASD AND ADAPTERS

_____________________________

+--- NOTICE: -------------------------------------------+

|                                                       |

| Configuration of PC Server S/390 systems can be       |

| confusing and difficult, especially for larger        |

| systems.  Use of ICPA or PS-CFG is highly             |

| recommended.  Consult with a Server Specialist if you |

| have any doubts or questions.                         |

|                                                       |

+-------------------------------------------------------+

TERMINOLOGY

This document will use the following terminology when

referring to the PC Server S/390 configuration:

SLOT      A MicroChannel slot on the system board into

which a MicroChannel adapter is inserted.

BAY       A position in the front of the system into

which a single device (hard drive, tape unit,

CDROM, etc.) can be inserted.

BANK      A group of similar 'Bays'.

The Bays and Banks are arranged and 'named' in the

following manner.

BANK A    Bank A consists of bays A1 and A2 which come

with the 2.88 MB diskette drive and CDROM drive

installed.

BANK B    Bank B Consists of bays B1 and B2.  These come

with the DAT tape drive and the 'glove box'

installed.

BANK C    Bank C is the first 'hard drive' bank with bays

C1 - C6.  The base (standard) hard drives will

be installed in this bank and bay C1 will be

'blocked' since the DAT tape uses the SCSI

address that would normally be used by a hard

drive in C1.  All models include a hot-swap

backplane installed in this bank.

BANK D    Bank D is the second hard drive bank with bays

D1 - D6.  All six bays are available for use.

The 'MYC' model comes with the backplane for

this bank preinstalled.

(C) Copyright IBM Corp. 1995                                               11

BANK E    Bank E is the third hard drive bank with bays

E1 - E6.  All six bays are available for use.

The 'MYC' model comes with the backplane for

this bank preinstalled.

INTERNAL

Maximum of 17 internal hard drives (with 1 inch form

factor).  Expanding beyond 5 drives  (into  bank 'D')

requires additional Power Supply and a 'drive bay

Backplane'.  Expanding into the third bank (more than 11

drives) requires another backplane.  (Note: The 'MYC'

model has the power supply and both optional backplanes

preinstalled.)  ICPA and PS-CFG will properly configure

power supply, backplanes and additional RAID/SCSI

adapters.

HARD DRIVES

94G2650   2.25GB SCSI Fast & Wide: 7ms access, 7200 RPM.

This will be the 'standard' drive for most

customers.

OTHERS    Other drives may be used for specialized

applications.  Check with your Server

Specialist.

94G2649 1.12GB SCSI Fast & Wide: 7ms access,

7200 RPM.  This will generally only be used in

special situations, possibly to provide more

r/w 'heads' for maximum performance.

94G2651 4.51GB SCSI Fast & Wide: 8ms access,

7200 RPM.  1.6 inch form factor, (only 3 drives

per hard drive bank).  May be useful in special

applications.

EXTERNAL DASD ENCLOSURES

________________________

Some users of the PC Server S/390 will find that they

need more disk storage capacity than the 30 Gigabytes

(useable, RAID-5) which is available within the system

unit.  For these users, the solution may be an external

expansion enclosure.  This section will provide an

introduction to the available expansion enclosures and

show some sample configurations.

12  Quick Reference

+---  NOTE: --------------------------------------------+

|                                                       |

| Configuration of external expansion units in          |

| conjunction with PC Server S/390 systems in optimal   |

| combinations can be difficult and confusing.  Persons |

| not very familiar with these units are encouraged to  |

| get assistance from a Server Specialist or other      |

| appropriately trained personnel.                      |

|                                                       |

+-------------------------------------------------------+

There are currently four external expansion units

available from IBM

o   3511

o   3516

o   3517  (SCSI Multi-Storage Enclosure)

o   3518  (Enterprise Expansion Enclosure)

The 3511 and 3516 are being phased out in favor of the

3517 and 3518.  Anyone considering the use of a 3511 or

3516 should ensure the components are available prior to

final commitment to these units.

All the expansion units and configurations discussed

below will assume the appropriate number of external SCSI

(RAID) connectors are available on the main system unit

to support the expansion configurations.  Each

configuration will include the appropriate number of SCSI

cables to connect the external unit to the system unit.

Although all of these enclosures can accept multiple disk

types, we will only discuss 2.25 GB and 4.5 GB disks, as

these are the ones most likely to be used in this

environment.

3511:

The 3511 is a flexible unit that can accept several disk

types, but it is also relatively complex to configure and

requires significant assembly.  The 3511 is NOT a

Hot/Swap capable unit.  The maximum storage capacity

(effective RAID-5) of a 3511 using 4.5 GB disks is 22.5

GB.  Components of the 3511 typically include:

35110V0   The actual floor standing enclosure

03H3231   SCSI F/W connector to SCSI F/W External Option

cable

06H3153   Upgrade 3511 to SCSI-Wide capable (2 required)

94G2648   Drive installation kit (one per hard drive)

Chapter 3.  DASD and Adapters  13

VARIOUS   Disk Drives

07H0387   2.25 GB disk drive  (max of 6 per

3511)

07H0834   4.5 GB disk drive (max of 6 per 3511)

SAMPLE CONFIGURATION

35110V0   Base Enclosure unit

03H3231   SCSI F/W connection cable

06H3153   Convert internal connections to Fast/Wide (2

required)

94G2648   Hard Drive installation Kit  (6; one per hard

drive)

07H0834   4.5GB Hard drive  8ms access time

3516:

The 3516 is a popular, hot/swap capable expansion unit

that can accept up to 7 hard drives.  It is relatively

simple to configure and install.  The maximum storage

capacity (effective RAID-5) of a 3516 using 2.25 GB disks

is 13.5 GB.  (Due to configuration and availability

considerations, the 2.25 GB drive is the only one likely

to be used in a 3516.)  Components of the 3516 typically

include:

35167V0   The actual floor standing or desk-top enclosure

06H3231   SCSI F/W connector to SCSI F/W external option

cable

94G3346   How/Swap tray; one needed per hard drive

07H0387   2.25 GB disk drive  (max of 7 per 3516)

SAMPLE CONFIGURATION

35167V0   Base Enclosure unit

06H3231   SCSI F/W connection cable

94G3346   Hot/Swap tray installation Kit  (7: one per

hard drive)

07H0387   2.25GB Hard drive  8ms access time (7 drives)

3517,  SCSI Multi-Storage Enclosure:

The 3517 is a new hot/swap capable expansion that uses

the new Tray type III.  It will accommodate up to five

2.25GB or 4.5GB hot/swap drives.  It is very easy to

configure and install.  The maximum hot/swap storage

capacity (effective RAID-5) of a 3517 using 4.5 GB disks

is 18.0 GB.  In addition to the Hot/Swap hard drives, up

14  Quick Reference

to two additional SCSI devices can be installed in the

3517.  This can be used for additional 4mm DAT tape

drives or non-Hot/Swap hard drives.  By using these

non-hot/swap drive bays for additional 4.5 GB disk

drives, total useable (RAID-5) capacity can be extended

to 27GB.  Components of the 3517 might typically include:

3517001   The actual floor standing or desk-top

enclosure

06H3231   SCSI F/W connector to SCSI F/W option cable

74G8631   4MM DAT tape drive

VARIOUS   Hard disk drives

70G9862   2.25 GB SCSI-II Fast/Wide Hot Swap

high performance HD-III (max 5)

70G9863   4.5 GB SCSI-II Fast/Wide Hot Swap

high performance HD-III (max 5)

94G3055   2.25 GB SCSI drive for use in

non-Hot/Swap drive bays (max 2)

94G3057   4.5  GB SCSI drive for use in

non-Hot/Swap drive bays (max 2)

SAMPLE CONFIGURATION

This configuration will provide a total of 22.5 GB

useable (RAID-5) disk space plus one 4mm DAT tape drive.

One of the 4.5GB drives will not be non-hot/swap.  The

4mm DAT is in addition to the one which comes standard

with the PC Server S/390

3517001   Base Multi-SCSI expansion unit

06H3231   SCSI F/W connection cable

70G9863   4.5GB SCSI-II Fast/Wide HD-III hard disk drive

(5 drives)

74G8631   4MM DAT tape drive

94G3057   4.5 GB SCSI drive for use in non-Hot/Swap drive

bay

3518,  Enterprise Expansion Enclosure

The 3518 is a new enclosure which uses the same frame and

layout as the PC Server 500/520 family.  In addition to

the three banks of hot/swap hard drive bays, it can also

accomodate two half-high or one full-hight 5.25" drive

(such as CD-ROM drives or Tape drives).  Each hard drive

bank can accomodate either 6 slim-height drives (1.12GB

or 2.25GB) or 3 half-height drives (4.5GB).  Two of the

banks can be 'daisy-chained' to provide up to six

half-height drives on a single SCSI channel.  Using a

combination of two SCSI channels and both 2.25GB and

4.5GB hard drives, a total of 33.75GB of useable (RAID-5)

Chapter 3.  DASD and Adapters  15

hot/swap disk capacity can be provided.  Between the PC

Server S/390 unit itself, the 3518, RAID/SCSI Adapter

considerations, internal/external SCSI connectors, DASD

Array layouts, disk sizes and capacities, and system

performance, configuration of this must consider many

different aspects.  Anyone not very familiar with these

systems are urged to seek trained server personnel for

assistance.  Some of the components of the Enterprise

Expansion Enclosure:

3518001   The actual floor standing enclosure

06H3231   SCSI F/W connector to SCSI F/W option cable

(may require multiple)

74G8631   4MM DAT tape drive for use in non-hot/swap bays

70G9864   Backplane to media bay cable:  connects devices

in top 'media bay' to the first H/S backplane.

94G4070   Backplane to backplane cable (to "daisy-chain"

two backplanes into one SCSI channel)

70G9876   Cable to connect 2nd or 3rd backplane to

external SCSI port

70G9855   Hot/Swap Backplane III (required when using

hard drives in the 2nd and 3rd banks)  Not

interchangable with PC Server 500 backplanes.

70G9739   Power Supply upgrade:  required when expanding

beyond the top bank of hard drives

VARIOUS   Hard disk drives

70G9862   2.25 GB SCSI-II Fast/Wide Hot Swap

high performance HD-III (max of 6 per

bank)

70G9863   4.5 GB SCSI-II Fast/Wide Hot Swap

high performance HD-III (max of 3 per

bank)

SAMPLE CONFIGURATION

This configuration will provide a total of 33.75GB

useable (RAID-5) disk space plus one 4mm DAT tape drive.

All hard drives will be hot/swap.  Two SCSI channels will

be required to support this configuration.

3518001   Base Enterprise Expansion Enclosure

06H3231   SCSI F/W connection cable (2 required)

70G9862   2.25GB SCSI-II Fast/Wide HD-III hard disk drive

(6 drives) will be used in the first hard drive

bank.

16  Quick Reference

70G9863   4.5GB SCSI-II Fast/Wide HD-III hard disk drive

(6 drives) will be used in the 2nd and 3rd hard

drive banks

74G8631   4MM DAT tape drive for use in non-hot/swap bays

70G9864   Backplane to media bay cable:  required to

connect the tape drive to the backplane and

SCSI channel

94G4070   Backplane to backplane cable (to "daisy-chain"

banks 2 and 3 into one SCSI channel with 6

4.5GB drives)

70G9876   Cable to connect 2nd backplane to external SCSI

port

70G9855   Hot/Swap Backplane III (2 required for the 2nd

and 3rd banks).

70G9739   Power Supply upgrade:  required since we are

using 2nd and 3rd bank of hard drives

Chapter 3.  DASD and Adapters  17

18  Quick Reference

CHAPTER 4.  OTHER COMPONENTS

____________________________

Some other components that may be needed or useful.

TAPES

There are several forms of tape support for the PC Server

S/390:

4MM DAT:  The 4mm DAT drive is included with every PC

Server S/390  It can be used by OS/2 using normal device

drivers.  Or, it can be used by the S/390 operating

system using the SCSI34x0 device manager.  The SCSI3480

driver makes the 4mm DAT appear to the S/390 system as a

3480 device, with the added benefit of the 4 - 10 GB

capacity of the DAT.

HURON ATTACHED TAPE DRIVES:  The S/370 Channel Emulator/A

(HURON) adapter can be used to attach 'real' 34x0 tape

devices, such as 3420, 3480 and 3490.  In this manner,

the tape drives can be used by the S/390 system just as

on a ES/9000 system, except for the performance

implications of the HURON adapter.

SCSI ATTACHED (OEM) TAPE DRIVES:  Another alternative is

to attach OEM produced 3420/3480/3490 media compatible

tape drives via the SCSI channel on the PC Server S/390

These devices are typically 'desktop' sized and range in

price from about $3000 (3420) to $14000 (3490E).  The

performance levels will also vary considerably, with the

higher cost models providing very good performance

(comparable to parallel channel on a ES/9000).  One of

these devices will require a SCSI address on an external

SCSI channel/connection.  On RAID systems it may be

appropriate to add a SCSI F/W adapter to provide this

connection rather than use the external RAID channel.

The 3490 (and 3490E) compatible SCSI tape drives must be

defined to the S/390 Operating System running on the PC

Server S/390 as a 3480 with IDRC (compaction).  In this

manner they will provide most of the 3490E capabilities,

but some of the extended functions may be limited or not

available.  The following tape drives have been tested

and are currently supported for use with the SCSI3480.SYS

driver:

o   3480 18 track

-   Fujitsu M2481, M2483, M2485

-   Overland Data C480, C490

(C) Copyright IBM Corp. 1995                                               19

o   4mm

-   IBM 4/10 GB (74G8631 or 74G8632), 55F9428

It is worthwhile noting that the IBM Brand 3490 SCSI

models CANNOT be used on the PC Server S/390.  The IBM

3490 requires a 'differential' SCSI interface while the

PC Server S/390 provides a 'single-ended' SCSI interface.

The following tape drives have been tested and are

currently supported for use with the SCSI3420.SYS driver:

o   3420  9 track

-   M4 Data  9914

-   Overland Data 5512,  5612,  TR995

-   Qualstar 1260S, 3404S, 3412S, 3414S, 3418S

o   4mm

-   IBM 4/10 GB (74G8631 or 74G8632), 55F9428

+---  NOTE: --------------------------------------------+

|                                                       |

| When purchasing a SCSI attached OEM tape drive        |

| through a vendor you should make sure the vendor      |

| understands and has experience with the PC Server     |

| S/390 and that the specific device manufacturer,      |

| model, and version has been shown to work with the PC |

| Server S/390                                          |

|                                                       |

+-------------------------------------------------------+

DISPLAYS

IBM markets two lines of quality color displays suitable

for use with the PC Server S/390  The Professional ('P')

series is a very high quality line which is recommended

when the system display is going to be used intensively.

The General Purpose ('G') series is a quality line of

displays.

Because of the high use of 'windows' and the GUI, we

recommend at least a 15" display.  If the display is

going to be used extensively, a 17" display is highly

recommended.

6553-503  15" "Professional" series.  1280x1024, Non

Interlaced, Multi-Frequency .26mm dot pitch.

DDC1 and DDC2B complient.

6554-603  17" "Professional" series.  1600x1200, Non

Interlaced, Multi-Frequency .26mm dot pitch.

Recommended best all-around display for use

with the PC Server S/390.

20  Quick Reference

6555-703  20" "Professional" series.  1600x1200, Non

Interlaced, Multi-Frequency .31mm dot pitch.  A

VERY nice display.

6542-103  (G40) 14" "Genral Purpose" series.  1024x768,

Multi-Frequency .28mm dot pitch.

6542-301  (G40) 14" "Genral Purpose" series.  1024x768

Non-Interlaced, Multi-Frequency .28mm dot

pitch. Reduced EMF Emissions,

6543-301  (G50) 15" "General Purpose series.  1024x768

Non Interlaced, Multi-Frequency .28mm dot

pitch.

6544-403  (G70) 17" "General Purpose" series.  1280x1024

Non Interlaced, Multi-Frequency .28mm dot

pitch,. reduced EMF emissions.

UNINTERRUPTIBLE POWER SUPPLIES (UPS)

Every PC Server S/390 should have a UPS. Period.  At

these price points, a UPS has to be one of the best

'values' in the industry.  For base servers, a 700 watt

UPS will provide power for at about 20 minutes.  For

larger configurations, a 1000 or 1400 watt UPS may be

appropriate.  20 minutes of protection will be sufficient

to cover most power outages and allow time for a clean

shutdown, if necessary.

94G3134   APC Smart-UPS 700.  4 outlets

94G3135   APC Smart-UPS 1000.  6 outlets

94G3136   APC Smart-UPS 1400.  6 outlets

The following table is summarized from APC's marketing

material for their Smart-UPS UPS models when used with

IBM PC Servers. The first row would apply to a PC Server

500 S/390 that has only the primary 434 Watt power supply

and is drawing full power (implying five hard drives in

Bank C). The second row assumes full population with hard

drives in banks C, D, and E and the required installation

of the secondary 220 Watt power supply.

Chapter 4.  Other Components  21

+------------------------+--------+-------+---------------------------------+

| PC SERVER MODEL        |  WATTS |   VA  |        RUNTIME IN MINUTES       |

+------------------------+--------+-------+--------+-------+--------+-------+

|                        |        |       | 700-VA | 1000-V| 1400-VA| 2200-V|

+------------------------+--------+-------+--------+-------+--------+-------+

| PC Server S/390        |   434  |  434  |   6.4  |   13  |   21   |   57  |

+------------------------+--------+-------+--------+-------+--------+-------+

| PC Server S/390, >5    |   654  |  654  |    -   |   7   |   12   |   32  |

| disks                  |        |       |        |       |        |       |

+------------------------+--------+-------+--------+-------+--------+-------+

MISCELLANEOUS OPTIONS

While most of these adapters can also be used with other

systems, they tend to be especially useful and

appropriate on the PC Server S/390.

46F3786   S/370 Channel Emulator/A:  Provides Parallel

"Bus and Tag" connection to traditional S/370

I/O Devices  (tape, printers, Comm.)  Maximum 2

per PC Server S/390. Not a 'high performance'

device.

70G7365   ServerGuard:  Provides additional monitoring

and remote operation abilities, including

remote or scheduled power off/on.

70G9739   Additional 220 Watt power supply.  Required

when expanding beyond the first hard drive bank

(C) (more than 5 drives) (Included with the MYC

model.)

70G9740   Hot/Swap Backplane and cables.  One needed for

each drive hard drive bank used beyond first

(C).  Each additional backplane accommodates 6

disk drives.  (Both optional backplanes are

included in the MYC model.)

70G8498   SCSI Fast & Wide Adapter:  For non-Array

systems.  A SCSI adapter will generally be

needed for each drive bank or backplane.  Has

external connector for use with external drive

tower or device (such as a SCSI tape drive).

70G9263   Additional Streaming RAID Adapter: Additional

RAID adapter needed when expanding into third

drive bank.  (Each RAID adapter controls up to

two banks or 12 drives.)

22  Quick Reference

CHAPTER 5.  DIAL-IN ACCESS

__________________________

Many customers have asked how to configure the PC Server

S/390 to support dial-in access via phone lines.  Typical

uses would include 'work-at-home', mobile users, and

off-site demo capabilities.  While there are a number of

ways to accomplish the dial-in objective, most users will

also want full LAN connectivity to allow access to other

resources on the LAN which the PC Server S/390 is

connected to.  IBM offers two types of products to

accomplish this full dial-in LAN connection while

allowing 3270 emulation access to the PC Server S/390

1.  LAN Distance is primarily a 'software' solution that

can run on the PC Server S/390 or any other suitable

system on the LAN.

2.  The 8235/DIALs product is primarily a 'hardware'

solution that attaches to the LAN with little or no

impact on the PC Server S/390 or other systems on the

LAN.

Both products have corresponding programs that run in the

remote workstations to provide the dial-in and connection

capabilities.  A more detailed description of each

product and the specific hardware and software products

required follows.

LAN DISTANCE

+---   NOTE: -------------------------------------------+

|                                                       |

| The Lan Distance product is being withdrawn from      |

| marketing effective April 26, 1996 (see announcement  |

| letter 996-059).  The program will still be supported |

| and is available as part of the "Warp Server" and     |

| "Warp Connect" products.  Although it is being        |

| formerly withdrawn, you may be able to find copies    |

| available via IBM resellers.                          |

|                                                       |

+-------------------------------------------------------+

The LAN Distance Server component runs on the PC Server

S/390 or other suitable PC on the LAN.  Remote users dial

into modems connected to either the standard PC serial

ports or to multi-port adapters such as the 'Artic

Multiport/2'.  It provides extensive security protection

and supports all popular communication protocols and

connectivities.  Once connected to the LAN Distance

(C) Copyright IBM Corp. 1995                                               23

server, remote users have all the facilities of the LAN,

as though locally connected to the LAN (except, of

course, for the difference in communication speeds).

Many different configurations are possible, with up to

128 concurrent users.  Here is a somewhat typical

configuration that would support up to 8 concurrent users

at speeds up to 28,800 bps.

+---------------------------------------------------------------+

| Table 2.   Sample 8 user LAN Distance Configuration           |

+----------+----------------------+-------+----------+----------+

|  PART    |  DESCRIPTION         |       |  PRICE   |  TOTAL   |

| NUMBER   |                      | QUANTI|Y(EACH)   | COST     |

+----------+----------------------+-------+----------+----------+

| 52G8364  | 8 Connection LAN     |   1   |  $       |   $      |

|          | Distance Server for  |       | 595.     | 595.     |

|          | OS/2                 |       |          |          |

+----------+----------------------+-------+----------+----------+

| 53F2604  | ARTIC Portmaster/2   |   1   |  $       |   $      |

|          | coprocessor adapter  |       | 1,313.   | 1,313.   |

|          | for Microchannel     |       |          |          |

+----------+----------------------+-------+----------+----------+

| 53F2610  | 8 Port RS-232        |   1   |  $       |   $      |

|          | interface            |       | 249.     | 249.     |

+----------+----------------------+-------+----------+----------+

| 53F2619  | 8 connection cable   |   1   |  $       |   $      |

|          |                      |       | 246.     | 246.     |

+----------+----------------------+-------+----------+----------+

| 92G7449  | 28.8 DataFAX         |   8   |  $       |   $      |

|          | External Modem, V.34 |       | 249.     | 1,992.   |

+----------+----------------------+-------+----------+----------+

|          |    Total             |       |          |    $     |

|          |                      |       |          | 4,395.   |

+----------+----------------------+-------+----------+----------+

Each remote user must also install and use a copy of LAN

Distance Remote, at a cost of $69. per remote

workstation.  LAN Distance Remote is available for OS/2

and Windows 3.1 users.

While this configuration is believed to be correct, all

customers should verify that any configuration ordered

will operate correctly.  If you plan to run the LAN

Distance Server on a PC Server S/390, we recommend using

a separate LAN Adapter port for LAN Distance.  A

Dual-ported LAN adapter works well for this purpose.

24  Quick Reference

8235 'DIAL IN ACCESS TO LANS' (DIALS)

The 8235 product offers a independent unit that has it's

own built-in LAN adapter connection and supports either

internal or external modems for remote users to dial

into.  A full range of security features are available,

including support for dialback and a TACAS client for DCE

Kerberos users.  Once connected to the 8235, remote users

have full use ot the LAN environment which the 8235 is a

part of.  A Wide range of configurations are possible,

for both Ethernet and Token Ring LANs, supporting many

different protocols and connection methods.  Using the

internal modems, multiple 8235 units can be stacked

together, providing a very compact installation.

A typical configuration to support up to 8 concurrent

dial in 28.8 kbps connections would be:

+---------------------------------------------------------------+

| Table 3.  Sample 8 user 8235 DIALs Configuration              |

+----------+----------------------+-------+----------+----------+

|  PART    |  DESCRIPTION         |       |  PRICE   |  TOTAL   |

| NUMBER   |                      | QUANTI|Y(EACH)   | COST     |

+----------+----------------------+-------+----------+----------+

| 38H5600  | 8235-031 8 port unit |   1   |  $       |   $      |

|          | with Token Ring LAN  |       | 5,739.   | 5.739.   |

|          | connection           |       |          |          |

+----------+----------------------+-------+----------+----------+

| 38H5575  | 28.8 V.34 internal   |   8   |  $       |   $      |

|          | modem unit for 8235  |       | 699.     | 5,592.   |

+----------+----------------------+-------+----------+----------+

|          |    Total  Cost       |       |          |   $      |

|          |                      |       |          | 11,331.  |

+----------+----------------------+-------+----------+----------+

Each remote user must also install and use a copy of the

DIALs remote client program.  There is no charge for

using as many copies of the client program as needed.

Remote users connect to the host network and work in

their OS/2, Windows, Windows 95, DOS, Macintosh, or UNIX

environment with no retraining or changes to their

applications

While this configuration is believed to be correct, all

customers should verify that any configuration ordered

will operate correctly.

Chapter 5.  Dial-In Access  25

IN ADDITION...

Both products offer multiple other features and

capabilities, such as WAN and inter-LAN connections,

Dial-out, remote administration, and choice of protocols

(IPX, TCP/IP, SNA, X.25, NetBIOS, 802.2, NetBEUI).  Since

these products supply only the 'connection services',

each remote workstation must have it's own application

programs such as:

1.  Communications Manager/2 for 3270

connection/emulation for OS/2

2.  PC/3270 for 3270 connection/emulation for DOS/Windows

3.  LAN Requestor to be a user/requestor on a LAN Server

network

26  Quick Reference

CHAPTER 6.  OBTAINING SOFTWARE AND CONFIGURATOR UPDATES

_______________________________________________________

Updated PC Configurators (ICPA and PS-CFG) and various

software components for the PC Server S/390 will

occasionally be made available for customer use.  These

updates will usually be distributed via the IBM PC

Company "Bulletin Board System" (BBS), its' associated

World-Wide-Web (WWW) page, and "File Transfer Protocol"

(FTP) site.  Files of interest to PC Server S/390 users

will generally be available from the PC SERVER  and OS/2

DRIVERS  sections of the FILES  area.

'SITES' OF INTEREST

BBS

The Bulletin Board System can be accessed by calling

(919) 517-0001.

PC Company Web Page

The PC Company Web page is HTTP://WWW.PCCO.IBM.COM .  To

directly access the "Files" section, use

HTTP://WWW.PCCO.IBM.COM/FILES.HTML

You can select the "PC Servers" or "OS/2 Drivers" links

to view and download the available software.

S/390 Web Page

For additional descriptive information about the PC

Server S/390, you may want to visit the S/390 Home Page

at:

HTTP://WWW.S390.IBM.COM/PRODUCTS/P390/P390.HTML

File Transfer Protocol Site

The FTP site is FTP.PCCO.IBM.COM   (all files are in the

/PUB  directory).  Logon as user ANONYMOUS.  To go direct

to the "PC Servers" directory, use

FTP.PCCO.IBM.COM/PUB/PC_SERVERS  and to access the OS/2

Drivers directory, use FTP.PCCO.IBM.COM/PUB/OS2_DRIVERS

Remember to transfer any program files in "Binary" mode

and file names are case sensitive.  The file ALLFILES.TXT

(C) Copyright IBM Corp. 1995                                               27

in the PUB directory contains a complete listing of all

files available and a brief description of each.

CONFIGURATOR UPDATES

There are currently two different configurators (ICPA and

PS-CFG) and they are available from two different sites.

ICPA

ICPA is available from the FTP, WWW, and BBS sites noted

above.  From the WWW, it is located in the FILES area,

then SOFTWARE FILES, OS/2 FILES, OS/2 APPLICATIONS. The

files you will need to run ICPA (USA version) will be:

1.  ICPAUSA.NWS     (news file)

2.  ICPAUSA1.EXE    (Self Extracting EXE file)

3.  ICPAUSA2.EXE    (Self Extracting EXE file)

4.  ICPAUSA3.EXE    (Self Extracting EXE file)

(Note: ICPAUSA4-8 are optional files which have

additional detail on many of the PC products.  They are

not required for most PC Server S/390 configuration

purposes.)

PS-CFG

The PS-CFG configurator for the USA is available from the

FTP site FTP.PS.BOULDER.IBM.COM  (198.17.57.66).  Login

this FTP site with userid of ANONYMOUS and password of

your userid.

The PS-CFG files are in the /PS/TOOLS/PSCFG  directory

and you will need the following files:

1.  PSC-USA1.EXE    (Self Extracting EXE file)

2.  PSC-USA2.EXE    (Self Extracting EXE file)

If you have a current copy of PS-CFG it has a complete

sample FTP session in the HELP  facility.

_____

OTHER FILES

A number of other files will be maintained on the BBS,

ftp server, and Web page.  Some of the ones of interest

include:

QREF390.ZIP The most current version of the PC Server

S/390 Quick Product Reference  (PKZipped file

containing a PostScript document).

28  Quick Reference

READ390.ZIP The most current version of the "READ-ME

First" document for the PC Server S/390

(PKZipped file containing a PostScript

document).

WARP390.ZIP A unique and special 'Boot Logo' for the PC

Server S/390, with instructions for installing.

Also includes a  .BMP  (Bitmap) version for use

as a background.

Chapter 6.  Obtaining Software and Configurator Updates  29

30  Quick Reference

CHAPTER 7.  CERTIFIED PC SERVER S/390 RESELLERS

_______________________________________________

IBM Direct can assist you in configuring your PC Server

S/390 and accompanying S/390 software.  They can help you

locate other IBM personnel who can provide additional

services, such as configuration, installation, and

migration.

IBM DIRECT     IBM Direct, IBM Corp.

Call 1-800-IBM-CALL (1-800-426-2255), Key

Code LE001

The Certified PC Server S/390 resellers below have been

to training classes on the PC Server S/390 and can

generally offer a full range of services, from initial

proposal through installation and migration/conversion.

CONTINUUM      Continuum of Austin, Texas.

Contact George Kindinger at (512)

338-7166.

CORNERSTONE    Cornerstone Systems, Inc. of Burbank,

California.

Contact Roy Jones at (714) 505-2741.

DEVTECH        DevTech Associates, Inc. of Naperville,

Illinois.

Contact John Denning, Sr., Vice President

of Marketing at (708) 355-4404.

DHI            DHI Computing Corporation of Provo, Utah

Contact Les Chappell at  (801) 429-2129

GIBRALTER      Gibralter Computer Group of Conyers,

Georgia.

Contact George Warren, VP Marketing,

404-929-3480.

HARTFORD       Hartford Computer Group, Inc. of

Inverness, Ill.

Contact Tony Graffia Jr., at  (708)

934-4461  x-4116

HEALTH CHARGE  Health Charge Corporation of Skokie, Ill.

Contact David Anderson,  at (312) 583-9800

(C) Copyright IBM Corp. 1995                                               31

INFORMATION TECHNOLOGY Information Technology Corporation

of Falls Church, VA.

Contact John Cotte at (703) 237-7370.

INTELLIWARE    Intelliware Systems, Inc. of Arlington,

Texas.

Contact Dick Ducotey at (817) 461-1700.

INTERPROCESS   Interprocess Systems, Inc. of Roswell,

Georgia.

Contact Gordon Chamberlain, President,

(770) 410-1700.

LINK           Link Computer Corporation of Altoona,

Pennsylvania.

Contact Robert Plants at (814) 946-3085.

PC COM         PC COM, Inc. of Tampa, Florida.

Contact Steve Friedman, President at (813)

288-9800.

QUEST          Quest Integration Services of Hudson, Ohio

Contact Timothy Paulus, Vice President, at

(216) 528-0448

REAL           Real Applications, Inc. of Woodland Hills,

California

Contact Mike Monroe, at (714) 440-7500.

STAN WEEKS     Stan Weeks, Inc. of Bellevue, Washington.

Contact Stan Weeks, at (206) 453-5166.

TECH-BEAMERS   Tech-Beamers, Inc. of Poughkeepsie, New

York

Contact Stanley Graham, Vice President, at

(914) 473-4444

UNIVERSAL      Universal Software, Inc. of Brookfield,

Connecticut.

Contact Gene Gilsenan, Vice President of

Marketing at (203) 792-5100.

VM ASSIST      VM Assist of San Francisco, Calif.

Contact Melissa Curry,  at (800) 616-1144

32  Quick Reference

CHAPTER 8.  SAMPLE CONFIGURATIONS

_________________________________

VSE Small distributed system

Configuration Pricing

as of 3/23/1996

Pricing Method:     List Price

No of free slots:   1                   Date Printed:    Mar 23 1996

Price File Date:    1 Mar/96            Program Version: V7.04

P/N              Description            Unit Price    Qty          Price

---              -----------            ----------    ---          -----

8641NYV  PC Svr 390 NYV P90 32MB/4 GB     21,659.00      1      21,659.00

AVAILABLE ONLY VIA IBM DIRECT

OR CERTIFIED RESELLERS

2 x 2.25 GB SCSI-2 F/W H'Disk

90MHz Pentium Processor

S/390 CMOS Microproc. Complex

Memory:       Std.    Maximum

ECC 70ns:  32 MB    256 MB

S/390:  32 MB    128 MB

L1 Cache:  16 KB     16 KB

L2 WBCache: 256 KB    256 KB

8 full length 32 bit slots

40MB/Sec Streaming Data Suppt

32-bit DMA Controller

Synchronous Channel Check

System Error Logging

SynchroStream Technology

2.88MB Diskette, 4/10 GB Tape

2x CD-ROM Dr. & ServerGuide

Parallel & Serial(2) ports

Mouse Port, Mouse

VPD & C2 Security Enabled

Enhanced Security LogicLock

SCSI-2 F/W Busmaster Adapter

SVGA-NI Graphics Board

OS/2 Warp with WINOS2

Communications Mgr/2 1.11

NetFinity

3 Year On-Site Warranty

Supports OS/2, S/390 app's

------end of system unit------

S390+32  (Std. S/390 Memory Card- 32 MB   Not Listed      1     Not Listed

Memory Upgrades (48-256 MB)

(C) Copyright IBM Corp. 1995                                               33

64MB (16MB ECC SIMMS)

92G7209  16MB 70ns ECC Memory Kit OBI         579.00      2       1,158.00

FC 2097

46F3786  S/370 Channel Emul Adapter/A       3,095.00      1       3,095.00

Hot Swap Disks

INTL Hot Swap Drives

Multiple 2GB Fast/Wide Disks

2x 2 GB SCSI-2 F/W HD 7ms

94G2650  2 GB SCSI-2 F/W HS Disk 8ms       1,079.00      2       2,158.00

LAN Adapters

Dual EtherStreamers (1-3)

73G7136  Dual EtherStreamer MC 32 Adapt       885.00      1         885.00

FC 2624

2-10BaseT ports

70G7365  PS/2 SERVERGUARD                     885.00      1         885.00

FC 9023

UPS Options

94G3134  APC Smart-UPS 700 - 4 outlets        479.00      1         479.00

6.2H x 5.4W x 14.1D (inches)

6544403  G70 17" FST Col. Monitor             819.00      1         819.00

Viewable screen area 15.7"

.28mm dot pitch, MPR-II

up to 1284 x 1024, 75 MHz NI

ISO & "plug & play" (VESA DDC)

Digital controls, tilt/swivel

53F2607  ARTIC Portmaster/2 (2MB)           1,699.00      1       1,699.00

FC 6368

72F0163  6-Port V.35 Interface Bd/A           765.00      1         765.00

72F0165  6-Port V.35 Cable                    619.00      1         619.00

Total Price            34,221.00

RATIONALE:

1.  Distributed only has 'copy' of central host data, so RAID

redundancy is not required.

2.  Relatively small VSE system, no additional S/390 memory required

3.  Also handles office LAN Server processing, so include additional

32MB RAM for OS/2 / LAN Server.

4.  Local network is Ethernet based, include dual EtherStreamer

5.  Attaching old printer, so include S/370 Channel Emulator/A

6.  Include UPS and ServerGuard for remote support facilities

7.  Inter-connected to central host and other remotes via

56KB lines using V.35 version of MultiPort/2 adapter.

34  Quick Reference

VM Development system

Configuration Pricing

as of  3/23/1996

Pricing Method:     List Price

No of free slots:   3                   Date Printed:    Mar 23 1996

Price File Date:    1 Mar/96            Program Version: V7.04

P/N              Description            Unit Price    Qty          Price

---              -----------            ----------    ---          -----

8641MYS  PC Svr 390A MYS P90 32M/6.75GB    25,569.00      1      25,569.00

AVAILABLE ONLY VIA IBM DIRECT

OR CERTIFIED RESELLERS

3 x 2.25 GB SCSI-2 F/W H'Disk

Supports RAID 0, 1, or 5

90MHz Pentium Processor

S/390 CMOS Microproc. Complex

Memory:       Std.    Maximum

ECC 70ns:  32 MB    256 MB

S/390:  32 MB    128 MB

L1 Cache:  16 KB     16 KB

L2 WBCache: 256 KB    256 KB

8 full length 32 bit slots

40MB/Sec Streaming Data Suppt

32-bit DMA Controller

Synchronous Channel Check

System Error Logging

SynchroStream Technology

2.88MB Diskette, 4/10 GB Tape

2x CD-ROM Dr. & ServerGuide

Parallel & Serial(2) ports

Mouse Port, Mouse

VPD & C2 Security Enabled

Enhanced Security LogicLock

SCSI-2 F/W Busmaster Adapter

SVGA-NI Graphics Board

OS/2 Warp with WINOS2

Communications Mgr/2 1.11

NetFinity

3 Year On-Site Warranty

Supports OS/2, S/390 app's

------end of system unit------

S390+32  (Std. S/390 Memory Card- 32 MB   Not Listed      1     Not Listed

Memory Upgrades (48-256 MB)

64MB (16MB ECC SIMMS)

92G7209  16MB 70ns ECC Memory Kit OBI         579.00      2       1,158.00

FC 2097

S/390 Memory Upgr (64-128 MB)    Not Listed      1     Not Listed

Upgrade to 64 MB ECC

26H2931  32 MB ECC S/390 Memory Card        2,569.00      1       2,569.00

Hot Swap Disks

INTL Hot Swap Drives

Chapter 8.  Sample Configurations  35

Multiple 2GB Fast/Wide Disks

6x 2 GB SCSI-2 F/W HD 7ms

70G9740  Hot Swap Backplane & Cable           189.00      1         189.00

Connects to SCSI-2 F/W or

RAID Adapter/A internally

(Adds 6 more SCSI connectors)

70G9739  PC SERVER POWER SUPPLY UPG           189.00      1         189.00

FC 3959

94G2650  2 GB SCSI-2 F/W HS Disk 8ms       1,079.00      6       6,474.00

LAN Adapters

Dual LANStreamer/A (1-3)

1 Dual LANStreamer Adapter/A

73G7137  Dual LANStreamer MC 32 Adapter       870.00      1         870.00

FC 2628

6339098  Token Ring Adapter Cable              36.00      2          72.00

FC 3390

UPS Options

94G3135  APC Smart-UPS 1000 - 6 outlets       679.00      1         679.00

FC 4006

8.5H x 6.7W x 17.3D (inches)

6554603  P70 Color Monitor (15.9")          1,069.00      1       1,069.00

15.9" viewable screen area

TCO-92, 0.26mm, Trinitron

up to 1600x1280

(replaces 17P)

96G2693  15-pin to 15-pin ID bit cable          0.00      1           0.00

(connects monitor cable to VGA

port on system - 1" long)

53F2607  ARTIC Portmaster/2 (2MB)           1,699.00      1       1,699.00

FC 6368

53F2610  8-Port RS232 Addition                295.00      1         295.00

FC 6362

53F2619  8-Port Cable                         246.00      1         246.00

FC 6366

Total Price            41,078.00

RATIONALE:

1.  VM System used for development and Office System support

2.  Critical data, protected via RAID and UPS

3.  Token Ring LAN Supports S/390 and significant LAN server activity.

Additional OS/2 memory required and double port LanStreamer.

4.  Additional 32MB S/390 RAM to support many users with little paging.

5.  MultiPort/2 adapter to provide 8 lines (up to 19.2 Kbs) to connect

to other systems in company

6.  Additional DASD for VM and LAN served disks

36  Quick Reference

MVS Large Application Development System

Configuration Pricing

as of  3/23/1996

Pricing Method:     List Price

No of free slots:   0                   Date Printed:    Mar 23 1996

Price File Date:    1 Mar/96            Program Version: V7.04

P/N              Description            Unit Price    Qty          Price

---              -----------            ----------    ---          -----

8641MYC  PC Svr 390A MYC P90 128M/13GB    37,599.00      1      37,599.00

AVAILABLE ONLY VIA IBM DIRECT

OR CERTIFIED RESELLERS

6 x 2.25 GB SCSI-2 F/W H'Disk

Supports RAID 0, 1, or 5

90MHz Pentium Processor

S/390 CMOS Microproc. Complex

Memory:       Std.    Maximum

ECC 70ns:  32 MB    256 MB

S/390: 128 MB    128 MB

L1 Cache:  16 KB     16 KB

L2 WBCache: 256 KB    256 KB

8 full length 32 bit slots

40MB/Sec Streaming Data Suppt

32-bit DMA Controller

Synchronous Channel Check

System Error Logging

SynchroStream Technology

2.88MB Diskette, 4/10 GB Tape

2x CD-ROM Dr. & ServerGuide

3 Backplanes, 2 free/12 drives

2nd 220W Power Supply std.

Parallel & Serial(2) ports

Mouse Port, Mouse

VPD & C2 Security Enabled

Enhanced Security LogicLock

SCSI-2 F/W Busmaster Adapter

SVGA-NI Graphics Board

OS/2 Warp with WINOS2

Communications Mgr/2 1.11

NetFinity

3 Year On-Site Warranty

Supports OS/2, S/390 app's

------end of system unit------

S390+32  (Std. S/390 Memory Card- 32 MB   Not Listed      1     Not Listed

S390+96  96 MB S/390 Memory Option        Not Listed      1     Not Listed

6554603  P70 Color Monitor (15.9")          1,069.00      1       1,069.00

15.9" viewable screen area

TCO-92, 0.26mm, Trinitron

up to 1600x1280

(replaces 17P)

96G2693  15-pin to 15-pin ID bit cable          0.00      1           0.00

Chapter 8.  Sample Configurations  37

(connects monitor cable to VGA

port on system - 1" long)

Internal Hot Swap Hard Drives

Multiple 2 GB F/W HSwap HD's

11x 2 GB SCSI-2 F/W HS HD's

70G9263  Fast/Wide SCSI-2 RAID Adapt/A      2,395.00      1       2,395.00

40 MB/sec data streaming

2 channels, 7 devices each

Uses two adjacent slots when

both channels are external

3-year warranty

94G2650  2 GB SCSI-2 F/W HS Disk 7.5ms     1,079.00      6       6,474.00

94G2650  2 GB SCSI-2 F/W HS Disk 7.5ms     1,079.00      5       5,395.00

LAN Adapters (MC)

Auto LANStreamer Adapter/A (1-

1 Auto LANStreamer Adapter/A

60G1592  Auto LANStreamer MC 32 Adapter       435.00      1         435.00

FC 1592

Supports UTP & STP

Includes RIPL

Supports 40MB/Sec Streaming

6339098  Token Ring Adapter Cable              36.00      1          36.00

FC 3390

Add'l SCSI for undefined reqts

70G8498  Fast/Wide SCSI-2 Adapter/A OBI       515.00      1         515.00

Does NOT incl. req'd cable(s)

3-year warranty

06H3231  S F/W Card-S F/W Extl Optn Cbl        75.00      1          75.00

Connects Fast/Wide Adapter to

extl Fast/Wide device (16-bit)

(requires separate terminator)

UPS Options

94G3136  APC Smart-UPS 1400 - 6 outlets       839.00      1         839.00

8.5H x 6.7W x 17.3D (inches)

FC 4008

46F3786  S/370 Channel Emul Adapter/A       3,095.00      1       3,095.00

SCSI Multi-Storage Enclosure

Cables to 1st external device

06H3231  S F/W Card-S F/W Extl Optn Cbl        75.00      1          75.00

Connects Fast/Wide Adapter to

extl Fast/Wide device (16-bit)

(requires separate terminator)

3517001  SCSI Multi-Storage Enclosure       1,269.00      1       1,269.00

Lockable door, 5 Hot Swap Bays

and 2 Bays for Tape Drives

370 Watt WW Power Supply

7.75" x 19" x 16"

3517 Disk & Tape Options

F/W H'Swap Tray III 3517/3518

74G8631  4/10 GB Int SCSI DAT Tape Dr.      1,729.00      1       1,729.00

Incl. -4 mm data cartridge

-4 mm cleaning cartridge

-4 mm diag. cartridge

-NovaBack backup S/W

38  Quick Reference

Replaces 8191359

Streaming Tape Drive Software

Mult. 4.51GB F/W HSwap HD III'

5x 4.51 GB SCSI-2 F/W HS HD's

70G9863  4.51 GB SCSI-2 F/W HS HD III       1,419.00      5       7,095.00

Total Price            68,095.00

RATIONALE:

1.  MVS Generally requires 128MB of S/390 RAM and at least 9GB

useable DASD space.  Choose MYF or MYC models.

2.  Large DASD config to support multiple levels of MVS and

development products.

3.  S/370 Channel Adapter to connect 3745 unit

4.  SCSI F/W adapter to connect external OEM 3480 compatible tapes

5.  Include 3517 external DASD enclosure for additional DASD capacity.

and second 4mm tape drive.

6.  No LAN Server activity, so no additional OS/2 RAM required.

7.  Single port LanStreamer sufficient for S/390 LAN Traffic.

BASE SOFTWARE PRICES

ESL Prices for basic software stacks fo reach operating system

platform.  Use these for 'ballpark' estimates only.  Exact price

estimates must be done via HONE/CFPROGS to ensure a proper

configuration and current prices.

VM/ESA:

Chapter 8.  Sample Configurations  39

+-------------------------------------------------------+

| Table 4. VMESA Software ESL Prices                    |

+----------------------+----------+----------+----------+

| PRODUCT NAME         | PROGRAM  | VER/REL  | ESL      |

|                      | ID       |          | PRICE    |

|                      |          |          | (OTC)    |

+----------------------+----------+----------+----------+

| VM/ESA Ver 2         | 5654-030 | 2.1      | $        |

|                      |          |          | 11,085.  |

+----------------------+----------+----------+----------+

| ICKDSF/VM            | 5684-042 | 1.16.0   |     NC   |

|                      |          |          |          |

+----------------------+----------+----------+----------+

| EREP                 | 5654-260 | 3.5.0    |     NC   |

|                      |          |          |          |

+----------------------+----------+----------+----------+

| HLASM                | 5696-234 | 1.2.0    | $        |

|                      |          |          | 2,120.   |

+----------------------+----------+----------+----------+

+----------------------+----------+----------+----------+

|  Sub-Total           |  VM-Base | 1.2.2    | $13,205. |

|                      |          |          |          |

+----------------------+----------+----------+----------+

| Other required       |          |          |          |

| software             |          |          |          |

+----------------------+----------+----------+----------+

| Final Total          |          |          |          |

+----------------------+----------+----------+----------+

VSE/ESA 2.1

40  Quick Reference

+-------------------------------------------------------+

| Table 5. VSE/ESA  Ver 2.1 ESL Prices                  |

+----------------------+----------+----------+----------+

| PRODUCT NAME         | PROGRAM  | VER/REL  | ESL      |

|                      | ID       |          | PRICE    |

|                      |          |          | (OTC)    |

+----------------------+----------+----------+----------+

| VSE Central Fun.     | 5686-066 | 6.1.0    |  $       |

|                      |          |          | 9,615.   |

+----------------------+----------+----------+----------+

|   Unique Code        |          |          |          |

| VSE/OLTEP REXX/VSE   |          |          |          |

| LANRES/VSE VSE/AF    |          |          |          |

| VSE/POWER FastCopy   |          |          |          |

| VSE/ICCF VSE/VSAM    |          |          |          |

| VSAM Space Mgt VSAM  |          |          |          |

| Back/Rest            |          |          |          |

+----------------------+----------+----------+----------+

| VSE/EREP             | 5656-260 | 3.5.0    |      NC  |

|                      |          |          |          |

+----------------------+----------+----------+----------+

| ICKDSF/VSE           | 5747-DS2 | 1.16.0   |      NC  |

|                      |          |          |          |

+----------------------+----------+----------+----------+

| ACF/VTAM(r)          | 5686-065 | 4.2.0    |   $      |

|                      |          |          | 4,095.   |

+----------------------+----------+----------+----------+

| CICS/VSE             | 5686-026 | 2.3.0    |          |

|                      |          |          | 5,880.   |

+----------------------+----------+----------+----------+

| DITTO                | 5688-052 | 3.2.0    |          |

|                      |          |          | 680.     |

+----------------------+----------+----------+----------+

| BTAM                 | 5746-RC5 | 1.1.0    |          |

|                      |          |          | 398.     |

+----------------------+----------+----------+----------+

| HLASM for VSE        | 5696-234 | 1.1.0    |          |

|                      |          |          | 545.     |

+----------------------+----------+----------+----------+

+----------------------+----------+----------+----------+

| Sub-Total            |  Base    |  2.1     | $        |

|                      | VSE      |          | 20,803.  |

+----------------------+----------+----------+----------+

| Other required       |          |          |          |

| Software             |          |          |          |

+----------------------+----------+----------+----------+

|  Final Total         |          |          |          |

+----------------------+----------+----------+----------+

Chapter 8.  Sample Configurations  41

MVS/ESA 5.2

42  Quick Reference

+-------------------------------------------------------+

| Table 6 (Page 1 of 2). MVS/ESA  Ver 5.2 ESL Prices    |

+----------------------+----------+----------+----------+

| PRODUCT NAME         | PROGRAM  | VER/REL  | ESL      |

|                      | ID       |          | PRICE    |

|                      |          |          | (OTC)    |

+----------------------+----------+----------+----------+

|   MVS/ESA JES2       |          |  5.2.0   |   $      |

|                      | 5655-068 |          | 9,350.   |

|                      |          |          |          |

+----------------------+----------+----------+----------+

|   TIOC               |          |  3.8.0   |          |

|                      | 5752-VS2 |          | NC       |

|                      |          |          |          |

+----------------------+----------+----------+----------+

|   HLASM              |          |  0.1.0   |          |

|                      | 5696-234 |          | 2,120.   |

|                      |          |          |          |

+----------------------+----------+----------+----------+

|   DFSMS/DFP          |          |  1.2.0   |          |

|                      | 5695-DF1 |          | 4,510.   |

|                      |          |          |          |

+----------------------+----------+----------+----------+

|   ICKDSF             |          |  9.16.0  |          |

|                      | 5655-257 |          | NC       |

|                      |          |          |          |

+----------------------+----------+----------+----------+

|   ACF/VTAM ESA       |          |  4.2.0   |          |

|                      | 5695-117 |          | 7,525.   |

|                      |          |          |          |

+----------------------+----------+----------+----------+

|   EREP               |          |  3.5.0   |          |

|                      | 5658-260 |          | NC       |

|                      |          |          |          |

+----------------------+----------+----------+----------+

|   TSO/E              |          |  2.4.0   |          |

|                      | 5685-025 |          | 2,035.   |

|                      |          |          |          |

+----------------------+----------+----------+----------+

|   ISPF               |          |  4.1.0   |          |

|                      | 5655-042 |          | 2,850.   |

|                      |          |          |          |

+----------------------+----------+----------+----------+

|   SMPE/E             |          |  1.8.1   |          |

|                      | 5668-949 |          | 1,665.   |

|                      |          |          |          |

+----------------------+----------+----------+----------+

+----------------------+----------+----------+----------+

|   Sub total          |  Base    |   4.3    | $        |

|                      | MVS      |          | 29,955.  |

+----------------------+----------+----------+----------+

| Other required       |          |          |          |

| Software             |          |          |          |

+----------------------+----------+----------+----------+

Chapter 8.  Sample Configurations  43

+----------------------+----------+----------+----------+

|  Final Total         |          |          |          |

+----------------------+----------+----------+----------+

TYPICAL CONFIGURATION COSTS

___________________________

These configurations and costs are for 'ballpark' purposes only.

+-----------------------------------------------------------------+

| Table 7.  VM/ESA  Minimum System                                |

+------------------------------------------+----------------------+

|   VM/ESA V 2.1  Base Pre-Config Software | $ 13,205.            |

|                                          |                      |

+------------------------------------------+----------------------+

|   8641-MYS (6.75G Array, 32MB)           | $ 25,569.            |

|                                          |                      |

+------------------------------------------+----------------------+

|   Addl Hwd. (Display, LAN Adptr, etc)    | $  2,000.            |

|                                          |                      |

+------------------------------------------+----------------------+

|    Total                                 | $ 40,774             |

|                                          |                      |

+------------------------------------------+----------------------+

+-----------------------------------------------------------------+

| Table 8.  VM/ESA  Typical System                                |

+------------------------------------------+----------------------+

| VM/ESA V 2.1 Base Pre-Config Software    | $ 13,205.            |

|                                          |                      |

+------------------------------------------+----------------------+

| Additional VM Software (VTAM.TCPIP, OV,  | $ 20,000.            |

| compilers, etc.)                         |                      |

+------------------------------------------+----------------------+

| 8641-MYF (11.25G Array, 128MB)           | $ 35,759.            |

|                                          |                      |

+------------------------------------------+----------------------+

| Addl Hwd. (Display, LAN Adptr, , etc.    | $  6,000.            |

|                                          |                      |

+------------------------------------------+----------------------+

|    Total                                 | $ 74,964             |

|                                          |                      |

+------------------------------------------+----------------------+

44  Quick Reference

+-----------------------------------------------------------------+

| Table 9.  VSE/ESA  Minimum  System                              |

+------------------------------------------+----------------------+

|   VSE/ESA 2.1  Base Software             | $ 20,803.            |

|                                          |                      |

+------------------------------------------+----------------------+

|   8641-MYS (6.75G Array, 32MB)           | $ 25,569.            |

|                                          |                      |

+------------------------------------------+----------------------+

|   Addl Hwd. (Display, LAN Adptr, etc)    | $  2,000.            |

|                                          |                      |

+------------------------------------------+----------------------+

|   Total                                  | $ 48,372.            |

|                                          |                      |

+------------------------------------------+----------------------+

+-----------------------------------------------------------------+

| Table 10.  VSE/ESA  Typical  System                             |

+------------------------------------------+----------------------+

|   VSE/ESA 2.1  Base Software             | $ 20,803.            |

|                                          |                      |

+------------------------------------------+----------------------+

| Additional VSE Software (COBOL, SQL,     | $ 10,000.            |

| etc.)                                    |                      |

+------------------------------------------+----------------------+

| 8641-MYF (11.25GB array, 128MB 390 RAM)  | $ 35,759.            |

|                                          |                      |

+------------------------------------------+----------------------+

| Addl Hwd. (Display, LAN Adptr, 370 Chan, | $  6,000.            |

| etc.)                                    |                      |

+------------------------------------------+----------------------+

|    Total                                 | $ 72,562.            |

|                                          |                      |

+------------------------------------------+----------------------+

+-----------------------------------------------------------------+

| Table 11.  MVS/ESA 5.2  Minimum System                          |

+------------------------------------------+----------------------+

|  MVS/ESA 5.2  Base Software              | $ 29,955.            |

|                                          |                      |

+------------------------------------------+----------------------+

|  8641-MYF (11.25 Array, 128MB)           | $ 35,759.            |

|                                          |                      |

+------------------------------------------+----------------------+

|  Addl Hwd. (Display, LAN Adptr, etc.)    | $  2,000.            |

|                                          |                      |

+------------------------------------------+----------------------+

|    Total                                 | $ 67,714             |

|                                          |                      |

+------------------------------------------+----------------------+

Chapter 8.  Sample Configurations  45

+-----------------------------------------------------------------+

| Table 12.  MVS/ESA 5.2  Typical System                          |

+------------------------------------------+----------------------+

|  MVS/ESA 5.2  Base Software              | $ 29,955.            |

|                                          |                      |

+------------------------------------------+----------------------+

|  Additional MVS Software (COBOL, CICS,   | $ 20,000.            |

| DB2, etc)                                |                      |

+------------------------------------------+----------------------+

|  8641-MYC (13.5GGB Array, 128MB)         | $ 37,599.            |

|                                          |                      |

+------------------------------------------+----------------------+

|  Addl Hwd. (display, LAN adptr, Disks,   | $  8,000.            |

| etc.)                                    |                      |

+------------------------------------------+----------------------+

|    Total                                 | $ 95,554.            |

|                                          |                      |

+------------------------------------------+----------------------+

PRECONFIGURED SOFTWARE STACKS

Software stacks are available for each of the s/390 operating

systems on a pre-configured CD-ROM. Each software component on

the CD must be separately licensed. These pre-configured CDs

are available as the following features.

+-------------------------------------------------------+

| Table 13. Preconfigured CD Software Stacks            |

+----------------------+----------------+---------------+

| PRODUCT NAME         | PROGRAM ID     | FEATURE       |

+----------------------+----------------+---------------+

|   OS/390             |  5645-001      |  5819         |

+----------------------+----------------+---------------+

|   MVS/ESA V5R2       |  5655-068      |  5809         |

+----------------------+----------------+---------------+

|   MVS/ESA V4R3       |  5695-047      |  5809         |

+----------------------+----------------+---------------+

|   VM/ESA V1R2.2      |  5684-112      |  5899         |

+----------------------+----------------+---------------+

|   VM/ESA V2R1        |  5654-030      |  5899         |

+----------------------+----------------+---------------+

|   VM/ESA SD0 V2      |  5684-026      |  5899         |

+----------------------+----------------+---------------+

|   VSE/ESA V1R4       |  5750-ACD      |  1453 (full)  |

+----------------------+----------------+---------------+

|   VSE/ESA V1R4       |  5750-ACD      |  1463         |

|                      |                | (partial)     |

+----------------------+----------------+---------------+

|   VSE/ESA V2R1       |  5690-VSE      |  2000         |

+----------------------+----------------+---------------+

46  Quick Reference

CHAPTER 9.  PC SERVER S/390 ORDER CHECKLIST

___________________________________________

Use this simple checklist to help order a system and make

sure  all  needed  components  are included in the order.

This is not intended to cover all  possible  systems  and

there may be good reasons to differ from the sequence and

suggestions herein.

System Unit

CHOOSE MODEL:

o   Need Array system? (Recommended).

__

-   If small system or less than 128MB of S/390

__

memory, select 8641-MYS

-   If large system, choose 8641-MYC

__

o   If non-array appropriate: 8641-NYV

__

S/390  MEMORY:    Choose total S/390 memory size:  Select

additional memory required:  32MB or 96MB card.

o   32MB (Base): Most VSE, small VM, very limited  use

__

MVS

o   64MB  (32MB  additional): Larger VSE, moderate VM,

__

and smaller MVS

o   128MB (96MB  additional):  Memory  intensive  VSE,

__

large VM, most MVS

SERVER MEMORY:  Choose total PC Memory size:

o   32MB (base): Sufficient for "S/390 only" systems.

__

o   Additional:    Add additional PC Memory to support

__

PC  Server  functions.    For  light  LAN   Server

workload,  add  32MB;  for  moderate  to heavy LAN

workload, add 64MB or more.    If  using  DataBase

oriented  applications such as DB2/2, consider 128

- 256 MB total PC memory.   (Special  emphasis  on

using memory to reduce I/O activity.)

You   should  normally  select  SIMM  configuration  that

miminizes the total number of SIMMs.

(C) Copyright IBM Corp. 1995                                               47

I/O Devices

DASD:

Most customers will use additional 2.25GB drives.

o   Use   Disk   Configuration  Aid  at  end  of  this

__

checklist for assistance

o   Determine total actual data GB required

__

o   If  Array  model,  adjust   for   redundant   data

__

'overhead'

o   If Array model, 'over-configure' to allow capacity

__

for growth without disrupting ARRAYs.

o   Order hard drives to meet GB requirements

__

o   Additional Power supply? (beyond 1st drive bank)

__

o   Additional Backplanes?  (2nd and 3rd drive banks)

__

o   Additional SCSI or RAID Controller(s)?

__

LAN ADAPTERS

o   Token Ring:?  Auto LANStreamer, Dual LANStreamer

__

o   Ethernet:  EtherStreamer, Dual EtherStreamer, Quad

__

Peermaster

-   Media: 10-Base-T (unshielded twisted pair)  or

10-Base-2 (thin Coax) ?

o   ATM, etc?

__

o   LAN Cables / connectors?

__

One LAN-adapter / port will be sufficient for most  S/390

only  usage.    For  LAN  based  workload, add sufficient

TokenRing or Ethernet ports to process that workload.

WAN ADAPTERS:

For SDLC / BSC communications via VTAM, etc.

o   Wide Area Connector(WAC)  RS-232, V.35, up to 2 lines

o   Multi-Port Adapter/2 RS-232, V.35 up to 8 or 6  lines

per adapter

OTHER I/O

o   S/370 Channel Emulator/A ?  (attach S/370 type I/O

__

Devices)

AVAILABILITY

o   UPS:    Every  PC  Server S/390 should have a UPS.

__

Should  size  UPS  to  provide  approximately   20

minutes run battery time:

-   APC  Smart-UPS 700:   Smaller systems, up to 5

drives

-   APC Smart-UPS 1000: Medium systems, up  to  11

drives

-   APC  Smart-UPS 1400: Larger systems, or longer

battery run time.

48  Quick Reference

o   ServerGuard: Very  useful  for  remotely  operated

__

systems.          Allows     remote     monitoring

shutdown/power-up control.

Displays

o   Size:  14, 15, 17"

__

o   General ('G') or Pro ('P') series

__

DISK CONFIGURATION AID

The  following table is intended to assist in determining

Hard Drive configurations.   It  is  based  on  following

assumptions:

o   Uses 2.25 GB Drives only

o   RAID configurations assume RAID-5

o   Configurations  are for maximum capacity, not maximum

performance

o   External enclosure configurations not shown

RAID Systems

Use this chart to help configure Array systems

Chapter 9.  PC Server S/390 Order Checklist  49

+--------------------------------------------------------------+

| Table 14.    ARRAY /  RAID  Systems   (MYS, MYF, MYC)        |

+--------+--------+--------+--------+--------+--------+--------+

| #      | ADD'L  | ADD'L  | ADD'L  |  #     | TOTAL  | DATA   |

| DRIVES | BACKPLA|EADAPTRS| POWER  | ARRAYS | CAPACIT| CAPACIT|

+--------+--------+--------+--------+--------+--------+--------+

|   3    |  No    |   0    |   No   |   1    |   6.75 |   4.50 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|   4    |  No    |   0    |   No   |   1    |   9.00 |   6.75 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|   5    |  No    |   0    |   No   |   1    |  11.25 |   9.00 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|   6    |  1     |   0    |   Yes  |   1    |  13.50 |  11.25 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|   7    |  1     |   0    |   Yes  |   1    |  15.75 |  13.50 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|   8    |  1     |   0    |   Yes  |   1    |  18.00 |  15.75 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|   9 *  |  1     |   0    |   Yes  |   2    |  20.25 |  15.75 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|  10    |  1     |   0    |   Yes  |   2    |  22.50 |  18.00 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|  11    |  1     |   0    |   Yes  |   2    |  24.75 |  20.25 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|  12 *  |  2     |   1    |   Yes  |   2    |  27.00 |  20.25 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|  13    |  2     |   1    |   Yes  |   3    |  29.25 |  22.50 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|  14    |  2     |   1    |   Yes  |   3    |  31.50 |  24.75 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|  15    |  2     |   1    |   Yes  |   3    |  33.75 |  27.00 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|  16    |  2     |   1    |   Yes  |   3    |  36.00 |  29.25 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

|  17    |  2     |   1    |   Yes  |   3    |  38.25 |  31.50 |

|        |        |        |        |        |        |        |

+--------+--------+--------+--------+--------+--------+--------+

50  Quick Reference

*   These configurations represent the 'crossover'

points where additional arrays are  required  to  be

defined.    In  'normal'  configurations  this would

require additional equipment (and expense)  with  no

corresponding  increase  in  capacity.    There may,

however, be cases where such a  configuration  would

be appropriate.

Non-RAID Systems

Use this chart to help configure non-array systems.

Chapter 9.  PC Server S/390 Order Checklist  51

+-------------------------------------------------------------------+

| Table 15.    Non-Array Systems       (NYV)                        |

+-----------+------------+-----------+------------+-----------------+

| # OF      | ADD'L      | ADD'L     | ADD'L      | TOTAL/DATA      |

| DRIVES    | BACKPLANE  | ADAPTRS   | POWER      | CAPACITY (GB)   |

+-----------+------------+-----------+------------+-----------------+

|    2      |    No      |   0       |   No       |     4.50        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|    3      |    No      |   0       |   No       |     6.75        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|    4      |    No      |   0       |   No       |     9.00        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|    5      |    No      |   0       |   No       |    11.25        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|    6      |    1       |   1       |   Yes      |    13.50        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|    7      |    1       |   1       |   Yes      |    15.75        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|    8      |    1       |   1       |   Yes      |    18.00        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|    9      |    1       |   1       |   Yes      |    20.25        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|   10      |    1       |   1       |   Yes      |    22.50        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|   11      |    1       |   1       |   Yes      |    24.75        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|   12      |    2       |   2       |   Yes      |    27.00        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|   13      |    2       |   2       |   Yes      |    29.25        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|   14      |    2       |   2       |   Yes      |    31.50        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|   15      |    2       |   2       |   Yes      |    33.75        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|   16      |    2       |   2       |   Yes      |    36.00        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

|   17      |    2       |   2       |   Yes      |    38.25        |

|           |            |           |            |                 |

+-----------+------------+-----------+------------+-----------------+

52  Quick Reference

IBM

Part Number: 051096

File Number: S/390

Printed in U.S.A.

DSMTB2451W TABLE SPLIT ON PAGE 43.

DSMMOM397I '.EDFETABL' WAS IMBEDDED AT LINE 776 OF 'QREF2'

DSMBEG323I STARTING PASS 2 OF 2.

DSMW32708E TEXT EXCEEDS RIGHT PAGE BOUNDARY ON PAGE 33.

DSMW32708E TEXT EXCEEDS RIGHT PAGE BOUNDARY ON PAGE 34.

DSMW32708E TEXT EXCEEDS RIGHT PAGE BOUNDARY ON PAGE 35.

DSMW32708E TEXT EXCEEDS RIGHT PAGE BOUNDARY ON PAGE 36.

DSMW32708E TEXT EXCEEDS RIGHT PAGE BOUNDARY ON PAGE 37.

DSMW32708E TEXT EXCEEDS RIGHT PAGE BOUNDARY ON PAGE 38.

DSMW32708E TEXT EXCEEDS RIGHT PAGE BOUNDARY ON PAGE 39.

DSMTB2451W TABLE SPLIT ON PAGE 43.

DSMMOM397I '.EDFETABL' WAS IMBEDDED AT LINE 776 OF 'QREF2'

DSMW32708E TEXT EXCEEDS RIGHT PAGE BOUNDARY ON PAGE 44.

DSMW32708E TEXT EXCEEDS RIGHT PAGE BOUNDARY ON PAGE 46.