LIMITED WARRANTY

The author  warrants to  registered users  that the software will

perform substantially in accordance with the accompanying written

material for  a period  of 90  days following registration of the

software. Any  implied warranties  are limited  to 90  days after

registration. Should the diskette(s) upon which BASIS is supplied

(after registering  the software) prove to be defective, they can

be returned within said 90 day period for replacement.

DISCLAIMER

The author  disclaims any  other warranties,  either  express  or

implied, including  but not  limited  to  implied  warranties  of

merchantability or  fitness  for  any  particular  purpose,  with

respect to the software and accompanying documentation.

The author  reserves the  right to revise and improve his product

as he  sees fit. This manual describes the product in its current

release  version,   and  may  not  pertain  correctly  to  future

releases.

NO LIABILITY FOR CONSEQUENTIAL DAMAGES

In no  event shall  the author,  or any  person involved  in  the

distribution of  BASIS, be  liable  for  any  damages  whatsoever

(incuding, without  limitation,  damages  for  loss  of  business

profits, business  interruption, loss of business information, or

other pecuniary  loss) arising out of the use or inability to use

this product,  even  if  the  author  has  been  advised  of  the

possibility of such damages.

BASIS Software and Documentation Copyright (C) 1988-94 by William

O. Giel. All Rights Reserved.

BASIS software  is owned by the author. It is licensed for use by

registration with  the author.  A single  license  to  use  BASIS

grants the  right to  use one  copy of  the software  on a single

computer. You may install the software on more than one computer,

provided that  only one  copy is  in use  at any  time.  Multiple

installations or network installations require a site license.

Modification, disassembly  and/or reverse  engineering  of  BASIS

executables are  not permitted,  and  will  render  any  warranty

granted above null and void.

DCA  Engineering  Software  is  a  registered  trademark  of  DCA

Engineering, Inc.  AutoCAD is a registered trademark of Autodesk,

Inc. CEADS is a registered trademark of Accugraph.

INTRODUCTION

BASIS  is   a  menu-driven  surveying  software  system  for  AT-

compatible personal  computers running under the MS-DOS operating

system (or  equivalent.) BASIS  was designed  and developed  by a

practicing land surveyor.

BASIS features  comprehensive computational capabilities. It also

can run  from within AutoCAD and offers several powerful drafting

enhancements.  These  drafting  enhancements  are  100  per  cent

compatible with  AdCadd/Softdesk (DCA  Engineering Software),  or

they can be used with "plain vanilla" AutoCAD as well.

BASIS 2.4  reflects revisions  and additions  resulting from over

five years  of daily  use in the workplace. Many improvements and

changes have been incorporated into the programs since its humble

beginning. However, this is not the final version. BASIS 2.4 will

remain a dynamic set of programs.

As we  prepare to enter the second half of the decade, most of us

are still  running some version of DOS. The promise of Windows or

OS/2 has  yet to  firmly emplace  itself with surveyors. The next

incarnation of BASIS is yet to be decided. We'll just have to see

what the future brings.

Once again,  I would  like to  thank all those who have supported

BASIS  by   registering.  As  always,  I  welcome  any  comments,

suggestions, and/or criticism of the system.

William Giel, LS

April 26, 1994

PERMISSION TO COPY

BASIS is  distributed under  the "Shareware"  concept.  This

means that  prospective users  can obtain the latest version

and documentation  (on disk) for a nominal distribution fee.

After using  the actual  software for  evaluation  purposes,

registration is encouraged.

BASIS is  not copy-protected  and distribution  to others is

permitted,  provided  that  documentation  and  registration

information is  provided along  with the  program diskettes,

and that  any fee  charged for  the copies  does not  exceed

$10.00.

Unregistered  copies  are  for  evaluation  and  educational

purposes only. Commercial use requires user registration.

The fee  for registration is $200.00. When you register, you

will receive  a copy  of the  latest version  of BASIS and a

printed version  of the  users'  manual.  Registration  also

entitles you  to free  technical support,  free upgrades  of

version 2.x  as they  are  produced,  and  of  course,  free

maintenance releases.

To register, please contact:

William O. Giel, LS

40 Briarwood Lane

Milford, CT 06460

(203) 877-9438

Table of Contents

Table of Contents

1.0 INSTALLATION...........................................1

1.01 SYSTEM REQUIREMENTS.................................1

1.02 RUNNING THE INSTALL PROGRAM.........................1

1.03 UNPACKING INDIVIDUAL FILES..........................3

2.0 GETTING STARTED........................................5

2.01 ORGANIZING DATA FILES...............................5

2.02 WHAT TO DO FIRST....................................5

2.03 RUNNING BASIS.......................................5

2.04 THE BASIS SYSTEM MENUS..............................6

3.0 SYSTEM DEFAULTS (F5)...................................9

3.01 DATA DIRECTORY......................................9

3.02 PRINTER HEADINGS...................................10

3.03 PRINTER OPTIONS (F6)...............................10

3.04 DISTANCE UNITS.....................................11

3.05 DIRECTION UNITS....................................11

3.06 DISPLAY PRECISIONS.................................11

3.07 SLOPE REDUCTION....................................11

3.08 RADIUS UNEQUAL TOLERANCE...........................11

3.09 COGO FILESIZE......................................12

3.10 POINT PROTECT......................................12

3.11 CUSTOMIZE COLORS...................................12

3.12 ENABLE MOUSE.......................................12

3.13 LIST DEFAULTS......................................13

3.14 SAVING THE NEW DEFAULTS............................13

4.0 DISK MANAGEMENT SYSTEM (F1)...........................14

4.01 CREATE CATALOG INDEX (F1)..........................14

4.02 CREATE PROJECT FILE (F2)...........................15

4.03 STORE COORDINATES (F3).............................16

4.04 CLEAR COORDINATES (F4).............................17

4.05 TRANSFER COORDINATES (F5)..........................17

4.06 LIST COORDINATES (F6)..............................19

4.07 DELETE DATA FILE (F7)..............................20

4.08 VIEW/EDIT CATALOG DATA (F8)........................21

4.09 DISPLAY/PRINT CATALOG DATA (F9)....................21

4.10 COPY DATA FILE (F10)...............................22

4.11 GENERIC IMPORT / EXPORT (^F1)......................23

4.12 CATALOG SEARCH (^F2)...............................25

4.13 EXPAND PROJECT FILE (^F3)..........................26

4.14 DISK MANAGEMENT SYSTEM HELP (^F4)..................26

4.15 COORDINATE COMPARISON (^F5)........................26

5.0 INTERACTIVE COORDINATE GEOMETRY SYSTEM (F2)...........27

5.01 CONVENTIONS........................................28

5.01.01 ENTERING ANGULAR VALUES......................28

5.01.02 ENTERING DISTANCES...........................30

5.02 THE COGO COMMAND LINE..............................30

5.02.01 SINGLE-KEYSTROKE COMMANDS....................31

PRINTER ON/OFF TOGGLE..............................31

POINT-PROTECT ON/OFF TOGGLE........................32

RESET COMMAND LINE.................................32

INSERTING COMMENTS.................................32

STORING A COORDINATE...............................32

5.02.02 BUILDING A COGO COMMAND......................32

5.02.03 TRAVERSING INTO A NEW POINT..................33

Table of Contents

5.02.04 INVERSING....................................33

5.02.05 BEARING - BEARING INTERSECTION...............34

5.02.06 BEARING - DISTANCE INTERSECTION..............35

5.02.07 DISTANCE - DISTANCE INTERSECTIONS............36

5.02.08 P.C. CURVE SOLUTION..........................37

5.02.09 SETTING MULTIPLE POINTS......................37

5.02.10 CURVE INVERSE................................38

5.02.11 P.I. CURVE...................................38

5.02.12 LIST COORDINATES.............................39

5.02.13 THREE POINT CURVE............................39

5.02.14 LIST UNUSED POINTS...........................40

5.02.15 AREA.........................................40

5.02.16 COORDINATE TRANSFORMATION....................40

5.02.17 EDM SLOPE REDUCTION..........................41

5.02.18 DIRECTIONS TO ANGLES.........................42

5.02.19 PREDETERMINED AREA PARTITIONS................42

5.02.20 TURNAROUNDS..................................45

Symmetric..........................................45

Offset, Off Tangent, Clockwise.....................47

Offset, Off Tangent, Counterclockwise..............47

Offset, Off Curve, Clockwise.......................47

Offset, Off Curve, Counterclockwise................47

5.02.21 DOS COMMAND PROCESSOR........................47

5.02.22 ORIENTATION..................................47

5.02.23 REGRESSION...................................50

5.02.24 RADIAL LAYOUT................................50

5.02.25 STATION AND OFFSET...........................51

5.02.26 POINT SEARCH.................................52

5.02.27 RIGHT-OF-WAY COMPUTATION.....................53

5.02.28 TANGENT LINES................................55

5.02.29 RIGHT-OF-WAY INTERSECTIONS...................56

Straight-Straight..................................56

Straight-Curved....................................56

Four-way, Curved-Curved............................58

5.02.30 MANUAL FORMFEED..............................58

5.02.31 STORE ELEVATION..............................59

5.02.32 DETAILED COGO HELP...........................59

6.0 FIELD TRAVERSE SYSTEM (F3)............................60

6.01 OPEN FIELD FILE....................................61

6.02 OPEN PROJECT FILE..................................61

6.03 ADD/ENTER TRAVERSE DATA - SPECIFICATIONS...........61

6.03.01 DESCRIPTIONS (Y/N)...........................62

6.03.02 ELEVATIONS (Y/N).............................62

6.03.03 SELECT TRAVERSE TYPE.........................62

6.03.04 STARTING POINT AND BACKSIGHT.................62

6.03.05 ENDING POINT AND FORESIGHT...................63

6.03.06 MAXIMUM ANGULAR ERROR........................63

6.03.07 MAXIMUM LINEAR ERROR.........................63

6.03.08 TYPE OF BALANCE..............................64

6.03.09 ANGLE BALANCE (Y/N)..........................64

6.03.10 ELEVATION BALANCE (Y/N)......................64

6.03.11 HARDCOPY (Y/N)...............................64

6.04 ENTERING THE TRAVERSE..............................64

6.04.01 ENTERING ANGULAR DATA........................64

Table of Contents

6.04.02 ENTERING DISTANCES...........................66

6.04.03 POINT NUMBERS................................66

6.04.04 SIDESHOTS....................................66

6.04.05 DESCRIPTIONS.................................66

6.04.06 TERMINATING DATA ENTRY.......................67

6.05 CHANGE TRAVERSE DATA (F4)..........................67

6.06 PRINT FIELD NOTES (F5).............................67

6.07 PROCESS TRAVERSE (F6)..............................67

6.08 EDIT TRAVERSE SPECIFICATIONS (F7)..................68

6.09 INSERT TRAVERSE DATA (F8)..........................68

6.10 DELETE TRAVERSE DATA (F9)..........................69

6.11 FIELD TRAVERSE SYSTEM HELP (F10)...................69

7.0 UTILITY SUPPORT MENU..................................70

7.01 AUTOMATIC DATA REDUCTION...........................70

7.02 CALCULATOR.........................................70

7.03 PRECISE EDM SLOPE REDUCTION........................75

7.04 DIRECTIONS TO ANGLES...............................76

7.05 SHORT LINE EDM REDUCTION...........................77

7.06 NAD27-83 TRANSFORMATION............................77

7.07 ELECTRO-OPTICAL EDM CALIBRATION....................79

7.08 VERTICAL CURVE DESIGN..............................81

7.08.01 COMPUTE GRADE TABLE (F1).....................81

7.08.02 FIXED POINT & PVC KNOWN (F2).................82

7.08.03 FIXED POINT & PVI KNOWN (F3).................83

7.08.04 TURNING ELEVATION & PVI KNOWN (F4)...........83

7.08.05 ENTER/EDIT ROADWAY BATCH FILE (F6)...........84

7.08.06 PROCESS ROADWAY BATCH FILE (F7)..............85

7.08.07 PRINT ROADWAY BATCH FILE (F8)................85

7.08.08 ROADWAY BATCH FILE STRUCTURE.................85

7.08.09 PRINTER TOGGLE (F9)..........................87

7.08.10 CUT/FILL CALCULATOR (F5).....................87

7.09 SIDESHOT DATA ENTRY PROGRAM........................87

7.10 PRINTER PLOTTER....................................89

7.11 EDIT SIDESHOT FILE.................................90

7.12 PROCESS SIDESHOT FILE..............................90

7.13 MEMORY RESIDENT CALCULATOR.........................91

7.14 DATUM SHIFTS.......................................92

7.15 CUSTOMIZING THE UTILITY SUPPORT MENU...............92

8.0 DRAFTING ENHANCEMENTS.................................94

8.01 INSTALLATION.......................................94

8.02 THE DRAFTING SEQUENCE..............................95

8.02.01 ACTIVATING THE ROUTINE FROM A DRAWING........95

8.02.02 FILENAMES....................................96

8.02.03 THE BASIS SEQUENCE MENU......................96

8.02.04 ENTERING A DRAFTING SEQUENCE.................97

8.02.05 ADDING TO A SEQUENCE.........................98

8.02.06 EDITING A SEQUENCE...........................98

8.02.06 GLOBAL EDITING...............................98

8.02.07 PRINT SEQUENCE...............................99

8.02.08 CREATE/RE-CREATE DXF FILE....................99

8.03 IMPORTING BASIS POINTS............................100

8.04 EXPORTING POINTS TO BASIS.........................101

8.05 DIRECTIONS TO ANGLES..............................102

8.06 SHORT LINE EDM REDUCTION..........................102

Table of Contents

8.07 RUNNING COGO FROM AUTOCAD.........................102

8.08 OTHER AutoLISP UTILITY PROGRAMS...................102

8.08.01 GRIDMKR.....................................102

8.08.02 LBLGRID.....................................103

8.08.03 HASH........................................103

8.09 DRAWING SUBDIRECTORIES............................104

8.10 RUNNING FIELD TRAVERSE FROM AUTOCAD...............104

8.11 CROSS SECTIONS AND PROFILES.......................104

8.12 RUNNING THE SIDESHOT PROGRAM FROM AUTOCAD.........105

8.13 RUNNING THE DISK MANAGEMENT SYSTEM FROM AUTOCAD...106

9.0 BATCHED COORDINATE GEOMETRY..........................107

9.01 INTERACTIVE BATCH COGO............................107

9.02 BATCH COMMAND PROCESSOR...........................108

9.03 EDIT/CREATE BATCH COMMAND FILE....................110

9.04 PRINT BATCH COMMAND FILE..........................111

9.05 RE-DEFINE DEFAULT EDITOR..........................111

9.06 SAMPLE BATCH COMMANDS.............................112

INDEX....................................................119

-1-

1.0 INSTALLATION

1.01 SYSTEM  REQUIREMENTS -  The following represents the minimum

equipment configuration for running BASIS:

1. 640 Kb RAM, AT Compatible (80286 CPU or greater)

2. Math Co-processor

3. Hard Disk

4. DOS 3.1 or greater

An EGA  display adapter, or better is recommended. However, BASIS

will run on monochrome (MDA) systems.

1.02 RUNNING  THE INSTALL  PROGRAM -  To install  BASIS on a hard

disk, insert BASIS diskette 1 of 2 into drive A or B and type

[d:]install

where 'd'  is the  drive letter  of the floppy drive in which the

diskette was  mounted. The BASIS installation program will appear

on the screen.

The installation  program will  request the  drive letter  of the

hard disk on which it will install the software. Press the letter

key that corresponds to the target drive for installation.

BASIS will  automatically copy  the program files from the floppy

into a  directory named  \BASIS.DIR on  the target  hard disk. If

this directory  does not exist, it will automatically be created.

After the  first diskette  is installed,  a  prompt  will  appear

requesting that  the second  diskette be  mounted in  the floppy.

Insert BASIS diskette 2 of 2 into the drive, and press ENTER.

After the  BASIS files  have  been  installed,  the  installation

program  will  ask  if  you  want  to  automatically  modify  the

AUTOEXEC.BAT  file.  BASIS  requires  that  two  DOS  Environment

strings be  placed into  AUTOEXEC.BAT. The  first, "BASIS", names

the drive  and directory where the BASIS programs were installed.

For example, if BASIS is installed on drive C, then the statement

SET BASIS=C:\BASIS.DIR

must appear  in AUTOEXEC.BAT. In the above example, it is assumed

that BASIS  was installed on drive C. If installed on a different

drive, then  the appropriate  drive letter should appear in place

of 'C:'  in the  example. Note  that a  trailing backslash is NOT

used.

Optionally, a  default text-editor can be named that will be used

by some  of the  BASIS programs.  For example,  to use  an editor

named C:\UTILS\TED, place the line

SET EDITOR=C:\UTILS\TED

Section 1 - Installation

-2-

in AUTOEXEC.BAT.

If you respond by pressing 'Y' for Yes, the original AUTOEXEC.BAT

file will  be saved  as AUTOEXEC.BAK,  and these  changes will be

automatically made. The installation program will ask you for the

name of  the default  text  editor  that  some  of  the  programs

require. By  default, the  program will  name  BCEDIT,  the  text

editor supplied  with BASIS, in the EDITOR environment string. If

you are  upgrading a  previous installation of BASIS, the program

will detect and update these statements if they already exist.

If you  press 'N'  and do  not allow  automatic  modification  of

AUTOEXEC.BAT, then  you MUST add these statements to AUTOEXEC.BAT

manually for  the software  to function properly. Unlike previous

versions of  BASIS, you  must at  least  include  the  SET  BASIS

statement even if the software is installed on drive C.

The installation program will then ask if you want to install the

AutoCAD drafting  enhancements. If you are not using AutoCAD, you

should respond  by pressing  'N' for  'No'. Otherwise,  if you do

have  AutoCAD,   and  would   like  to   install   the   drafting

enhancements, respond by pressing 'Y' for 'Yes'.

If you  select 'N'  and decide  that  you  want  to  install  the

drafting enhancements  later on,  you can run the install program

with the  /D option.  With this  option, the installation program

will only  install the  drafting enhancements.  To do this, mount

the installation disk in a floppy drive and type

[d:]install /D

where [d:] is the drive in which the diskette is mounted.

If you  selected 'Y'  (or if you run INSTALL with the /D option),

the installation  program will  request the  name of  the AutoCAD

program directory.  The default  is C:\ACAD.  Next, a prompt will

appear requesting  the directory  of the ACAD.PGP file (a file of

external commands  described  in  AutoCAD's  documentation).  The

default is the AutoCAD program directory named above. However, if

you are using software such as DCA engineering software, ACAD.PGP

will probably  reside in a different directory than AutoCAD, such

as C:\DCA\CIVIL. Be sure that you name the correct directory.

The installation  program will add a few external commands to the

ACAD.PGP file,  saving the original ACAD.PGP file as ACADPGP.BAK.

If  an   existing  BASIS  installation  is  being  upgraded,  the

appropriate lines will be modified as necessary.

The installation  program will copy the drafting enhancement Lisp

files and  executables into the AutoCAD program directory. And it

will  copy   BASIS  point   blocks   into   a   directory   named

C:\ACAD\BLOCKS. If  this directory  does not  exist, it  will  be

created. If  you already  have a  directory named C:\ACAD\BLOCKS,

Section 1 - Installation

-3-

and have  a DCA  point block  named POINT.DWG  stored in  it, the

BASIS POINT.DWG  will overwrite  it. Functionally,  there  is  no

difference, although  the text  attributes  of  the  BASIS  point

blocks are  smaller than  DCA's and  are a  slant simplex  style,

rather than standard txt style.

After the  installation is  complete, a  prompt will  appear that

will allow  rebooting the  computer (unless  you did  not  select

automatic modification  of AUTOEXEC.BAT.)  You  must  reboot  the

computer in  order to make the modifications to AUTOEXEC.BAT take

effect.

Note: BASIS  installs a  graphic headliner  program, as well as a

screen-saver. Both  these programs  require as  a minimum, an EGA

display. If  your display hardware does not support EGA graphics,

then delete  the files  BASIS1.EXE and  SAVER.EXE from  BASIS.DIR

after installation.

1.03 UNPACKING  INDIVIDUAL FILES - Although you'll probably never

need it,  a utility  program named LZHUF.EXE is provided on disks

one and two of the distribution set.

This program  lets you  unpack (or  pack) individual files if you

ever need  only one  file from  the originals  and don't  want to

spend the time for a complete re-installation.

The program usage for packing a file is:

[d:][path]LZHUF E [d:][path]filename [d:][path]filename

For unpacking:

[d:][path]LZHUF D [d:][path]filename [d:][path]filename

In either  of the above, [d:] and [path] represent optional drive

and path  specifiers. The  first file  is the source file and the

second file is the target.

To distinguish  between packed  and unpacked  files,  BASIS  uses

modified extensions, as indicated below:

Packed Extension         Unpacked Extension

$EX                      EXE

$CO                      COM

$BA                      BAT

$HL                      HLP

$DA                      DAT

For example,  suppose  that  you  want  to  re-install  the  file

COGO.EXE, the  interactive Cogo  program. This  file is stored on

the  diskette   labelled   PROGRAMS   2,   as   a   packed   file

\SURVEY\COGO.$EX.

Section 1 - Installation

-4-

If you place the original diskette in drive A, you could type

LZHUF D A:\SURVEY\COGO.$EX C:\BASIS.DIR\COGO.EXE

As mentioned,  you'll probably never have to do this. But you may

find LZHUF  handy for  packing your  own  files,  such  as  large

AutoCAD drawings, so they take up less space on your disk. If you

find that  you are  using LZHUF  very  frequently  for  your  own

purposes, I  recommend that  you obtain  a copy  of the excellent

PKZIP shareware  compression utilities  which provide  comparable

compression with  much  greater  convenience.  Unfortunately  ZIP

files are  not compatible with those created using LZHUF. You can

obtain a copy of PKZIP from most major computer Bulletin Boards.

LZHUF  is   a  public-domain  implementation  of  the  Lempel-Ziv

compression algorithm  with Huffman  coding, written  by Haruyasu

Yoshizaki. It  was translated  to English  by  Haruhiko  Okumura,

sysop of  the Science SIG of PC-VAN, a Japanese computer bulletin

board. The  BASIS  installation  program  uses  portions  of  the

original LZHUF.C source code for unpacking compressed files.

Section 1 - Installation

-5-

2.0 GETTING STARTED

2.01 ORGANIZING  DATA FILES  - Although data can be stored in the

same  subdirectory   as  the   BASIS  programs,  good  hard  disk

management dictates that you store BASIS data files in a separate

directory from  the programs, unless you plan to store data files

onto floppy  diskettes (of  course, floppies are much slower than

the hard disk.)

You should  let BASIS store all its data files in this directory.

Although you  can create a separate directory for each project or

client, this will be very inconvenient and you will find yourself

constantly changing  the data  directory in  the System  Defaults

program (described  in the  next section.). Having all BASIS data

files in one directory also makes it very easy to make backups of

the entire directory, either to diskettes, or preferably to tape,

if your system is equipped for it.

This directory  can be  on any  drive, and  branch off  the  root

directory, or  any other subdirectory. Subdirectories are created

using the  DOS "mkdir"  or "md"  command. Refer to the DOS manual

for instructions  on using  this utility  (Also refer  to section

3.01).

The BASIS Disk Management System will manage more than 7000 files

per subdirectory, depending on available RAM. More information on

this subject  will be  given in  the sections  explaining  System

Defaults and the Disk Management System.

2.02 WHAT  TO DO  FIRST -  Following the  initial installation of

BASIS, the following steps are recommended:

1. Create a data subdirectory using the DOS "md" utility.

2. Run BASIS (see section 2.3).

3. Configure the BASIS System Defaults (see section 3.0).

4. Create the Catalog Index (see section 4.01).

2.03 RUNNING  BASIS  -  If  the  installation  instructions  were

followed, the  program modules  now reside  on the hard disk in a

subdirectory  named  BASIS.DIR.  To  run  BASIS,  log  into  this

directory, type BASIS2 and press ENTER. This invokes the start-up

batch file  BASIS2.BAT. You  must run BASIS using this batch file

to ensure that the system runs properly.

As  an   alternative,  you  may  copy  the  startup  batch  file,

BASIS2.BAT into  the root  directory of  drive C. This eliminates

the need  to log  into \BASIS.DIR. Simply type BASIS2 at the C:\>

prompt, and press ENTER.

Section 2 - Getting Started

-6-

You may  also launch  BASIS2 from  a parent  batch file.  If your

version of DOS supports the 'CALL' statement (DOS 3.3 or greater)

you can place the line

CALL D:\BASIS.DIR\BASIS2

in your  batch file,  where D: should be the drive that the BASIS

software and BASIS2.BAT reside.

If you  are using  an earlier  DOS version, you can launch BASIS2

from a batch file with the statement

COMMAND/C D:\BASIS.DIR\BASIS2

where, again, D: is the drive where BASIS was installed.

Since  BASIS   will  remain   in  the  BASIS.DIR  directory  upon

termination, your  batch file  should follow  the line that calls

BASIS2 with a line that restores the directory and drive that you

wish to be current.

For example,  assuming that  BASIS was  installed on drive E, you

could create a short batch file named B.DAT in the root directory

of drive C:

CALL E:\BASIS.DIR\BASIS2

C:

CD \

Then you could invoke BASIS by simply typing B at the C:\> prompt

and pressing ENTER.

When you  run BASIS2,  a graphic  headliner screen  appears next.

Press ENTER  to bring  up the BASIS System Menu. If the headliner

remains on  screen for  60 seconds  and no  keys are pressed, the

"Sleeping Surveyor" screen-saver will automatically be displayed.

You can  press any  key while  the screen-saver  is displayed  to

bring up the BASIS System Menu.

If you  grow tired  of the  graphic headliner, you can use a text

editor to delete the line

%BASIS%\BASIS1.EXE

from BASIS2.BAT.

2.04 THE  BASIS SYSTEM  MENUS - BASIS is menu-driven. Rather than

making you  memorize obscure  commands, or  look up commands on a

"crib sheet",  all of the functions of BASIS are accessed through

a series  of menus.  The  System  menus  are  full-screen,  while

specific function  menus are  smaller pop-up  menus.  Most  menus

allow you to select an item in two or more ways.

Section 2 - Getting Started

-7-

The System menus, such as the BASIS Main Menu shown in Figure 2.1

allow you  to  select  an  item  by  pressing  the  corresponding

function key. On menus where more than ten choices are given, the

shifted function keys are used. (F11 and F12 are not used because

older keyboards do not support them.)

Pop-up menus  are similar.  If selections  are displayed  with  F

keys, then  you can  press  the  corresponding  function  key  to

activate your selection. Or, you can use the cursor movement keys

to position  the selection  bar on  your choice,  and then  press

ENTER. If  F keys  are not  displayed for an item (such as a Help

Menu) you  can  select  by  pressing  the  first  letter  of  the

selection. If more than one selection have the same first letter,

pressing the letter again will jump to the next selection. Or you

can use  the cursor  movement keys  to select, and press ENTER to

run.

If you  have a  Microsoft compatible  mouse, you  can select menu

items by  simply clicking on them. To click on an item, place the

mouse cursor  on the  item you want to select, and press the left

mouse button.

BASIS Main Menu

F1 - Disk Management System

F2 - Coordinate Geometry System

F3 - Field Traverse System

F4 - Utility Support Menu

F5 - System Defaults

F6 - DOS Command Processor

F7 - DCA/AutoCAD

F8 - Batched Cogo System

ESC - Exit

Figure 2.1 - The Main Menu

Throughout BASIS,  the ESC  key is  always used  to return to the

previous level.  Avoid hard  resets, as this could result in data

loss or  corruption. The  use of  the ESC  key insures  that data

files are properly closed.

If you are using a mouse, you can click on the bottom line of the

menu (where the ESC - Exit selection is displayed) to exit.

The following sections of the manual describe the individual menu

items of the Main Menu in detail.

Whenever a system menu is displayed, a digital clock will display

the current time. If no keys are pressed, or no mouse movement is

detected for  a period  of five  minutes, the "Sleeping Surveyor"

Section 2 - Getting Started

-8-

screen-saver will  appear on  the screen  (unless you deleted the

file SAVER.EXE  because you do not have EGA-compatible graphics.)

Pressing any  key while the screen-saver is active will bring you

back to the menu.

Section 2 - Getting Started

-9-

3.0 SYSTEM DEFAULTS (F5)

The System  Defaults program is executed by pressing function key

five (F5)  from the  BASIS Main  Menu (or clicking on any part of

that item with the mouse.)

This program allows you to change certain default characteristics

of the  system to suit your taste. A pop-up menu will appear that

allows you  to select  those items  that can  be changed. You can

also list the current system defaults from this menu.

These consist of the following:

Data Directory

Headings

Printer Options

Linear Units

Direction Units

Display Precisions

Slope Reduction Formula

Unequal Radius Tolerance

Default Cogo Filesize

Default Point Protect

Enable Mouse

You can also customize the colors that BASIS uses for its various

screens from the System Defaults menu.

To select  an item,  you can  use the  cursor  movement  keys  to

position the  selection bar on your choice, and then press ENTER.

Or, you  can press the first letter of your choice and then press

ENTER. (If  more than  one selection  has the  same first letter,

pressing that  letter again  will select the next matching item.)

If you  have a  mouse, you can place the mouse cursor on the item

and click the left mouse button.

3.01 DATA DIRECTORY - If a data subdirectory has been created, it

is necessary  to tell the BASIS system what it is. Selecting this

item brings up a dialog box that prompts for the name of the data

subdirectory.

BASIS will follow the usual DOS convention for finding files. The

best way  to assure  that the  files will be found is to give the

complete path specification.

For example, if a data subdirectory named COGODAT was created off

the root directory of drive D, type in

d:\cogodat

and press ENTER.

Section 3 - System Defaults

-10-

If the  data is  being written  to a floppy drive, then the drive

letter followed by a colon should be typed in, such as

a:

IMPORTANT -  Note that  a trailing  backslash is  never  entered.

BASIS automatically  precedes filenames  with  a  backslash  when

accessing files.

The data  subdirectory can  be changed by returning to the system

defaults program  from the  main menu,  and typing in a different

data path.  Some BASIS  programs may allow you to change the data

directory at  run-time, however  for most programs the choice you

enter here will be the directory that is used for data files.

When the  dialog box  appears,  the  current  data  directory  is

displayed as  the default  response. The  right and  left cursor,

backspace, delete,  home and  end keys are activated, and you can

use them  to position  the cursor  within the input field to make

any changes. Or you can simply re-type a new directory name.

3.02 PRINTER  HEADINGS   -  This selection brings up a dialog box

that lets  you enter  custom printout headings, such as a company

name and address.

You should  note  that  these  are  only  default  headings.  All

programs that  generate  printouts  will  alway  let  you  supply

headings at  run-time. What  you type  in here  will be  used  as

default reponses  to those  heading prompts  when they occur. The

use of  upper and  lowercase letters  is permitted.  Headings are

centered at printout time.

You may  elect only  to fill  in the  first two  headings, with a

company name and address, for example. The other two lines can be

filled in  at run-time  with a  job name  and location,  or other

information particular to a job.

As with  the data  directory field,  the heading  fields  can  be

edited with  the cursor  control keys, backspace, delete,home and

end keys.

3.03 PRINTER OPTIONS - This selection brings up a dialog box that

lets you  select or  set four  options related to printing. These

are the  printer address,  number of  lines per  page,  automatic

initial formfeed, and single/double spacing option.

Automatic Initial  Formfeed - Turning this option on ensures that

the printout  for a  program will  begin on  a new page. If it is

turned off,  then you must manually set the printer at the top of

a form  at the  start of a new program. If you find that a manual

formfeed is usually performed, such as to detach previous sheets,

then turning the auto-formfeed off will help to conserve paper.

Section 3 - System Defaults

-11-

Lines per page - This option is used to establish the page length

for printout  pagination. Usually,  it will  be between 55 and 66

lines  per   page.  If   desired,  however,   pagination  can  be

effectively cancelled by entering a large number, such as 9999.

Printer Port  - This  default  tells  BASIS  where  to  find  the

printer. In  most cases,  the printer  will be  plugged into  the

primary   parallel port, which the system identifies as "PRN", or

"LPT1".

Printout spacing  - This  option lets you select either single or

double-spaced printout.

3.04 DISTANCE UNITS - This selection lets you toggle this default

option from  feet to meters, or back. This default will determine

the units  of area computations (acres vs. hectares) and distance

conversions in  the field traverse system. Metric conversions are

made using  the U.S.  Survey Foot  value. For  typical  surveying

operations in the United States, set this option to FEET.

3.05 DIRECTION  UNITS -  This option  toggles between AZIMUTH and

BEARINGS, which  will control  what the  system  uses  to  output

directions, and  in some  cases, what to expect as input, such as

in the  Cogo programs.  Azimuths are  assumed to be reckoned from

North, if chosen.

3.06 DISPLAY  PRECISIONS -  This selection brings up a dialog box

that lets you specify the display precision of angles, distances,

coordinates and  elevations. A  value of  '0' indicates  that  no

fractional part is to be displayed.

Angular precision  can be either a '0' or a '1', indicating whole

seconds, or tenths of a second, respectively.

Coordinates, distances,  and elevations can be displayed with '0'

through '8' decimal places.

3.07 SLOPE  REDUCTION -  This selection  lets you  select whether

curvature and refraction corrections will be applied whenever any

of the  programs reduce zenith angle/slope distance measurements.

(The only  program  that  will  not  correct  zenith  angles  for

curvature and  refraction even if this option is turned on is the

Sideshot Utility program.)

3.08 RADIUS  UNEQUAL TOLERANCE  - When  performing certain  curve

operations in  Cogo, unequal  radius  distances  between  entered

point numbers  can indicate  an error  in  data  entry  or  point

numbering. The  program will  compare radii, and if unequal by an

amount greater than the value specified here, further computation

Section 3 - System Defaults

-12-

will be  aborted, and  an error  message  displayed.  Since  many

compound and  reverse curve  recomputations  may  involve  slight

roundoff error, or marginally unequal radii, do not set too small

a value  for the  tolerance. A  value of  .002 is  a good working

value.

3.09 COGO  FILESIZE -  The value  set  here  will  determine  the

default filesize  for project  file creation. Again, it is only a

default value,  and when creating coordinate files, the user will

always be  able to overide it. File creation will be discussed in

detail later  in this  manual. For  now, suffice  it to  say that

files can be anywhere from 50 to 30,000 points in size (depending

on available  disk space). You should set the filesize default to

the value most frequently used, perhaps 1000.

3.10 POINT  PROTECT - This option, if on, will cause a warning to

be displayed  if an  attempt is  made to  store coordinates  to a

point already  in use  (unless the coordinates are the same) when

using the  Cogo programs. When that happens, you can either enter

a new point number, or overwrite the existing coordinates. If the

point-protect option is turned off, no warning will be displayed,

and  the  point  will  be  overwritten.  When  running  the  Cogo

programs, you will again be able to toggle this option on or off,

however it will always default to the value set at this time when

the Cogo programs are started.

3.11 CUSTOMIZE  COLORS -  This  selection  brings  up  the  color

selection screen.  You can  use this  screen to set various color

options that all the BASIS programs will use.

When you  select colors, a miniature display in the bottom of the

screen will  show how  they look.  You  can  select  main  window

foreground,background, highlight  and input  colors, as  well  as

pop-up window colors.

Use the cursor movement keys to position the selection bar on the

screen component  for which  you want to select a color, and then

use the '+' and '-' keys to toggle through the available colors.

3.12 ENABLE  MOUSE -  This selection  lets you disable the mouse,

even if the program detects the presence of one.

For users  who have  experienced delays  when  executing  various

BASIS programs  due to  slow mouse  initiallization with  certain

mouse drivers (i.e. CalComp's CCMOUSE driver), this option can be

used to disactivate the BASIS mouse routines.

Select this  item from the defaults menu, and press 'Y' to enable

the mouse, or 'N' to disable the mouse.

Section 3 - System Defaults

-13-

3.13 LIST DEFAULTS - Selecting this option causes a pop-up window

to appear that shows the current settings of all system defaults.

When you  are finished  looking at the display, press any key, or

click either  the left  or right  mouse button  to return  to the

System Defaults Menu.

3.14 SAVING  THE NEW DEFAULTS - When you are finished editing the

defaults, select  QUIT (or  simply press ESC.) A message box will

appear asking  if you want to save any changes that may have been

made. The  default is YES, and you can simply press ENTER to save

the new defaults. Pressing 'N' (for No) will not save any changes

that you may have contemplated.

If you are using a mouse, simply click on the YES or NO buttons.

Section 3 - System Defaults

-14-

4.0 DISK MANAGEMENT SYSTEM (F1)

The Disk  Management System  is executed by pressing function key

one (F1)  from the  main menu. When selected, the Disk Management

menu (see  Figure 4.1)  will appear on the screen, from which the

various programs  that comprise  the system  can be  accessed. As

with any  other  system  menu,  selections  are  made  by  either

pressing the  corresponding function key, or by clicking the item

with the mouse.

The Disk  Management System  can be  broken down  into three main

categories:

1. File Creation

2. File Handling

3. Coordinate Handling

As with  all other  menus and programs of BASIS, pressing the ESC

key returns program control to the previous level.

Disk Management System

FILE CREATION                   FILE HANDLING

F1 - Create Catalog Index       F7 - Delete Data File

F2 - Create Project File        F8 - Display/Edit Catalog Data

F9 - View/Print Catalog Data

COORDINATE HANDLING            F10 - Copy Data File

F3 - Store Coordinates         ^F1 - Generic Import/Export

F4 - Clear Coordinates         ^F2 - Catalog Search

F5 - Transfer Coordinates      ^F3 - Expand Project Filesize

F6 - List Coordinates          ^F4 - Help

Figure 4.1 - The Disk Management Menu

4.01 CREATE  CATALOG INDEX  (F1) -  The catalog index file is the

heart of  the BASIS Disk Management System, and creating it using

this routine is the next step after creating a data subdirectory.

The  catalog   index  is   only  created   once  for   each  data

subdirectory, and  is used by the BASIS system to cross-reference

BASIS filenames  with their  corresponding DOS disknames, as well

as for  storing other  important information  about the file. The

catalog index  is what gives BASIS its mini-database capabilities

for sorting,  or searching out files from commentary information.

It also gives the user the ability to use filenames 50 characters

in length, rather than the DOS restricted eight-character names.

Press function key one (F1) to activate this program. The catalog

creation window  will appear,  and the  user will be prompted for

the data  path. The  default is  the path named previously in the

Section 4 - Disk Management System

-15-

System Defaults program. To accept the default path, simply press

ENTER. If a different data path is desired, type it in, using the

same conventions  explained in  section 3.01  of this manual, and

press ENTER.  The amount of free space available on the disk will

be displayed,  and the  user will  be asked  if it  is desired to

proceed. Respond by typing 'Y' or 'N' (upper or lowercase). It is

not necessary to press ENTER.

If a  catalog index already exists for the named subdirectory, an

error message  will appear  on the screen. A second catalog index

will not be created by the program, as this would confuse BASIS.

If no  catalog index  exists on  the named path, then the program

will create  it, and  program control  will return  to  the  Disk

Management menu.

If you  are using  floppy disks  for  storing  project  files,  a

catalog index  file must  exist on each floppy. If you are trying

to use  the Copy  Data File  program (see Section 4.10) to copy a

project file  to a  floppy, then the target floppy must contain a

catalog index  file. Wherever BASIS project files are going to be

saved, a  catalog index file must exist to keep track of the file

and its  various parameters.  Note, however,  that this  does not

apply to exporting generic ASCII coordinate files.

4.02 CREATE  PROJECT FILE  (F2) - To create a project file, press

function key  two (F2).  The project  file creation  screen  will

appear.

Once a  catalog index has been created, this function can be used

to create  a project,  or coordinate  file. Other  system modules

will also  give the  opportunity to  create project  files  under

certain circumstances,  and the  procedure at  those times is the

same as the following.

The first field to be entered is the filename. Alphanumeric names

up to  50 characters long are permitted. The first six characters

of the  filename cannot  contain the characters * , ?, or a blank

space, and  may only  contain those characters allowed by DOS for

filenames. Filenames  are not case-sensitive, that is, the system

does not  recognize any  difference between  upper and  lowercase

letters.

A feature  of the BASIS Disk Management System is that it permits

filenames   such   as   FAIRFIELDAVENUE,   FAIRFIELDSTREET,   and

FAIRFIELDROAD, as  examples. The  above files would be recognized

as separate  files, and  would  be  saved  to  disk  with  unique

disknames    (FAIRFI0.PRJ,     FAIRFI1.PRJ    and    FAIRFI2.PRJ,

respectively). This  latter  action  is  handled  by  the  system

internally, and  need not  be a  concern of  the user.  Up to 100

files with the same first six characters are permitted.

Section 4 - Disk Management System

-16-

The next  prompt requests  the desired filesize, with the default

filesize appearing  after  the  prompt.  To  accept  the  default

filesize, simply  press ENTER.  Otherwise, respond  by typing the

desired filesize  (minimum 50  points, maximum 30,000 points) and

pressing ENTER.  There will be a momentary pause while the system

prepares the  file on  the disk.  If there  is insufficient  disk

space, a  warning will  flash, and the file creation process will

be terminated.

The next  series of  prompts request commentary information about

the file,  and the responses to these are optional. To bypass any

prompt, simply  press ENTER.  Remember that these fields are used

to search  for files  in the Catalog Search Program. To enjoy the

database  management   capability  of   the  system,   users  are

encouraged to answer the prompts!

Once all  prompts are responded to (or bypassed) the file Catalog

will be  updated with  the file  information,  and  control  will

return to the Disk Management System.

The ESC  key can  be used  at any  time to abort the project file

creation. Do  not use  the ESC key as a shortcut to bypassing the

commentary fields, as it will cancel the file creation process.

4.03 STORE  COORDINATES (F3)  -  This  program  is  activated  by

pressing function key three (F3). Once a project file is created,

the Store  Coordinates Program  can be  used to  place coordinate

data into  the file (Coordinates can also be stored from the Cogo

Program).

The first  prompt to  appear  requests  the  name  of  the  BASIS

coordinate file  to receive  the points.  The default response is

the last file used. If the file that you name does not exist, you

will be given the option to create it.

The next  prompt to  appear asks  if you  want to  store randomly

numbered points,  or a  sequential series of points.If you select

'R' for  random, then  you will  have to enter a point number for

each coordinate  you store. If you press 'S' for sequential, then

the computer  will automatically  supply sequential point numbers

for each coordinate you enter.

The next  prompt requests  the data  format. Several  formats are

available  to   provide  flexibility   for  entering   northings,

eastings, elevations,  and descriptions.  Select the  number that

corresponds to the format you desire.

If you  selected "Sequential"  storage, you will then be prompted

for the  starting and  ending points  of the  series you  wish to

store.

Then, depending  on which  storage format you chose, prompts will

appear requesting  the  optional  coordinate  shifts.  These  are

Section 4 - Disk Management System

-17-

constant values  that will be added to coordinate values that you

enter. For example, suppose you enter a North shift of -90000 and

an East shift of -370000. If you enter a northing of 99276.33 and

an easting  of 375324.65,  the  coordinates  will  be  stored  as

9276.33 and 5324.65. The default values for the coordinate shifts

are zero. You can accept the defaults by simply pressing ENTER.

The coordinate  entry screen  will then  appear.  While  entering

points, if  a point  is already  in use,  the current  coordinate

values will  appear as default responses to the prompts for data.

If you  accept the  default value  by simply pressing ENTER, then

any  shift   value  will  NOT  be  applied...the  value  will  be

unchanged. If  you change  the value,  the shift, if any, will be

applied

Coordinate storing  can be terminated at any time by pressing the

ESC key.  Points are  automatically saved  to disk  as  they  are

entered, and no special commands to save the data are required.

4.04 CLEAR  COORDINATES (F4)  - Function  key four (F4) initiates

the  Clear  Coordinates  program,  which  performs  the  opposite

operation from the Store Coordinates program.

The first  prompt requests the BASIS project filename. Because of

the consequences  of deleting  coordinates, there  is no  default

response. You  must type  in the  name of the file from which you

want to delete coordinates.

The next  prompts request  the starting  and ending point numbers

for the sequence of points to be deleted.

Next, a  series of  three prompts  ask  if  you  want  to  delete

northings and  eastings,  elevations  and/or  descriptions.  This

gives you  the option  to selectively delete values. For example,

you may  only be  interested in  clearing out elevations. In that

case, respond to the northing and easting prompt with 'N' for No.

Type 'Y' for elevations, and 'N' for descriptions.

To initiate  clearing coordinates,  a final  prompt requests  the

user to  type 'yes'  (upper or lowercase) and press ENTER. Typing

anything else will cancel the clear operation.

The ESC key aborts the program if pressed, and returns control to

the Disk Management System.

4.05 TRANSFER  COORDINATES (F5)  -  This  program,  activated  by

function key  five (F5),  can be  used to  transfer sequential or

random points  across files,  and if  desired, in  different data

subdirectories. It can also be used to transfer a block of points

within the same file (with limitations).

Section 4 - Disk Management System

-18-

BASIS will  prompt for  source and  target data  directories  and

files. The  default paths  are the  current   "DATA AT:" defaults

which  have  been  set  previously  (see  section  3.01).  Simply

pressing ENTER  at these  prompts will  log in  the default  data

directories.  To  change  the  directory,  type  in  the  desired

directory name  exactly as  described in  section 3.01. Enter the

source and target BASIS filenames at the appropriate prompts.

The next  prompt asks if you want to transfer points at random or

sequentially.Pressing  'S'   will  select   sequential  transfer.

Pressing 'R' selects random transfer.

If you  select sequential  transfer, prompts for the starting and

ending point  numbers in  the source  file and the starting point

number in  the target  file will  appear. The  program  will  not

accept a  target starting  point that will result in over-running

the target  filesize. If  you are  transferring points within the

same file,  the transfer  regions cannot  overlap. If  they do, a

warning will appear.

If random  transfer is selected, then the program will prompt for

the point  number of  the target file to begin storing the random

points that  are entered.  If  the  maximum  target  filesize  is

reached during  a random transfer, the program will automatically

terminate and return control to the Disk Management System menu.

While random transfer within the same file is permitted, you must

be careful  that you do not transfer to a new point, and then try

to recall that point's old value. This is similar to transferring

sequential points  to an  overlapping region. The program permits

random transfers  within the  same file  to give you flexibility.

However, it is up to you to do it sensibly.

Unused points will be transferred as unused points.

If Random  transfer is  selected, a  prompt will  appear to allow

selection of  a hardcopy record of the random points transferred.

Respond by  pressing 'Y'  to  enable  printing,  or  'N'  for  no

printout. The default response is 'N'.

If the  hardcopy option  is enabled, then prompts will appear for

the printout  headings. The default headings are those set in the

System Defaults configuration. Simply press ENTER at each heading

prompt  to  accept  the  default.  Otherwise,  type  the  desired

message, and press ENTER.

The next  series of prompts will allow setting the combination of

northings, eastings,  elevations and  descriptions that are to be

transferred. In  some cases,  the user  may desire  to only  send

northings and  eastings. At other times, only elevations may need

to be transferred. Respond to the corresponding prompts by typing

Y or N, as required.

Section 4 - Disk Management System

-19-

If desired, it is also possible to shift the northings, eastings,

or elevations by constant amounts. For example, a file containing

state plane  coordinates could  be reduced  to a project datum by

entering a  negative value  for northing shift and easting shift.

Elevations on  one datum  could be converted to a different datum

by entering the corresponding elevation shift. The default values

for the datum shifts are always zero. Remember, the value that is

entered is always added to the source coordinate/elevation in the

target file.  The source  value remains  unchanged. To decrease a

value, enter a negative shift value.

If  Sequential  transfer  was  selected,  do  not  interrupt  the

process, as  doing so  may corrupt  the target  file.  If  Random

transfer was  selected, pressing  ESC will terminate the transfer

routine, and restore the Disk Management System Menu.

4.06 LIST  COORDINATES (F6)  - The  coordinate listing routine is

accessed  by  pressing  function  key  six  (F6)  from  the  Disk

Management  System   menu.  Various  combinations  of  northings,

eastings, elevations, and descriptions can be listed. If desired,

a listing  of UNUSED  points can  be generated,  rather than used

points.

The first  prompt to  appear  requests  the  name  of  the  BASIS

coordinate file  to be  listed. The  name of  the last  file used

appears as  the default  response. You  can accept the default by

pressing ENTER, or you can type the name of the desired file.

After responding to the filename prompt, the program will request

the starting  and ending  points for  the  listing.  The  default

responses are  for the  beginning of the file, point number zero,

and the highest point number, which depends on the filesize.

Next, the  available listing  options will  be  displayed.  These

permit  listing  several  combinations  of  northings,  eastings,

elevations  and  descriptions.  Select  the  listing  option  you

desire. The default is option number one, northings and eastings.

Next a  prompt appears asking if you want a printout. The default

response is 'N' for no printout. If you want hardcopy, press 'Y'.

Next, a  prompt will  appear allowing  selection of  used  points

("coordinates") or  unused points.  Respond by typing C or U. The

default response is 'C' for coordinates.

An "ALL  OK (Y/N)"  prompt will  appear  next.  Review  the  data

entered at  this point.  If there  is something  that you want to

change, press 'N'. The default response is 'Y'.

If a  coordinate listing  was  selected  (as  opposed  to  unused

points), a few more option prompts will appear.

Section 4 - Disk Management System

-20-

The    coordinates/elevations     can    be     displayed    with

northing/easting/elevation shifts  applied to the original values

(the  file  coordinates/elevations  do  not  change  -  only  the

displayed or printed values). The default datum shifts are zero.

The precision  of the  coordinates/elevations  can  be  selected,

ranging from zero to eight decimal places. The default values are

the display precisions selected in the System Defaults program.

If you  enabled hardcopy,  prompts  for  printout  headings  will

appear next.  The default  responses are  those set in the System

Defaults configuration.

If hardcopy  is NOT  selected, the  coordinates will be displayed

one screen  at a  time. To continue listing, press ENTER when the

display pauses, and the next screen "page" will appear.

If hardcopy is enabled, the coordinates will scroll up the screen

as they  are printed.  The display  will not  pause as it does if

hardcopy is disabled.

To terminate the listing, press the ESC key.

(If hardcopy is selected, the printer may continue to print for a

few moments  after pressing the escape key, depending on the size

of the printer buffer, however program control will revert to the

Disk Management System menu once the buffer is cleared.)

4.07 DELETE  DATA FILE  (F7) - This routine, selected by pressing

function key  seven (F7),  is used  to permanently  remove a data

file (of  any type)  from  the  BASIS  catalog,  and  delete  the

corresponding disk file from the data subdirectory.

Use this  routine with  caution, as the file removal procedure is

non-reversible!

Respond to  the filename prompt by entering the BASIS filename. A

warning will  appear on  the screen  advising  the  user  of  the

consequences of  this routine.  To permanently remove a file from

BASIS, type 'YES' or 'yes' to the prompt and press ENTER.

CLEANCAT is  a utility included with BASIS that removes "garbage"

from the  BASIS Catalog  Index. This  garbage is  in the  form of

records for  "Deleted" project  files, which take up space in the

catalog index, even though the actual project files are no longer

on disk.  In the  present version  of BASIS, a catalog entry uses

483 bytes of disk space.

To remove  these  useless  entries,  run  CLEANCAT.  To  use  the

program, shell  out (F6  from the main menu) and type CLEANCAT at

the DOS  prompt. The  program will  create a  back up copy of the

original catalog named CATALOG.BAK, and store the cleaned version

in CATALOG.DAT,  which is  the name  of the  normal catalog index

file. If there is no need for the original catalog, the /d option

Section 4 - Disk Management System

-21-

can be  used, which  will cause  the program to delete the backup

once the  cleaned version  has  been  successfully  created.  For

example, to run the program with the delete option type

cleancat /d

at the  dos prompt.  (Note: this  program must  be run  from  the

BASIS.DIR directory.)

The cleaned  catalog will  allow for  quicker file  finding  when

opening project  files, and  take up  less space  on the disk. Of

course, if there are not very many deleted files, the improvement

will not be very significant.

4.08 VIEW/EDIT  CATALOG DATA  (F8) -  Pressing function key eight

(F8) evokes  the catalog  edit program.  This routine  allows the

user to  edit or  modify  the  commentary  information  that  was

entered in the file creation process.

This routine  is also  useful for simply viewing this information

for a  desired file. After typing the BASIS filename, in response

to the  filename prompt,  the file information will appear on the

screen. If  no changes  are to be made, simply press ESC, and the

filename prompt will reappear, to allow entering another filename

for viewing/editing.  Hit the ESC key if no other files are to be

viewed.

If changes  are desired,  simply edit  or retype  each  field  as

needed, pressing  ENTER after  each prompt. The current value for

each field  is the default response. Note that some fields cannot

be changed or edited.

4.09 DISPLAY/PRINT  CATALOG DATA  (F9)  -  This  routine  permits

alphabetically sorting (optional) and displaying all files in the

BASIS catalog  in the  subdirectory of  the users  choice. It  is

accessed by  pressing  function  key  nine  (F9)  from  the  Disk

Management System menu. File data can be displayed to the printer

or screen.

The first  prompt to  appear requests  the data subdirectory. The

default value  is the  current "DATA  AT:" default. To accept the

default, simply  press ENTER,  otherwise enter  the  subdirectory

specification, and press ENTER.

The next prompt allows selecting alphabetic sorting of the files.

Respond by  pressing 'Y'  or 'N'.  If there are many files in the

catalog, it may take a second or two to perform the sort.

You can  also limit  files listed  to those created on or after a

certain date. If you respond to this prompt with 'Y', you will be

prompted for a date.

Section 4 - Disk Management System

-22-

A "list  box" of BASIS files will be displayed on the screen. You

can use  the Page Up and Page Down keys to browse the list. Press

ESC when you are done.

The next prompt asks if you want to view individual file data. If

you press  'Y', detailed file data for each file in the list will

be displayed,  starting with  the first file. If you press 'N' or

ESC program  control returns  to the  Disk Management  menu.  The

default response  is 'N'.  If you press 'Y', then individual file

data will  appear on  the screen, one file at a time. Use PgUp or

PgDn to scroll forward or backward through the files.

The program  will also  ask if  you want hardcopy of the detailed

listing. If  you select  'Y', the information will be sent to the

printer in  addition to  being  displayed  on  the  console.  The

default response is 'N'.

While file data is being displayed or printed, you can cancel the

program at  any time  by pressing  ESC. If hardcopy is on, it may

take a few moments before the printer buffer clears.

4.10 COPY  DATA FILE  (F10) -  This routine was formerly known as

the Back-Up Data File routine in the first version of BASIS.

Pressing function  key ten  (F10) activates the program, which is

used to  make copies  of a  file onto a floppy disk, or any other

data subdirectory.  It can  also be  used to copy files stored on

floppy disks  onto the hard disk. A BASIS catalog must be present

on the diskette or in the destination directory.

Unlike  the  Transfer  Coordinates  program,  this  program  will

automatically adjust  the  target  filesize  to  accommodate  the

source file.  This is  useful if  the source  filesize  has  been

increased  since   the  last  back  up  was  made.  If  there  is

insufficient room  on the target disk for the new file, a warning

will be issued and the program will terminate.

The first prompt that appears requests the source data directory.

The default data directory is the BASIS system default. To accept

this, press ENTER, otherwise enter the desired path.

Next, respond  to the request for the source filename by entering

the BASIS filename, and pressing ENTER.

If the  above information  is entered  correctly, a  prompt  will

appear for  the target  path. The  default target  path is floppy

drive A.  (If you  entered drive  A or B as the source directory,

the default  will be  the BASIS  default directory  on  the  hard

disk.) To  accept the default, press ENTER, otherwise type in the

path desired.  The program  will assume  that the  backup will be

given the same BASIS filename as the source.

The program  will not  permit attempting  to backup  a file  over

itself, nor will it permit copying within the same directory.

Section 4 - Disk Management System

-23-

A message  box will  appear, displaying  file data for the source

file, and  also the  target file,  if a  file with  the same name

exists in  the target directory. Since the program will overwrite

the target  file if  it exists,  and  modify  the  target  file's

catalog data,  a prompt  requests confirmation  to  initiate  the

process. Type 'Y' to proceed, or 'N' to abort the process.

If the  target file does not exist at the destination, it will be

created, and  the information  in the source catalog listing will

be copied  to the  destination catalog as well. The catalog data,

and time  and date stamps on the copied file will be identical to

the source file.

A "progress  bar" will appear briefly, graphically indicating the

percent of  completion as the file is copied. Normally, copying a

file takes only a few seconds.

The ESC  key can be used at any time to terminate the program and

return control to the Disk Management System menu.

4.11 GENERIC  IMPORT /  EXPORT (^F1)  - Pressing the SHIFT and F1

key simultaneously will initiate the import/export program.

This program is used to convert BASIS binary coordinate files, or

portions thereof, into ASCII text files for transmission to other

software systems  that also support generic data exchange. It can

also be used to import coordinates from other systems in the same

manner.

The first prompt asks if you want to send coordinates to an ASCII

file, or  receive coordinates  from an  ASCII  file.  Respond  by

pressing 'S'  or 'R'.  The  default  response  is  'S'  for  send

coordinates.

The next  prompt requests  the name  of the BASIS coordinate file

that will  be involved with the transfer. The default response is

the name  of the  last file  used. Accept the default by pressing

ENTER, or  type  in  a  new  file  name.  If  you  are  receiving

coordinates, and  the file that you name does not exist, you will

be given the option to create it.

Next,  you  will  next  be  given  the  option  to  restrict  the

coordinates  sent  or  received  by  specifying  a  window.  Only

coordinates that fall within the window will be considered valid.

A window  can be  either rectangular or circular. Respond to this

prompt by  pressing 'B'  (for Box), 'C' (for circle), or 'N' (for

no window.) The default response is 'N'.

If you  select 'B',  for a box or rectangular window, you will be

prompted for  the maximum and minimum northing and easting values

of the window. If you select 'C', for a circular window, you will

Section 4 - Disk Management System

-24-

be prompted  for the  northing and easting of the center, and the

radius of the circle.

Next, a  pop-up window  will  appear,  displaying  the  available

transfer formats. These are:

1. Northing, Easting

2. Easting, Northing

3. Northing, Easting, Elevation

4. Easting, Northing, Elevation

5. Northing, Easting, Elevation, Description

6. Easting, Northing, Elevation, Description

7. Point Number, Northing, Easting

8. Point Number, Easting, Northing

9. Point Number, Northing, Easting, Elevation

10. Point Number, Easting, Northing, Elevation

11. Point Number, Northing, Easting, Elevation, Description

12. Point Number, Easting, Northing, Elevation, Description

A prompt  will appear requesting that you select the format. Type

in the  number of the format you want to use. (If you are working

with a  windowed group  of points,  then you must select a format

that includes a point number.)

If you  selected a format that does not include the point number,

you will be asked to supply codes for invalid or unused points. A

sequence of  ASCII  points  without  point  numbers  needs  these

invalid point  codes to serve as fillers for unused points in the

sequence.  You  can  supply  invalid  coordinate  codes,  and  if

applicable, invalid  elevation codes. The default code for either

is -999999.  Accept the default by pressing ENTER, or type in the

value that  you want  to use  (which can  be any  combination  of

letters or digits.)

If you  are sending  coordinates  to  an  ASCII  file,  the  next

prompt(s) will  request the  precision of the coordinates and, if

applicable, elevations.  You can  send coordinates and elevations

with precisions ranging from zero through eight decimal places.

The next  prompt asks  for the field delimiter, or character that

separates the  coordinates. This  can be  a space,  a comma, or a

CRLF (carriage  return/linefeed character).  Generally, you  will

use either  a space  or a  comma. Some other programs may require

that the  data be  separated by  placing each value on a separate

line.  In  this  case,  use  the  CRLF.  Press  the  number  that

corresponds to  your choice  (1-space, 2-comma  or  3-CRLF.)  The

default is 1, for a space delimiter.

If you  are receiving  coordinates, and  a format  that does  not

include point  numbers was  chosen, you  will then  be  asked  to

supply the starting point number for storing the imported points.

Section 4 - Disk Management System

-25-

If you  are sending  coordinates, you will be asked to supply the

starting and  ending point  numbers of  the block of points to be

sent.

Finally, you will be prompted for the name of the ASCII file. You

can enter  a complete  DOS file specification, including path and

extension. If  you are  sending coordinates  and the file already

exists, a warning message will appear.

The program  will then  prompt "ALL  OK (Y/N)?"  and give you the

opportunity to review the data that you have entered. If you want

to cancel the transfer press 'N'. The default response is 'Y'.

4.12 CATALOG  SEARCH (^F2)  - Pressing the SHIFT key and function

key two  (F2) simultaneously  calls the  Catalog Search  program.

This routine  allows the  user to  search for  files  that  match

certain input  values in  the commentary information fields which

are entered  when a  file is  created. To  take advantage of this

feature, it is important that these fields be entered during file

creation.

The first prompt to appear requests the data directory to search.

The default  is the system default data directory. Press ENTER to

accept the default, or type in the directory you want to search.

Next, a  dialog box  appears, in  which you  can enter the search

strings for any of the fields. You must enter at least one search

string. You  can search for a file by name, street, town, client,

file number,  creator, and  file type.  Searches  are  not  case-

sensitive. That  is, the  program does  not  distinguish  between

upper and lowercase letters.

Press ENTER  to bypass  any field.  When you  get to  a field for

which you  want to  find matching  files, type  in the string you

want to match. Wildcards '*' and '?' are permitted.

An asterisk(*)  matches  any  characters.  A  question  mark  (?)

matches any characters up to the character following the question

mark. The  main difference  between *  and ?  is that  ? must  be

followed by  at least  one character.  *  does  not  have  to  be

followed by a character.

For example,  suppose you  want to find all files that pertain to

Main Street.  Since you  may have typed Main St. rather than Main

Street when you created the file, you could be sure that you find

all files  by searching  for MAIN S*. (MAIN_S? will find MAIN ST,

but it wont find MAIN STREET.)

Or, suppose  your not  sure how  you may  have spelled  a clients

name. You  could have  spelled Smith  or Smythe.  You could  find

either by  searching for  SM?TH* or SM?TH?. However, SM? will not

work, and  SM* will.  Why  not?  SM*  works  because  it  matches

anything that  begins with  SM. SM? does not work because it only

matches SM or SM# (# being any printable character.)

Section 4 - Disk Management System

-26-

Any matching files will be displayed in a list box. You can press

Page Up or Page Down to scroll up or down through the listing.

Press ESC when done.

4.13 EXPAND PROJECT FILE (^F3) - Coordinate files can be enlarged

up to  a maximum  of 30,000 points by pressing the combination of

SHIFT and function key three (F3).

You will  be prompted  for the  BASIS filename  of the file to be

expanded. The  default is  the last  file used.  Press  ENTER  to

accept the default, or type the filename.

The current  filesize will  be displayed,  and the  program  will

prompt for  the new  filesize, which  must be  larger.  Filesizes

cannot be decreased, only enlarged.

The program  will ensure  that there  is enough room on the disk,

and if so, expand the file to the new size.

As with  all other  programs, the  ESC key will return control to

the Disk Management System menu.

4.14 DISK  MANAGEMENT SYSTEM  HELP (^F4)  - Shifted  function key

four (^F4)  activates the  Help system.  When pressed,  a  pop-up

window will appear, listing the various routines available in the

Disk Management  System. To  view a  Help  screen,  position  the

selection bar  on the  topic of  interest  by  using  the  cursor

movement keys,  or by  pressing the first letter of the item, and

then press  ENTER. If  you are  using  a  mouse,  click  on  your

selection.

When the Help screen appears, the up/down cursor keys, as well as

PgUp and  PgDn are  activated and permit scrolling the help text.

To return  to the  Help menu,  press ESC.  To return  to the Disk

Management menu, press ESC again.

4.15 COORDINATE  COMPARISON (^F5)  - Shifted  function  key  five

(^F5) invokes  the project  file comparison  routine. You may use

this routine  to compare  the contents of files. Differences will

be displayed  in the  screen window,  including points in-use vs.

unused points,  and points  with differing coordinate values. You

may find  this program  useful if a project file is inadvertently

worked on  separate computers,  and you  wish to  consolidate the

file to one.

Section 4 - Disk Management System

-27-

5.0 INTERACTIVE COORDINATE GEOMETRY SYSTEM (F2)

Pressing function  key two (F2) from the main menu, activates the

Interactive Coordinate  Geometry program.  This  is  one  of  the

largest programs  of the  system, with  many functions and pop-up

utilities.

The first  prompt to  appear will  request the  name of the BASIS

project, or  coordinate file.  The default  is the last file that

was used.  To accept the default, press ENTER, otherwise, enter a

new filename.  If the  file does not exist, you will be given the

option to create it.

Next, the program will prompt for the printout headings, with the

defaults being  the headings  selected  in  the  System  Defaults

program. Simply  press ENTER to accept the defaults, or else type

in the desired headings. (The printout headings will only be used

if hardcopy  is enabled   from  the Cogo  screen.  This  will  be

described shortly.)  After you  answer the  file name and heading

prompts, the Cogo screen will appear.

Take a  moment to  look over  the screen. Notice the filename and

filesize are  displayed in  a message  bar across  the top of the

screen. If  you press  CAPS LOCK,  NUM LOCK  or SCROLL  LOCK, the

status of  those buttons is displayed in the lower left corner of

the screen.  A hint to press F10 for a help screen appears at the

lower right  corner. About two-thirds down the screen, the status

of the point-protection and printer toggles are displayed. Point-

protect may be either ON or OFF, depending on the default setting

you used  in the  System Defaults  program. The printer is always

OFF when  you first  start Cogo.  Just below  this status line, a

prompt appears,  "FROM POINT?". The program is waiting for you to

enter some  data. This  section of the screen is called the "cogo

command line."

Most cogo  operations are  performed from  the cogo command line.

You respond  to a series of prompts, supplying the known data and

skipping the unknown data. The program computes the unknowns.

You can  over-ride the  usual prompts  to solve  numerous special

problems by entering a command prefix. Pop-up utilities and other

types of  solutions are  activated by  pressing various  function

keys. Generally,  when a function key is pressed, a dialog box or

pop-up window  will appear,  either requesting  more  information

about the problem, or displaying a menu with additional choices.

Cogo features  an advanced  command line expression parser, which

will intelligently  determine the  problem it  is being  asked to

solve based  on the data that is entered. There is no need to use

separate command  prefixes, or function keys to perform traverse,

inverse, bearing-bearing,  bearing-distance,or  distance-distance

computations. The  program will  decide which  problem  to  solve

based on the information that you enter.

Section 5 - Interactive Coordinate Geometry

-28-

If you press F10, a pop-up display of available command prefixes,

and function  key  utilities  will  appear.  This  handy  "quick-

reference" to special commands can be viewed at any time when you

are using  cogo (unless you're entering data in a dialog box.) If

you need  more detailed help on a particular topic, you can press

^F10 (SHIFT F10) while the command index is displayed, and a menu

of detailed help topics will appear. You can select a topic using

the cursor movement keys, first letter or mouse.

Detailed help  is an  "on-line" version of this manual. The topic

of interest  will be displayed and you can use the cursor up/down

keys, or  Page Up/Page  Down keys  to scroll  forward or backward

through the  text. When  you are finished, press ESC to return to

the previous level, until you are back in cogo.

This section  of the  manual will explain the various options and

features of cogo in detail.

5.01 CONVENTIONS  - As  with any  other program,  there are a few

rules to  follow when using BASIS Cogo. These primarily deal with

how to  enter values.  BASIS has  extensive  error-trapping,  and

should correctly  report the  source of  most errors to the user,

however to avoid frustration, new users should read the following

section carefully.

5.01.01 ENTERING  ANGULAR VALUES - There are two modes of angular

input, which  are determined by selecting BEARINGS or AZIMUTHS in

the System Defaults program (see section 3.05).

To explicitly enter a bearing, the format QDDMMSS.SSS... is used,

where Q  refers to  the quadrant  code (1-NE,2-SE,3-SW,4-NW),  DD

refers to  the Degrees,  MM refers  to the  minutes,  and  SS.SSS

refers to  the seconds,  and optional fractional seconds. Leading

and trailing zeros must be filled in. For example, a bearing of N

45�30'30" W would be entered as 4453030 . N 9�01'00" E is entered

as 1090100 . Notice that leading and trailing zeros are filled in

the latter example. A bearing of S 23�27'34.3" E would be entered

as 2232734.3

If azimuths  are the selected mode, then the corresponding format

is DDDMMSS.SSS...  where  DDD  represents  the  degrees,  MM  the

minutes, and SS.SSS... represents seconds and optional fractional

seconds. Again,  leading and  trailing zeros  must be entered. To

enter an  azimuth of  270�23'11", 2702311  would be  typed at the

keyboard. An azimuth of 9�14'00" would be keyed in as 0091400.

Bearings  can   also  be  entered  implicitly,  by  defining  two

endpoints, provided the coordinates of the selected endpoints are

stored. For  example, entering  456*457 at  a bearing prompt will

result in the bearing that exists between points numbered 456 and

457 being  used in  the computation.  Calling for an unused point

Section 5 - Interactive Coordinate Geometry

-29-

number in a defined bearing will cause an error message to appear

on the screen.

A frequently  used bearing  can also  be stored  as a bearing (or

azimuth) number. A maximum of ten stored directions is permitted.

Directions are stored using the ^ operator.

For example,  suppose that  a bearing  of N  45�35'20" W is being

repeatedly used,  perhaps on  a tier of lots in a subdivision, or

in coordinating  a structure  for layout.  Rather than repeatedly

key in  the bearing explicitly, or even by defining endpoints, it

could be  stored as  a bearing number, and then invoked by simply

entering that  bearing number in response to a bearing prompt. To

store the  bearing as stored direction number one, the first time

it is  used enter  4453521^1. From  that point  on, typing  1  in

response to a bearing prompt will produce that bearing.

The ^  operator can  also be  applied  to  defined  bearings  and

azimuths. To  store the  direction defined  by points 27 to 28 as

stored direction number two, enter 27*28^2 at the next bearing or

azimuth prompt.  From that  point on, simply typing 2 in response

to a direction prompt will result in that direction being used in

the computation.

Stored bearings  are saved  along with the project file, and will

remain there until overwritten with a different stored bearing.

Bearings and  azimuths can  also be  incremented  by  an  angular

value.

For example,  suppose it  is desired to add an angle of 72�12'45"

to a  bearing of  N 14�23'57"  E. This  can be  done by  entering

1142357+721245. Angles can be similarly subtracted.

The convention  for angles  is not  as strict as for bearings and

azimuths. The angle convention is DDMMSS.SSS... which is similar,

only LEADING zeros do not have to be entered as with bearings.

For example,  suppose it  was desired  to subtract  an  angle  of

0�08'07" from  a bearing  of N 01�23'04" W. This would be entered

as 4012304-807.

The end  result of  angle arithmetic can be stored as a direction

number as  well, by  simply  including  ^#  at  the  end  of  the

expression. For the last example, if it was desired to store that

value as  stored direction  number one, then the expression would

be 4012304-807^1.  More than one angle can be added to a bearing.

The only  limitation is  that a  maximum  of  30  characters  are

permitted in response to BASIS bearing/azimuth prompts.

Angles  can   also  be   added   or   subtracted   from   defined

bearings/azimuths. For  example 100*101+302120  would result in a

adding 30��21'20"  to the defined bearing from point 100 to point

101.

Section 5 - Interactive Coordinate Geometry

-30-

To facilitate  the entry  of bearings  that are  90, 180, and 270

degrees apart  from a  previously stored,  or defined  bearing, a

quadrant indicator  of .1,.2,and  .3 can  be  used  respectively.

(This will  not work with explicit bearings...BASIS will read the

decimal and number as fractional seconds.)

For example,  to produce  a bearing  that is 90 degrees clockwise

from the  bearing between  points 25  and  26,  enter  25*26.1  .

Similarly, a  bearing 270 degrees clockwise from a bearing stored

as bearing  number 7  is entered  as 7.3 in response to a bearing

prompt. These  can also  be stored  by appending ^# at the end of

the expression.

5.01.02 ENTERING DISTANCES - Distances can be entered explicitly,

that is,  by entering  the actual  distance,  or  implicitly,  by

entering two  known coordinates  that  define  a  distance.  This

version  of   BASIS  has  no  provision  for  storing  distances.

Arithmetic is also permitted.

For example,  to enter  a distance  of 100.55  feet, simply  type

100.55 in response to a distance prompt.

To enter a distance defined by points numbered 301 and 302, enter

301*302 in response to a distance prompt.

If two  partial distances  are to  be added,  such  as  in  taped

measurements, enter the two distances separated by a '+' sign. To

enter  the  sum  of  50.13  and  35.87  as  one  distance,  enter

50.13+35.87  in  response  to  a  distance  prompt.  Subtraction,

multiplication and  division are  also supported.  This is useful

for setting  the midpoint  of a line. To enter a distance halfway

between points   301  and 302, enter 301*302/2 in response to the

distance prompt.

There is  one catch  to using multiplication. Suppose you want to

traverse across  the frontage  of ten lots that are 50 feet wide.

If you  enter 10*50,  BASIS will think that you are trying to use

the defined  distance between  point 10  and point  50. This will

probably not  be the  distance that  you intend.  If one of these

points is  unused, BASIS  will display  an error  message. If you

want to  force the  multiplication, you  must include  a  decimal

point in  at least  one of  the multipliers,  such as  10*50.  or

10.*50  This tells BASIS that you want to perform arithmetic, and

not use a defined distance.

5.02 THE COGO COMMAND LINE - This section will explain how to use

the BASIS  Cogo command  line, which  is the  primary data  entry

mechanism of  Interactive Cogo.  BASIS allows the user to build a

"command" sequence  by responding  to prompts and pressing ENTER.

When all  information is  entered, the  final keystroke is always

Section 5 - Interactive Coordinate Geometry

-31-

the backslash character, or '\', which tells BASIS to process the

built-up command.

5.02.01 SINGLE-KEYSTROKE  COMMANDS AND  COMMAND PREFIXES  - As we

begin,  let's  examine  a  few  of  the  available  function  key

utilities, and the command prefixes.

Press F10,  to bring  up the command index. The index is shown in

Figure 5.1.

Index Of Commands

F1 - Printer Toggle            ^F1 - Turnarounds

F2 - Point Protect Toggle      ^F2 - Orientation

F3 - Clear Command Line        ^F3 - Regression

F4 - Area/Predetermined Area   ^F4 - Radial Layout

F5 - List Unused Points        ^F5 - Calculator

F6 - Coordinate Transformation ^F6 - Station/Offset

F7 - EDM Slope Reduction       ^F7 - Point Search

F8 - Directions to Angles      ^F8 - Right-of-Way

F9 - DOS Command Processor     ^F9 - R.O.W. Intersections

F10 - Command Index

Command Prefixes

$ - Comment                     P - Solve P.I. Curve

S - Store Coordinate            L - List Coordinates

I - Curve Inverse               T - Three-point Curve

.......R - Solve P.C. Curve            O - Tangent Lines

ESC - E X I T

Figure 5.1

Many of  these functions  will be  discussed in  detail  shortly,

however a few will be described here.

First, function key one, (F1) toggles the printer selection on or

off. The  default is  off. If  hardcopy of  the  cogo  output  is

desired, press  F1 once.  Notice the screen now reflects "PRINTER

ON" above the command line. To turn hardcopy off, press F1 again.

If the  printer is  not selected  when you try to toggle it on, a

message box  will appear  to inform  you that  the printer is not

ready. You  can press 'I' to ignore the warning, or 'R' to retry.

If you  choose ignore,  then the  program will cancel the call to

toggle the printer on.

If you  enable the  printer successfully, but somehow deselect it

afterwards, a  built-in critical  error handler  will detect  the

off-line condition when you attempt to print. Cogo will appear to

hang while  the program attempts to establish communications with

the printer.  It may  take a  minute or  two, until  the computer

gives up and displays a message box allowing you to Abort, Retry,

Ignore or Fail.

Section 5 - Interactive Coordinate Geometry

-32-

Even though  the error handler gives you four choices, you really

only have  two that are effective. You can either get the printer

back on-line  and press  'R' for  retry, or  press 'A' for abort,

which will terminate the cogo program.

Function key  two (F2)  is the  point protect toggle. The initial

state of  this toggle  depends on what was selected in the System

Defaults program.  To  change  the  current  setting,  press  F2.

Pressing the key will toggle, or switch the point protect mode on

then off,  then on,  etc.  The  status  of  point  protection  is

displayed near  the printer  status, above the command line. With

point protect  'ON', a  warning will  be issued  whenever a  used

point is  about to be overwritten. The user can enter a different

point number,if  desired, or  simply press ENTER to overwrite the

point.

Function key  three (F3)  clears the  current command  line,  and

resets the data entry sequence. This key can be used if a mistake

is made, and the user wants to start building a command again.

The '$' prefix is used if the user wishes to enter a comment.

For example,  at the  FROM POINT? prompt, type $ and press ENTER.

The prompt will change to COMMENT, and the user can type in a one

line comment.  Hit the  backslash key  '\' when  the  comment  is

entered. It  will appear to scroll up onto the screen, and if the

printer is on and selected, it will be printed out.

The 'S' prefix is typed if it is desired to store a coordinate.

To enter  a coordinate,  such as a point of beginning, type S and

press ENTER  at the  FROM POINT? prompt.The prompt will change to

NORTHING?. Enter  the northing  and press  ENTER. Respond  to the

EASTING? prompt  by entering the easting and pressing ENTER. Cogo

now prompts  for POINT NO?. Enter the point number, only now that

the command  is complete,  type the  backslash character  '\'  to

process the command.

Pressing ENTER  here would  do no harm, however, it would have no

effect. The  backslash must  be typed  to process the command. In

fact, BASIS  has a pretty good idea when you should be typing the

backslash, and  will display  a reminder  if it has no additional

prompts for data.

5.02.02 BUILDING A COGO COMMAND - Probably the first operation to

be performed  with a  new project file is the entry of a starting

coordinate.

The first  prompt that  always appears  for a new command line is

FROM POINT?.  To store  a coordinate,  type S,  and following the

procedure described  in the  above paragraph,  enter  a  starting

coordinate, such  as 5000 north, and 5000 east. After hitting the

backslash character, the entered coordinates will scroll into the

Section 5 - Interactive Coordinate Geometry

-33-

display portion  of the  screen, and be printed if the printer is

on and  hardcopy is  selected. The  command  line  then  restores

itself to the FROM POINT? prompt and awaits a new command.

5.02.03 TRAVERSING  INTO A  NEW POINT  - The next operation after

storing a  coordinate might be traversing to a new point from the

starting point  entered above.  Respond to  the  FROM  POINT?  by

entering the point number you wish to start the next course from.

Simply pressing ENTER will default to the last point computed.

For our  example, simply press ENTER, as we wish to traverse from

the last point which was stored.

The next  prompt that  will appear  is the  BEARING?  prompt  (or

AZIMUTH? prompt,  if that  directional mode  was selected  in the

System Defaults  program). Respond  by entering  a bearing, using

the conventions  described in section 5.01.01. Since there are no

other points  in this new file, and no bearings have been stored,

it is  necessary to  enter an  explicit bearing, such as 1453030,

for N 45�30'30" E, and press ENTER.

The next prompt requests a distance. Following the conventions of

section 5.01.02, enter an explicit distance, say 100.55 feet, and

press ENTER.

Cogo is now prompting TO POINT?, and needs the point number to be

assigned to  the newly computed coordinate. Enter a number within

the limits of the filesize, say '2', and press the BACKSLASH.

Pressing ENTER  instead of  BACKSLASH will  have no effect. BASIS

knows from the data that you have entered that you have entered a

complete command.  If you do not press BACKSLASH, a reminder will

appear.

To summarize  building a  traverse command  line, using the above

example, the sequence is as follows:

1 (ENTER) 1453030 (ENTER) 100.55 (ENTER) 2(BACKSLASH)

You'll notice  that once  a command is processed, and the results

scroll into  the display  area of  the screen,  the command  line

resets itself to the FROM POINT? prompt.

5.02.04 INVERSING  BETWEEN TWO  KNOWN POINTS - To inverse between

two known  points, enter the first known point in response to the

FROM POINT?  prompt, and  press ENTER.  The next  prompt requests

BEARING? Normally, to traverse a new point in, a bearing would be

entered here.  To INVERSE, however, simply enter the second known

point, and  press BACKSLASH. The bearing and distance between the

two entered points will scroll into the display window.

In this  case, pressing  ENTER rather  than BACKSLASH  after  the

second point  is typed  will not work. BASIS can't be sure if you

are entering  a direction  or a  point, so  it will not display a

Section 5 - Interactive Coordinate Geometry

-34-

reminder. Nothing bad happens if you do accidentally press ENTER.

BASIS will  simply prompt  for a  distance. You  can still  press

BACKSLASH to compute the inverse.

You must  press BACKSLASH to let BASIS know that you have entered

a complete  command. When  it  examines  this  command,  it  will

realize that you want to inverse between the two numbers.

Inversing between  two known  points, say points numbered 1 and 2

is accomplished as follows:

1 (ENTER) 2 (BACKSLASH)

A series  of "consecutive  inverses" can  also be  computed.  For

example, suppose it is desired to inverse 1 to 2, 2 to 3, 3 to 4,

etc., and  end at  19 to  20. Simply enter the first point number

and the final point number as a negative, as follows:

1 (ENTER) -20 (BACKSLASH)

You can also perform a consecutive inverse in reverse order, such

as

20 (ENTER) -1 (BACKSLASH)

5.02.05  BEARING   -  BEARING  INTERSECTION  -  A  frequent  Cogo

computation involves  computing the coordinates of a new point by

intersecting two  known bearings  through two other known points.

Of course, these bearings can not be parallel to each other!

In the  above example  of traverse,  two points,  1 and  2,  were

computed. As  a new  example, compute the coordinates of a point,

to be  numbered as  point  number  3,  which  is  formed  by  the

intersection of a line having a bearing of S 14�52'10" W, through

point 2, and a line bearing S 34�14'20" E through point 1.

The procedure  for building  this command line is very similar to

that for  the traverse,  only when  the prompts  for the  unknown

elements appear, in this case, distances, simply press ENTER.

The first prompt, always FROM POINT?, is answered by entering one

of the known points. Enter 2, and press ENTER.

In response  to the  following BEARING?  prompt, enter  the known

bearing through that point, 3145210, and press ENTER.

Bypass the  ensuing DISTANCE?  prompt by  pressing ENTER  with no

value entered.

Type 3 in response to the SOLVE POINT? prompt, and press ENTER.

A second  BEARING? prompt  will appear.  Respond by  entering the

other known bearing, 2341420, and press ENTER.

Section 5 - Interactive Coordinate Geometry

-35-

Bypass the second DISTANCE? prompt simply by pressing ENTER.

Finally, enter  the second  known point,  1, in  response to  the

final TO  POINT? prompt  and  press  BACKSLASH,  since  the  cogo

command is completed.

The coordinates  of the  unknown point  will be computed, and the

results of the computation will scroll into the display window.

To summarize, the command was built up like this:

1 (ENTER) 3145210 (ENTER) (ENTER) 3

(ENTER) 2341420 (ENTER) (ENTER) 2 (BACKSLASH)

5.02.06 BEARING - DISTANCE INTERSECTION - Another frequently used

Cogo computation  involves computing  the coordinates  of  a  new

point formed  by the  intersection of  a known  bearing through a

point, with  a known  distance from  another point.  This is also

called the line-circle intersection.

Experienced surveyors  know that  unless a  line is  tangent to a

circle, if it intersects the circle, it intersects at two points.

These two  solutions  are  often  called  the  "near"  and  "far"

solutions. The "near" solution is the intersection closest to the

initial point and the "far" solution is the farthest.

When the  program requests  the  SOLVE  POINT?,  and  the  "near"

solution is  desired, enter  the solve point number preceded by a

minus sign,  such as -3. If, the "far" solution is desired, enter

the point number normally, without a minus sign.

Again using  points numbered  1 and  2, which  have already  been

computed, compute  the coordinates  of a point, to be numbered as

4, formed by intersecting a line bearing N 26�13'20" E from point

1 and  a distance of 75.00 feet from point 2. The "near" solution

is the desired solution.

Starting at  the FROM  POINT? prompt, enter the first known point

number, 1, and press ENTER.

At the  BEARING? prompt,  enter the  known bearing  through  this

point, 1261320, and press ENTER.

Bypass the first DISTANCE? prompt by pressing ENTER.

Enter the  SOLVE  POINT?  number  as  -4  (Remember,  the  "near"

solution) and press ENTER.

Since the next BEARING? is unknown, simply press ENTER.

Respond to  the next  prompt, DISTANCE?,  by entering  the  known

distance, 75, and pressing ENTER.

Section 5 - Interactive Coordinate Geometry

-36-

Finally,  enter   the  second  known  point,  2,  and  press  the

BACKSLASH. The  results of  the computation  will scroll onto the

screen.

The procedure is summarized as follows:

1 (ENTER) 1261320 (ENTER) (ENTER)

-4 (ENTER) (ENTER) 75 (ENTER) 2 (BACKSLASH)

These intersections  do not always have to be computed as bearing

-distance intersections.  Distance-bearing intersections are also

allowed. Also, it does not matter if the bearing runs opposite to

what is  given as  input... the program will detect and correct a

reversed bearing in the output.

5.02.07 DISTANCE  - DISTANCE  INTERSECTIONS -  The  third  common

intersection problem  involves intersecting  two known  distances

from two  points. The  sum of  the two  distances must be greater

than or  equal to the distance between the two points. If the sum

is equal  to the  distance between  the points, there is only one

solution, otherwise, there are two solutions.

The BASIS  Cogo convention  is  as  follows.  To  solve  for  the

"clockwise" solution,  enter the  SOLVE POINT?  as a normal point

number,  with   no  preceding  sign.  If  the  "counterclockwise"

solution is desired, enter the SOLVE POINT? as a negative.

For example,  suppose the  coordinates of a point, to be numbered

as 5,  are formed  by intersecting  a distance of 64.50 feet from

known point  2, with a distance of 71.22 feet from known point 1.

The "counterclockwise" solution is desired.

Respond to  the initial  FROM POINT?  prompt by  entering 2,  and

press ENTER.

Bypass BEARING?  by pressing  return, since  the bearing  is  not

known.

Type 64.50  in response  to the first DISTANCE? prompt, and press

ENTER.

Enter the SOLVE POINT? as -5, since the counterclockwise solution

is desired.

Bypass the next BEARING? prompt, as it is also unknown.

Respond to  the second  DISTANCE? prompt   by  entering 71.22 and

pressing ENTER.

Type 1,  followed by  the BACKSLASH  in response  to the final TO

POINT? prompt.  The results  of the  computation will scroll onto

the screen.

Section 5 - Interactive Coordinate Geometry

-37-

To repeat, the command was entered as follows:

2 (ENTER) (ENTER) 64.5 (ENTER)

-5 (ENTER) (ENTER) 71.22 (ENTER) 1 (BACKSLASH)

5.02.08 P.C.  CURVE SOLUTION  - If the P.C. and radius point of a

curve are known, the P.T can be solved from the arc length, chord

length or central angle.

To call  this routine, type the command prefix R at a FROM POINT?

prompt, and press ENTER.

A prompt  requesting the  point number  of the  P.C. will appear.

Enter the P.C. point number, and press ENTER.

Next, the  program will  request the  point number  for the P.T.,

which is  the point  being solved.  Enter the  point number to be

assigned to the new point, and press ENTER.

Next, a  prompt appears,  requesting the radius point. The radius

point  is   entered  with   a  negative  sign  if  the  curve  is

counterclockwise, or  to the  left. If the curve is clockwise, or

to the  right, the  radius point is entered as a normal, positive

number (no positive sign is needed).

Finally,  a  prompt  appears,  requesting  either  a  "Delta"  or

interior angle,  or a "Chord" or long chord distance, or an "Arc"

or arc  length. Enter whichever is known, preceding the data with

the appropriate code letter (D,C, or A, respectively).

For example,  suppose it is desired to compute a point on a curve

whose radius  is point  number 4,  and whose P.C. is point number

2.(These points  are convenient  to  use  since  they  have  been

computed in  a previous  example.). Starting  from point 2, it is

desired to  advance  counterclockwise  along  the  curve  an  arc

distance of  50.00 feet,  and solve  a new  point numbered 6. The

command sequence is as follows:

R (ENTER) 2 (ENTER) 6 (ENTER) -4 (ENTER) A50 (ENTER)

The newly  computed curve  and curve  data will  scroll into  the

display window.

5.02.09 SETTING  MULTIPLE POINTS  ON A  LINE OR  CURVE - A useful

feature of  BASIS Cogo is the ability to set multiple points on a

line or  curve when  using  a  traverse  or  P.C.  curve  command

sequences. For  example, suppose  five points  must be set, 50.00

feet apart,  starting from known point numbered 1, along the same

bearing from  point 1  to point 2. Start numbering the new points

with 10. The command line is very simple:

Section 5 - Interactive Coordinate Geometry

-38-

1 (ENTER) 1*2 (ENTER) 50 (ENTER) 10.5 (BACKSLASH)

There is  little difference  from  the  normal  traverse  command

sequence, only  a ".5"  has been  tacked onto  the  SOLVE  POINT?

response. If  30 new  points were  needed, then  10.30 would have

been the response.

The same  feature can  be applied  to the  P.C. Curve  (R prefix)

function. When  the P.T.  point number  is requested, including a

fractional part  tells the system to compute multiple points. For

example, using  the  same  curve  from  the  example  in  section

5.02.08, suppose  it is  desired to set five points on the curve,

separated by  chords of  10.00 feet, and starting assigning point

numbers to  the new points at point 7. The command sequence is as

follows:

R (ENTER) 2 (ENTER) 7.5 (ENTER) -4 (ENTER) C10 (BACKSLASH)

The computed curve data will scroll into the display window.

This feature,  combined with  the ability to divide distances and

angles, can  be very  useful for  dividing lines  and curves into

equal segments.

5.02.10 CURVE  INVERSE -  Once a  curve is  computed,  the  curve

information can  be displayed  anytime by using the Curve Inverse

routine ( I prefix ).

To call  this routine, type I in response to a FROM POINT? prompt

and press ENTER.

The next  series of  prompts will  request the  P.C.,  P.T.,  and

Radius Point,  respectively. Enter  the point numbers that define

the curve.  Remember,  to  inverse  counterclockwise,  enter  the

radius point  number as  a negative.  The  command  sequence  for

inversing the curve computed in section 5.02.08 would look like

I (ENTER) 2 (ENTER) 6 (ENTER) -4 (BACKSLASH)

The curve information will scroll into the display window.

5.02.11 P.I. CURVE - The P.I. curve option is invoked by typing P

in response  to the  FROM POINT?  prompt. It  is used  to solve a

curve having  a known  P.I. and  tangent directions, and either a

radius or tangent length.

The first prompt requests the P.I. point number. Next the bearing

to the  P.C. is  requested. This  can be an explicit, defined, or

stored bearing.  Then the bearing to the P.T. is requested. BASIS

will then  request a  initial solve  point number. Then, either a

tangent distance, or radius length must be entered, preceded by a

T, or an R, respectively.

Section 5 - Interactive Coordinate Geometry

-39-

At this  point, the user can press the Backslash key, and process

the curve.

However, pressing  ENTER, rather  than BACKSLASH, will invoke one

more prompt,  for WIDTH? Here, the user can supply the width of a

right-of-way, and  BASIS will  compute  the  inside  and  outside

curves,  assuming  the  line  curve  entered  previously  is  the

centerline.

For a  single line curve (no width), BASIS will compute the P.C.,

R.P., and  P.T., in  that order, using the initial solve point as

the P.C.  If a  width is  entered, then  additional point numbers

will be  assigned to  the inside  P.C. and P.T., and outside P.C.

and P.T., in that order.

Suppose a  P.I. is  established as point number 25. The direction

to the  P.C. will  be on  the line  to point  number 24,  and the

direction to  the P.T.  will be  on the  line to point number 26.

Start assigning  points with  point  number  50.  The  centerline

radius is  to be 275.00 feet, and the Right-of-Way width is 60.00

feet. The command sequence is as follows:

P (ENTER) 25 (ENTER) 25*24 (ENTER)

25*26 (ENTER) 50 (ENTER) R275 (ENTER) 60 (BACKSLASH)

5.02.12 LIST  COORDINATES -  The command  prefix L can be used to

provide a  listing of  coordinates for a specified range of point

numbers. Type  L in  response to  a FROM  POINT? prompt. The next

prompt will request the starting point number of the listing, and

the  next  prompt  will  request  the  ending  point.  Enter  the

appropriate values, terminating entry with a Backslash character.

The coordinates  will scroll  into the display window, one screen

"page" at a time.

For example,  to list coordinates from point 1 through point 100,

the command sequence is as follows:

L (ENTER) 1 (ENTER) 100 (BACKSLASH)

5.02.13 THREE  POINT CURVE  - A  curve can be fitted to any three

points, provided they are not on a straight line. This routine is

called by  typing T  in response  to a  FROM POINT?  prompt,  and

entering the  point numbers  of the  three known  points, and the

point number to be assigned to the radius point.

For example,  compute the  curve that passes through points 1, 2,

and 3, and store the radius point as point number 11. The command

line looks like this:

T (ENTER) 1 (ENTER) 2 (ENTER) 3 (ENTER) 11 (BACKSLASH)

Section 5 - Interactive Coordinate Geometry

-40-

5.02.14 LIST  UNUSED POINTS - The unused point listing is invoked

by pressing  function key  5. This  routine can  be called at any

time, while  entering any  command. It  is  non-destructive,  and

returns to the Cogo program exactly where it left off.

Press function  key 5  (F5). A  window will pop-up on the screen,

requesting the  starting and  ending point numbers for the unused

point listing.  Respond by  entering the  desired range, pressing

ENTER after  each entry.  By default,  the  starting  and  ending

points are zero and the highest point, respectively.

The unused  point numbers will fill the window, one window "page"

at a time.

Pressing the  ESC key at any time will terminate the unused point

listing.

5.02.15 AREA  - Pressing  function key four (F4) will call up the

point-to-point area routine. To return to Cogo from this routine,

press the ESC key.

The first prompt to appear requests the starting point number for

the figure.  Enter the  starting  point  number,  and  press  the

BACKSLASH or  ENTER key.  Continue entering point numbers in this

fashion, inversing  around the figure. To compute the area, close

into the starting point number.

Handling curves  are no problem. If the next point is a P.C. (not

necessarily tangent, just the beginning of a curve) enter it as a

negative, and press BACKSLASH. A prompt for the P.T. will appear.

If the  P.T. is  also the  beginning of  another curve  such as a

P.C.C or  P.R.C., then it also is entered as a negative. The next

prompt  requests  the  radius  point  number.  If  the  curve  is

counterclockwise, then enter the radius as a negative number.

Note that  when calculating  the area,  either the  BACKSLASH  or

ENTER can be used to enter a point number.

To return to Interactive Cogo from the Area function at any time,

press the ESC key.

5.02.16 COORDINATE  TRANSFORMATION -  This utility  is invoked by

pressing function key six (F6). It will request data that enables

the rotation  and translation  of a block of coordinates, defined

by  the   user.(Note:  Scaling   is   not   performed   by   this

transformation).

The first  point requested  is  the  rotation  point.  Enter  the

rotation point  number and  press ENTER. The file coordinates for

that point  will be  displayed, and  the program will request new

coordinates. The  defaults for  the new  coordinates are  the old

Section 5 - Interactive Coordinate Geometry

-41-

coordinates. To accept the defaults ( i.e. rotation only ) simply

press ENTER  for each, otherwise enter the new coordinate values,

followed by ENTER.

Next, a  point number  is requested  that will  define  an  "old"

bearing from  the rotation point. Enter a point number, and press

ENTER.

The new  , rotated  bearing of  this line is then requested. This

can be entered as an explicit, defined, or stored bearing, and it

determines the rotation angle of the transformation. Press ENTER.

Finally, a  range of points are requested. Enter the starting and

ending points  of the  series, and press ENTER after each. Review

the data  in the  window, and respond to the ALL OK (Y/N)? prompt

by typing  Y or  N (upper or lowercase). The selected points will

be transformed based on the data that was input.

Pressing ESC  at any  time, will  abort the  routine  and  return

control to the Cogo program.

5.02.17 EDM  SLOPE REDUCTION  - This  is a non-destructive pop-up

utility  that   allows  the   user  to   reduce  slope   distance

measurements. It  is activated  by pressing  function  key  seven

(F7). You can return to cogo at any time by pressing ESC.

The first  prompt  requests  the  user  to  enter  the  units  of

measurement for  the EDM,  either feet  or  meters.  The  default

response is  whatever distance  units  were  set  in  the  System

Defaults program.  Accept the default by pressing ENTER, or press

'F' for  feet or  'M' for  meters.  (When  the  routine  computes

horizontal and vertical distances, they will be displayed in both

feet and meters.)

The current default for curvature and refraction corrections will

be displayed in the upper right corner of the window. If you need

to change  this setting,  you must do so from the System Defaults

program.

Next, ZENITH  ANGLE? requests the measured zenith angles. As many

as eight  repititions of  zenith angles  can be  entered,  either

direct, or  inverted, and  an average  will be  taken. The  angle

format is  DDMMSS.SSS... Press ENTER after each angle is entered.

After typing the last zenith angle, press ENTER twice.

If an  error is made while entering data, and cannot be corrected

using BACKSPACE,  pressing function key three (F3) will reset the

routine.

Next, SLOPE  DISTANCE? prompts  for the  measured slope distance.

Again, up  to eight  repititions of  a measurement can be entered

and averaged.  Enter each  measurement, and  press  ENTER.  After

typing the last distance measurement, press ENTER twice.

Section 5 - Interactive Coordinate Geometry

-42-

The horizontal  and vertical  distances will  be displayed. Press

any key  to continue  entering data,  or press  ESC to  return to

cogo.

It is  possible to  enter distances  in alternate  units to those

selected. For example, some surveyors measure EDM distances first

in feet,  and then  a second  time in  meters for a check. If you

selected FEET as the input distance units, you can enter a metric

distance as  a negative  number. The  program will  convert it to

feet. If  you selected METERS as the input units, you would enter

a distance measured in feet as a negative number.

The program  will display  the mean  of  the  zenith  angles  and

distances. It  will also  perform some  simple range checking for

gross errors and warn you if it detects bad data.

5.02.18 DIRECTIONS  TO ANGLES  - This is a non-destructive pop-up

utility that  allows the  user to  reduce direction readings made

with a  theodolite to angles. It is accessed by pressing function

key eight (F8).

The format  for entering  angles is  DDDMMSS.SSS..., and  leading

zeros need  not be  entered. Trailing  zeros,  however,  must  be

entered.

The first  prompt  requests  the  BACKSIGHT  CIRCLE?.  Enter  the

initial backsight reading, and press ENTER.

Next the  FORESIGHT CIRCLE?  is requested.  Enter  the  foresight

circle reading and press ENTER.

FORESIGHT CIRCLE?  appears again.  Enter the  inverted  foresight

circle reading and press ENTER.

Finally BACKSIGHT  CIRCLE?  appears  again.  Enter  the  inverted

backsight circle reading, and press ENTER.

The average angle-right, from the backsight to the foresight will

be displayed.  A warning  will be  displayed if  the  direct  and

inverted angles  differ by  more than  30 seconds. Hit any key to

continue with more data, or press ESC to return to cogo.

If  you  make  a  typing  mistake  and  can't  correct  it  using

BACKSPACE, press  F3 to  reset the  routine.  You  can  exit  the

program any time by pressing ESC.

5.02.19 PREDETERMINED  AREA PARTITIONS  - Parcels  of a  required

area can  be computed  in two  ways. The first method is known as

the "Hinge"  method, and  the second  is  known  as  the  "Three-

Bearing" method.

Section 5 - Interactive Coordinate Geometry

-43-

Both methods  are accessed  by pressing  function key  four (F4),

which is  the normal  area-computation program.  However,  rather

than enter  all the  points of  a closed  figure, the  last known

point is  entered with  .1 for the "Hinge" method, or with .2 for

the "Three-Bearing" method.

HINGE AREA  - Figure  5.2 is  an example  of  a  typical  "Hinge"

problem.

Begin the  area sequence  with the  hinge point.  In the example,

this is  point number  one (1). Enter subsequent point numbers in

the normal fashion, pressing ENTER or BACKSLASH after each.

However at the last known point, point number five (5), enter 5.1

and press  ENTER or  BACKSLASH. The  .1 tells the computer that a

"Hinge" area  solution is desired. The current area of the figure

is displayed, and several prompts will appear.

The first  prompt requests  the known  bearing or  azimuth of the

line from  the last  known point.  The second prompt requests the

point number  for the  point to  be  solved.  Finally,  the  area

desired (in  square feet, or square meters if in metric units) is

requested. Note that this is the total area required, and not the

difference from  the partial  area displayed on the screen. Press

ENTER or  BACKSLASH after  responding to each prompt. The bearing

can be  entered in  any allowable cogo format (explicit, defined,

stored, etc.).  If a  solution  is  possible,  the  newly  solved

courses will  scroll onto the screen, and the new total area will

be displayed.

THREE-BEARING AREA  - Figure  5.3 illustrates  a typical  "Three-

Bearing" area problem.

Select a  point to  start  from  and  proceed  around  the  known

portions of  the figure  in normal  fashion,  pressing  ENTER  or

BACKSLASH after  each point  number entered. For the sake of this

example, assume that point entry began with point number one (1).

At the last known point, point number four (4), enter 4.2. The .2

tells the  system that a "Three-Bearing" solution is desired. The

current area of the figure will be displayed, and several prompts

must be answered.

The first  three of these prompts will request the three bearings

to close  the figure.  These can  be  entered  in  any  allowable

format, but must be entered in their proper order of occurence.

Next, the point number of the first of two points to be solved is

requested. The  second solve  point number will be  automatically

numbered one higher than the first. The routine will not accept a

point  number  that  will  result  in  over-running  the  maximum

filesize.

Section 5 - Interactive Coordinate Geometry

-44-

(Intentionally Left Blank...Substitute Figures 5.2 and 5.3)

Section 5 - Interactive Coordinate Geometry

-45-

Finally, the desired area (in square feet, or square meters if in

metric mode)  will be requested. The responses to all prompts are

followed by  a BACKSLASH or ENTER. The solved courses will scroll

into the screen, and the solved area will be displayed.

As with  any Cogo function, the results of the above calculations

will be  printed on  the printer  if it is toggled ON. Used point

warnings will also be active, if point-protect is toggled ON.

To return to interactive Cogo at any time, simply press ESC.

5.02.20 TURNAROUNDS  - Five different types of turnarounds can be

processed automatically  by pressing  SHIFT and  function key one

(F1) simultaneously.  A pop-up  menu  will  appear  on  the  Cogo

display from which the desired type can be chosen. The Turnaround

menu is shown in figure 5.4.

TURNAROUNDS

F1  - Symmetric

F2  - Offset, Off Tangent - CW

F3  - Offset, Off Tangent - CCW

F4  - Offset, Off Curve - CW

F5  - Offset, Off Curve - CCW

Figure 5.4 - The Turnaround Menu

These different  turnarounds are  shown in  figure figure 5.5a-e,

and can  be selected  by pressing  the associated  function  key,

using the  cursor control  keys to  position the highlight on the

desired type,  and pressing ENTER, or by clicking with the mouse.

Pressing ESC  returns control  to the  Cogo screen.  A dialog box

will appear  when a  turnaround is  selected that will prompt for

the turnaround parameters.

The first  option is the symmetric turnaround, as shown in figure

5.5a. To  solve this  turnaround, either the center of the circle

or the centerline P.C. must be known, as well as the bearing from

the center  to the  P.C. or  the bearing  from the  P.C.  to  the

center, respectively. The bearing can be entered in any allowable

Cogo format (defined, explicit, stored, etc.).

Other required  information includes  the turnaround  radius, the

return radius,  the right-of-way  width, and the optional tangent

width. Some designers prefer a tangent between the return and the

turnaround, while  others may not. To solve with a tangent, enter

the desired  tangent length  (the default  is 20  feet) otherwise

enter zero (0) if a tangent is not desired. Other defaults are 50

feet for  the turnaround  radius, 30  feet for the return radius,

and 50  feet for  the right-of-way width. To accept the defaults,

simply press  ENTER at  each prompt,  otherwise enter the desired

value and press ENTER.

Section 5 - Interactive Coordinate Geometry

-46-

(Intentionally Left Blank...Substitute Figure 5.5)

Section 5 - Interactive Coordinate Geometry

-47-

The point  numbers of  the turnaround will be solved sequentially

as indicated  in figure  5.5a and proceed counterclockwise around

the circle.  If a  tangent length  of zero  (0) is  entered,  the

additional tangent points will be omitted.

The second  type of turnaround is the offset turnaround, off of a

tangent, or  straight right-of-way,  with the  turnaround  radius

curving clockwise from the P.C.

Required information  includes either  a center  point, or a P.C.

point, along with either a bearing from center to P.C. or P.C. to

center, respectively.  Note that  the P.C.  is not the centerline

P.C., as  with the  symmetric turnaround,  but rather  the actual

P.C. or  beginning  point  of  the  turnaround  curvature.  Other

information required  is similar to the symmetric turnaround. The

solution provides  a sequential  series of point numbers as shown

in figure 5.5b.

The third type of turnaround is the same as the above, except for

turnarounds that  curve counterclockwise  from the P.C. This type

is shown in figure 5.5c.

A fourth  type of  turnaround that  can be  solved is  an  offset

turnaround off  a curved right-of-way, curving clockwise. This is

similar to  the offset turnaround off the tangent, in that either

the P.C. or center point must be known, however in this case, the

radius point  of the  right-of-way curve  must also be known. The

turnaround solved is shown in figure 5.5d.

Finally, a counterclockwise offset turnaround off a curved right-

off-way is  available, which  is similar  to the  type  described

above. This type is shown in figure 5.5e.

5.02.21 DOS  COMMAND PROCESSOR  - Pressing function key nine (F9)

permits the user to "shell out" to DOS, and perform any DOS level

activities. To return to Cogo, type EXIT while in DOS.

This is  a non-destructive  utility, and  returns to Cogo exactly

where it left off.

Because Interactive  Coordinate Geometry has grown into a "memory

hog", you will have a limited amount of RAM in this secondary DOS

session to  run applications. However, you should have no trouble

running any DOS level utilities or small programs.

5.02.22 ORIENTATION  - Pressing  SHIFT and  function key two (F2)

calls the least squares orientation routine. This function solves

for the  "best fit"  of a  polygon formed  by  observed  monument

coordinates to  a theoretical  polygon defined  by  deed  or  map

coordinates. It  will NOT  re-scale the observed coordinates, and

angles between observed coordinates will not be changed. In order

Section 5 - Interactive Coordinate Geometry

-48-

to satisfy  the least  squares condition,  at least  three points

must be known in both systems

When executed, a pop-up window will appear on the screen, and the

prompt Read from disk (Y/N)? will appear. If the routine is being

used for  the first  time, respond  by pressing N. Points entered

for solving  the orientation can be saved to disk and edited with

a text  editor,  or  word  processor.  The  disk-read  option  is

available for  calling a  previously saved orientation file which

may have  been edited,  or for  adding additional observations to

the file, and reprocessing the orientation.

The next  set  of  prompts  request  the  point  numbers  of  the

theoretical and   observed control point pairs. The "theoretical"

points are  the points  in the desired coordinate system, and the

"baseline" points are the corresponding points to be transformed.

Respond to  each prompt by entering the appropriate point number.

A weight  factor between one and ten can be entered for each pair

of  coordinates,   which  may   be  desirable   when  fitting  to

monumentation of  variable stability  or reliability.  Use higher

weights for more reliable monuments, and lower weights for lesser

points. The  default weight  is five.  To  obtain  an  unweighted

solution, simply assign the same weight to all points.

When all  common points  (a minimum  of three  pairs)  have  been

entered, simply  press  ENTER  in  response  to  a  "theoretical"

prompt.

A  prompt   will  then   appear,  asking   if  you  want  a  full

transformation (translation  and  rotation),  or  rotation  only.

Press '1'  for full  transformation or '2' for rotation only. The

default is  '1', for  full transformation.  You  can  accept  the

default by simply pressing ENTER.

If you  select "rotation  only", a  prompt will appear requesting

the point number of the control point to rotate around. Enter the

point number of the theoretical point. The program will "pin" the

polygons through  this control  point and  determine the rotation

angle that produces the best fit at the other control points.

The routine  will calculate  the transformation  parameters,  and

display the residuals and error vectors in the window. The UP and

DOWN arrow  keys, as  well as the PG UP and PG DN keys are active

at this  time and  can be  used to  scroll through the display of

residuals, if it exceeds the window size.

Excessively high values are indicative of errors, or possibly due

to attempting  to orient  to a  point  that  is  grossly  out  of

position. If  such is  the case, eliminate the suspected point(s)

by using  the space bar to highlight the line to be rejected, and

pressing CTRL  E. The line will then be printed in reverse-video,

and that  pair of  points will  be removed  from the control set.

Repeat this procedure to reject all outlying points. If you press

Section 5 - Interactive Coordinate Geometry

-49-

CTRL E a second time on a reject, it toggles the reject flag off,

and the line is restored to normal.

Press ENTER  to exit  the display  window. If any rejections were

selected, the control set will automatically be re-processed, and

the new results displayed on the screen.

Save to  disk (Y/N)? provides the opportunity to save the control

points (theoretical  and baseline) to a file, which can be edited

with a  text editor,  or added to later on when more observations

are made.  To save the orientation points, press Y. To bypass the

save, press N. If the file is saved, it will be saved to the same

data directory  as the  project file,  under the same diskname as

the project  file, except  that the  file extension  will be .LSO

(for Least  Squares Orientation). The format for this file is the

theoretical  point  number,  baseline  point  number  and  weight

factor, separated  by one or more spaces. The next pair starts on

a new line.

The next  prompt permits  editing the orientation file, providing

it exists  on disk. To skip the edit session, respond by pressing

N. To  proceed with editing, press Y.  The routine can invoke any

text editor  or word  processor that  has been  placed in the dos

environment using the "set editor" command.

For example, if a text editor named "ED" is the editor of choice,

then place the line

set editor=ed

in  the  AUTOEXEC.BAT  file  that  automatically  runs  when  the

computer is  turned on.  The BASIS  installation program may have

automatically placed  this line  in your AUTOEXEC.BAT file if you

permitted it.

If the  program is  unable to  find an  editor name  in  the  DOS

environment, it  will automatically  use BCEDIT,  the text editor

supplied with BASIS.

Once the  editing  session  is  completed,  and  the  editor/word

processor  is   exited,  program   control  will  return  to  the

orientation window.  (If a  word processor is being used, be sure

to specify  non-document, or  unformatted mode,  when saving  the

edited file  back to  disk.)   Upon returning  to the orientation

routine, a  prompt will  give the  option to reprocess the edited

file. Respond by pressing Y or N.

The next  prompt to  appear is  Transform Coordinates (Y/N)? This

allows the user to perform a general transformation to any point,

or series of points in the "baseline" system, to convert into the

theoretical system, based on parameters computed from the control

set.If the  standard deviations are acceptable, and it is desired

to transform  a series  of points,  press Y. Otherwise, to bypass

actual coordinate transformation, press N.

Section 5 - Interactive Coordinate Geometry

-50-

If transformation is selected, prompts will appear requesting the

starting and  ending point numbers for the series of points to be

transformed.  Unused   points  within   this  range  will  remain

unchanged.

Next, a prompt appears asking where you want to start storing the

transformed points.  You can either overwrite the old coordinates

with the  transformed points, or you can shift them to some other

location in  the project file, thus leaving the original block of

coordinates intact.  The default  response is  the  same  initial

point  number   as  the   transformation  block,   resulting   in

overwriting the  old coordinates  with the  new coordinates. If a

different point  is selected,  the program  will not  permit  the

blocks to overlap.

Finally, a  message requests  that you  press 'Y' to proceed with

the transfer. Press 'N' to cancel.

The orientation  procedure can be aborted at any time by pressing

the ESC key, which will return control to the cogo screen.

5.02.23 REGRESSION  - This  routine will fit a straight line to a

series of  points  by  linear  regression,  and  is  accessed  by

pressing SHIFT and function key three (F3) simultaneously.

A pop-up  window will  appear, requesting  the point  numbers  to

which the  line will be fitted. Enter the known point numbers. To

satisfy the  least squares  condition, a  minimum of three points

must be entered. A maximum of 50 points can be entered.

Once all  the points  are entered, simply press ENTER in response

to a  Point on line: prompt. The bearing or azimuth of the fitted

line, and  a point  on the  new line  will be displayed. You will

have the option to store both the direction and the point.

To store  the direction  of the  line, respond  to the  prompt by

pressing Y. Then enter a number between one and 10. The direction

will be  stored as  a conventional  stored bearing  (see  section

5.01.01).

To store  the coordinates  of the  point-on-line, respond  to the

prompt by  pressing Y,  and then  enter the desired point number.

The default point number is the highest point number in the file.

The regression  procedure can  be aborted at any time by pressing

the ESC  key, which  will return  control to the interactive cogo

screen.

5.02.24 RADIAL LAYOUT - Pressing SHIFT and function key four (F4)

executes the  radial stakeout  computation utility.  This routine

Section 5 - Interactive Coordinate Geometry

-51-

will compute  azimuth angles  (clockwise) and distances to points

from a station and backsight point.

The first prompt to appear allows the user to select hardcopy. If

hardcopy was  toggled on in Interactive Cogo, then the default is

Y. If  hardcopy was  off in  cogo, then  the default  will be  N.

Accept the  default by  pressing ENTER,  or press  Y or N to turn

hardcopy on or off.

The next  prompt requests  the point  number for  the  instrument

station. Enter  the point  number and press ENTER. Next enter the

point number  of the  backsight station,  and  press  ENTER.  The

occupied point  number,  the  backsight  point  number,  and  the

direction of  the backsight  will scroll into the display window.

If hardcopy  is selected,  this data  will also  be sent  to  the

printer.

The next  prompt will  request the  point to be staked. Enter the

point number,  and press  ENTER. The  point number, azimuth angle

from the  backsight, and  distance from the instrument point will

scroll into  the display  window. This prompt will be repeated to

permit entry  of a  series of  layout points until the program is

ended by pressing ESC.

The instrument  station and  backsight can be changed at any time

by pressing  function key  one (F1).  Prompts for  the instrument

station and  backsight point will appear, and after responding to

them, the program will resume requesting layout points.

To exit  the program  at any time, press ESC to return control to

Interactive Cogo.

5.02.25 STATION  AND OFFSET - Pressing SHIFT and function key six

(F6) while  in  Interactive  Cogo  activates  the  Station/Offset

routine. This  routine is similar to the radial stakeout routine,

only rather  than displaying  angles and  distances, stations and

left or right offset distances are displayed.

The first prompt to appear allows the user to select hardcopy. If

hardcopy was  toggled on in Interactive Cogo, then the default is

Y. If  hardcopy was  off in  cogo, then  the default  will be  N.

Accept the  default by  pressing ENTER,  or press  Y or N to turn

hardcopy on or off.

The next  prompt  requests  the  point  number  for  the  initial

station. Enter  the point number and press ENTER. If the baseline

is a  curve, enter  the PC  as a  negative number. Next enter the

point number  of a  station on  the line, and press ENTER. If the

baseline is  a curve, enter the Radius Point, positive for curves

to the right (CW), or negative for curves to the left (CCW).

Next, a  prompt appears  requesting  the  station  value  of  the

initial station.  The default  is 0  (for 0+00).  If the  initial

Section 5 - Interactive Coordinate Geometry

-52-

station has  some other  value,  it  may  be  entered  here.  For

example, if  the initial station is 12+25.35, then enter 1225.35.

Calculated stations  will be  referenced  to  this  station.  The

occupied point  number, the backsight point number, the direction

of the  backsight and  the initial station value will scroll into

the display  window. If hardcopy is selected, this data will also

be sent to the printer.

The next  prompt will  request the  point to be offset. Enter the

point number, and press ENTER. The point number, station and left

or right  offset will scroll into the display window. This prompt

will be  repeated to  permit entry  of a  series of offset points

until the program is ended by pressing ESC.

The initial  station and  P.O.L. (or Radius Point) can be changed

at any  time by  pressing function  key one (F1). The prompts for

the intial  station and  P.O.L. (or Radius Point) will appear and

after responding  to them,  the program  will  resume  requesting

offset points.

To exit  the program  at any time, press ESC to return control to

Interactive Cogo.

5.02.26 POINT SEARCH - Pressing SHIFT and function key seven (F7)

while in  Interactive Cogo  brings up the "Point Search" utility.

This routine,  given a center coordinate and radius, will display

a list of all points within the defined circle.

A pop-up window will appear, requesting a focus point number. You

can enter  either a  pont number,  or a northing and easting. The

focus point  is the  center point  of the  circular  area  to  be

searched.

To enter  a point  number, type  in the  point number  and  press

ENTER. To  enter a northing and easting, press ENTER at the point

number prompt without typing anything. Then type the northing and

easting in response to the prompts, pressing ENTER after each.

Next, two  prompts appear,  requesting the  maximum  and  minimum

search radii.  If you  enter a minimum radius of zero, all points

within the circle defined by the maximum radius will be reported.

If you  enter a minimum value, all points within the donut-shaped

area between the minimum and maximum radii will be displayed.

Finally, the starting and ending point numbers for the search are

entered. The  default responses are point number 0 for a starting

point, and the highest permissible point number as the end point,

i.e. the entire file will be searched. The portion of the file to

be searched  can be  limited by  entering different start and end

points.

When running  the POINT  SEARCH utility,  after finding  matching

points, the  program will continue requesting more search points,

Section 5 - Interactive Coordinate Geometry

-53-

rather than  immediately returning  to the  cogo command line. If

you want  to go  back to  the command  line,  simply  press  ESC.

Otherwise, enter  another  focus  point  and  search  data.  This

feature will  be greatly  appreciated when  you are  running this

handy utility for a series of focus points.

5.02.27 RIGHT-OF-WAY  COMPUTATION -  Pressing SHIFT  and function

key eight  (F8) brings  up the  right-of-way computation routine.

This routine  permits automatic  computation of parallel easement

or right-of-way  limits, referred to a known centerline or offset

baseline.

When activated,  a window  will appear  requesting the  following

information:

RIGHT OFFSET:  Enter the  parallel offset  distance for the right

side of the easement from the known baseline. A value of zero can

be entered for one-sided solutions.

LEFT OFFSET: Enter the parallel offset distance for the left side

of the  easement from  the known  baseline.  As  with  the  right

offset, a value of zero is used for one-sided easements.

Note that  "right" or  "left" is  based on an observer's right or

left looking  up station,  or from  the starting  baseline  point

towards the ending baseline point.

You can  effectively enter  two "left"  offsets by  entering  the

right as  a negative.  You may  also enter  two right  offsets by

entering the left as a negative.

STARTING DIRECTION:  Enter the direction of the starting terminus

of the  easement. Directions can be entered in any of the bearing

or azimuth formats permitted in Interactive Cogo.

ENDING DIRECTION:  Enter the  direction of the ending terminus of

the easement.

SOLVE POINT:  Enter the  point number  for the first point of the

solution. Subsequent  new points  will be  numbered  sequentially

from this  first point number in a clockwise direction.around the

easement. When entering this number, consider the final number of

solve points. If the filesize of the current project file will be

exceeded, an  error message  will appear, and control will return

to Interactive Cogo.

BASELINE POINT:  Enter centerline or baseline points in the order

that they  occur. These  points should  be angle  points  in  the

reference line, although the program will later detect and ignore

points on line if they are inadvertently entered. A maximum of 35

points may be entered.

Section 5 - Interactive Coordinate Geometry

-54-

(Intentionally Left Blank...Substitute Figure 5.6)

Section 5 - Interactive Coordinate Geometry

-55-

After each centerline point is entered, press RETURN. At the last

point, press RETURN twice.

Refer to figure 5.6a to see how these values relate to an example

problem.

As the  solution is computed, a traverse around the easement will

scroll onto  the  screen,  starting  and  closing  at  the  first

baseline point. If Point-Protect is on, the familiar point-in-use

warning will  appear on  the screen should the program attempt to

overwrite a used point.

There are  circumstances when  the left  or right offset distance

results in  a "butterfly"  or overlap  of the  computed  easement

lines. The  conditions that  lead to  a "butterfly" are generally

due to  offset distances  that are  two large  and  geometrically

inconsistent with  the length  and angular  relationship  of  the

centerline  or   baseline.  Since   this  is  a  relatively  rare

occurence, the  program will  not check  for butterflies. It will

compute parallel offset lines and their intersection points, even

if they overlap. See figure 5.6b.

5.02.28 TANGENT LINES - You can construct lines from a point that

are tangent  to a  circle, or  lines tangent to two circles using

this command.

Type the  letter 'O'  in response  to a FROM POINT? prompt on the

cogo command line.

The prompt  FIRST POINT?  will appear.  If you are constructing a

line from  a point  that is  tangent to a circle, enter the point

number of  the point.  If you  are constructing a line tangent to

two circles, enter the radius point of the first circle.

Next,  the   prompt  RADIUS(-  IF  LEFT)?  appears.  If  you  are

constructing a  tangent line  from a  point to  a circle,  simply

press ENTER  to skip this entry. However, if you are constructing

a tangent  to two  circles, enter  the radius length of the first

circle (corresponding  to the  first radius point entered above.)

If you  want the  tangent to  start on the left side of the first

circle (as  you face towards the second circle), enter the radius

as a negative number.

The prompt  SECOND POINT? appears next. Enter the radius point of

the circle to which the tangent will be constructed.

Then, RADIUS(-  IF LEFT)? appears. Enter the radius length of the

circle. If  you want  the tangent  drawn to  the left side of the

circle (as  viewed from  the first  point), enter the radius as a

negative number.

Finally, SOLVE POINT? appears. Enter the name of the point number

for storing  the coordinates of the point of tangency. If you are

Section 5 - Interactive Coordinate Geometry

-56-

solving for  a circle-circle tangent, two points will be computed

and stored  beginning with  this point number. As with any of the

cogo routines,  if the  points are  in use, a warning will appear

and you  will have the opportunity to overwrite or re-enter a new

point number.

5.02.29 RIGHT-OF-WAY  INTERSECTIONS  -  Pressing  SHIFT  F9  will

present a pop-up menu of intersection computations that BASIS can

automatically  perform.   These  correspond  to  bearing-bearing,

bearing-distance,  and   distance-distance  line   intersections,

except that  provisions are  made for  R.O.W.  width  and  return

radii. The  program computes  the centerline  intersection point,

and the  P.C., radius  point and P.T. for all returns. Points are

numbered sequentially,  starting with the solve-point number that

you specify.

Computations are  performed for  a four-way intersection. You can

disregard some  of the  points if  you are  only concerned with a

three-way intersection.

From the  menu, select  the intersection  type that  you want  to

calculate.  You   can  make   your  selection   by  pressing  the

corresponding function  key, or  by clicking  on your choice with

the mouse. You can also use the cursor movement keys to highlight

your choice, and then press ENTER.

The first  selection, F1,  presents a dialog box requesting known

data for  an intersection  of two straight rights-of-way. A point

on the  centerline of each right-of-way must be known, as well as

the directions.  You can enter the direction using any acceptable

BASIS format,  such as  explicit, defined,  stored, etc. You must

also enter the width for each right-of-way, which need not be the

same. You can enter the width using any BASIS distance format.

After providing  the right-of-way data, you must then provide the

return radius  for the  interection return  or fillet curves, and

the starting  point number  for the series of points that will be

computed.

The first  point computed  will be  the intersection point of the

centerlines. The  remaining points will be computed starting with

the P.C.  of the  first return  to the right of the first R.O.W.,

(as you  face the  intersection from the known point on the first

right-of-way) proceeding  sequentially around the intersection in

a counter-clockwise direction. Refer to Figure 5.7a.

The second  selection, F2, presents a dialog box requesting known

data for an intersection of a straight and curved right-of-way. A

point on  the centerline, and direction of the straight right-of-

way is required. You can enter the direction using any acceptable

BASIS format, such as explicit, defined, stored, etc.

Section 5 - Interactive Coordinate Geometry

-57-

(Intentionally Left Blank - Substitute Figure 5.7)

Section 5 - Interactive Coordinate Geometry

-58-

You must  also provide  the radius point for the curved right-of-

way, as  well as  its centerline radius. Enter the width for each

right-of-way using  any BASIS  distance format.  You may also use

any BASIS format for entering the centerline radius of the curved

right-of-way.

After providing  the right-of-way data, you must then provide the

return radius  for the  interection return  or fillet curves, and

the starting  point number  for the series of points that will be

computed. As  with bearing-distance  line intersections, you must

let the  program know if you want the near or far solution. Enter

the solve point as a negative number for the near solution, or as

a positive number (unsigned) for the far solution.

The first  point computed  will be  the intersection point of the

centerlines. The  remaining points will be computed starting with

the P.C.  of the  first return  to the right of the first R.O.W.,

(as you  face the  intersection from the known point on the first

right-of-way) proceeding  sequentially around the intersection in

a counter-clockwise direction. Refer to Figure 5.7b.

The third  selection, F3,  presents a dialog box requesting known

data for an intersection of a two curved rights-of-way. Here, the

radius point  and centerline  radisu of each right-of-way will be

requested. You  can also  use BASIS distance formats for entering

the right-of-way widths.

After providing  the right-of-way data, you must then provide the

return radius  for the  interection return  or fillet curves, and

the starting  point number  for the series of points that will be

computed. As  with distance-distance line intersections, you must

let  the   program  know   if   you   want   the   clockwise   or

counterclockwise solution.  Enter the  solve point  as a negative

number for the counterclockwise solution, or as a positive number

(unsigned) for the clockwise solution.

The first  point computed  will be  the intersection point of the

centerlines. The  remaining points will be computed starting with

the P.C.  of the  first return  to the right of the first R.O.W.,

(as you  face the  intersection clockwise  along the first curved

right-of-way) proceeding  sequentially around the intersection in

a counter-clockwise direction. Refer to Figure 5.7c.

5.02.30 MANUAL  FORMFEED -  The 'F'  prefix is  typed  if  it  is

desired to advance the printer to the top of a new sheet.

At the  'FROM POINT?'  prompt in cogo, or the 'STARTING POINT' or

'TO POINT?'  prompts in  area mode,  pressing F,  followed  by  a

BACKSLASH will  send a formfeed command to the printer, and reset

the programs internal line and page counters.

If you  press ENTER  rather than  BACKSLASH after  typing  the  F

prefix, a  prompt FORMFEED(Y/N)?  will appear, requiring that you

Section 5 - Interactive Coordinate Geometry

-59-

confirm your  request. Press Y followed by a BACKSLASH to proceed

with the formfeed. To cancel, type N (or anything besides Y.)

5.02.31 STORE  ELEVATION -  The 'Z'  prefix is  typed  if  it  is

desired to store an elevation.

At the  'FROM POINT?'  prompt in  cogo, type Z and press ENTER. A

prompt will appear requesting the elevation to be stored. Type in

the elevation and press ENTER. The next prompt requests the point

number for  the elevation. Type in the point number and press the

BACKSLASH character.  If point-protect is enabled, a warning will

appear if the assigned point has a different elevation value. You

can overwrite  the value  by pressing  ENTER, or  you can enter a

different point number.

5.02.32 DETAILED  COGO HELP  - At  the beginning of this section,

you were  told about  the F10  function that  displays  the  cogo

command index.  From this  display, you  can press  F1 to  access

detailed help.

You can  also access detailed help directly from cogo by pressing

SHIFT   F10. This will display the index to help topics. Select a

topic using  the cursor  movement keys  or first letter and press

ENTER or click on the topic with the mouse.

When viewing  Help text, the cursor up/down keys, as well as PgUp

and PgDn  can be  used to  scroll the text. Pressing ESC from the

Help screen  will return  control to  the  preceding  Help  menu.

Pressing ESC again will return to the preceding level.

Section 5 - Interactive Coordinate Geometry

-60-

6.0 FIELD TRAVERSE SYSTEM (F3)

The Field  Traverse System  is accessed  by pressing function key

three (F3) from the main menu.

This system  permits entry,  processing and  adjustment of  field

survey data in a variety of formats. The field data is saved in a

separate  field  file  to  which  data  can  be  added,  deleted,

inserted, or  changed. The  system has  several options which are

selected  by  the  user,  including  three-dimensional  traverse,

alphanumeric  descriptions   of  points,   types  of  adjustment,

hardcopy, and  others. These  options will be discussed in detail

in the following sections of the manual.

The Field  Traverse System  is extensively  modified  from  BASIS

Version 1.8.  While the  data entry procedures are identical, the

system will  handle 900  observations (compared  to  350  in  the

previous version).  The menu system has been redesigned, and edit

functions are now accessible from the FTS main menu.

Because the  program has  been completely  redesigned,  the  new,

improved Field  traverse files  are not  compatible with previous

versions of  BASIS. The  new  field  files  are  saved  with  the

extension .BFT  (for Basis  Field Traverse)  and  the  new  Field

Traverse program  will display a warning if you attempt to access

a traverse  file in the old format. A conversion utility has been

supplied that  you can use to convert old Field Traverse Files to

the new format. This program is named OLD2NEW.EXE.

To run  the program, shell to DOS from the BASIS Main Menu. Then,

at the DOS command prompt, type

OLD2NEW

The program  will automatically  convert any  old Field  Traverse

files that are found in the current default BASIS data directory.

The Field Traverse System menu is shown in figure 6.1.

F1 - Open Field File

F2 - Open Project File

F3 - Add/Enter Traverse

F4 - Change Traverse Data

F5 - Print Traverse Data

F6 - Process Traverse

F7 - Edit Traverse Specs

F8 - Insert Traverse Data

F9 - Delete Traverse Data

F10 - Help

Figure 6.1 - Field Traverse Menu

Section 6 - Field Traverse System

-61-

You can  select the  item of  your choice  by either pressing the

corresponding function  key, using  the cursor  movement keys  to

highlight the  item and  pressing ENTER,  or by clicking with the

mouse.

6.01 OPEN  FIELD FILE  - A  field data file is opened by pressing

function key  F1. This  will prompt  for the filename of the data

file for  storing raw  field data.  If the  file named  does  not

exist, the  user will  be given  the opportunity  to  create  it.

Creating the  field file  is similar  to creating a project file,

and commentary information can be entered which can later be used

to identify  the file,  or as  search criteria  for the  "Catalog

Search" routine (see section 4.12).

A field  file must  be opened  to use  the other  routines of the

Field Traverse  System. You  may choose to create only one or two

field  files   which  are   re-used   for   different   projects.

Alternately, you  may wish  to create  a separate  field file for

each project  file. This  is entirely  up to you, as a field file

can be cross-referenced to any project file at any time.

6.02 OPEN  PROJECT FILE  - Pressing  function key F2 at the Field

Traverse menu  lets you  enter the  name  of  an  existing  BASIS

coordinate file  to receive  processed coordinates.  If the  file

does not exist, you will be given the option to create it.

A project  file need  not be opened to enter raw data. It is only

necessary to  name a  project file  before using function key F6,

the  "Process  Traverse"  routine,  since  coordinates  are  only

computed and saved with that procedure.

6.03 ADD/ENTER  TRAVERSE DATA  - SPECIFICATIONS  - To enter field

data, once the field file has been opened, press function key F3.

If the file contains data from a previous session, a warning will

appear on  the screen,  and you  will have  the option to cancel,

overwrite or add to the existing data.

If no  previous data exists, or if the overwrite option is taken,

the traverse  specification screen  will appear. This screen will

let you  select the  type of  traverse, starting and ending data,

balance options,  rejection  limits,  and  hardcopy  option.  The

specification input screen is shown below in Figure 6.2.

The program  will prompt  through the  specification screen.  The

responses  to  some  prompts  are  mandatory,  while  others  are

optional.

If you  choose to add data to an existing file, the specification

screen will  not appear,  and you  will be  presented with a data

entry screen.  You can  then add  data  to  the  file,  based  on

specifications that were previously entered.

Section 6 - Field Traverse System

-62-

Descriptions (Y/N):      Elevations (Y/N):         Traverse Type:

AVAILABLE TRAVERSE TYPES

1...Horizontal Angle, Horizontal Distance, Elevation

2...Horizontal Angle, Horizontal Distance, Elevation Difference

3...Horizontal Angle, Slope Distance, Zenith Angle

Starting Point Data         Point:

Northing:                 Easting:                 Elev:

Starting Backsight Data     Point:

Northing:                 Easting:                 Bearing:

Ending Point Data           Point:

Northing:                 Easting:                 Elev:

Ending Foresight Data       Point:

Northing:                 Easting:                 Bearing:

Maximum Angular Error (seconds per station):

Maximum Linear Error - One Part in:

Compass/Transit/No Balance (C/T/N):    Vertical Adjustment (Y/N):

Adjust Angles (Y/N):       Hardcopy (Y/N):

Figure 6.2 - Traverse Specification Screen

6.03.01 DESCRIPTIONS  (Y/N): -  Pressing 'Y'  in response to this

prompt will  enable the  Description  field  of  the  data  entry

routine. If  you do  not want  to  enter  descriptors,  then  the

response should be 'N', which is also the default.

6.03.02 ELEVATIONS  (Y/N): -  The response  to this  prompt  will

determine if  the traverse  entered is  three-dimensional or two-

dimensional. If 'Y' is pressed, then you should be ready to enter

elevation data  when entering  the traverse.  Pressing  'N'  will

suppress all prompts for elevation data. The default is 'N'.

6.03.03 SELECT  TRAVERSE TYPE  (1,2 or 3): - Here, you select the

format for  data entry. If elevations are disabled, then the type

of elevation  can be ignored. Horizontal Angles include bearings,

azimuths, interior/exterior  angles, and  deflection  angles.  If

Traverse Types 2 or 3 are chosen, and elevations are enabled, the

user will  be prompted  for h.i.'s and h.r.'s when entering field

data. All  Traverse Types  will  let  you  enter  sideshots.  The

default is Traverse Type 1.

6.03.04 STARTING  POINT AND BACKSIGHT: - This sequence of prompts

requests the  starting point number, or coordinates, and optional

backsight point number, coordinates, or bearing/azimuth.

Section 6 - Field Traverse System

-63-

If the  starting point  number is  known then  it can be entered,

otherwise, the explicit coordinates of the starting point must be

entered. If  elevations are  enabled, then  a starting  elevation

must be  entered. If  a  starting  point  number  is  given,  and

elevations are  enabled, then  that point must have an associated

elevation on file with its coordinates.

Responding to  the Backsight  prompts are  optional, however,  if

bypassed, the  program will  expect that  an explicit  bearing or

azimuth be  entered for  the first  leg of  the  traverse.  If  a

backsight point  is known,  it can  be entered  as either a point

number, explicit  coordinates, or  as a bearing/azimuth. The same

conventions for  entering a  bearing/azimuth  in  the  Coordinate

Geometry System  apply here,  and the  user can enter an explicit

bearing/azimuth,  a   defined  bearing/azimuth,   or   a   stored

bearing/azimuth if desired.

6.03.05 ENDING  POINT AND  FORESIGHT -  Providing ending  data is

optional, however  if the  traverse is  to be  balanced,  or  its

closing precision  evaluated, then the ending coordinates must be

supplied. If  elevations are  enabled, a closing elevation can be

entered to permit an elevation balance, and similar evaluation of

elevation closure.

The closing  coordinates can  be entered as a point number, or as

explicit coordinates.

If a  foresight point number, coordinates, or bearing/azimuth are

known, then it can be entered. This is the line that the "closing

angle" will  be turned  into, from the ending point. If an ending

coordinate is  not given,  but a  foresight azimuth  is, then the

program will  assume that this azimuth is the theoretical azimuth

of the  last leg  of the  traverse, and  there will be no closing

angle (since  there was no closing point). The angle balance will

be modified accordingly. This last feature is useful when closing

into a solar azimuth on the last leg of an open traverse.

6.03.06 MAXIMUM ANGULAR ERROR (SECONDS PER STATION): - This entry

lets you  set a  rejection limit  which will  abort the  traverse

balance if  the angular  error exceeds  the specified  limit. The

default is  ten seconds  per station. Exceeding the entered value

will cause  the computer  to sound an audio alarm, and cancel the

balance. The  user can  then review the printout to determine the

source of the error.

6.03.07 MAXIMUM  LINEAR ERROR  - ONE  PART IN:  - This  rejection

limit will cancel the balance if the ratio of linear error (after

the angles  are  balanced)  exceeds  the  specified  amount.  The

default is  7500. As  with the angular rejection limit, exceeding

the value  entered will  sound the audio alarm, and terminate the

Section 6 - Field Traverse System

-64-

balance. You  can then review the data to determine the source of

the error.

6.03.08 TYPE OF BALANCE (COMPASS, TRANSIT, NONE) - This selection

allows the  user to  choose the type of balancing procedure to be

used. The  appropriate response  is C,  T, or N for Compass Rule,

Transit Rule,  or No  Balance, respectively. Coordinates are only

sent to  the project  file when  the balance  is executed.  If  a

rejection limit terminates the balance procedure, the coordinates

will not  be sent to the project file. The default is the Compass

Rule.

6.03.09 ANGLE BALANCE (Y/N): - You have the option to disable the

angle balance.  If disabled,  then the  unbalanced angles will be

used in the traverse balance routine. The default is 'Y'.

6.03.10 ELEVATION  BALANCE (Y/N):  - If  elevations are  enabled,

then you  can select  a proportional elevation balance. An ending

elevation must  be available  to process  the elevation  balance.

This elevation can either be stored with the ending point number,

or entered explicitly with the ending coordinates.

6.03.11 HARDCOPY (Y/N): - This option lets you select hardcopy of

the traverse  as it  is balanced.  The first  printout is the raw

traverse. If  angle-balance is  enabled, the next printout is the

traverse  results  after  the  angles  are  balanced.  The  final

printout is the balanced traverse.

ALL OK (Y/N)? appears next, to give you the opportunity to review

and, if  necessary, correct the specifications. Press Y to accept

the traverse  specs, or  press N to re-enter. If re-entering, the

previously entered values will appear as defaults to the prompts.

If you  want to  clear a  field while re-entering data, press the

DEL key.

6.04 ENTERING  THE TRAVERSE  - Once  the traverse  specifications

have been  completed, and  accepted, the  field data entry screen

will appear.  Certain entry fields will be highlighted, depending

upon the  type of  traverse selected.  These are  the fields that

require a  response. For  example, if elevations are off, none of

the elevation-related  fields will  appear highlighted,  and  the

user will not be prompted for any data of that type, such as h.i.

or h.r.  Over 1000  entries are  possible, depending on available

memory. (Note: extended or expanded memory is not used.)

6.04.01 ENTERING  ANGULAR DATA  - Angles  can be  entered in four

ways, namely bearings, azimuths, left or right angles, or left or

right deflections.  These different input modes are identified by

Section 6 - Field Traverse System

-65-

typing the  respective prefixes  B, A,  R, or D before the value.

Once an  angular mode  is given, it is not necessary to repeat it

for subsequent entries, unless the mode changes.

Azimuths and  Bearings are  entered using  the  same  conventions

described  in  COGO  for  explicit  bearings.  Note  that  stored

bearings/azimuths, or defined bearings/azimuths are not permitted

in Field  Traverse. Angles  and Deflections are entered using the

same conventions  described in  COGO also.  See section  5.01  to

review the angular conventions.

If  a   starting  backsight  was  not  given  when  the  traverse

specifications were  entered, the  program will  expect that  the

first traverse entry (also called a "record") will contain either

a bearing  or azimuth. If it does not, then an error message will

appear, and the user will have to repeat the entry.

The following  examples will  illustrate the  correct method  for

entering angular data.

Example of First Bearing Entry:

B3723455 - this is equivalent to S 72�34'55" W.

Subsequent Bearing Entries:

2315624 - for S 31�56'24" E. The prefix B is not

required if the preceding entry was in

the same mode.

Example of First Azimuth Entry:

A0452312 - for an azimuth of 45�23'12". If this

is the first azimuth entry, or if the

preceding entry were given in a diff-

erent mode, then the A prefix is needed.

Subsequent Azimuth Entries:

0121756 - for 12�17'56". As long as the preceding

entry is an azimuth, the A prefix is not

needed.

Example of Left/Right Angles:

R-1034759 - for an angle 103�47'59" to the left.

Notice the minus sign is used to rep-

resent an angle "left". Also note that

the R prefix is needed, unless the

previous mode was the same.

Subsequent Left/Right Angles:

1863455 - for an angle 186�34'55" to the right. The

Section 6 - Field Traverse System

-66-

minus sign isn't needed, since angles to

the right are considered positive. The R

prefix is not needed either, as long as

the preceding entry was in the same mode.

Example of Left/Right Deflections:

D23420  - for a deflection angle of 2�34'20" to the

right. The D prefix identifies that the

angle is a deflection. If the deflection

is to the left, then a minus sign is used

as in the previous example. Note that

leading zeroes are not needed.

Subsequent Deflection Angles:

-125042 - represents a left deflection of 12�50'42".

Note the minus sign, to indicate the angle

is to the left. Also note the D prefix is

not needed, as long as the previous entry

was also a deflection angle.

6.04.02 ENTERING  DISTANCES -  Distances are  entered  by  simply

typing the desired distance and pressing ENTER. Note that defined

distances are not permitted.

A built  in metric conversion is available, and can be invoked by

entering the  distance as  a negative value. The direction of the

conversion depends  on the  default distance units. If the system

default is  set for  feet, a  metric distance  is  entered  as  a

negative for  conversion  to  feet.  Conversely,  if  the  system

default is  for meters, a negative distance would perform a feet-

to-meters  conversion.  The  same  rules  apply  for  elevations,

elevation differences, h.i.'s and h.r.'s.

6.04.03 POINT  NUMBERS -  It is  only required that you enter the

first point  number. After  that, the  default point  number will

automatically increment  by one.  You can  accept the  default by

pressing ENTER at the prompt, or override by entering a different

point number.

6.04.04 SIDESHOTS - Sideshots are indicated by entering the point

number as  a  negative  number.  Subsequent  point  numbers  will

default to  the next  point, as  above, but also as negatives. To

terminate a series of sideshots, simply enter a point number as a

positive value.

6.04.05 DESCRIPTIONS  - If Descriptions are enabled, then you can

enter up  to 10  alphanumeric characters to describe a point. Any

characters are  converted to  uppercase, so  it does  not  matter

Section 6 - Field Traverse System

-67-

whether they  are typed  in as  upper or  lowercase letters.  The

description will always defaults to the previous description.

6.04.06 TERMINATING  DATA ENTRY  - Once  all field  data has been

entered, press  the ESC  key. The  system  will  prompt  for  the

closing Angle.  If known,  enter the  closing angle following the

standard conventions for angular data ( see section 6.04.01 ). If

there is  no closing  angle, simply bypass by pressing ENTER with

no data entry.

6.05 CHANGE  TRAVERSE  DATA  (F4)  -  Pressing  function  key  F4

activates the field data editor. This routine lets you modify, or

change individual  field data  entries or  "records". To edit any

record, enter  the record  number in  response to the prompt, and

press return.  You can  also edit  a record by entering the point

number (as  a negative  number.) The  program will  then find the

record(s) that  is associated  with the point number you specify.

The current  entry will  appear on  the highlighted  input  line.

Pressing ENTER  in  response  to  the  prompts  will  accept  the

previously  entered  values  as  defaults.  To  change  a  value,

overtype it with the new value, and press ENTER.

When all editing is completed, press ESC. Before returning to the

previous level,  the closing  angle will  appear. Press  ENTER to

accept the current value, or enter a new value to change it.

6.06  PRINT   FIELD  NOTES   (F5)  -  A  hardcopy  of  the  field

specifications and  field data  file can  be printed  by pressing

function key  F5 from  the main  menu. You  will be  prompted for

printout headings.  As in  other printout  routines, the headings

supplied in the System Defaults configuration will be the default

headings, and  are accepted  by simply pressing ENTER. Otherwise,

type in the desired headings, and press ENTER.

6.07 PROCESS  TRAVERSE (F6)  - To  compute the traverse and store

coordinates into  a project  file, press function key F6 from the

main menu.

In order  for this  routine to execute, it is necessary to open a

BASIS project file, prior to its selection. Failure to do so will

cause a  warning to  appear on  the screen, and return control to

the Field Traverse System menu.

A field  data file  must also  be open. Usually it will be, since

the "Process"  routine is usually executed immediately after data

entry or editing. If not a warning will appear on the screen, and

the you  will have  to open  a field  file prior  to running  the

"Process" function.

Section 6 - Field Traverse System

-68-

If  the   hardcopy  option   was   selected   in   the   traverse

specifications, then  the standard  printout heading prompts will

appear. Respond  to the prompts in the usual manner. The traverse

data will  be processed  according to  the  options  set  in  the

traverse specifications file. If a balance option, other than "No

Balance", was  selected, and  any of  the  rejection  limits  are

exceeded, the  audio alarm will sound and the computation will be

aborted. The  user must then determine the cause of the rejection

and correct  either the  entry, or the measurements themselves to

obtain a satisfactory run. However, if realistic rejection limits

are set  forth in the specifications, and the field work and data

entry are  relatively blunder-free,  a successful  run should  be

obtained, and  the processed  coordinates will  be passed  to the

named BASIS project file.

IMPORTANT -  COORDINATES ARE ONLY PASSED TO THE PROJECT FILE UPON

SUCCESSFUL EXECUTION OF THE "PROCESS TRAVERSE" ROUTINE.

6.08 EDIT TRAVERSE SPECIFICATIONS (F7) - Pressing function key F7

will   activate    the   specification   editor.   The   traverse

specification screen  will appear on the screen, and you can edit

the modifiable  entries. Certain  parameters may  not be changed.

These are   the Elevation and Description On/Off toggles, and the

Traverse Type  Number. These  cannot be  edited because  doing so

would invalidate the format of the field data entries.

If these values must be changed, then it will be necessary to re-

enter the traverse. All other values can be modified, or deleted.

The prompts  will default  to the  previously entered  values. To

delete a value, simply press the space bar, and hit ENTER. To re-

enter a value, simply type in the new desired value.

The specifications  editor is  also useful  to modify the balance

option, and  see the  difference  in  results  of  the  available

balance types.

It can  also be  used to  change the starting or ending points if

traverse data is added or deleted from the field file.

Of course,  it is  also useful  to correct  mistakes  made  while

entering traverse specifications, which are only discovered after

attempting to balance the traverse.

6.09 INSERT TRAVERSE DATA (F8) - At times, it may be necessary to

insert an  omitted record  into the  field  data  file.  This  is

accomplished by pressing function key F8 from the main menu.

To insert  a record,  you must  enter the  existing record number

that the  new record  is to be inserted BEFORE. To insert records

after the  last entry,  use the "ADD FIELD DATA" option described

in the preceding section.

Section 6 - Field Traverse System

-69-

Note that  when a  record  is  inserted,  all  subsequent  record

numbers are automatically incremented by one. Thus, if a group of

records are  to be  inserted, it is suggested that you start with

the highest  insertion record,  and work backwards. This way, the

original record numbers are valid for each insertion.

Press ESC when all inserted data has been entered.

6.10 DELETE TRAVERSE DATA (F9) - To delete a data record from the

field file, press function key F9 from the main menu.

Respond to the prompt by entering the record number of the record

to be deleted. The record will appear on the screen, and a verify

prompt "Are  You Sure  (Y/N)?" will  appear beneath it. To delete

the record  press Y.  To cancel  the deletion,  press N (Upper or

Lowercase are permitted).

Similar to data insertion, deletions will cause subsequent record

numbers to  decrement by  one. It is suggested you start deleting

at the  highest numbered  record, and  work  backwards,  so  that

subsequent deletions  can be  referred  to  the  original  record

numbers.

Press ESC when all deletions have been made.

6.11 FIELD  TRAVERSE SYSTEM  HELP (F10)  - To  activate the Field

Traverse Help  System, press  F10 from  the Field Traverse System

menu. A menu of available help topics will appear.

Select the  help topic by either pressing the first letter, or by

using the  cursor movement keys, and pressing ENTER. You can also

click on a topic with the mouse.

When viewing  Help text, the cursor up/down keys, as well as PgUp

and PgDn  can be  used to  scroll the text. Pressing ESC from the

Help screen  will return  control to  the  preceding  Help  menu.

Pressing ESC again will return to the preceding level.

Section 6 - Field Traverse System

-70-

7.0 UTILITY  SUPPORT MENU  - The  Utility Support  Menu  provides

access to  a group  of stand  alone surveying  utility  programs,

which are  useful for solving problems not specifically addressed

by the main system modules.

Starting with  version 1.8 of BASIS, the ability to customize the

Utility Support Menu has been added.

7.01 AUTOMATIC  DATA REDUCTION - This routine is designed for use

with ASCII  data files  imported from  the HP-41CX, created using

TRIG2 software  in that unit. LINK ( see section 7.06 ) and TOPC1

(HP-41 program) are required to upload the data to the computer.

Files created  on the  CMT MC-V  hand-held computer  using  TRIGL

software can  also be processed, after they have been uploaded to

the desktop using Kermit.

The HP-41  software is  designed for the Kern E-Series electronic

theodolites, in  conjunction with  the DIF-41 interface. The MC-V

version works  with any of the theodolites supported by the MC-V.

Documentation is  provided separately  for these programs. Please

contact the author of BASIS for additional information.

7.02 CALCULATOR  - A  "pop-up" RPN  scientific  calculator  (non-

resident version)  is provided.  It is also available from within

COGO and  Field Traverse  by pressing  shifted function  key five

(^F5). Using  the calculator  is nearly identical to using the HP

series  of   calculators.  A   memory-resident  version   of  the

calculator, along  with an  AutoCAD Drawing  Menu and  Loader are

available as a separate package from the author of BASIS.

USING THE  CALCULATOR - To run the program, press the appropriate

function key  from the Utility Support Menu, or Shift F5 while in

COGO or  Field Traverse.  A calculator  window will appear on the

screen. The "display" is in the upper left portion of the window.

The 'X'  register is  the calculator  display. The  Y, Z,  and  T

registers are the other three registers in the stack. Last X will

contain the  last contents  of the  X register,  and  is  updated

whenever a  math  function  is  performed  that  involves  the  X

register. The right side of the window lists available functions,

and the  lower section  of the  window displays  the ten  storage

registers.

EXITING vs  PASTING - When you are finished using the calculator,

press ESC  to return  to the  previous screen.  You also have the

option of  pasting the  value in  the 'display' into the previous

application, by pressing INS.

For example,  suppose you  activated the calculator during a cogo

distance prompt.  After performing  a quick side calculation, you

arrive at  a number  that you want to use in the distance prompt.

Press INS,  and the value you just calculated will be pasted into

Section 7 - Utility Support

-71-

the waiting  distance prompt  in  Cogo.  When  pasting,  whatever

appears in  the X  register (display)  of the  calculator will be

transferred, exactly  as it appears. In some cases, such as error

messages, or  scientific notation,  it may  not be appropriate to

paste the result.

OPERATIONS AND FUNCTIONS - Functions are accessed by pressing the

appropriate key  on the  computer,  the  same  as  if  an  actual

calculator  were  being  used.  Some  functions  are  denoted  by

lowercase letters  and some  by uppercase  letters. The lowercase

letters are  obtained by  simply pressing  the  letter  key.  The

uppercase letters  are obtained  by pressing the SHIFT and letter

key at  the same time. It does not matter if the CAPS LOCK is on.

The program will know if it is and adjust itself accordingly. For

example, to calculate the sine of a number in the X register (the

"display") press  the 'S'  key, even  if CAPS LOCK is on. For the

Arc Sine,  press SHIFT and 'S' simultaneously, again even if CAPS

LOCK is on.

If  you  are  familiar  with  Hewlett-Packard  calculators,  then

reverse polish notation (RPN) should be quite familiar. With RPN,

the two  operands of  a calculation  are first  "entered" and the

operation is  then performed.  To add  2 and  2, for example, the

keystrokes would  be 2  ENTER 2 +. Notice the sum, 4, is in the X

register, and that the stack has dropped down. Subtraction is the

same. To  subtract 3  from 5 the keystrokes would be 5 ENTER 3 -.

The difference,  2, will  appear in the X register. To divide 100

by 3,  key 100  ENTER 3  /. The  quotient will  appear in  the  X

register. Multiplication is performed using the * key.

The stack  makes it easy to perform chained calculations, without

resorting to storing intermediate values in registers, or writing

them down. For example, perform the following calculation:

(20 + (200 + 300)/3) x 45

the keystrokes could be 20 ENTER 200 ENTER 300 + 3 / + 45 *

Most people,  once they  become accustomed  to RPN, never want to

switch back  to the  algebraic type  of calculator (the ones with

the "=" signs).

Since  RPNCALC  uses  a  non-destructive  windowing  process,  it

restores the  screen exactly as it was upon exiting. This feature

should make  it useful  to programmers  who may  wish to spawn it

from within their own applications

LIST OF  FUNCTIONS -  ADD (+), SUBTRACT (-), MULTIPLY (*), DIVIDE

(/) -  The basic  four  functions  operate  using  the  X  and  Y

registers, and  place the  result in  the X  register. They  also

cause the stack to drop.

SINE (s),COSINE  (c),TANGENT  (t)  -  The  natural  trigonometric

functions of the number in the X Register will be returned to the

Section 7 - Utility Support

-72-

X Register.  The value  of X  prior to  the trig function will be

placed in  Last X.  Note the program expects angular values to be

in decimal degrees.

ARC SINE  (S), ARC  COS (C) and ARC TANGENT (T) - These functions

return the  inverse trigonometric function of the number in the X

register. Last  X will receive the number in the X register prior

to the inverse trig function. The number returned is an angle, in

decimal degrees.

CHS (m)  - This  function is  used to toggle a positive number in

the X Register to a negative, and vice-versa.

X<>Y (\)  - This  function is  used to swap the contents of the X

and Y registers.

X2 (x)  - This function will square the number in the X register,

and update Last X.

SQRT (X) - This function returns the square root of the number in

the X register, and updates Last X.

1/X (i)  - This function returns the inverse of the number in the

X register, and updates Last X.

Last X  (l) - This function recalls the contents of Last X to the

X  register,  and  pushes  the  stack  up  (unless  the  previous

operation was ENTER).

->DEG (d)  - This  function converts  a number  in  DD.MMSSSSS...

format in the X register to decimal degrees, and updates Last X.

->DMS (D)  - This  converts a  number in decimal degrees in the X

register to DD.MMSSSSS... format, and updates Last X.

Y^x (y)  - This function will raise a number in the Y register to

the power contained in the X register. Last X will be updated and

the stack will drop.

Log X (g) - This returns the logarithm (base 10) of the number in

the X register, updating Last X.

Ln X  (n) -  This function returns the natural logarithm (base e)

of the number in the X register, updating Last X.

->Polar (p) - This function takes an angle in the Y register, and

a length  in the  X register,  and will return the y-component in

the Y register and the x-component in the X register. Last X will

be updated.

->Rect (r)  - Given  the y component in the Y register, and the x

component in  the X register, this function will return the angle

in the  Y register, and the length in the X register. Last X will

be updated.

Section 7 - Utility Support

-73-

10^x (G) - This function returns 10 to the power of the number in

the X register, updating Last X.

e^x (N)  - This  function returns  e raised  to the  power of the

number in the X register, updating Last X.

x! (F)  - This returns the factorial of a positive integer in the

X register,  updating Last X. The highest permissible argument is

170.

FIX (f)  - This function permits setting the decimal precision of

the X  register (the display). A precision of 0 through 9 decimal

places may  be selected.  This will  not cause  the numbers to be

rounded internally. It only affects the displayed value.

EEX (e)  - This  function is  used to  enter exponents of 10. For

example, to  enter 1.2367e-15, key in '1.2367', press 'e' and key

in '15  m' (the  'm' is  to toggle  the sign  of the  exponent to

negative).

SCI (E)  - This function changes the display format to scientific

notation. As  with FIX, a decimal precision of 0 through 9 places

can be selected.

STO ([) - This function places the value in the X register into a

selected storage register (0-9).

Register  arithmetic   is  permitted.   Numbers  can   be  added,

subtracted, multiplied  or divided into the contents of a storage

register,  with   the  result  being  placed  in  the  particular

register.

For example,  to accumulate  the contents  of the X register into

register 8, the keystrokes [ + 8 will do the job. To multiply the

contents of  register 5  by the  number in  the X  register,  the

sequence [ * 5 will accomplish the job. To divide the contents of

register 3  by the number in the X register, the keystrokes are [

/ 3.

RCL (])  - This  recalls the  value from a selected register into

the  X  register,  and  lifts  the  stack  (unless  the  previous

operation was ENTER).

R# (R) - This causes the stack to "roll down" one register.

R^ (U) - This causes the stack to "roll up" one register.

PI (P)  - This  places the  value of  PI (3.141592654) into the X

register, pushing up the stack (unless the previous operation was

ENTER).

CLREGS (Z) - This function clears all storage registers.

Section 7 - Utility Support

-74-

<- (Backspace)  - Pressing  the BACKSPACE key will backspace over

entered keystrokes and/or clear the X register.

D->R (>)  - This  function will convert a decimal degree value in

the X register to radians, and update Last X.

R->D (<)  - This  function converts  a  radian  value  in  the  X

register to decimal degrees, updating Last X.

HMS+ ({) - This adds an angle in DD.MMSS format in the X register

to an  angle (also  in DD.MMSS  format) in  the Y  register,  and

displays the  result in  DD.MMSS format in the X register. Last X

is updated.

HMS- (})  - This  function will subtract a DD.MMSS angle in the X

register from one in the Y regster, placing the difference in the

X register. It will also update Last X.

EXIT (ESC) - The ESC key is used to exit to DOS, or return to the

application program from which the calculator was called.

ERROR MESSAGES - The programs feature extensive error trapping to

guard against  calculation errors,  such as  dividing by zero, or

attempting to take the square root of a negative number.

When a  math error  is detected,  the calculator  will "beep" and

issue an  error message  in the  display. To resume calculations,

press any  key. The  calculator will reset to the status prior to

the error.

The error messages that may appear consist of the following:

OUT OF  RANGE -  An argument(s)  of the  called function  was not

within the  permissible limits for the particular operation, i.e.

attempting to take the square root of -1.

SINGULARITY - The arguments would result in a singularity, i.e. 0

raised to the -2 power.

OVERFLOW -  The operation  would result  in a number greater than

the largest permissible number.

UNDERFLOW -  The operation would result in a number less than the

lowest permissible number.

TLOSS -  Argument(s) would  result in  total loss  of significant

digits, i.e. sin(10e+70).

PLOSS - Argument(s) would result in a partial loss of significant

digits.

DIV BY ZERO - An attempt was made to divide a number by zero.

Section 7 - Utility Support

-75-

DATA ERROR  - A  general error  message, generated if none of the

above apply.

7.03 PRECISE EDM SLOPE REDUCTION - For high accuracy requirements

and trigonometric  levelling, this  utility should  be used. This

routine corrects  for earth  curvature, refraction and parts-per-

million, and  computes the  horizontal and vertical distance from

slope  distance   measurements  and  zenith  angles.  Up  to  ten

repititions of  zenith angle  readings and  distance measurements

can be entered.

The first  prompt requires  that the  type of  measurement units,

meters or  feet, be entered. The default is meters. To accept the

default, simply  press ENTER.  To select feet, press F. If meters

are chosen, then the program will expect all distances, including

instrument and reflector heights to be entered in meters. If feet

are selected, then all distances should be in feet. However, if a

distance is  entered as  a negative, then the program will assume

that it  is in  the alternate  units, and  convert to  the chosen

units.

The next  series of  prompts request  the printout headings, with

the defaults  being the  headings defined  in the System Defaults

routine (see  section 3.0). To accept the default heading, simply

press ENTER  in response  to each  prompt, otherwise  type in the

desired heading and press ENTER.

The next  screen displays the default instrument constants, which

can be  obtained from  the technical specifications supplied with

the EDMI,  and several  other default  options. BASIS is supplied

with constants  that should  suffice for  most infra-red  EDMI's.

Note that  this  routine  is  intended  for  use  with  lightwave

instruments only. It is not suitable for microwave instruments.

To accept  the defaults,  simply press ENTER. To edit the default

values, press  the appropriate function keys and make the desired

changes. Press  ENTER to  save the  new values.   For example, if

temperature is  measured  in  Fahrenheit,  rather  than  Celsius,

select the  temperature units  function to  change that  default.

Similarly, if  pressure  is  measured  in  inches  of  Mercury,or

millimeters, rather  than millibars,  select that option and make

the  appropriate   change.Once  changed,   the  new   table  will

automatically appear the next time this program is invoked.

After accepting  the instrument specifications and constants, the

data entry window will appear.

The printer can be toggled ON or OFF by pressing function key one

(F1).  Temperature  and  pressure  can  be  entered  by  pressing

function key  two (F2). The data entry screen can be re-set using

function key  three (F3)  and the  ESC key will return control to

the Utility menu.

Section 7 - Utility Support

-76-

Initially, a  default temperature and pressure will appear. These

correspond to  values that will produce a parts-per-million (PPM)

correction of  zero (0).  Therefore, if  the  PPM  correction  is

desired, then  the actual  observed atmospheric  data  should  be

entered by  pressing function key two (F2). Also, temperature and

pressure are  considered for  correcting  the  zenith  angle  for

refraction.

Comment? appears in the input window. If desired, an alphanumeric

comment can be typed in. Press ENTER.

Next the  prompts for  h.i and  h.r. appear. Enter the instrument

height and  reflector height  in the  appropriate units.  If both

heights are  the same,  these prompts  can be  bypassed by simply

pressing ENTER  in response  to each. This will result in a value

of zero (0) being assigned to the h.i. and h.r.

Next, Slope Distance: appears. Enter the measured slope distance,

and press  ENTER. The  Slope Distance:  prompt will  continue  to

appear to  allow  entry  of  repeated  measurements.  Up  to  ten

repititions are  permitted. When all distances have been entered,

simply press ENTER with no data entry.

After the  slope distances  have been  entered, the  program then

prompts for  Zenith Angle:.  Enter the  observed zenith angles in

DDDMMSS.S format. For example, a direct zenith of 90�31'30" would

be entered  as 903130.  An inverted zenith of 269�23'14" would be

entered as 2692314. Up to ten zenith readings are permitted. When

all zenith  angles have  been entered, simply press ENTER with no

data entry.

The horizontal  distance, and  mark-to-mark elevation  difference

will be  displayed on  the screen.  To clear the screen for a new

observation, press  ENTER. If the printer toggle is ON, this will

also cause the data to be printed.

To return to the Utility menu, press ESC at any time.

7.04 DIRECTIONS  TO ANGLES  - This  utility  is  used  to  reduce

theodolite directions  to  a  clockwise  angle  from  an  initial

backsight.

Backsight: prompts  for the  initial direction  of the backsight.

Enter in DDMMSS.S format and press ENTER. Leading zeroes need not

be entered.

Foresight: prompts  for the direction to the foresight. Enter and

press ENTER.

Foresight: prompts  for the  inverted foresight  direction. Enter

and press ENTER.

Section 7 - Utility Support

-77-

Backsight: prompts  for the  inverted backsight  direction. Enter

and press ENTER.

The  clockwise   angle,  from  backsight  to  foresight  will  be

displayed. Pressing F3 will re-set the entry sequence if an error

is made  while entering data. Pressing ESC will return control to

the Utility menu.

7.05 SHORT  LINE EDM  REDUCTION - This routine is essentially the

same as  the precise  EDM reduction in operation, however it does

not  display   vertical  differences,   nor  does   it   consider

temperature,  pressure,   curvature,  refraction,  or  parts-per-

million. It  will display horizontal distances on the screen. The

user can  select either  feet or meters as input units. Up to six

zenith angle  or distance  repititions  can  be  entered.  It  is

intended for  short measurements,  or where high precision is not

required.

Pressing ESC returns control to the Utility menu

7.06 NAD27-83  TRANSFORMATION -  A computer  program  to  perform

rigorous transformation of State Plane Coordinates from NAD 27 to

NAD 83  has been added to the BASIS Utility Support Package. This

is a  dedicated version  of GENXFORM,  which has been modified to

act upon the BASIS coordinate file structure.

The utility will perform a least squares two-parameter similarity

transformation of  plane coordinates  on the North American Datum

of 1927  (NAD 27)  to the  new NAD  83 datum.  It is  based on  a

transformation model of the form

E83 = aX27 - bY27 + c

N83 = bX27 + aY27 + d

and  features  a  weighted  interpolation  of  residuals  at  the

transformed points  based on  computed residuals  at the  control

(common) points.

A minimum of three points common to both systems must be known to

satisfy the least squares condition, and a maximum of 100 control

points can  be entered.  The control  set can be saved to a file,

and recalled  as needed  to transform  coordinates within a given

area.

The first  prompt to  appear, Hardcopy (Y/N)?, allows the user to

select printing  a report of the transformation. If a printout is

desired, press Y, otherwise press N.

Read From  Disk (Y/N)?  will appear next. Since this is the first

time the  software is  being used,  the response  will be N. If a

Section 7 - Utility Support

-78-

file of control points had been saved from a previous session, it

could be read in by responding to this prompt by pressing Y.

If the  response is  Y, the next prompt to appear will be Control

Point File:,  and the  user must  enter the  name of  the file in

which the  control set  is stored.  Include the  drive letter and

path if  different from  the current  default. Upon  reading  the

file, the computer will display the metric conversion factor used

when the  file was created, and then proceed to permit data entry

of additional stations.

If the  response was  N, then  the next  prompt, Convert  NAD  83

Coordinates To  Feet (Y/N)?  , will  appear. Since  the  National

Geodetic Survey  is only  publishing coordinates in meters, it is

more efficient  to enter the published metric values directly. If

it is  desired to convert the output coordinates to feet, respond

by pressing  Y. Otherwise, if the output coordinates are to be in

meters, or  if the  NAD 83 control point values have already been

converted to  feet, respond  by pressing N. If the above response

was Y,  a second  prompt will  appear,  requesting  U.S  Feet  or

International Feet  (U/I)? Respond  by pressing U or I, depending

on the desired conversion.

The next  series  of  prompts  will  request  the  control  point

information. This  data will  consist of the station name, NAD 83

northing and Easting, and NAD 27 northing (Y) and easting(X). The

prompt Station:  will appear. Type in the name of the station, up

to 17 characters in length, and press ENTER. The next two prompts

will request  the NAD 83 northing and easting values. Type in the

data, pressing  ENTER after  each.  The  next  two  prompts  will

request the  NAD 27 northings and eastings, assumed to be in U.S.

survey feet.  Enter the  coordinates, pressing  ENTER after each.

Finally, ALL  OK (Y/N)?  will appear in the lower right corner of

the screen.  Review the data for the control point, and if all is

correct press  Y. If a mistake was made, press N and re-enter the

data for the point.

Repeat the above process for each control point. When all control

points have  been entered (a minimum of three) simply press ENTER

in  response   to  the   next  prompt.  This  will  initiate  the

computation of the transformation parameters.

Note: The  program can be aborted at any time by pressing the ESC

key. However,  any data  that has  not been saved will be lost in

doing so.

The results  of the  transformation computation  will scroll into

the display  window. If it exceeds twenty lines in length, the UP

and DOWN  ARROW keys,  as well  as PAGE  UP  and  PAGE  DOWN  are

activated, and  the user  can scroll  up  or  down  to  view  the

residuals and error vetors for each control point.

If it  is desired to reject a control point, use the space bar to

move the  highlight to  the desired  point. Pressing  CTRL E will

Section 7 - Utility Support

-79-

cause the  line to be printed in reverse video, and the line will

be rejected  from the  control set. After the residuals have been

examined, press either ESC or ENTER.

If any  points were  rejected, the  set will be re-processed, and

the above  procedure is  repeated. If  there were  no rejections,

then the  standard errors  of the residuals and the error vectors

will appear  on the  screen, along  with the  prompt Save To Disk

(Y/N)?. In most cases, if the data is being entered for the first

time, the  user will  probably want  to save the control set, and

should respond  by pressing Y. To bypass the save press N. If the

data is  being saved,  the next prompt will request Control Point

File:. Type  in the file specification, including drive, and path

if different  from the  current default.  Include an extension to

the filename,  if desired. The file specification must conform to

the usual DOS restrictions for naming files.

The next  prompt will  permit the  user to  transform a  file  of

coordinates,  if   desired.  Transform  Coordinates  (Y/N)?  will

appear.  If  it  is  desired  to  transform  a  file  of  NAD  27

coordinates, respond  by pressing  Y. To  bypass  transformation,

press N.  If Y was selected, the program will request the name of

the basis  project file,  and then the beginning and ending point

numbers for the points to be transformed.

The control point file has been structured to permit easy editing

with  a   text  editor  or  word-processor  (in  non-document  or

unformatted mode).  The first line contains the metric conversion

factor to be used (1.0, if no conversion is desired). The control

point data  follows. For each control point, there are two lines.

The first  contains the station name, and the second contains the

NAD 83 Northing, Easting and NAD 27 Northing, Easting. Coordinate

values must be separated by one or more spaces, and a line cannot

exceed 80  characters in length. Station names can use the entire

line,  but   the  program   will  only  recognize  the  first  17

characters. All lines must end with a ENTER.

7.07 ELECTRO-OPTICAL  EDM CALIBRATION  - This  program performs a

least squares  analysis of  measurements made  over a calibration

base line.

The first  screen  appears  displays  the  default  EDMI  values,

including nominal index of refraction and carrier wavelength (per

manufacturer), instrument  and prism  constants, if  any need  be

applied, and  units for  measurement of temperature and pressure.

These values  will  usually  be  the  same  for  any  calibration

performed by  the  same  office.  When  actual  observations  are

entered later  in the program, the user will have the opportunity

to overide any default.

If this  is the  first time the program is being used, it will be

necessary to enter the known baseline values. Otherwise, the next

screen to  appear will  display the  published baseline data that

Section 7 - Utility Support

-80-

was used  the last  time the  program was  run.  If  a  different

baseline is being used from the one that is displayed, respond to

the Change  Baseline Data  (Y/N)? by  pressing Y.  To accept  the

baseline that is shown on the screen, press either N or ENTER.

When entering  baseline  data,  the  first  prompt  requests  the

distance units,  either meters  or feet. Select the approropriate

response, keeping in mind that if the baseline data is entered in

meters, all  observations made  on the baseline, including h.i.'s

and h.r.'s  must be  in meters.  Similarly, if feet are selected,

all measurements  must be  in feet.  Respond to the prompts until

all data  for the baseline is entered. Station names can be up to

eight alphanumeric  characters in length. If meters are selected,

be sure  to enter  the station elevations in meters. A maximum of

five baseline  stations is  permitted, which  should be more than

enough since  NGS calibration  baselines are normally established

with only four stations.

(Note: The program will only search the current directory for the

default file and  baseline file. If the program is being run from

BASIS these files will be found in the BASIS.DIR subdirectory. If

a default  file is  not found,  the program  will assume  typical

values for  an infra-red  EDMI. If  a baseline file is not found,

then the program will prompt for the baseline data.)

After the  baseline data  is accepted, the user will be given the

option to read a file of observed data from disk. This option can

be chosen  to reprocess  the analysis  or edit  the observations.

Respond to  the prompt  by pressing  Y if an observation file has

been saved  from a  previous session. If no file exists, or a new

set of observations is being entered, press N.

If Y  is pressed,  the next  prompt will  request the name of the

file. The  usual DOS  conventions for  file specifications apply,

however it  is not  necessary to  include any file extension. the

program will automatically append the extension .EDM to the named

file. If an extension is typed by the user, it will be ignored. A

prompt will  then appear  asking the  user if  the file  is to be

edited. To  bypass editing, press N. The program will analyze the

measurements and  display the  results  of  the  test.  If  Y  is

pressed, the  program will  run the obserevd data entry sequence,

with the  observed data  values appearing as default responses to

the various prompts.

If an  observation file  was not  read, or if the edit option was

selected, the  observed data  entry screen  will appear next. The

various data  items associated  with an  EDMI calibration are now

entered. This  data includes  date of  test (  entered in  MMDDYY

format ),  personnel, time  of test  ( morning  or  afternoon  ),

instrument make  and model,  instrument  serial  number,  nominal

index and carrier wavelength, temperature and pressure units, and

the number  of measurements made over the baseline ( a maximum of

20 and  a minimum  of three  ). The  program will  prompt ALL  OK

(Y/N)? after  this data  is entered.  Review the information, and

Section 7 - Utility Support

-81-

press  Y if everything is correct. Press N if there is a mistake,

and re-enter the data.

Next, the  program will request the actual measurements made over

the baseline. When entering the names of the stations, be sure to

exactly match the names of the stations in the published baseline

file. Failure to match a known station will cause the computer to

issue an  audible chirp,  and  display  an  error  message.  Also

remember that  the units  of all  distances, including instrument

and reflector  heights, must  be consistent with the units of the

baseline file ( either meters or feet ).

Once  the   data  is   entered,  the  analysis  computation  will

automatically be  executed, and  the  results  displayed  on  the

screen. The  program will  perform the  Student's  t-distribution

analysis to  determine if  the  constant  and  scale  errors  are

statistically significant  at the 1 per cent level and report the

results.

When the  analysis is complete, the user will be given the option

to print out the test data to the standard printer.

Program control will return to the Utility Menu when completed.

7.08 VERTICAL CURVE DESIGN - The Vertical Curve Design Program is

a powerful  utility that  allows  computation  of  station  grade

elevations  along   vertical  curves   and  connecting  tangents,

computation of vertical curve lengths that satisfy several design

criteria, batch  processing of  grades for  entire roadways,  and

conversion of cut/fill distances to feet and inches.

To activate  this utility,  press the  corresponding function key

from the Utility Support Menu.

The Vertical  Curve Design screen will appear, along with prompts

for printout  headings. The  default headings are those that were

set in  the System  Configuration Program.  To accept the default

headings, simply  press ENTER  at each prompt. Otherwise, type in

the desired heading and press ENTER.

The main  functions of  this program  are selected  from the menu

that appears at the top of the display. You can choose an item by

either pressing  the corresponding function key, or by moving the

highlight bar  to the  desired item using the cursor control keys

and pressing ENTER.

7.08.01 COMPUTE  GRADE TABLE  (F1) -   Pressing  F1  activates  a

routine  that  computes  grade  elevations  along  a  tangent  or

vertical curve. The following information will be needed:

G1(%) - Enter the beginning grade in per cent (such as 5.2 or

-3.0 ).

Section 7 - Utility Support

-82-

G2(%) - Enter the ending grade in per cent.

PVI Station:   Enter the vertex station in feet (that is, 2700.00

for station  27+00). If  the PVC  station is known, or preferred,

simply press  ENTER and  the prompt  will request the PVC station

instead. If  a previous PVI station appears as a default, and you

want to enter a PVC, press DEL to clear the field, and then press

ENTER to toggle over to the PVC prompt.

PVI Elevation:  Enter the elevation of the PVI. If you toggled to

a PVC  station, then  this prompt will request the PVC elevation,

rather than the PVI elevation.

Length: Enter the length of the vertical curve.

Increment: Enter  the desired station increment. When the program

first runs, the stationing default is 50 feet.

A table  of grade elevations will scroll into the display window.

A prompt  requesting any  odd stations  for which  elevations are

desired will  appear. Enter  any desired  stations (in  feet) and

press ENTER. To return to the menu, simply press ENTER or ESC.

Stations along  a tangent can be processed also. Simply enter the

same value  for the  ending grade  (G2) that  was entered for the

beginning grade.  This tells  the program that the computation is

not along  a curve.  Then, toggle to the PVC option and enter the

starting station  and elevation.  Enter the length of the tangent

over which  stations and  elevations are  needed at the "Length:"

prompt.

7.08.02 FIXED  POINT &  PVC KNOWN  (F2) -  This  routine  permits

solving for the length of a vertical curve that will pass through

a fixed  station and  elevation. The  beginning and ending grades

must be known, and in this case, the station and elevation of the

PVC. The following prompts will appear:

G1(%) - Enter the beginning grade in per cent (such as 5.2 or

-3.0 ).

G2(%) - Enter the ending grade in per cent.

PVC Station: - Enter the known PVC station, in feet.

PVC Elevation: - Enter the known PVC elevation.

Station: -  Enter the  known station  that the  curve  will  pass

through, in feet.

Elevation: -  Enter the  elevation of  the above station, through

which the curve must pass.

Section 7 - Utility Support

-83-

The length of the vertical curve satisfying the entered condition

will be computed and printed in the display window.

7.08.03 FIXED  POINT & PVI KNOWN (F3) - This routine also permits

solving for the length of a vertical curve that will pass through

a fixed  station and  elevation. The  beginning and ending grades

must be known, and in this case, the station and elevation of the

PVI. The following prompts will appear:

G1(%) - Enter the beginning grade in per cent (such as 5.2 or

-3.0 ).

G2(%) - Enter the ending grade in per cent.

PVI Station: - Enter the known PVI station, in feet.

PVI Elevation: - Enter the known PVI elevation.

Station: -  Enter the  known station  that the  curve  will  pass

through, in feet.

Elevation: -  Enter the  elevation of  the above station, through

which the curve must pass.

The length of the vertical curve satisfying the entered condition

will be computed and printed in the display window.

7.08.04 TURNING  ELEVATION &  PVI KNOWN  (F4) - This routine also

permits solving  for the length of a vertical curve, of which the

highest  or  lowest  elevation  (turning  point)  is  known.  The

beginning and  ending grades must be known, and in this case, the

station and  elevation of  the PVI.  The following  prompts  will

appear:

G1(%) - Enter the beginning grade in per cent (such as 5.2 or

-3.0 ).

G2(%) - Enter the ending grade in per cent.

PVI Station: - Enter the known PVI station, in feet.

PVI Elevation: - Enter the known PVI elevation.

Elevation: -  Enter the  elevation of the highest or lowest point

(turning point) of the curve.

The length of the vertical curve satisfying the entered condition

will be computed and printed in the display window.

Section 7 - Utility Support

-84-

7.08.05 ENTER/EDIT ROADWAY BATCH FILE (F6) - This routine is used

to create or edit a batch file of station and vertex data that is

used to process stations and elevations for an entire roadway.

By default, the editor used to create or edit roadway batch files

is BCEDIT.COM,  the same  editor used with Batched Cogo, which is

supplied with BASIS. This is a no-frills 80-column screen editor,

which functions  like a  rudimentary word-processor.  Although it

lacks the  features found  in more sophisticated text-editors, it

is very easy to use, and quite effective for the task at hand.

When you  invoke the  Enter/Edit option  from the  Vertical Curve

Design Menu,  you will  be asked  to supply the name of the batch

file to  edit. If  the file  exists, it  will be  loaded into the

editor. If  it does  not exist, it will be created. Roadway batch

files are  named like  any DOS  file. However,  the program  will

automatically append  the extension  ".RBC". Therefore, it is not

necessary to  type an extension. In fact, the program will "beep"

at you if you do attempt to add an extension. Roadway Batch Files

will be  stored in  the current data directory that is set in the

System Configuration program.

The batch file will appear on the screen, one command per line. A

"help" bar  will appear at the bottom of the screen. You can move

around using the cursor keys, PgUp, PgDn, Home and End. CTRL-PgUp

or CTRL-Home  positions the  cursor at  the top  of the file, and

CTRL-PgDn or  CTRL-End positions  the cursor at the bottom of the

file. Characters can be added by typing them in, or deleted using

Backspace or  Del. If you look at the "help" bar, you'll notice a

few editor  commands preceded  by a  '^' symbol. This means press

CTRL in  conjunction with the specified key. For example, to kill

a line (delete the line the cursor is currently at) press CTRL-K.

To restore  a line  that has been killed, press CTRL-Y (to "yank"

it back,  or "undo"  the kill).  Press F1 to end the edit session

and save  the edited BCF. Press F2 to save the BCF without ending

the session,  or press  CTRL-Z to quit the session without saving

any edited data.

You may  be used  to a  different text-editor, such as the Norton

Editor, Brief,  MicroEmacs, Wordstar,  or many  others.  You  can

force BASIS  to use  your editor using the DOS "set" command. Add

the line

SET EDITOR=myeditor.xxx

to your  AUTOEXEC.BAT file,  where 'myeditor.xxx'  is the name of

your favorite  editor. Be sure that your editor is in the default

path, or  that the path is included in the SET statement, so that

it can be found when BASIS calls it.

You don't have to use BCEDIT if you don't want to, however unless

you re-define the default editor, BASIS will use BCEDIT. Once you

re-define an  editor, it will always be the default editor, until

you re-define something different.

Section 7 - Utility Support

-85-

Please note  that BASIS will call the editor with the name of the

roadway batch  file as  a command line argument. Your editor must

be capable  of accepting  the edit  file  on  the  command  line.

Fortunately, most  do. Also,  if your editor calls in any support

or overlay  files, you'll have to locate them in \BASIS.DIR since

the DOS path only applies to executable files.

7.08.06 PROCESS  ROADWAY BATCH  FILE (F7) - After creating and/or

editing a roadway batch file, this routine is used to process the

data. It  will generate  a listing  of stations,  elevations  and

vertical curve data, as if each item was processed interactively.

The program  will check  that obvious  errors do not exist within

the data,  issuing a  warning message and terminating if an error

is found.  If hardcopy  is desired,  be sure  that the printer is

toggled on (using function key nine, F9).

7.08.07 PRINT  ROADWAY BATCH FILE (F8) - A roadway batch file can

be sent  to the  printer with this function. The printer does not

have to  be toggled  on when  this routine  is called, as it will

automatically attempt to send data to the printer, after checking

that the  printer channel  is active and ready to receive output.

Note that  only the  commands of  the batch file will be printed.

They will  not be  processed. This  is useful for checking a long

batch file,  or simply for keeping a hardcopy record of the batch

file.

7.08.08 ROADWAY  BATCH FILE  STRUCTURE -  A  roadway  batch  file

contains station/elevation  and vertex/curve-data information. It

may contain  non-processing comment  lines (useful  when  someone

else reads  over the  batch file)  and may also contain a station

increment override,  if an increment other than the default of 50

feet is desired.

A short sample batch file appears as follows:

;This is a comment

I 25

S 100 124.56

V 450.36 131.66 L300

V 675.12 128.32 M3.5

V 901.25 124.33 S850 125.00

S 1250 129.75

The above  example uses  all  vertex/curve-data  types  that  are

available.

Section 7 - Utility Support

-86-

The first  line is  an optional comment line, the first character

of which  must be a semi-colon. Comment lines can appear anywhere

in the file, and the processor simply ignores them.

The next  line is  the stationing increment specifier, which must

always be  the first  actual non-comment  line in  the  file.  Of

course, if  the default  increment of 50 feet is acceptable, this

line can  be omitted.  However, if  a station  increment is being

set, it  must be the first processing line in the file. Note that

the increment  line always starts off with an I (upper- or lower-

case) followed  by one  or more spaces, and the desired increment

value. In  the above  example, the  increment was  changed to  25

feet.

The next  line contains  the initial starting station, and begins

with the  letter S (upper- or lower-case) followed by one or more

spaces, the  station value  (in feet), and the station elevation,

separated by  one or  more spaces.  In  the  above  example,  the

station is  100 feet  (for 1+00), at an elevation of 124.56. Note

that a  roadway batch  must begin  with an  S (station/elevation)

record, before any vertex records can be used.

The next  three lines  contain various  forms of  vertex records,

which describe  vertical curves.  Vertex records  consist of  the

letter V  (upper- or  lower-case) followed  by  the  station  and

elevation of the PVI. The vertical curve is then specified by one

of three ways.

A length  of curve  can be given as in the first vertex record of

the sample.  In the example, L300 represents a length of curve of

300 feet.  Note that  no spaces are used between the L prefix and

the length of 300 feet.

In the  second vertex  record of  the example,  the length of the

curve is  not known,  however a middle ordinate, or distance from

the PVI  to the  curve, is  entered, from  which the program will

calculate the  length. Note  that there  are no spaces separating

the M  prefix from  the value  of 3.5  feet. Also note that the M

value is always positive. The program will apply the value in the

proper direction for sag or crest curves.

The  third  vertex  record  demonstrates  the  remaining  option,

passing the  curve through  a fixed  station and  elevation.  The

letter S  is used to indicate a fixed station, and is followed by

the station in feet, and elevation of the point through which the

curve must  pass. The  program will calculate the length of curve

to  satisfy   the  condition.  Note  that  there  are  no  spaces

separating the  S prefix  from the  station, however  one or more

spaces separate  the station  and elevation.  In the example, the

third vertex  record specifies  a vertical  curve with  a PVI  at

9+01.25, PVI elevation of 124.33, passing through station 8+50 at

an elevation of 125.00.

Section 7 - Utility Support

-87-

The final  record of  the sample  file is  the ending station and

elevation. This  is in  the form  of an S record. In the example,

the ending  station is  12+50,  elevation  129.75.  Just  as  the

roadway batch  must start with a station record, it also must end

with a  station record.  Attempting to  end with  a vertex record

will result in a "Premature End-of-File" error message.

Batch files  are also  permitted that run from station to station

with  no   vertex  records,   which  may  be  useful  in  certain

circumstances, such  as establishing  finish grades  along sewers

and  drains  for  layout  purposes.  Note  that  infinite  grades

(vertical) are  not permitted, which may occur if a user attempts

to enter an abrupt grade change, such as a drop manhole.

7.08.09 PRINTER  TOGGLE (F9) - This is used to toggle the printer

ON or  OFF. The  program will reflect the current hardcopy status

in the  lower right  corner of  the screen.  When  attempting  to

toggle the printer ON, the program will check the printer status.

If the  printer is not powered-on, a message will appear that the

printer is  not ready,  and give  the option  to Ignore or Retry.

Turn the  printer on,  and press  R for retry. Otherwise, press I

for ignore,  and the  program will  disregard the previous toggle

command.

7.08.10 CUT/FILL  CALCULATOR (F5)  - This  is a simple calculator

that converts  cut or  fill distances  in decimal  feet to  feet,

inches and  eighths. A  prompt will request the finished grade. A

second prompt  requests the  existing  grade,  such  as  a  stake

elevation. The  program will  then  calculate  the  cut  or  fill

distance,  displaying   the  answer   in  decimal  feet,  and  in

feet/inches. Press  ESC to  return to the menu. If the program is

requesting an existing grade, pressing ESC resets the calculator,

and pressing ESC a second time returns to the menu.

7.09 SIDESHOT  DATA ENTRY  PROGRAM  -  The  Sideshot  Program  is

designed to  facilitate rapid data entry of sideshot observations

from previously  established points.  While  the  field  traverse

system has  the capability  to process  sideshots,  the  Sideshot

Program provides  three additional  data types,  as well  as  the

ability to  change data  types, setups and backsights without re-

entering traverse specifications.

Select Sideshot  Data Entry  from the  Utility  Support  Menu  by

pressing the corresponding function key. The Sideshot screen will

appear, requesting  the name of the project file. Enter the BASIS

project file name.

Next a  prompt appears  asking if  you want  to save the sideshot

data to  a file. If you respond by pressing 'Y', then the program

will ask for an eight-character file name. This file will be used

to save  the raw  sideshot data.  If you anticipate adjusting the

Section 7 - Utility Support

-88-

values of  the control  points later  on, you can use the Process

Sideshot File program to recompute all the sideshots in the file.

You can  also use  the Edit Sideshot File program to edit the raw

data using a text editor.

Next, prompts will appear requesting the printer headings.

A window will then appear from which the initial data options are

selected.

You can  turn elevations and descriptions ON or OFF by responding

to the  prompts with  Y or  N. If you turn these options off, you

will not be prompted for their values when entering data.

Next select  a data  type from the list of six available choices.

Type 1  applies to data measured using horizontal angles (left or

right),  horizontal   distances,  and   optional  elevations  (if

elevations are ON). Type 2 is for horizontal angles and distances

as with  the first  type, but  with optional delta elevations and

reflector heights.  Type 3  is used with horizontal angles, slope

distances and  zenith angles.  Type 4 applies to data obtained by

reading horizontal  angles, three stadia hairs and zenith angles.

Type 5  is for  horizontal angles,  stadia distances  and  zenith

angles. Type  6 is  used for  points located by plus stations and

right or left offsets from a baseline, with optional elevations.

If elevations are ON, prompts for h.i.'s, h.r.'s and rod readings

will appear where appropriate.

If mode  4 is  being used, a stadia factor of 100 will be used to

compute the  distance. Mode  5 assumes  that the stadia intercept

has already  been multiplied  by a  stadia factor  for the stadia

distance, however  it will  compute the  horizontal and  vertical

distances using stadia formula.

As with  the Field  Traverse system, distances, h.i.'s and h.r.'s

entered as  negatives will  be converted to alternate units. That

is, if the system defaults distance units are in feet, a negative

distance will  be assumed  to be in meters, and converted to feet

before being  used in  computations. Conversely,  if the  default

distance  units   are  in  meters,  negative  distances  will  be

converted from  feet to  meters. The  conversion option  does not

apply to elevations, rod readings or elevation differences. These

values must  be entered in the same units for which the system is

configured.

When  entering  sideshots,  several  function  keys  are  active.

Funtion key  one (F1) toggles the printer on or off. Function key

two (F2) toggles the automatic point-protection option on or off.

Function key  three (F3)  is used to clear the current data entry

line and start over. Function key four (F4) is used to change the

instrument station  point  number  and  backsight  point  number.

Function key five (F5) pops up a window to list unused coordinate

Section 7 - Utility Support

-89-

point numbers.  Function key  six (F6)  is used  to  change  data

types.

Coordinates are  written to  the project  file when  each shot is

entered. There  are no  traverse files  to process  or edit. If a

mistake is made, simply re-enter the shot and overwrite the point

number.

Please note  that horizontal  angles must be either left (entered

as negatives)  or right angles. No provision is made for entering

bearings or  deflections. As  a result, angles are simply entered

in DDDMMSS  format. Angle prefixes, as used in the Field Traverse

System are not necessary and should not be used.

Press ESC to exit the program.

7.10 PRINTER  PLOTTER -  The BASIS  Printer Plotter  is a utility

that lets you plot points on a 9-pin dot-matrix printer.

The program  lets you enter point numbers, and it then determines

the orientation  to  an  8.5  x  11  inch  sheet  of  paper  that

accomodates the  largest plot  scale. You can accept the computed

scale, or  enter a  smaller scale. The program will then plot the

points and point numbers, along with a north arrow.

The first  prompt requests the BASIS project file. Enter the file

name and press ENTER. Pressing ESC terminates the program.

The next  prompt requests  how you  want to  enter point numbers,

either random points, or sequential blocks of points. Press R for

random points, or S for sequential points.

If you  select random  points, the  next prompt  requests a point

number. Type  in the point number and press ENTER. Once you enter

a point,  notice that  the default for the next point is the next

highest point.  You can  accept the default by pressing ENTER, or

type in  a different  point number.  Continue entering individual

points until  all points  have been entered. Note that a point is

not considered  'entered' until  you press  the ENTER  key. After

entering the last point number, press ESC when you are finished.

If you  select sequential  points,  the  next  prompt  to  appear

requests the  starting point  number for a series of points. This

is followed by a prompt requesting the ending point number of the

series. Unused  points within  the  series  will  be  ignored.  A

sequence can  be entered in either ascending or descending order.

When you are finished entering sequence blocks, press ESC.

The program will then compute the best orientation for the paper.

The largest  possible scale will be displayed. You can accept the

computed scale  or enter a smaller scale. Pressing ESC terminates

the program.

Section 7 - Utility Support

-90-

Next the  program requests  that you  position the printer at the

top of  a new  sheet. Press  any key  once you have done this, or

press ESC to cancel the program.

The program  will then  plot the  points. You can cancel plotting

while the printer is working by pressing ESC.

The program  has been  tested on  the  Epson  LX-810,  RX-80  and

Panasonic 1080i, 1091i and 1191 printers. However, it should work

on any compatible 9-pin dot-matrix printer.

Plot accuracy  is 1/72  of an  inch, depending  on the mechanical

quality of the printer.

IMPORTANT NOTE TO USERS OF 24-PIN PRINTERS...

such as the Panasonic 1124, Epson LQ series and others - In order

for the  Printer-Plotter program  to function correctly, you must

make a small change in the file UTILMENU.DAT.

From the  BASIS Main  Menu, press  F6 to shell to DOS, and at the

DOS command prompt type

BCEDIT UTILMENU.DAT

Then using the cursor control keys, move the cursor to the end of

the line for the Printer-Plotter and change it to

Printer-Plotter,pplot /24

Press F1 to save the corrected file.

If you  do not  do this  and you  attempt to  plot with  a 24-pin

printer, the  plotted points  WILL NOT  SCALE. Conversely, if you

change to  a 9-pin  printer, you  must remove  the '/24' that was

added. A message at the bottom of the program window will display

the current configuration as a reminder.

Plotting accuracy with 24-Pin printers is 1/60 of an inch.

7.11 EDIT  SIDESHOT FILE  - Once you have created a sideshot file

you can  use the  ASCII text-editor  supplied with BASIS (or your

own favorite,  if you've  defined it).  At the  prompt, enter the

eight-character file  name that  you  used  when  you  saved  the

sideshot data. Simply modify the entries that need to be changed,

and save  the file.  In  order  to  make  the  changes  you  make

effective, you'll  have to re-process the sideshot file using the

Process Sideshot File program

7.12 PROCESS  SIDESHOT FILE  - When  you want to process an ASCII

file of  sideshots that  you've edited,  or  possibly  after  re-

adjusting control  points, then  select this  program. Enter  the

Section 7 - Utility Support

-91-

eight-character filename  of the  sideshot data  file to process.

Next, enter  the name of the BASIS project file. The program will

automatically re-compute your sideshots.

7.13 MEMORY  RESIDENT CALCULATOR  - If  you find  the  BASIS  RPN

Calculator to  be a  handy utility,  the memory resident version,

TSRCALC.EXE is  even handier.  With this  program, you can access

the calculator from within any DOS program by simply pressing the

hotkey that activates the program.

To activate  the program,  type  \BASIS.DIR\TSRCALC  at  the  DOS

command prompt.  This will  load the  program into memory, and it

will stay there. By default, ALT F10 is the hotkey combination to

activate the  program. If  you want  to remove  the program  from

memory, the default hotkey to remove it is ALT F1.

You will  probably find  it convenient  to load  TSRCALC in  your

AUTOEXEC.BAT file, so that it is automatically loaded into memory

when  you  boot  your  computer.  Since  TSRCALC  uses  about  54

kilobytes of  RAM, you may also choose to load it "high" with DOS

5.0 or higher, or with a memory manager like 386Max or QEMM386.

Please note  that the calculator will only be active when you are

in "text" video mode. If you are running AutoCAD and want to pop-

up the  calculator, you must press the F1 key to toggle into text

mode.

If you  wish to change the hotkeys, you can assign different keys

when you load TSRCALC by using the /K command line switch.

The syntax   is  /Kaaabbb,   where aaa  and bbb  are  three-digit

groups that  correspond to the caculator and un-install key. Each

group  must   contain  three   digits,  with  leading  zeroes  if

necessary. And, of course, you can't use the same key code twice.

You can use any of the following:

Key Code       Corresponding Hot-Key

016 to 025     ALT Q,W,E,R,T,Y,U,I,O,P

030 to 038     ALT A,S,D,F,G,H,J,K,L

044 to 050     ALT Z,X,C,V,B,N,M

084 to 093     SHIFT F1 to F10

094 to 103     CTRL F1 to F10

104 to 113     ALT F1 to F10

114            CTRL PrtSc

115            CTRL left arrow

116            CTRL right arrow

117            CTRL End

118            CTRL PgDn

119            CTRL Home

120 to 131     ALT 1,2,3,4,5,6,7,8,9,0,-,=

132            CTRL PgUp

Section 7 - Utility Support

-92-

For example, if you wanted to use the keys CTRL-F1 and CTRL-F2 to

run the  calculator and  un-install (in  that order)  the command

line would look like

\BASIS.DIR\TSRCALC /K096097

If you   make  a  mistake with  this option, TSRCALC will use the

default hot-keys (and display a message to this effect.)

Memory-resident programs  are actually  "renegade" programs, that

may not  be fully  compatible with  all systems. Try TSRCALC from

the command  line of  your computer before you install it in your

AUTOEXEC.BAT file. TSRCALC has been successfully implemented on a

wide variety  of clones,  and has  proven very  popular with many

users. In  all probability, if BASIS runs on your system, TSRCALC

will run on it as well.

NOTE: TSRCALC, as the rest of the BASIS programs, requires a math

coprocessor (or a 486DX CPU.)

7.14 DATUM  SHIFTS -  This program permits translation and linear

scaling of  horizontal and  vertical coordinates in BASIS project

files. It  is especially  useful for metric conversions, shifting

project datum  coordinates to  state plane  coordinates,  or  for

converting to different vertical datums (linear variation only!)

While this is a handy program, and worth having as a utility, use

it with  caution. Careless  use could  wreak havoc  on a  project

file!

7.15 CUSTOMIZING  THE UTILITY SUPPORT MENU - You may  want to add

some "home-brew"  utilities of  your own  to the  Utility Support

menu. You may also want to re-order the items in the menu.

To facilitate this, the Utility Support Menu program has been re-

designed to  read an  ASCII file  named UTILMENU.DAT  (which must

reside in  the BASIS  program sub-directory).  You can  edit this

file using  a text-editor,  a word-processor  (in ASCII mode), or

the batch-command  screen editor  BCEDIT which  is supplied  with

BASIS.

UTILMENU.DAT, as supplied with BASIS looks like:

Sideshot Editor,sshot

Vertical Curves & Grades,vcurve

Directions to Angles,dangle

Short-Line EDM Reduction,edm

RPN Calculator,\basis.dir\rpncalc

Precise Slope Reduction,edmred

HP-41 Link,hpibm

MC-V Link,mcv

Automatic Data Reduction,afdr

Section 7 - Utility Support

-93-

Serial File Transfer,blink

NAD 27-83 Transformation,xform

Lightwave EDM Calibration,edmcal

Printer-Plotter,pplot#

You'll notice  that each  line of this file corresponds to a menu

item in the Utility Support Menu.

Each line consists of two fields, separated by a comma.

The first  field contains the name of the selection, as it should

appear in the menu. For example, in the second line

Vertical Curve Design,vcurve.exe

"Vertical Curve  Design" is  the selection  that appears  in  the

menu.

The next  field,"vcurve.exe", is  the file  specification of  the

program that  the menu  will call. Notice that the first field is

separated from the second field by a comma.

If you  add programs to the menu, be sure that you press ENTER at

the end  of each  line  that  you  add.  Please  note  that  only

executable programs  (.EXE and  .COM)  can  be  called  from  the

Utility Menu.  You may  include a  path preceding  the executable

name if the program is not in the BASIS.DIR directory.

Command line  arguments are  also supported  starting with  BASIS

2.0. Comand  line arguments should follow the program name and be

separated by  spaces, exactly  as you would enter them at the DOS

prompt.

Section 7 - Utility Support

-94-

8.0 DRAFTING  ENHANCEMENTS - BASIS features several routines

designed to  work within  the DCA/AutoCAD environment. While

the  use  of  DCA  software  is  highly  recommended,  these

routines will also work with un-enhanced AutoCAD.

8.01 INSTALLATION - The drafting routines are contained on a

separate diskette, but can be installed at the same time the

main installation program is used. If the install program is

used, you  will be  given the option to install the drafting

enhancements.

You can  also use  the installation  program to install only

the Drafting Enhancements by using the /D option. Insert the

installation diskette into a floppy drive and type

[d:]install /D

where [d:]  is the  drive in  which the diskette is mounted.

Follow the  directions that  appear on  the screen,  and the

program will install only the drafting routines.

When complete,  remove the original diskette from the floppy

drive, and  store it  in  a  safe  place,  preferably  at  a

different location.

If you  do not  use the  install program,  then use  the DOS

'COPY' command to copy all files from the \ACAD directory of

the Drafting  Enhancement diskette into your AutoCAD program

directory. Copy the files from the \ACAD\BLOCKS directory of

the diskette  into a  directory named C:\ACAD\BLOCKS on your

hard disk. If this directory does not exist, you must create

it, and  it MUST  be on  drive C. (If everybody used AutoCAD

Release 11,  this  wouldn't  be  necessary.  But  to  ensure

successful operation  with earlier  versions, the Lisp files

supplied with  BASIS only  look for  BASIS  Blocks  at  this

location.)

In addition  to placing  the necessary  routines on the hard

disk, the  AutoCAD file  ACAD.PGP will  be modified to allow

AutoCAD to call the new programs. The new lines that will be

added to ACAD.PGP are:

DMS,C:\BASIS.DIR\DMSMENU.EXE,256000,,0

DANG,C:\BASIS.DIR\DANGLE.EXE,88000,,0

EDM,C:\BASIS.DIR\EDM.EXE,88000,,0

DRAFT,C:\BASIS.DIR\DRAFTSEQ.EXE,256000,,0

BAS,C:\BASIS.DIR\BASISIMP.EXE,128000,,0

BASEXP,C:\BASIS.DIR\BASISEXP.EXE,128000,,0

COGO,C:\BASIS.DIR\COGO.EXE,500000,,0

FIELD,C:\BASIS.DIR\FLDTRAV.EXE,500000,,0

BASX,C:\BASIS.DIR\BASSECT.EXE,128000,,*

SSHOT,C:\BASIS.DIR\SSHOT.EXE,256000,,0

Section 8 - Drafting Enhancements

-95-

The drive letter will be different if you installed BASIS on

a  drive  other  than  C.  If  you  are  manually  modifying

ACAD.PGP, you  should substitute  the drive  letter on which

BASIS is installed for C: in the above entries.

The installation  program will  save the  original  ACAD.PGP

file as  ACADPGP.BAK. If  you are  re-installing an  updated

version, the  installation program  will not  duplicate  the

above  lines,  although  it  will  modify  them  as  needed,

depending on the nature of the updated programs.

NOTE -  If DCA  Software is not being used, be sure that the

system variables "USERR1" through "USERR5" are not to set to

any values  other than zero. DCA Software makes use of these

values, and  this program  will  interpret  the  DCA  values

correctly. However,  other third-party  enhancement software

may use  these variables  in  different  ways,  which  would

definitely confuse  BASIS! BASIS  Software only  reads these

values. It does not alter them.

One final  note concerns  the BASIS  point block, POINT.DWG.

This block  is compatible  with DCA's  point block  with the

same name. However, the BASIS point block is styled slightly

different from  the DCA  point block.  The BASIS point block

uses a  slant simplex  text style (DCA uses standard "TXT"),

and the  text is  equivalent to  L50, which  is smaller than

DCA's. If  you prefer  the DCA  point block, do not copy the

BASIS POINT.DWG  into  C:\ACAD\BLOCKS,  or  be  prepared  to

restore the  DCA POINT.DWG from your original DCA diskettes.

If you  use the  install program, the BASIS point block will

overwrite  the   DCA  point   block  if   it  is   saved  in

C:\ACAD\BLOCKS. If  you are  using DCA  Release 11 this will

not be a problem.

8.02 THE  DRAFTING SEQUENCE  - The Drafting Sequence Utility

lets you  create, edit  and process  files of point-to-point

plotting instructions  using BASIS coordinate point numbers.

You can  plot lines  and arcs,  as either  line or  polyline

entities. Data  entry is  similar to  entering points for an

area traverse in cogo.

8.02.01 ACTIVATING  THE ROUTINE  FROM A  DRAWING - To invoke

the  Drafting  Sequence  Program  from  within  the  AutoCAD

Drawing Editor,  type (load "seq") at the command prompt. Be

sure to include the parentheses. (This can also be placed on

a menu.  See the  AutoCAD manual  for details on customizing

menus, or  contact the  author of BASIS.) The graphic screen

will be  turned off (on single screen systems) and the BASIS

Drafting Sequence window will appear.

Section 8 - Drafting Enhancements

-96-

8.02.02 FILENAMES  - The first prompt to appear will request

the name  of the BASIS coordinate file that you want to work

with. The  default will  be the  last file used, and you can

accept the  default by simply pressing ENTER. If you want to

use a  different file, type in the name of the desired file,

and press  ENTER. If  you change your mind, and want to quit

the program, you can press ESC to return to AutoCAD.

Next, the program requests the name of the Sequence/DXF file

name. Keep  in mind that this program will create two files.

The first  is the  binary 'sequence' file, that contains the

points numbers in the sequence for connecting them together.

The second  file is  the DXF  text file,  that contains  the

actual information  AutoCAD uses to import the entities into

the drawing.  Both  files  share  the  same  name;  however,

sequence files end with .SEQ and DXF files end with .DXF.

When you  enter the  Sequence/DXF file  name, only enter the

first eight  letters (or  less) and do NOT add an extension.

The program  will automatically append either a .SEQ or .DXF

extension as needed.

The default  Sequence/DXF file  name  is  the  same  as  the

drawing name. You can accept it by simply pressing ENTER, or

you can type a new name.

8.02.03 THE  BASIS SEQUENCE  MENU - At this point, the BASIS

Sequence Menu  appears. Select  the  function  you  want  by

either pressing the corresponding function key. You can also

select a  function by  using the  cursor  movement  keys  to

highlight your  selection, and  then pressing  ENTER. Or, if

you are  using a  mouse, you  can click on the item that you

want.

Generally, the  order of steps for creating and processing a

BASIS drafting sequence is:

1.   Press F1  to either  start a new sequence, or add to an

existing sequence.

2.   Press F5 to create (or re-create) a DXF file.

3.   Press ESC to return to AutoCAD.

Once you  start a  sequence, any  data  that  you  enter  is

automatically saved  to disk.  You can  come back  later  to

edit, add  or delete  entries and  perform 'global' edits of

layers and point numbers. If you're using DCA and you decide

that you want to change the base point and rotation angle of

a drawing,  you can  're-create' the  DXF file,  and the new

transformation factors  will  automatically  be  taken  into

consideration.

Section 8 - Drafting Enhancements

-97-

8.02.04 ENTERING  A DRAFTING  SEQUENCE - Entering a drafting

sequence is  very easy.  Select F1 - Enter/Add Sequence Data

from the  menu. If  the  sequence  file  already  exists,  a

warning will  appear, and  you  will  be  given  options  to

overwrite the  file, add new data at the end of the existing

file, or  quit back  to the menu. Select what you want to do

by pressing either 'O', 'A' or 'Q'.

A series  of prompts  will then appear, requesting the point

numbers that  you want  to connect.  Simply enter  the point

numbers to connect in response to the FROM: and TO: prompts.

Curves  are  entered  by  entering  the  FROM:  point  as  a

negative, which  indicates the P.C. of the curve. Next enter

the P.T. point in response to the TO: prompt. A third prompt

appears,  requesting   the  RADIUS  PT:.  If  the  curve  is

clockwise, enter  the point  number of the radius point as a

positive number.  If the curve is counterclockwise, enter it

as a negative. The procedure is nearly identical to the Cogo

Area Computation.

Several function keys are activated when entering a drafting

sequence, and the "Hotkey" menu appears at the bottom of the

window.

Function key one (F1) is the "Recover" key, which resets the

current sequence  record to  the FROM: prompt. It is used if

an error  is made  and the  current  sequence  must  be  re-

entered.

Function key  two (F2)  is the  "Backup"  key,  and  permits

stepping backwards  to the  previous sequence if a series of

errors were made.

Function key  three (F3)  is the  "Plot Thru"  key,  and  if

pressed in  response to the TO: prompt allows the generation

of a sequence of lines.

For  example,   suppose  points   one  through  45  must  be

connected. Rather  than enter  each individual line, enter 1

in response  to the  FROM: prompt, then press F3 in response

to the  TO: prompt.  The prompt  PLOT THROUGH:  will appear.

Enter the  last point  number and  press return. The program

will automatically  create the  sequences to  connect points

one thru 45.

Function key  four (F4)  allows selection  of  the  sequence

layer from  the available  layer table.  To  change  layers,

press F4 , enter the desired layer name, and press ENTER. To

view the  layer table,  respond to  the LAYER: prompt with a

question mark.  The default  layer is  the current  sequence

layer. If  you enter  a '?', a list of available layers will

appear on the screen.

Section 8 - Drafting Enhancements

-98-

When you  are finished  entering points, press ESC to return

to the menu.

8.02.05 ADDING  TO A  SEQUENCE - If the "Add" data option is

selected for  an existing sequence/script file, then the new

sequence data  will  simply  be  appended  to  the  existing

sequence file. Otherwise, operation is identical to that for

entering a new sequence, as described above.

8.02.06 EDITING  A SEQUENCE - To edit a sequence file, press

function key two (F2) from the menu.

To edit  a record, simply respond to the Record No. To Edit:

prompt by  typing in  the sequence  number that  needs to be

changed. The  "Old" sequence  record will  be displayed, and

the user  will be  prompted  for  the  "New"  sequence.  The

default responses are the "Old" values.

You can  also locate  sequence records  to edit by searching

for a point number. Press F2 and enter the point number that

you want  to find.  If found,  the program  will display the

record containing the point number for editing. If you press

F3, the  program will  search for the next record containing

that point.

You can  also delete  records. If  you want  to  delete  the

record that  is currently displayed, press F4. A prompt will

appear requesting  confirmation. Respond  by pressing 'Y' to

delete the  record. You  should be  aware that when a record

number is  deleted, all  subsequent record numbers drop down

by one.

When all editing is completed, press the ESC key.

8.02.06 GLOBAL  EDITING -  Pressing function  key three (F3)

activates the global edit routine. Sometimes, you might want

to make a change that affects many records. For example, you

might want  to change the layer for records 100 through 300.

You can do this using global editing.

Global editing  lets  you  change  layers,  or  shift  point

numbers for  a block  of records. The first prompt to appear

requests your  selection of  either  of  the  two  permitted

global edits.  Press 1  to edit  layers, or press 2 to shift

point numbers. Of course, you can press ESC to cancel global

editing and return to the menu.

If you  are editing layers, the next prompt requests the new

layer that  you want to use for the block of points. Type in

the desired  layer.  Typing  '?'  will  display  a  list  of

available layers.

Section 8 - Drafting Enhancements

-99-

If you  want to  shift point  numbers,  enter  the  constant

number that you want to be added to all point numbers in the

block of  records you are editing. This number can be either

positive or  negative. For  example, suppose  you created  a

sequence for  connecting points  one through  100. Later you

realize that you really meant to connect points 1001 through

1100. You  would enter a shift value of 1000 and apply it to

the block of records.

Once you have selected the type of edit, and entered the new

data, the  program will  ask for  the  starting  and  ending

record numbers  for the  corrections. By default, the entire

sequence is selected. Accept the defaults by simply pressing

ENTER at  the prompts  for the  starting and  ending  record

numbers. Otherwise,  enter the  actual record  numbers  that

define the block you want to edit.

8.02.07 PRINT  SEQUENCE -  Pressing function  key four  (F4)

activates the print sequence routine.

When this  routine is  activated,  you  can  supply  printer

headings, and  generate a paginated hardcopy of the sequence

file.

You can cancel printing that is in progress by pressing ESC.

8.02.08 CREATE/RE-CREATE  DXF FILE  -  Typically,  the  last

thing you'll  want to  do before  returning to AutoCAD is to

create a  DXF file  of the  entities in your sequence. To do

this, press function key five (F5) from the menu.

Prompts will  appear  requesting  the  starting  and  ending

record numbers  for the  DXF file.  By default,  the  entire

sequence will  be processed,   but you can restrict creation

to a  specific block of records if you don't want to process

all the records.

Next, a  prompt appears  requesting if  you  want  LINES  or

POLYLINES.

If you  select 'L'  for LINES,  each  sequence  record  will

result in  an individual drawing entity, either a LINE or an

ARC, in the DXF file.

If you  select 'P'  for POLYLINES,  the program  will create

single polyline  entities for  all continuous  sequences  of

points. (Note:  Continuity is determined by sequence records

starting at  the same  point number that the previous record

ended at. A layer change will disrupt continuity, and result

in a new entity being started.)

Section 8 - Drafting Enhancements

-100-

8.03  IMPORTING   BASIS  POINTS   -  You  can  import  BASIS

coordinate points,  along with  elevations and descriptions,

into your  AutoCAD drawing  using this  routine. If  you are

using DCA  software, the  program will  automatically detect

the rotation  and base  point of  your  drawing  and  import

points properly.

At the AutoCAD command prompt, type

(load "basimp")

You could  place this  expression in  a custom  menu. If you

want to do this, refer to the AutoCAD manual for information

on customizing menus.

The first  prompt to  appear requests  the name of the BASIS

coordinate file.  The default  is the  last file used, which

you can  accept by simply pressing ENTER. Otherwise, type in

the name of the BASIS file you want, and press ENTER.

Next, the  program asks if you want to set a window boundary

for importing  points. You  can use  this option  if you are

only interested in extracting points from the file that fall

within a  rectangular region.  If you  press 'Y'  to  use  a

window boundary,  then  you  must  define  the  boundary  by

entering the maximum and minimum northings and eastings.

Next, the  program asks  if you  want to import descriptions

and elevations,  along with  the point numbers. Press 'Y' in

response to  each prompt  if you do. The default for each is

'N' which you can select by simply pressing ENTER.

If you selected 'Y' for elevations, the program will request

the decimal  precision of  the  elevation  values.  You  can

select 0 through 8 decimal places.

Next, the program asks if you want to enter point numbers at

random, or  if you  want  to  import  sequential  blocks  of

points. Press  'S' for  sequential points, or 'R' for random

points.

If you  select 'R', then you must enter each point number to

import and press ENTER after each. When you have entered all

the points you want to import, press ESC.

If you  selected 'S' for sequential points, then the program

will prompt  for the  starting and  ending point  numbers of

blocks of  points. You  can enter  more than one block. When

you are done, press ESC.

When you  return to  AutoCAD, you  will be asked if you want

'Standard' point  blocks, or  'Current-layer' point  blocks.

Standard  point   blocks  are   nearly  the  same  as  DCA's

Section 8 - Drafting Enhancements

-101-

coordinate  point   blocks  (the   text  style  is  slightly

different.) With standard point blocks, the point node comes

in on  the current layer, the point number comes in on layer

PNTS,  the   elevation  comes  in  on  layer  ELEV  and  the

description comes  in on  layer  DESC.  Current-layer  point

blocks come  in to  the drawing  with the point node and all

attributes on  the current layer, rather than PNTS, DESC and

ELEV. Either are compatible with DCA software.

8.04 EXPORTING  POINTS TO BASIS - This routine is similar in

operation to  the BASIS Import Routine, only it sends points

in the opposite direction. To export points from the drawing

to a BASIS coordinate file, type

(load "basexp")

at the AutoCAD command prompt.

There  will   be  a  pause  while  the  AutoCAD  points  are

transferred to  a temporary  file. The  length of this pause

will depend  on the  number of points in the drawing and the

speed of your computer. A counter will indicate the progress

of the point selection.

The first  prompt to  appear requests  the name of the BASIS

coordinate file.  The default  is the  last file used, which

you can  accept by simply pressing ENTER. Otherwise, type in

the name of the BASIS file you want, and press ENTER.

Next, the  program asks if you want to set a window boundary

for exporting  points. You  can use  this option  if you are

only interested  in saving  points in  the  file  that  fall

within a  rectangular region.  If you  press 'Y'  to  use  a

window boundary,  then  you  must  define  the  boundary  by

entering the maximum and minimum northings and eastings.

Next, the  program asks  if you  want to export descriptions

and elevations,  along with  the coordinates.  Press 'Y'  in

response to  each prompt  if you do. The default for each is

'N' which you can select by simply pressing ENTER.

The next prompt requests if you want to protect points. With

Point-Protect  enabled,   the  program  will  NOT  overwrite

existing  BASIS  coordinates.  Press  'Y'  to  enable  point

protection.

The program  now requests  blocks of  points to  save in the

BASIS coordinate file. You can enter more than one block.

Press ESC when you are done, to return to AutoCAD.

Section 8 - Drafting Enhancements

-102-

8.05 DIRECTIONS  TO ANGLES  - The  BASIS Utility  to convert

theodolite  directions  to  clockwise  angles  is  available

within AutoCAD.  To invoke  this utility,  type DANG  at the

AutoCAD command prompt. See section 7.09 for instructions on

using this utility.

8.06 SHORT  LINE  EDM  REDUCTION  -  This  utility  is  also

available from  within AutoCAD.  Type  EDM  at  the  command

prompt.  Instructions   on  the  use  of  this  routine  are

contained in section 5.02.17

8.07 RUNNING  COGO FROM  AUTOCAD - A very convenient feature

of the  BASIS system  of enhancements  is that it allows the

Interactive Coordinate  Geometry routines to run from within

AutoCAD. To  call the  Cogo routine, simply type COGO at the

AutoCAD command  prompt. The familiar BASIS Cogo screen will

appear, and  the user  can perform Cogo calculations in that

environment. Upon  exiting, the  AutoCAD drawing editor will

be restored.

Note that  new points  created  using  Cogo  must  still  be

imported to the drawing using the BASIS Import routine. This

gives the  user complete  control over what appears and what

does not appear on the screen.

8.08  OTHER  AutoLISP  UTILITY  PROGRAMS  -  A  few  utility

programs are  included with  the BASIS Drafting Enhancements

package.  These   are  AutoLISP  routines  for  drawing  and

labelling  a  coordinate  grid  at  a  user  specified  grid

interval, and  a routine  for spotting  tic-marks at desired

angle-points.

8.08.01 GRIDMKR  - This  routine is executed by typing (load

"gridmkr") at the AutoCAD command prompt. Be sure to include

the parentheses.  The routine will prompt for the diagonally

opposite corners of the area where the coordinate grid is to

appear. These  points can  be selected  using the  mouse,  a

digitizer, or  by entering  X,Y  AutoCAD  coordinates.  When

selecting these  points, a  highlighted box  will appear  on

screen (  similar to  the "crossing"  window). Next a prompt

will appear requesting the grid interval. If DCA is present,

the default  will be whatever interval satisfies a five inch

grid requirement. If DCA is not present, the default will be

200 feet.  Enter the  desired grid interval and press ENTER,

or simply  press ENTER  to accept the default. A rectangular

grid will  be drawn  that includes  the area specified. Note

that the grid will be drawn on the current layer.

Section 8 - Drafting Enhancements

-103-

8.08.02 LBLGRID  - This utility permits the grid lines drawn

using GRIDMKR  to be  labeled with the appropriate Northings

and Eastings.

Since the  grid area  will most  likely be  trimmed  to  the

borders  of   the  sheet,  the  first  prompt  requests  the

endpoints of  the alinement line for labeling the northings.

This line  is the  "guide line" and the grid label text will

start at  this line,  where it intersects each grid line. In

most cases,  a line  will be  selected that is parallel to a

sheet edge, however any line can be chosen.

Next, a prompt appears requiring that the lowest Northing to

be  labeled   be  indicated   by   picking   any   grid-line

intersection on  the lowest East to West grid line ( i.e the

lowest northing) to label. Similarly,the highest Northing to

be labeled  is then  indicated by picking an intersection on

the highest  grid line to label. The next prompt is the grid

interval. The  default is  for a  five inch  grid if  DCA is

present, otherwise  the default  is  for  a  200  foot  grid

interval. To  accept the  default, press  ENTER.  Otherwise,

enter the desired labeling interval, and press ENTER.

A series  of prompts,  similar to  the  above,  will  appear

requesting the same data for labeling the Eastings.

The grid  lines will  be labeled  in the current text style,

and on the current layer.

8.08.03 HASH  - This program can be used to place Hashmarks,

or Tics at angle-points between lines, or at P.C.'s, P.T.'s,

etc.

The hashmarks  will extend  equally on  either side  of  the

point (  unlike AutoCAD  tic marks  ), and  will bisect  the

angle formed  by the  two intersecting lines. For P.C.'s and

P.T.'s, the hashmark is drawn at 90� to the tangent.

To invoke  the utility,  type (load  "hash") at  the AutoCAD

command prompt.  If DCA  is present,  the hashmark  will  be

sized so that it is 0.15 inches in length on the drawing. If

DCA is  not present,  then the  program will  prompt for the

desired hashmark length (in feet). For example, if a drawing

is to  be plotted at a 50 scale, and the hashmarks are to be

0.15 inches  long, respond  to the length prompt by entering

7.5 (feet).

Select Hashmark  Point: prompts  for the  selection  of  the

point where  the hashmark is to be placed. The default OSNAP

mode is  for Intersec, however this can be overridden in the

usual fashion. Select the point, using a pointing device, or

by entering  the X,Y  AutoCAD Coordinates.  Next,  a  prompt

appears  requesting  that  a  point  on  one  of  the  lines

Section 8 - Drafting Enhancements

-104-

intersecting at  the hashmark  point be selected. This point

is for  direction only.  Next a  prompt appears requesting a

point on  the second line. The routine uses this information

to calculate  the direction  of  the  hashmark  so  that  it

bisects the  angle between  the two  lines selected. In some

cases, such  as a  point of  curvature, the bisection option

may be undesirable. In cases such as this, pick any point on

the tangent  as the  first point  on line, then simply press

ENTER in  response to  the prompt  for the second point. The

hashmark will be drawn at 90� to the first line selected.

To exit  the routine,  simply press  ENTER at  the prompt to

select the hashmark point.

8.09 DRAWING  SUBDIRECTORIES -  As supplied,  the  BASIS.BAT

boot program  will load  and run  DCA/AutoCAD from a drawing

subdirectory named  DRAWINGS. This  is  easily  modified  by

changing the  line in  this batch  file from CD \DRAWINGS to

whatever  subdirectory   the  user   wishes  to  define  for

drawings. This  is done  using a text editor, word processor

(in non-document  mode) or  with BCEDIT,  which is  supplied

with BASIS.  If the  BASIS2.BAT startup  batch file is used,

then a  prompt will  request the drawing directory each time

AutoCAD is called from the BASIS Main Menu.

8.10  RUNNING  FIELD  TRAVERSE  FROM  AUTOCAD  -  The  Field

Traverse System  can be  executed from  the AutoCAD  drawing

editor. Be sure the ACAD.PGP file contains the line

FIELD,C:\BASIS.DIR\FLDTRV.EXE,500000,,0

If  the  Drafting  Enhancements  were  installed  using  the

installation program,  this will  automatically be  added to

ACAD.PGP. To  run the  Field Traverse  System, type FIELD in

response to the AutoCAD command prompt.

8.11 CROSS  SECTIONS AND PROFILES - This program is designed

to facilitate  entering terrain  profiles and  sections that

are field run with a total station.

To use  this program,  the files BASSECT.EXE and BASSECT.LSP

must be in the AutoCAD program directory. Also, the line

BASX,C:\BASIS.DIR\BASSECT.EXE,128000,,*

must appear in the ACAD.PGP file.

From the AutoCAD drawing editor, enter the command

(load "bassect")

Section 8 - Drafting Enhancements

-105-

A window  similar to  the BASIS  Import Points  window  will

appear on the screen.

A series  of BASIS points numbers can be entered to define a

profile or cross-section.

As with  the standard  BASIS point  import routine,  you may

select "Standard"  DCA-type blocks,  where  the  point  node

comes in  on the current layer, the point number comes in on

layer PNTS,  the elevation  comes in  on layer  ELEV and the

description comes  in on  layer DESC.  Or,  you  may  select

"Current" layer  point blocks, where the node, point number,

elevation and description all come in on the current layer.

If DCA is being used, the point text will be sized according

to the horizontal scale factor of the drawing. If DCA is not

detected, the  program will  request the size of the text in

feet. You should consider what scale you intend to plot, and

choose your text size accordingly.

Remember, by  default DCA  scales a  drawing at 1"= 1' until

you select  a horizontal  scale in  SETUP. If an appropriate

scale is not selected, the point text attributes will appear

very small. It's a good idea to create prototype drawings at

various scales  so you  don't have  to go  through the SETUP

procedure all the time.

The program  then requests  that you pick the starting point

to draw  the cross-section  or profile. Points will be drawn

from left  to right  from this  starting point, according to

the sequence  in which  they were  entered. The  AutoCAD 'Y'

coordinate will  be based  on the  elevation  of  the  point

relative to  the starting  point that is picked. The AutoCAD

'X' coordinate will be based on the horizontal distance from

the previous  point. For this reason, you may want to draw a

grid first,  and place  the starting  point at some point on

the grid based on its horizontal position and elevation.

If DCA  is detected,  the elevations  may be  plotted at  an

exaggerated scale,  if the  horizontal  and  vertical  scale

factors are  not set  equal to each other. Usually, profiles

are drawn  at an  exaggerated vertical  scale, while  cross-

sections are  plotted with  the same horizontal and vertical

scales.

If DCA  is not  detected, the  program  will  prompt  for  a

vertical exagerration factor. For example, if the horizontal

scale is  40 feet  to the  inch, and the vertical is to be 4

feet to the inch, use a vertical exaggeration factor of 10.

8.12  RUNNING  THE  SIDESHOT  PROGRAM  FROM  AUTOCAD  -  The

Sideshot Data  Entry program  can also  be executed from the

Section 8 - Drafting Enhancements

-106-

AutoCAD drawing  editor. Be  sure the ACAD.PGP file contains

the line

SSHOT,C:\BASIS.DIR\SSHOT.EXE,256000,,0

If  the  Drafting  Enhancements  were  installed  using  the

installation program,  this will  automatically be  added to

ACAD.PGP.  To  run  the  Sideshot  program,  type  SSHOT  in

response to the AutoCAD command prompt.

8.13 RUNNING  THE DISK  MANAGEMENT SYSTEM FROM AUTOCAD - The

Disk Management System can also be executed from the AutoCAD

drawing editor. Be sure the ACAD.PGP file contains the line

DMS,C:\BASIS.DIR\DMSMENU.EXE,256000,,0

If  the  Drafting  Enhancements  were  installed  using  the

installation program,  this will  automatically be  added to

ACAD.PGP. To  bring up the DMS menu, type DMS in response to

the AutoCAD command prompt.

Section 8 - Drafting Enhancements

-107-

9.0  BATCHED   COORDINATE  GEOMETRY   -  Batched  Coordinate

Geometry is  invoked by  pressing function  F8 from the main

menu. As  can be  seen from  the Batch  Cogo Menu,  shown in

Figure 9.1,  this system  of programs  consists of  two main

functions,  Interactive   Batch  Cogo   and  Batch   Command

Processor, and  two  support  functions,  Edit/Create  Batch

Command File  and Print Batch Command File. An option to re-

define the  default editor  is also  provided. Press  ESC to

return to the previous menu.

F1 - Interactive Batch Cogo

F2 - Batch Command Processor

F3 - Edit/Create Batch Command File

F4 - Print Batch Command File

F5 - Re-define Default Editor

F6 - Help

Figure 9.1 - The Batched Cogo Menu

9.01 INTERACTIVE BATCH COGO - To execute this program, press

function key F1 from the Batch Cogo Menu.

This function  is essentially the same as normal Interactive

Cogo. The only noticeable difference is that after supplying

the usual  BASIS project  filename and  printer headings,  a

prompt window  appears requesting  the name  of  the  "Batch

Command File".

A Batch  Command File  (BCF) is where all Cogo commands will

be  recorded.   You  may  enter  up  to  eight  alphanumeric

characters  to   name  a   BCF,  following  the  normal  DOS

conventions of allowable characters for filenames. Please do

not append  an extension  to the  name, or  a path. You will

only be allowed to enter eight characters. Pressing ESC will

bring you back to the Batch Cogo menu.

BASIS will  automatically append  the extension  .BCC to the

name you  enter, and  either look  for or create the file in

the current  default data  path, as  specified in the System

Configuration Program  (the same  place BASIS  project files

are saved.)

If the  file is a new file, the program will ask if you want

to  create  it.  Press  Y  or  ENTER  to  create  the  file.

Otherwise, press N or ESC.

If the  file already exists, you will be given the option to

add Cogo  commands to  it, over-write  the existing data, or

quit (and  re-enter a  filename.) Press O to overwrite, A to

add data, or Q to quit. The default response is A.

Section 9 - Batched Coordinate Geometry

-108-

If you  select either  A or  O, then  a final  ALL OK (Y/N):

prompt will appear. Press Y or ENTER to proceed, or N or ESC

to re-enter the filename.

Once you  enter  Batch  Cogo,  you  will  find  that  it  is

identical to  Interactive Cogo.  The only difference is that

every command  you process  is being  saved in  the BCF  you

specified.

This applies  to almost all Cogo commands. However, the pop-

up utilities  are not  passed to  the BCF.  These consist of

Least Squares  Orientation,  Linear  Regression,  EDM  slope

reduction,   Directions    to   Angles,    Radial    Layout,

Station/Offset, Point  Search, Unused Point Listing  and the

Scientific Calculator.  These functions  are  available  for

interactive use,  but will  not  be  written  to  the  Batch

Command File.

All other  genuine Cogo  operations are  passed to  the BCF,

including turnarounds,  area and  defined area computations,

intersections,   curve    routines,   comments,   coordinate

translation and rotation, consecutive inverse, etc

Be aware  that invalid  commands will not be saved. If BASIS

flashes an "Invalid..." message, that command was ignored as

far as  the BCF  is concerned.  However,  if  you  edit  the

invalid command  and successfully  reprocess it,  it will be

saved.

Also be  aware that  the original  command is what is saved.

For example,  if you were traversing from point 155 to point

156 the command may look like

155 B1453030 D165.26 156

but when you press the backslash key to process the command,

BASIS may  warn you that point 156 is in use, at which point

you enter  157 to store the point at number 157, rather than

156. The original command requesting storage at point 156 is

what is  saved. However,  you will  be given  the option  to

enable point-protect  when you  run the  Process Batch  file

program later on.

9.02 BATCH  COMMAND PROCESSOR  - This function is invoked by

pressing function key F2 from the Batch Cogo Menu.

After creating  a Batch Command File (BCF) using Interactive

Batch Cogo (described above) or by creating or editing a BCF

using the editor, this program is used to process it.

After  entering  the  BASIS  project  filename  and  printer

headings, you  will be  prompted for the name of the BCF. As

with the  batch creation  program, enter  an eight-character

Section 9 - Batched Coordinate Geometry

-109-

filename,  following  the  DOS  convention  for  permissible

characters in filenames. Do not include a path or extension.

BASIS  automatically   appends  .BCC  as  an  extension  and

searches the  default data  path  specified  in  the  System

Configuration Program.

If the  named file  is not found, the program will re-prompt

for the BCF name.

If the  file is  found, BASIS  will then check to see if the

current project file is the same file that was used when the

BCF was  created. If  it is not, then a warning message will

be displayed, informing you that the current project file is

not the same as the originally linked project file. You will

be allowed  to proceed  if you type "YES" in response to the

prompt. BASIS  will disregard the originally linked file and

assume that  you want  to process  the BCF  into the current

project file.  If you type anything else, you will return to

the menu.

Next, a  prompt will appear requesting the maximum number of

errors that  will terminate  the run. The default is one (1)

error. You  may decide  that you  will tolerate  more errors

before causing  the run to terminate, but normally, you will

probably want to abort after one error.

Chances are,  if you created the BCF using Interactive Batch

Cogo, you  won't encounter  errors.  However,  if  you  have

created the  BCF using the editor, or have edited a BCF, you

may inadvertently  introduce errors  in the  command syntax.

For example,  you may  attempt to  traverse  from  an  empty

point. The  batch  processor  performs  the  same  check  as

Interactive Cogo  at run  time, and will keep a count of the

errors it  encounters.  Of  course,  bad  commands  will  be

ignored.

The next  prompt to  appear asks  if  hardcopy  is  desired.

Respond by  pressing Y  if a printout is desired, or N if no

hardcopy is needed.

Next, the  program will  ask if  you want  the point-protect

option turned  on. Respond  by pressing  Y to  enable point-

protect, or  N to  turn it off. If you enable point-protect,

BASIS will advise you to remain at the console to respond to

any "Point in Use" messages that may appear.

Finally, "ALL  OK (Y/N)?" appears. Review the parameters you

have specified,  and if all is satisfactory, press Y. If you

want to change something, press N.

If you  have selected  hardcopy, be sure that the printer is

selected and  loaded with  paper. The  batch processor  will

execute your BCF very quickly.

Section 9 - Batched Coordinate Geometry

-110-

9.03 EDIT/CREATE  BATCH COMMAND  FILE - A Batch Command File

(BCF) is  simply an  ASCII  text  file  that  contains  cogo

commands. You can edit BCF's created using Interactive Batch

Cogo, or you can create a BCF from scratch. Editing a BCF is

easy, however  you should  be familiar with the structure of

Cogo commands.  Most of these will be obvious if you've been

using Interactive  Cogo. However,  a  few  will  be  new  in

appearance.  The   new-looking  command   lines  consist  of

turnaround commands, translate and rotate, area, and defined

area.

Before getting  into the  actual process  of editing  a BCF,

let's talk  about the  editor itself.  BASIS 1.8 is supplied

with an ASCII text editor named BCEDIT, which is the default

editor. This  is a  no-frills 80-column screen editor, which

functions like  a rudimentary  word-processor.  Although  it

lacks the  bells-and-whistles of  the "big boys", it is very

easy to use, and quite effective for the task at hand.

When you  invoke the  Edit option  from the Batch Cogo Menu,

you will  be asked to supply the name of the BCF to edit. If

the file  exists, it  will be  loaded into the editor. If it

does not exist, it will be created.

The BCF  will appear  on the screen, one command per line. A

"help" bar  will appear at the bottom of the screen. You can

move around using the cursor keys, PgUp, PgDn, Home and End.

CTRL-PgUp or  CTRL-Home positions  the cursor  at the top of

the file,  and CTRL-PgDn or CTRL-End positions the cursor at

the bottom  of the  file. Characters  can be added by typing

them in,  or deleted  using Backspace or Del. If you look at

the "help" bar, you'll notice a few editor commands preceded

by a  '^' symbol.  This means press CTRL in conjunction with

the specified  key. For  example, to kill a line (delete the

line the  cursor is currently at) press CTRL-K. To restore a

line that  has been killed, press CTRL-Y (to "yank" it back,

or "undo"  the kill).  Press F1  to end the edit session and

save the edited BCF. Press F2 to save the BCF without ending

the session,  or press  CTRL-Z to  quit the  session without

saving any edited data.

You may  be used  to a  different text-editor,  such as  the

Norton Editor,  Brief, MicroEmacs, Wordstar, or many others.

You can  redefine your favorite editor as the default editor

by pressing  function key  F5 from  the Batch Cogo Menu, and

entering the  name of  your editor.  Be sure  to include the

path, unless your editor can be found in the DOS path.

You don't  have to  use BCEDIT if you don't want to, however

unless you  re-define the  default editor,  BASIS  will  use

BCEDIT. Once  you re-define an editor, it will always be the

default editor, until you re-define something different.

Section 9 - Batched Coordinate Geometry

-111-

Please note that BASIS will call the editor with the name of

the BCF  as a  command line  argument. Your  editor must  be

capable of  accepting the  edit file  on the  command  line.

Fortunately, most  do. Also,  if your  editor calls  in  any

support or  overlay files,  you'll have  to locate  them  in

\BASIS.DIR since  the DOS  path only  applies to  executable

files.

The first  line of  the BCF  always contains the name of the

BASIS project  file it was originally linked with. If you're

creating a  new BCF,  place the name of the project file you

intend to  link with in the first line. The remainder of the

file contains  the  Cogo  commands.  If  you've  been  using

Interactive Cogo,  you're already  familiar with most of the

Cogo commands.

If neither  a DOS  environment editor,  nor a default editor

are found, then BASIS will use BCEDIT.

9.04 PRINT  BATCH COMMAND  FILE -  Pressing function  key F4

calls a  program to  print a listing of a Batch Command File

(BCF). The  first prompt requests the name of the BCF. Enter

the name  of the  desired file,  up  to  eight  alphanumeric

characters  in   length,  and   following  the   usual   DOS

conventions for naming files. As with any of the other Batch

functions, do  not include any extension or directory, since

BASIS automatically  places  the  extension  .BCC  onto  the

filename you  enter, and  expects to  find the  file in  the

default data  path specified  in  the  System  Configuration

Program.

Next, enter  the printer headings, check that the printer is

loaded with  paper,  and  press  any  key  to  generate  the

listing. You  can cancel  executuion at any time by pressing

ESC.

9.05 RE-DEFINE  DEFAULT EDITOR  - By default, BASIS uses the

BCEDIT.EXE 80-column screen editor for editing Batch Command

Files. You  can easily change this default, so that when you

call the  editor, BASIS  loads your  favorite text-editor or

word processor.

Simply press  function key  F5 from the Batch Cogo Menu, and

type in the name of the editor of your choice. If necessary,

include the  drive and  directory in which it is located. If

it is in the DOS Path, this won't be necessary.

In order  to function properly, the editor or word processor

must be capable of accepting the edit file as a command line

argument, since  this is  how BASIS  will call  it. If  your

editor uses  external help  files or  overlays,  then  those

Section 9 - Batched Coordinate Geometry

-112-

external  files   will  have  to  be  saved  in  \BASIS.DIR,

otherwise your  editor probably  will not be able to find or

load them.  You may also try the DOS APPEND command, if your

version of DOS supports it.

If you  are using  a word processor, make sure that you save

the edited  file as  a straight  ASCII file (non-document or

unformatted) since  BASIS will  not  recognize  the  special

control codes  that most word processors place in their .DOC

files.

There is  one other  way to specify a default editor without

using this  option. You  can use  the DOS  'set' command  to

place  the   name  of   the  default  editor  into  the  DOS

environment string.  For  example,  to  specify  the  Norton

Editor, place the following line in AUTOEXEC.BAT

set editor=ed.com

When you  edit a  Batch Command File, BASIS will first check

to see  if an  editor is  specified in  the DOS environment.

Next,  it  looks  for  \BASIS.DIR\DFLTNAM6.DAT,  which  will

contain a  user-specified editor,  created using F5 from the

Batch Cogo Menu. If it finds this file then the editor named

within will  be used.  If neither  a DOS environment editor,

nor a default editor are found, then BASIS will use BCEDIT.

9.06  SAMPLE   BATCH  COMMANDS  -  Let's  review  the  basic

commands. The  parenthetical remarks  are explanations,  and

are not a part of the command.

Traverse

20 B1453030 D200.56 21

(From 20, N 45�30'30" E, 200.56, to 21)

123 B5*6 D12.3+52.4 127

(From 123, bearing 5*6, distance 12.3+52.4, to 127)

15 B1 D34*35 16

(From 15, stored bearing #1, distance 34*35, to 16)

and so on, exactly like Interactive Cogo.

Inverse

1 2

(Inverse between 1 and 2)

1 -15

(Consecutive inverse, 1 to 2, 2 to 3,...,14 to 15)

Section 9 - Batched Coordinate Geometry

-113-

Intersections

14 B3274530 500 B1*2 15

(From 14, S 27�45'30" W, solve 500, bearing 1*2, to 15)

33 D100.56 -250 D256.00 35

(From 33, distance 100.56, solve 250 [ccw], distance 256.00,

to 35)

216 B8 -495 D300 100

(From 216,  stored bearing  8, solve  495  [near],  distance

300.00, to 100)

Curve Inverse

I 200 202 -201

(PC 200, PT 202, RP 201 [ccw])

R Curve

R 200 201 300 A100.33

(PT 200, RP 201 [cw], solve 300, arc length 100.33)

R 301 -302 303 D213015

(PT 301, RP 302 [ccw], solve 303, Delta 21�30'15")

R 781 500 999 C1*2

(PT 781, RP 500 [cw], solve 999, Chord distance 1*2)

P Curve

P 2 2*1 2*3 100 R250.00 50

(PI 2, bearing PI to PC 2*1,bearing PI to PT 2*3, solve 100,

Radius 250.00, width 50.00)

P 100 1453030 4323120 201 T50.56

(PI 100,  bearing PI  to PC N 45�30'30" E,bearing PI to PT N

32�31'20" W,

solve 201, Tangent 50.56, no width specified)

Section 9 - Batched Coordinate Geometry

-114-

Comment

$Here's a comment statement

List Coordinates

L 1 25

(List coordinates from 1 through 25)

Store Coordinate

S 1000 1500 1

(Northing 1000.00, Easting 1500.00, store as point 1)

Three Point Curve

T 14 15 16 100

(Known points 14, 15, 16, solve point 100)

Translate/Rotate

X 503 99721.36 376554.23 576 3454039 500 700

(Rotate  about  503,  new  northing  99721.36,  new  easting

376554.23, old bearing to point 576, new bearing S 45�40'39"

W, start translate/rotate at 500, end at 700)

Turnarounds

C TYPE1 50 20 30 50 200 3 1*3

(Type 1 [Symmetric], turnaround radius 50.00, tangent 20.00,

return radius  30.00, R.O.W.  width 50.00,  solve point 200,

centerline PC point 3, PC to Center bearing 1*3)

C TYPE1 60 0 25 50 125 -75 75*74

(Type 1  [Symmetric], turnaround  radius 60.00,  no  tangent

[0], return  radius 25.00,  R.O.W. width  50.00, solve point

125, center point 75, bearing center to centerline PC 75*74)

C TYPE2 50 0 30 60 175 164 164*163.1

(Type 2  [Offset, off tangent, clockwise], turnaround radius

50.00, no  tangent [0],  return radius  30.00, R.O.W.  width

60.00, solve  point 175,  PC point 164, bearing PC to Center

164*163.1)

Section 9 - Batched Coordinate Geometry

-115-

C TYPE2 60 30 30 60 215 -200 200*199

(Type 2  [Offset, off tangent, clockwise], turnaround radius

60.00, tangent  30.00, return  radius  30.00,  R.O.W.  width

60.00, solve  point 215, Center point 200, bearing Center to

PC 200*199)

C TYPE3 50 0 30 60 175 164 164*163.1

(Type 3  [Offset, off tangent, counterclockwise], turnaround

radius 50.00,  no tangent  [0], return  radius 30.00, R.O.W.

width 60.00,  solve point  175, PC  point 164, bearing PC to

Center 164*163.1)

C TYPE3 60 30 30 60 215 -200 200*199

(Type 3  [Offset, off tangent, counterclockwise], turnaround

radius 60.00,  tangent 30.00,  return radius  30.00,  R.O.W.

width 60.00,  solve point  215, Center  point  200,  bearing

Center to PC 200*199)

C TYPE4 50 0 30 60 175 164 165

(Type 3  [Offset, off  curve, clockwise],  turnaround radius

50.00, no  tangent [0],  return radius  30.00, R.O.W.  width

60.00,  solve   point  175,   approach  radius   point  164,

turnaround PC point 165)

C TYPE4 60 30 30 60 215 200 -201

(Type 3  [Offset, off  curve, clockwise],  turnaround radius

60.00, tangent  30.00, return  radius  30.00,  R.O.W.  width

60.00,  solve   point  215,   approach  radius   point  200,

turnaround center point 201)

C TYPE5 50 0 30 60 175 164 165

(Type 3  [Offset, off  curve, counterclockwise],  turnaround

radius 50.00,  no tangent  [0], return  radius 30.00, R.O.W.

width 60.00,  solve point  175, approach  radius point  164,

turnaround PC point 165)

C TYPE5 60 30 30 60 215 200 -201

(Type 3  [Offset, off  curve, counterclockwise],  turnaround

radius 60.00,  tangent 30.00,  return radius  30.00,  R.O.W.

width 60.00,  solve point  215, approach  radius point  200,

turnaround center point 201)

Tangent Lines

O 3 5 R-25 100

(Construct a  line from  point 3,  tangent to a circle whose

radius point  is 5,  having a radius of 25.00, solve tangent

point 100,  on the  left side  of the  circle as viewed from

point 3 looking toward point 5.)

Section 9 - Batched Coordinate Geometry

-116-

O 3 R35.50 5 R-25.75 200

(Construct a  line tangent  to a circle centered at point 3,

having a  radius of  35.50 and  tangent to  a second  circle

centered at  point 5  with a  radius  of  25.00.  Solve  two

tangent points  starting with  point 200.  Compute the first

tangent point on the right side of the first (35.50) circle,

and the  second tangent point on the left side of the second

(-25.75) circle, as viewed from point 3 looking toward point

5.)

Right-of-Way Intersections

W TYPE1 1 1453030 50 2 2453030 50 25 1000

(TYPE 1  - Compute  intersection of  two straight rights-of-

way, first  centerline point 1, bearing N 45�30'30" E, width

50.00, second  centerline point  2, bearing  S 45�30'30"  E,

width 50.00,  return radius 25.00, start solution with point

1000.)

W TYPE2 1 1453030 50 2 500 50 25 -1100

(TYPE 2  - Compute  intersection of  a straight right-of-way

with a  curved  right-of-way,  first  centerline  point  (of

straight R.O.W.)  is point  1, bearing  N 45�30'30" E, width

50.00, radius  point of curved R.O.W. is point 2, centerline

radius 500.00,  width  50.00,  return  radius  25.00,  start

"near" solution at point 1100.)

W TYPE3 1 500 50 2 500 60 25 -1200

(TYPE 3  - Compute intersection of two curved rights-of-way,

first radius point 1, centerline radius 500.00, width 50.00,

second radius  point 2,  radius 500.00,  width 60.00, return

radius 25.00,  start "counterclockwise"  solution  at  point

1200.)

Area Computations

AREA                (begin area computation)

201                 (starting point)

202

203

-204                (negative, if PC)

206                 (PT, negative if PCC or PRC)

205                 (RP, negative if CCW)

207

201                 (close to POB, compute area)

END AREA            (end area computation, return to regular

Cogo)

AREA                (begin area computation)

15                  (starting point)

Section 9 - Batched Coordinate Geometry

-117-

16

17

18

15                  (close to POB, compute area)

AREA                (immediately process another area, no

END AREA)

19                  (new starting point)

18

17

20

19                  (close to POB, compute area)

END AREA            (end area computation, return to regular

cogo)

$PARCEL "B"         (insert a comment)

AREA                (restart area computation)

101                 (starting point)

102

103

104

101                 (close to POB, compute area)

END AREA            (end area computation, return to regular

cogo)

Defined Area Computation

AREA                (begin area computation)

30                  (starting point [hinge])

31.1                (last known point)

B31*32 36 15000     (Bearing  31*32, solve point 36, need

15000 s.f)

END AREA

AREA                (begin area computation)

30                  (starting point)

31.2                (last known point, compute 3 bearing

slide)

B31*32 B32*33 B33*30 34 43700

(bearing[1] 31*32, bearing[2] 32*33,

bearing[3] 33*30,first solve point 34,

need 43700 s.f.)

AREA                (immediately process another  area, no

END AREA)

101                 (starting point)

102

-103                (PC)

-105                (PRC)

104                 (RP)

107                 (PT of second curve)

-106                (RP of second curve, ccw)

108

109

Section 9 - Batched Coordinate Geometry

-118-

101                 (close to POB)

END AREA            (end area computation, return to regular

cogo)

Section 9 - Batched Coordinate Geometry

-119-

ACAD.PGP  94                    Calibration of EDMI 79

Adding Angles to Bearings       Catalog Search 25

and Azimuths 29               Change File Size    26

Adding Distances    30          Cleancat,garbage removal 20

Adding Quadrants    30          Clearing Coordinates     17

Adjusting  the Traverse  67     Closing Angle, in Field

Adjustment Option   64            Traverse     67

Angle Balance  64               Cogo Command Line   30

Angular Data, Field             Cogo, Within AutoCAD     102

Traverse     64               Colors, customizing 9

Angular Error, Maximum   63     Colors,Customizing  12

Area Computation    40          Comments, Inserting 32

Area Computation,               Comparing Coordinates    26

Predetermined     42          Comparing Project Files  26

AutoCAD                         Compass Rule Adjustment  64

Running Cogo Within    102    Consecutive Inverse 34

Running Field Traverse        Conventions

within    104                Entering Azimuths 28

Running the Disk                Entering Bearings 28

Management System          Conventions, Cogo   28

within    106              Coordinate comparison    26

Running the Sideshot          Coordinate Geometry

Program within 105           System,Interactive     27

Automatic Data Reduction 70     Coordinates

Automatic Formfeed Option         Clearing     17

10                              Inversing in Cogo 33

Azimuths, Entering in Cogo        List Unused in Cogo    40

28                              Listing 19

Azimuths, Entering in Field       Listing in Cogo   39

Traverse     64                 Storing 16

Backing-Up Data Files    22       Storing a Single  32

Backslash Command                 Transferring 17

Terminator   30                 Transforming 40

Batch Cogo                        Traversing in Cogo     33

Batch Command Processor       Copying Data Files  22

108                        Create Project File 15

Edit Batch File   110         Creating the Catalog Index

Interactive Batch Cogo 107      14

Print Batch File  111         Cross-sections 104

Re-define Default Editor      Cul-de-sacs    45

111                        Curvature Correction     11

Sample Batch Commands  112    Curve Inverse  38

Bearing-Bearing                 Curve Solutions

Intersection 34                 Curve Inverse     38

Bearing-Distance                  Known P.C.   37

Intersection 35                 Known P.I.   38

Bearings, Entering in Cogo        Three-Point Curve 39

28                            Customizing the Utility

Bearings, Entering in Field       Menu    92

Traverse     64               Cut/Fill Calculator 87

Building a Cogo Command  32     Data Files

Calculator, Memory Resident       Backing-Up   22

Version 91                      Catalog Index     14

Calculator,Pop-up   70            Copying 22

Index

-120-

Creating     15               Editing Field Data  67

Default Subdirectory   9      EDM Calibration     79

Deleting     20               EDM Slope Reduction

Expanding    26                 Precise 75

Exporting    23                 Short Lines  77

Filesize, Default Cogo 12     EDM Slope Reduction, Short

Importing    23                 Lines   41

Maximum Number    5           Elevation Balance   64

Naming  15                    Elevations, Field Traverse

Organization 5                  62

Searching For     25          Elevations, storing in COGO

Datum Shifts   92                 59

Datum Transformation     77     Ending Data, Field Traverse

Defaults, System    9             63

Defaults,Listing    13          Entering Distances  30

Defaults,Saving     13          ESC - The Escape Key     7

Defined Directions by Point     Expanding Data Files     26

Numbers 28                    Exporting Data 23

Defined Distances by Point      Field Data File     61

Numbers 30                    Field Traverse 60

Deflection Angles   64          Field Traverse, from within

Deleting Data Files 20            AutoCAD 104

Deleting Field Data 69          Filenames, Allowable     15

Descriptions, in Field          Filesize, Default Cogo   12

Traverse     66               Formfeed  58

Descriptions,Field Traverse     Hardcopy Option, Field

62                              Traverse     64

Direction Units, Default 11     HELP key  31

Disk Management System   14     Help,Cogo 28

Disk Management System, from    Help,Detailed Cogo  59

within AutoCAD    106         Hinge-Point Predetermined

Display Precisions, Default       Area    43

11                            Importing Data 23

Displaying and Printing         Inserting Field Data     68

Coordinates  19               Installing BASIS    1

Distance Units, Default  11     Instrument Constants     75

Distance-Distance               Interactive Batch Cogo   107

Intersection 36               Interior/Exterior Angles 64

Distances, Entering in Field    Intersections

Traverse     66                 Bearing-Bearing   34

DOS Command Processor    47       Bearing-Distance  35

Drafting Enhancements             Curved-Curved R.O.W.   58

Coordinate Grids  102           Distance-Distance 36

Drafting Sequence 95            Straight-Curved R.O.W. 56

Drawing Subdirectories 104      Straight-Straight R.O.W.

Exporting Points to BASIS          56

101                        Intersections, R.O.W.    56

Hashmarks    103              Inverse, Curve 38

Importing BASIS Points 100    Inversing between

Installation 94                 consecutive points     34

Labeling the Grid 102         Inversing in Cogo   33

Edit Batch Command File  110    Linear Error, Maximum    63

Editing and Viewing Catalog     Linear Regression   50

Data    21                    Lines Per Page, Default  10

Index

-121-

List Defaults  13               Screen-Saver   7

Listing Coordinates in Cogo     Searching the Catalog    25

39                            Sideshot Files

Listing Unused Points in          Creating     87, 90

Cogo    40                      Processing   90

LZHUF.EXE 3                     Sideshot Program    87

Main menu 6                     Sideshots, from within

Midpoint of a line  30            AutoCAD 105

Mouse,Enabling or Disabling     Sideshots, in Field

12                              Traverse     66

Multiple Points on a Line       Single Keystroke Commands

37                              31

Multiplying Distances    30     Slope Reduction Formula  11

NAD83 Transformation     77     Spacing,Printout    11

No Balance Option   64          Specifications, Editing  68

OLD2NEW Conversion Utility      Specifications, Traverse 61

60                            Starting Data, Field

Orientation, Least Squares        Traverse     62

47                            Station and Offset  51

Plotter,printer     89          Store Elevation     59

Point Numbers, in Field         Stored Bearings and

Traverse     66                 Azimuths     29

Point Search   52               Storing a Single Coordinate

Point-Protect, Default   12       32

Point-Protect ON/OFF Toggle     Storing Coordinates 16

32                            Subdirectory, Default    9

Print Batch Command File 111    System Requirements 1

Printer Headings, Default       Tangent Lines  55

10                            Theodolite Directions to

Printer ON/OFF Toggle    31       Angles  42, 76

Printer Plotter     89          Three-Bearing Predetermined

Printer Port, Default    11       Area    43

Printing Field Data 67          Three-Point Curve Solution

Printing the Catalog     21       39

Printout, new form  58          Tolerance, Radius Unequal

Process Batch File  108           11

Processing Field Data    67     Transferring Coordinates 17

Profiles  104                   Transformation, Coordinate

Radial Layout  50                 40

Radius Unequal Tolerance,       Transit Rule Adjustment  64

Default 11                    Traverse Entry, Field

Re-define Default Editor 111      Traverse     64

Re-entering Cogo Commands       Traverse Specifications  61

32                            Traverse Types, Selecting

Refraction Correction    11       62

Rejection Limits, Field         Traversing in Cogo  33

Traverse     63               Trig Levelling, with Data

Reset Command Line  32            Collector    70

Right-of-Way Computation 53     Trigonometric Levelling,

Right-of-Way Intersections        Precise 75

56                            Turnarounds

Roadway Batch Files 83            Offset, Off Curve,

Running BASIS  5                     Clockwise 47

Saving System Defaults   13

Index

-122-

Offset, Off Curve,

Counterclockwise    47

Offset, Off Tangent,

Clockwise 47

Offset, Off Tangent,

Counterclockwise    47

Symmetric    45

USERR# System Variables  95

Utility Programs    70

Vertical Curve Design    81

Index