Toshiba lap top computers and Jaws2

The following is some information on the Toshiba line of lap top

computers, your model may be discussed, read through it for some

helpful information.

The Toshiba keyboard layout is different than a desk topcomputer.

Therefore we have designed a special Jaws keyboard layout for the

Toshibas.  The default or standard Jaws macro file for the Toshiba

is called "jawstosh.jmf", the "tosh" is short for Toshiba.  If you

already have your synthesizer installed and want to load Jaws then

follow the generic installation steps.  If you need more help read

the section specific to your Toshiba.

Generic software installation

1.  Copy the Jaws distribution disk onto your hard disk or another

floppy.  If you are not sure how to do this refer to the section in

the Jaws manual about "Installing the Software".

2.  Copy the "JAWSTOSH.JMF" file to "JAWS.JMF".  This copies the

Toshiba keyboard layout into the default file, so it will be loaded

automatically at boot up.  This line will do it for you:

COPY JAWSTOSH.JMF JAWS.JMF

3.  Setup a batch file or an autoexec.bat file to load your

synthesizer software if necessary and Jaws.  To load Jaws just use

the name of the synthesizer version, like "J_ACNTPC" for the Accent

or "J_SB" for the Sounding Board or "J_SONIX" for the Symphonix.

It should start talking.  If it doesn't refer to your synthesizer

manual to be sure the proper synthesizer software is loaded, or

refer to the Jaws manual for your particular version of Jaws and

synthesizer to be sure you are doing all the steps correctly.

Toshiba keyboard layout

The easiest way to find the Jaws keys on the Toshiba keyboard is to

turn on help mode.  Hold  down the Insert key (second key to the

right of the space bar) and press the "1" on the numbers row (above

the "q").  Then explore the keyboard, the keys will speak their

names when pressed.  Here is the Toshiba-Jaws key board layout:

Prior Line = Up Arrow

Next Line = Down Arrow

Say Current Line= Shift-Up Arrow

Prior Word = Shift-left Arrow

Next Word = Shift-Right Arrow

Say Current Word= Shift-Down Arrow

Prior Character = Left Arrow

Next Character = Right Arrow

Say Current Character = Shift-End

Say up to Cursor = Shift-Enter

Say from Cursor = Shift-Page Down

Jaws cursor on = key in top right corner,

PC Cursor on = key left of Jaws cursor key, second from right top

corner.

On some models, like the t3100, the Jaws and Pc cursor keys are

reversed.

Say all = control Scroll Lock, usually the top right key.

Read top line = Shift-Home

Read bottom line = Shift-Page Up

Route PCJAWS  to PC = shift-Jaws cursor on, top right corner

Route PC to JAWS = Shift-PC cursor on, second from top right

corner.

Position of active Cursor = Period/Delete, to left of left arrow.

Position of in-active Cursor = insert-Period/Delete

With the help mode on you can also discover the locations of the

special keys specific to certain applications, like the Word

Perfect macros for the spell checker or the files menu/revealed

codes mode.  Try control-up arrow, control down arrow, and alt-end

for some of these functions.

toshiba 3100

If you have a Toshiba 3100 lap top you may want to use the hard

drive for some of your Jaws software.  We recommend that you create

a Jaws subdirectory on the hard drive and copy the entire Jaws

distribution disk over to it.  If your system was not

installed by Henter-Joyce follow these steps:

1.  Install the synthesizer in the expansion slot.  This can be

very complex so be careful or let us do it.  The slot is on the

back of the computer, 2 screws hold on a cover.  Remove the cover,

slide in the synthesizer and use the same screws to hold in the

synthesizer.  Be sure it is "seated" properly in the slot, be

gentle, do not force it, but be sure it is firmly inserted into the

side connector.

2.  Boot up your system with no floppy in the disk drive, or with

a boot floppy in the A drive.

3.  Create a Jaws subdirectory: enter "c:" and press enter; enter

"md\jaws", press enter; then enter "cd\jaws" and press enter.

4.  Then copy all the files from the Jaws distribution disk: put

the Jaws disk in the disk drive; Enter

"copy a:*.* c:\jaws";

5.  If you are using an Accent synthesizer you can use our pre-

defined config.sys and autoexec.bat files.  Copy the autoexec.t31

file and the config.t31 file to the root directory.  The following

commands will do this, replacing Autoexec.bat and Config.sys files

already their.  If you do not want this, maybe you are using a

different synthesizer,  then you should look at our files and add

the appropriate commands to your files.

Enter "cd\", press enter;

enter "copy \jaws\autoexec.t31 \autoexec.bat" add press enter;

enter "copy \jaws\config.t31 \config.sys" and press enter.

Re-boot the computer with no disk in the A drive and let it load up

with speech from the hard drive.  You should hear it say "Accent

Ready" then your name and "Jaws Ready".

If your computer is already in "resume mode" you will need to press

the reset button.  On the left side, towards the front is a small

cover, open it by pulling down from the top.  Inside is a button,

press it to reset the computer.  It will beep once, press enter,

then wait for it to load after the next beep.

Now you should be able to use the Jaws utilities like the macro

editor without having the boot floppy in the drive, it will be

loaded from the hard drive.

You may want to set up the Toshiba to use "resume" mode:  enter

"c:test3"; enter "0" and press enter;  move the cursor down to

"Resume mode"; use left or right arrow to select "resume"; press

F10, then "9", then enter.  Now when you turn it off you will hear

two beeps, this means that you are in resume mode.  When you turn

it back on it will not need to re-boot, it will already be up and

running where you left it.

toshiba 1000 SE

If you have a Toshiba 1000 SE lap top you may want to use the RAM

drive for some of your Jaws software.  We recommend that you leave

the Jaws program itself on the floppy drive: this is named

"J_ACNTPC.EXE" for the Accent SE synthesizer, or "J_SONIX.EXE" if

you have a Symphonix synthesizer.  You should also leave the Accent

driver "SPKEMS.DVC" or the Symphonix driver on the floppy.  These

files take up a lot of memory and you do not need them on the RAM

drive.

If your system was not installed by Henter-Joyce follow these

steps:

1.  Install the synthesizer in the modem slot.  This can be very

complex so be careful or let us do it.

2.  Boot up your system with no floppy in the disk drive.

3.  Create a boot disk:  put in a new disket in the floppy drive;

enter "c:";

then enter "format a:/s", if it says you have the wrong

parameters then you do not have a high density floppy.  No

problem, for low-density floppies use the syntax below:

enter "format a:/s/t:80/n:9";

High density floppies have 1.4 megabytes, low density have 720

kilobytes of storage.

4.  Then copy all the files from the Jaws distribution disk: put

the Jaws disk in the disk drive; Enter

"xcopy a:\ b:\/s"; when it says to put in the destination disk or

the "B" disk put in the newly-formatted disk we did above;  When it

asks to put in the source or "A" disk put in the Jaws disk again;

Repeat these steps until the copy is complete.

5.  Now copy the autoexec.t1s file and the config.t1s file to the

proper file names (this step may not be necessary, the install

procedure may have already done it).  The following commands will

do this, if you do not want to use our configuration then you

should look at our files and add the appropriate commands to your

files to get the configuration you want.

enter "copy \autoexec.t1s \autoexec.bat" add press enter;

enter "copy \config.t1s \config.sys" and press enter.

Now you should have a boot disk that will also load up the Accent

driver and Jaws.  Re-boot the computer with the Jaws boot disk in

the A drive and let it load up with speech.  You should hear it say

"Accent Ready" then your name and "Jaws Ready".

If your computer is already in "resume mode" you will need to press

the reset button just behind the power supply input

connection.  Use a skinny object to press in the recessed button

inside the small hole.  It will beep once, press enter, then wait

for it to load after the next beep.

Then we want to format the D drive and copy the Jaws utilities over

to it:

1.  Enter "c:format d:", press "y" when asked and press enter.  It

may ask for the current volume label, just type it in.  If you do

not know this press enter, let it abort, then enter "c:label d:"

and it will tell you the current label.  Then repeat the step above

to format the D drive.

2.  Enter "a:jcopyse", this will copy the Jaws utility files over

to the D drive.

3.  re-boot the computer with your Jaws boot disk in the a drive.

It should boot up with speech and be defaulted to the D drive.

Now you should be able to use the Jaws utilities like the macro

editor without having the boot floppy in the drive, it will be

loaded from the RAM drive.

You should at this time set up the Toshiba to use "resume" mode:

enter "c:setup10"; move the cursor down to "Resume mode"; use left

or right arrow to select "resume"; press enter and "y".  Now when

you turn it off you will hear two beeps, this means that you are in

resume mode.  When you turn it back on it will not need to re-boot,

it will already be up and running where you left it.  Just give it

about 5 seconds and it will be ready to speak.