Introduction

The New Enhanced AT (NEAT[(r)]) CHIPSet[tm] from Chips &

Technologies has brought many of the memory and performance

features of an 80386 system to the less expensive 80286 CPU.

In addition, it offers features not found in any previous

personal computer system.  Among these, user-controllable

shadow RAM, bus timing, wait state control and LIM EMS 4.0

emulation, all in a jumperless configuration.

To control all of these features, a ROM-based Setup routine

stores appropriate values into CMOS registers contained in

three of the NEAT[(r)] chips.  To simplify configuration for

new users, a minimal setup routine is accessible under the

"Easy NEAT CHIPSet Register Setup" option.  This

"stripped-down" setup contains all of the mandatory setup

information and is fully detailed in the JE3010 and JE3011

Owner's Manuals.

To access all of the features of the NEAT[(r)] CHIPSet[tm], the

user will have to be acquainted with the NEAT[(r)] registers

themselves.  These registers are accessible under the "Advanced

NEAT[(r)] CHIPSet[tm] Register Setup" option and are fully

explained in this supplement.  Before consulting this, read the

Owner's Manual and complete the installation and setup of your

new computer system.

WARNING!  It is conceivable that contradictory settings may be

stored in the CMOS registers that can prevent the system from

booting.  In this event, attempt the re-initialization

procedure that follows several times before calling for

technical assistance.  If you are still unsuccessful, please

refer to the Obtaining Assistance section of this document.

ATTEMPTING TO RE-INITIALIZE THESE SETTINGS EITHER BY REMOVING

THE BATTERY OR CMOS DEVICES CAN RESULT IN PERMANENT DAMAGE TO

THE MOTHERBOARD AND WILL VOID ANY WARRANTY.

CMOS Reset      To re-initialize the NEAT[(r)] CMOS registers, follow this

procedure:

[C1] Turn on the computer

[C1] Wait 20 seconds

[C1] Press and hold down the [Key: ] key

[C1] Press the reset button for at least 3 seconds

[C1] When the memory test starts, release the [Key: ] key.

This procedure will reinitialize the CMOS setup and the

Extended CMOS settings to their default values.  Of course, the

Extended CMOS Setup and possibly the CMOS Setup will need to be

run again to set the proper values.

Memory Types

The NEAT[(r)] 80286 motherboard can utilize its RAM in four

different configurations:  base, shadow, extended and expanded

(EMS).  As more memory is added to it (up to eight megabytes),

this flexibility increases.

If 1024Kb (1Mb) has been installed on it, the residual 384Kb

above 640Kb can be used for either shadow RAM or Extended

memory.

Base            The memory addresses from 0Kb to 640Kb comprise the base memory

of the system.  This is the memory that DOS manages and is

sometimes referred to as "conventional memory". Some

applications require the full amount of base memory.

Shadow          In all IBM software compatible computers, the addresses from

640Kb to 1024Kb are reserved for the system BIOS ROMs and ROMs

of I/O expansion cards.  In systems with 1024Kb of RAM, the

memory addressed from 640Kb to 1024Kb would normally be lost.

The NEAT[(r)] Motherboards have the ability to copy the

contents of the BIOS ROMs into this unused RAM, disable the

ROMs and run the BIOS routines from RAM.  This process is

referred to as "shadowing."  The main advantage to the shadow

feature is the increase in system performance.  Most ROMs have

access times of only 200 to 400ns.  Since these boards

typically operate with memory faster than 80ns, the shadowed

BIOS routines can attain a 200 to 400% increase in performance.

If shadow RAM is enabled during the Extended Setup procedure,

the following events will take place after a reset or power on:

1) After the POST, the AMI BIOS looks at the Extended Setup

parameters and finds that you have shadow features enabled.

2) It looks at the BIOS locations that are to be shadowed.

3) It then takes each BIOS (at the selected locations) and

copies their complete contents into RAM addressed at the same

location.

4) The BIOS ROMs are then disabled.  Their operations are then

executed from shadow RAM.

Note:  Shadow RAM cannot be enabled when the "memory

relocating" feature in advanced setup is used.

Extended        The memory addresses above 1024Kb are referred to as extended

memory.  While the 80286 CPU is capable of using this memory,

DOS and other PC applications can only address 1024Kb.

Therefore, most applications are unable to take advantage of

extended memory.  Some examples of programs that can utilize

extended memory are:  VDISK, VCACHE, Framework II and AutoCAD.

The NEAT[(r)] Motherboard takes any memory that is not used for

EMS (Expanded Memory Specification), and uses it for Extended

memory with the starting address beginning at 1024Kb.

If 1024Kb is installed, the 384Kb of memory that is above the

640Kb base may be used for extended memory.  To select this

option, set bit 6 on at register 6BH in the advanced setup

program.  If more than 1Mb is available, DO NOT use this

option.

Note:  The "memory relocating" feature cannot be enabled when

the "shadow" feature is used.

Expanded        The NEAT[(r)] Motherboards conform to the Lotus/Intel/Microsoft

(EMS)           Expanded Memory Specification version 4.0 (LIM EMS 4.0).

Expanded memory is not addressed as part of the system RAM.

Instead, it is partitioned from the rest of the RAM and is

accessed in pages by programs that support LIM EMS 4.0.  Note

that LIM EMS 4.0 is downward-compatible with older LIM EMS

specifications (ie. LIM EMS 3.2).

RAM that is partitioned for EMS is removed from the available

extended memory, reducing the total RAM displayed at POST.  The

interface between applications software and EMS is a software

driver that is provided on diskette with the motherboard.

Some examples of programs that can utilize expanded memory

are:  Lotus 1-2-3 v2.01, Freelance Plus v3.0, Manuscript;

Microsoft File, Word, Works; Ashton-Tate dBase III/IV,

Framework II; Software Publishing Corp. Professional File;

Borland Sidekick Plus and AutoCAD.

Examples        A system with 8192Kb of RAM could be configured with the

following memory arrangement:  640Kb of conventional memory

with all shadow features enabled.  6144Kb (6M) of expanded

memory, leaving 1024Kb that is automatically configured as

extended memory.

With this configuration, the POST memory test will count to

1664Kb which is 640Kb conventional plus 1024Kb extended.  The

384Kb is used for shadow and expanded memory addressing, plus

the actual 6Mb set aside for expanded memory, is not included

in the POST memory count.  The BIOS tests the 384Kb RAM

separately, while the expanded memory is tested by using one of

the options offered by the driver software (explained later).

Alternate       While there are a variety of non-DOS operating systems (ie.

Operating       Xenix, PC/MOS, etc.) which we will not discuss, there are some

Systems         alternate operating environments that overlay DOS giving the

user additional functionality.  These include Microsoft's

Windows/286 and Quarterdeck's DesqView.  These operating

systems load from DOS and assume control of the system allowing

advanced windowing, applications control and memory management

features.  Both Windows/286 and DesqView utilize base, extended

and expanded memory for optimum performance.  Follow the

recommendations of the software manufacturer for configuring

the memory to achieve the best performance.  Typically, this

calls for a small amount of extended memory and as much

expanded memory as possible.

Extended Setup

During each setup, use [Key: ][Key: ][Key: ]

[Key: ] to move the highlight bar.  Use [Key: ] or

[Key: ] to change parameters.  Each setup screen will

display a window describing each option.  Use the [Key: ]

key to exit the current setup screen.

Entering        Reset or power up the system.  The system will go through POST

Extended Setup  and display the size of the memory being tested.  Note that

this test can be bypassed by pressing the [Key: ] key.

This option is useful when the memory on the system is quite

large, although It is a good idea to let the test count

through.

Immediately after the memory test, the following prompt will

appear on the screen:

Press <DEL> key to run SETUP/EXTD-SET

Press the [Key: ] key to get into the Setup Mode.  Note

that [Key: ] key will initiate the setup mode only when

the above message is present on the screen.  A moment after the

[Key: ] is pressed, this menu will appear:

EXIT FOR BOOT

RUN CMOS SETUP

RUN XCMOS SETUP

Use [Key: ][Key: ] and highlight "RUN XCMOS SETUP"

and press [Key: ].  The Extended Setup screen appears as

follows:

NEAT CHIPSET SETUP PROGRAM

MAIN MENU

EASY NEAT CHIPSET REGISTER SETUP

ADVANCED NEAT CHIPSET REGISTER SETUP

ENABLE/DISABLE VIDEO & MAIN BIOS SHADOW

WRITE CMOS REGISTERS AND EXIT

DO NOT WRITE CMOS REGISTERS AND EXIT

Enable/         Highlight this option and press [Key: ].  The display

Disable Video   will change to a graphic depiction of the CMOS registers as

and Main BIOS   shown in the following diagram.  Setting these bits to '1'

Shadow          enables the SHADOW RAM function for that bit.  We recommend

enabling all shadow options for maximum performance.

SETUP SHADOW RAM FOR 212

[LN Length:RM]

MAIN  BIOS SHADOW AT F0000H,64Kb ->0

VIDEO BIOS SHADOW AT C0000H,16Kb ->0

VIDEO BIOS SHADOW AT C4000H,16Kb ->0

Shadowing the main ROM BIOS will improve overall system speed.

Shadowing the video ROM BIOS will offer a 20% to 30% increase

in video performance.  You must have a minimum of 1Mb of RAM

and the Relocate Option must be disabled (82C212, 6BH, set bit

6 to 0).  This may cause the screen to flicker with some

systems.  If any video problems are encountered, turn off the

video shadow feature.

Note:  The "shadow" features cannot be enabled if the "memory

relocating" feature is used.

Press [Key: ] to return to the main menu.

Advanced NEAT   Highlight "Advanced NEAT CHIPSet Register Setup" and press

[(r)] Register  [Key: ].  The system will warn you that improper setup

Setup           may prevent the computer from operating.  If this occurs, refer

to the CMOS Reset section at the beginning of this supplement.

Next, the display will change to a graphic depiction of the

CMOS registers as shown in the following diagrams.  The first

diagram shows default settings for boards with the 82C212.  The

second diagram shows default settings for boards with the

82C212B.

A description of the currently selected bit's function will

appear on the right portion of the screen.  Any permissible

values will also be displayed.  All bits marked as "R"eserved

cannot be changed.  For clarity, we will discuss the options

available at each bit in the following sections.

82C212          *           BITS   7 - 0

82C211   60H -> 00  0 0 R 0 R 0

61H -> 0 1  00  01  01

62H -> RR   01  10  00

82C212   64H -> 0    00   RRRRR

65H -> 0 0 0 0 1 1 1 0

66H -> 1       RRRRRRR

67H -> 0 0 0 0 0 0 0 0

68H -> 0 0 0 0 0 0 0 0

69H -> 0 0 0 0 0 0 0 0

6AH -> **  *     RRRRR

6BH -> * 1 1 0  10  11

6CH -> **    *   RRRRR

6DH -> 0100       0000

6EH -> 00   00  00  00

6FH -> 000  RR   1 0 R

82C206   01H -> 11  00  00  0 0

82C212B         *           BITS   7 - 0

82C211   60H -> 00  0 0 R 0 R 0

61H -> 0 1  00  01  01

62H -> RR   01  10  00

82C212B  64H -> 0    01   RRRRR

65H -> 0 0 0 0 1 1 1 0

66H -> 1 0 0     RRRRR

67H -> 0 0 0 0 0 0 0 0

68H -> 0 0 0 0 0 0 0 0

69H -> 0 0 0 0 0 0 0 0

6AH -> **  *     RRRRR

6BH -> * 1 1 0  10  11

6CH -> **    *  0 RRRR

6DH -> 0100       0000

6EH -> 00   00  00  00

6FH -> 000  0 R  1 1 R

82C206   01H -> 11  00  00  0 0

Note:  "*" indicates that the default values will automatically

adjust for the system RAM configuration.  The correct values

should be present, however, check them to be sure.

82C211

The 82C211 Bus Controller consists of the following functional

sub-modules:  Reset and Shut Down Logic, Clock Generation and

Selection, CPU State, Bus State, Bus Arbitration and Refresh

Logic, NMI Generation Logic, Numeric Coprocessor Interface.

60H             PROCCLK Register RA0

7,6             82C211 Revision Number

This bit is not revisable by the user.

5               Alternate CPU Reset

This bit is not revisable by the user.

4               Processor Clock Select

0:   PROCCLK = CLK2IN

1:   PROCCLK = BCLK

This determines the source of the PROCCLK signal and should be

set for CLK2IN.  This will divide the CLK signal by two.  For

example, the 32MHz oscillator will be divided by two, providing

a 16MHz timing signal for the 80286 processor.

2               Local Bus Ready TIMEOUT NMI

0:   Disable

1:   Enable

This option is set by the ROM BIOS and need not be changed.

0               Local Bus Ready TIMEOUT

This bit is not revisable by the user.

61H             Command Delay Register RA1

7               Address Hold Time Delay

0:   Disable

1:   Enable

This option is set by the ROM BIOS and need not be changed.

6               Quick Mode Enable

0:   Disable

1:   Enable

This option is set by the ROM BIOS and need not be changed.

5,4             AT Bus 16 Bit Memory Command Delay

00:  Delay is 0 BCLK Cycles

01:  Delay is 1 BCLK Cycles

10:  Delay is 2 BCLK Cycles

11:  Delay is 3 BCLK Cycles

This option is set by the ROM BIOS and need not be changed.

3,2             AT Bus 8 Bit Memory Command Delay

00:  Delay is 0 BCLK Cycles

01:  Delay is 1 BCLK Cycles

10:  Delay is 2 BCLK Cycles

11:  Delay is 3 BCLK Cycles

This option is set by the ROM BIOS and need not be changed.

1,0             AT Bus I/O Cycle Command Delay

00:  Delay is 0 BCLK Cycles

01:  Delay is 1 BCLK Cycles

10:  Delay is 2 BCLK Cycles

11:  Delay is 3 BCLK Cycles

This option is set by the ROM BIOS and need not be changed.

62H             Wait State Register RA2

5,6             16 Bit AT Cycle Wait State

00:  0 Wait States

01:  1 Wait States

10:  2 Wait States

11:  3 Wait States

This option is set by the ROM BIOS and need not be changed.

3,2             8 Bit AT Cycle Wait State

00:  2 Wait States

01:  3 Wait States

10:  4 Wait States

11:  5 Wait States

This option is set by the ROM BIOS and need not be changed.

1,0             Bus Clock (BCLK) Source Select

00:  BCLK = CLK2IN/2

01:  BCLK = CLK2IN

10:  BCLK = ATCLK

11:  Reserved

This option selects the bus clock (BCLK) source.  Default is

CLK2IN/2.  In a system with a 32MHz oscillator, CLK2IN will be

16MHz and the resulting bus speed will be 8MHz (CLK2IN/2).

When this option is selected, you have an advantage of being

able to switch the system speed down twice.  Once with the

keyboard (system switches to 8MHz), and once with the turbo

switch (system switches to 4MHz).  This can come in handy when

some software needs to run near IBM-PC speed.

Changing this bit to 01 would result in a bus speed of 16MHz.

Attempts to change the clock speed via keyboard, will lock up

the system.  The bus speed cannot be faster than the system

speed.  Also, be aware that most AT expansion cards are

incapable of exceeding 10MHz.  This option is most useful when

the source oscillator has been replaced.

82C212(B)

The 82C212 and the 82C212B memory controllers consists of the

following functions:  RAM Page / Interleave Mode, Shadow RAM,

Lotus Intel Microsoft (LIM) Expanded Memory Specification (EMS)

version 4.0 emulation.

64H             Version Register RB0

7               NEAT[(r)] Memory Controller Identifier

This bit is not revisable by the user.

6,5             82C212 Revision Number

This bit is not revisable by the user.  For the 82C212, this

value will be 00.  For the 82C212B, this value will be 01.  If

you get a 00 and you have an 82C212B chip, you should execute a

CMOS Reset as explained at the beginning of this supplement.

65H             ROM Configuration Register RB1

In most cases all Shadow RAM should be set from the other menu

"ENABLE/DISABLE VIDEO AND MAIN BIOS SHADOW".  However, if you

have a special application, you may set specific bits here.

NOTE:  Please use caution when using this option.  Many ROMs

will have problems.  Hard disk drive controller card ROMs, for

example, should not be shadowed.  Other ROMs (i.e. a Floppy

controller card ROM) can possibly be shadowed without

problems.  If you have any operation problems with a ROM you

have shadowed, disable this option.

If you have a BIOS ROM that is located at a specific address

and you wish to provide shadow RAM for this BIOS, you must set

three bits:

[C1] Set one of the RAM bits from register 65H (bit 7, 6, 5 or

4) within the address range of the BIOS to be shadowed, to the

Read Only/Write Protected state.

[C1] Disable one of the ROM bits from register 65H (bit 3, 2,

1, or 0) within the address range of the BIOS to be shadowed.

[C1] Enable one (or more, depending on the BIOS size) of the

bits from registers 67H to 69H.  The first bit enabled should

be equal to the starting address of the BIOS to be shadowed.

If the BIOS to be shadowed occupies an address area larger than

the area of one of these bits, you must enable the next

consecutive bit (or bits) until the complete BIOS address area

is shadowed.

7               SHADOW RAM at C0000H to CFFFFH

0:   Read/Write Enable

1:   Read Only (Write Protected)

6               SHADOW RAM at D0000H to DFFFFH

0:   Read/Write Enable

1:   Read Only (Write Protected)

5               SHADOW RAM at E0000H to EFFFFH

0:   Read/Write Enable

1:   Read Only (Write Protected)

4               SHADOW RAM at F0000H to FFFFFH

0:   Read/Write Enable

1:   Read Only (Write Protected)

3               ROM at C0000H to CFFFFH

0:   ROM Enabled

1:   ROM Disabled

2               ROM at D0000H to DFFFFH

0:   ROM Enabled

1:   ROM Disabled

1               ROM at E0000H to EFFFFH

0:   ROM Enabled

1:   ROM Disabled

0               ROM at F0000H to FFFFFH

0:   ROM Enabled

1:   ROM Disabled

66H             Memory Enable-1 Register RB2

7               Address Map For RAM in 512Kb to 640Kb Area

0:   RAM on I/O Channel

1:   RAM on System Board

6               256Kb to 512Kb Memory status (82C212B only)

0:   Memory on system board

1:   Memory on AT-bus

5               0Kb to 256Kb Memory Status (82C212B only)

0:   Memory on system board

1:   Memory on AT-bus

67H             Memory Enable-2 Register RB3

7               SHADOW RAM at BC000H to BFFFFH

0:   Disable

1:   Enable

6               SHADOW RAM at B8000H to BBFFFH

0:   Disable

1:   Enable

5               SHADOW RAM at B4000H to B7FFFH

0:   Disable

1:   Enable

4               SHADOW RAM at B0000H to B3FFFH

0:   Disable

1:   Enable

3               SHADOW RAM at AC000H to AFFFFH

0:   Disable

1:   Enable

2               SHADOW RAM at A8000H to ABFFFH

0:   Disable

1:   Enable

1               SHADOW RAM at A4000H to A7FFFH

0:   Disable

1:   Enable

0               SHADOW RAM at A0000H to A3FFFH

0:   Disable

1:   Enable

68H             Memory Enable-3 Register RB4

7               SHADOW RAM at DC000H to DFFFFH

0:   Disable

1:   Enable

6               SHADOW RAM at D8000H to DBFFFH

0:   Disable

1:   Enable

5               SHADOW RAM at D4000H to D7FFFH

0:   Disable

1:   Enable

4               SHADOW RAM at D0000H to D3FFFH

0:   Disable

1:   Enable

3               SHADOW RAM at CC000H to CFFFFH

0:   Disable

1:   Enable

2               SHADOW RAM at C8000H to CBFFFH

0:   Disable

1:   Enable

1               SHADOW RAM at C4000H to C7FFFH

0:   Disable

1:   Enable

0               SHADOW RAM at C0000H to C3FFFH

0:   Disable

1:   Enable

69H             Memory Enable-4 Register RB5

7               SHADOW RAM at FC000H to FFFFFH

0:   Disable

1:   Enable

6               SHADOW RAM at F8000H to FBFFFH

0:   Disable

1:   Enable

5               SHADOW RAM at F4000H to F7FFFH

0:   Disable

1:   Enable

4               SHADOW RAM at F0000H to F3FFFH

0:   Disable

1:   Enable

3               SHADOW RAM at EC000H to EFFFFH

0:   Disable

1:   Enable

2               SHADOW RAM at E8000H to EBFFFH

0:   Disable

1:   Enable

1               SHADOW RAM at E4000H to E7FFFH

0:   Disable

1:   Enable

0               SHADOW RAM at E0000H to E3FFFH

0:   Disable

1:   Enable

6AH             Bank 0/1 Enable Register RB6

7,6             Bank 0/1 DRAM Types

00:  Disabled

01:  256Kb and 64Kb Combination

10:  256Kb DRAMS

11:  1M DRAMS

5               Bank 0/1 Number Of RAM Banks Used

0:   1 Bank, Non Interleaved

1:   2 Banks

6BH             DRAM Configuration Register RB7

7               Page/Interleaved Mode Enable

0:   DRAMS used in Normal Mode

1:   DRAMS used in Page/Interleaved

The JE3010 and JE3011 use an advanced memory accessing scheme

incorporating both Paged and Interleaved memory techniques.

These methods are available when

two or four banks of RAM are installed in the motherboard.

This option will permit the installation of slower RAM (i.e.

150 or 120ns) to operate in the motherboard.  When either two

or four banks are installed, paging and interleaving can both

occur, resulting in greatly improved memory performance.  These

techniques require RAM chips that have a CAS before RAS

design.  There are no provisions for paging or interleaving

when only one bank or only three banks of RAM are installed.

This feature can only be enabled if you have two or four banks

of memory filled with the same size DRAM.  This option is not

valid if you have only one or three banks of DRAM or if your

banks contain mixed size memory (ie. 256Kb and 64Kb).

6               Relocate DRAM at 640Kb to above 1Mb

0:   Do not relocate RAM (Reserve for SHADOW)

1:   Relocate 80000H-FFFFFH to 100000H-11FFFFH

This option allows only a 1Mb system to take the 384Kb of

memory (located between 640Kb and 1Mb) and remap it to be

addressed above 1Mb so that it can be used as "extended"

memory.

If the motherboard has more than 1Mb installed, this bit should

be set to 0.  If not, you will be remapping this RAM onto other

RAM.

NOTE:  You will not be able to enable any shadow RAM features

if you choose to relocate RAM.

5               RAM Access Wait States

0:   Zero Wait States

1:   One Wait State

In non-interleaved operation, 80ns or faster DRAM is required

for 0ws access.  In interleaved operation, 150ns or faster DRAM

is all that is required.

4               EMS Enable Bit (requires driver)

0:   EMS Disabled

1:   EMS Enabled

This option allows extended memory to be remapped as expanded

memory (EMS).  Therefore, you must have at least 1.5Mb to use

this feature.

NOTE:  Please read the section "Setting up EMS" located later

in this manual for a complete step by step procedure for

completely setting up EMS memory.

3,2             EMS Memory Access Wait State

00:  0 Wait States

01:  1 Wait State

10:  2 Wait States

11:  Reserved

This option is set by the ROM BIOS and need not be changed.

1,0             RW ROM Access Wait State Control

00:  0 Wait States

01:  1 Wait State

10:  2 Wait States

11:  3 Wait States

This option is set by the ROM BIOS and need not be changed.

6CH             Bank 2/3 Enable Register RB8

7,6             Bank 2/3 DRAM Types

00:  None

01:  Reserved

10:  256Kb DRAMS

11:  1M DRAMS

5               Bank 2/3 Number of banks

0:   1 Bank, Non Interleaved

1:   2 Banks

4               Interleave type (82C212B only)

0:   2 Way Interleave

1:   4 Way Interleave

Only with the 82C212B, 4-way interleaving is possible when all

four banks are filled with the same RAM type (i.e. banks 0 and

1 filled with 1Mb chips AND banks 2 and 3 filled with 1Mb

SIPPs), allowing for more performance.  4-way interleaving can

be enabled by setting register 6CH (bit 4) to a 1.  This bit is

reserved on the regular 82C212.

If all four memory banks contain the same size DRAM, select

option 1.  If you have only two memory banks of DRAM or if

banks 0 and 1 do not contain the same size memory as banks 2

and 3 select option 0.

6DH             EMS Base Address Register RB9

7,6,5,4         Expanded Memory Base Address

0000:  C0000H,C4000H,C8000H,CC000H

0001:  C4000H,C8000H,CC000H,D0000H

0010:  C8000H,CC000H,D0000H,D4000H

0011:  CC000H,D0000H,D4000H,D8000H

0100:  D0000H,D4000H,D8000H,DC000H

0101:  D4000H,D8000H,DC000H,E0000H

0110:  D8000H,DC000H,E0000H,E4000H

0111:  DC000H,E0000H,E4000H,E8000H

1000:  E0000H,E4000H,E8000H,EC000H

The default of 0100 should be fine for most applications.

Ensure that none of the four selected addresses conflicts with

other peripherals in your system.  For example, hard and floppy

controllers often use C0000H, C4000H, C8000H or CC000H.

3,2,1,0         EMS Page Reg I/O Base Address

0000:  208H/209H

0001:  218H/219H

0101:  258H/259H

0110:  268H/269H

1010:  2A8H/2A9H

1011:  2B8H/2B9H

1110:  2E8H/2E9H

The default should be fine for most applications.  Ensure that

neither of the two I/O addresses selected conflicts with other

peripherals in your system.  For example, COM4: is usually

2E8H.

6EH             EMS Address Extension Register RB10

7,6             EMS Page 0 Position

00:  1M to 2M of EMS Memory

01:  2M to 4M of EMS Memory

10:  4M to 6M of EMS Memory

11:  6M to 8M of EMS Memory

This option is set by the ROM BIOS and need not be changed.

5,4             EMS Page 1 Position

00:  1M to 2M of EMS Memory

01:  2M to 4M of EMS Memory

10:  4M to 6M of EMS Memory

11:  6M to 8M of EMS Memory

This option is set by the ROM BIOS and need not be changed.

3,2             EMS Page 2 Position

00:  1M to 2M of EMS Memory

01:  2M to 4M of EMS Memory

10:  4M to 6M of EMS Memory

11:  6M to 8M of EMS Memory

This option is set by the ROM BIOS and need not be changed.

1,0             EMS Page 3 Position

00:  1M to 2M of EMS Memory

01:  2M to 4M of EMS Memory

10:  4M to 6M of EMS Memory

11:  6M to 8M of EMS Memory

This option is set by the ROM BIOS and need not be changed.

6FH             Miscellaneous Register RB12

7,6,5           Set EMS Memory Size

000:   0 to 512Kb

001:   1Mb

010:   2Mb

011:   3Mb

100:   4Mb

101:   5Mb

110:   6Mb

111:   7Mb

This should be set to reflect the total amount of expanded

memory you desire.  This amount is deducted from the available

extended memory.

4               External EMS mapper (82C212B only)

0:   External EMS disable

1:   External EMS enable

This feature is not supported by the existing LIM EMS standard

and is for future use.

2               Enable RAS Timeout Counter

0:   Disable

1:   Enable

This option is set by the ROM BIOS and should not be changed.

1               Enable CPUA20 onto A20

0:   Enable

1:   Disable

This option is set by the ROM BIOS and should not be changed.

82C206

01H             Clock and Wait State Control

7,6             XIOR/XIOW Wait States

00:  1 I/O Wait State

01:  2 I/O Wait States

10:  3 I/O Wait States

11:  4 I/O Wait States

This option is set by the ROM BIOS and need not be changed.

5,4             16-Bit DMA Wait States

00:  1 DMA Wait State

01:  2 DMA Wait States

10:  3 DMA Wait States

11:  4 DMA Wait States

This option is set by the ROM BIOS and need not be changed.

3,2             8-Bit DMA Wait States

00:  1 DMA Wait State

01:  2 DMA Wait States

10:  3 DMA Wait States

11:  4 DMA Wait States

This option is set by the ROM BIOS and need not be changed.

1               EMR Bit

0:   Disabled

1:   Enabled

This option is set by the ROM BIOS and need not be changed.

0               CLK Bit

0:   DMA CLK = SCLK / 2

1:   DMA CLK = SCLK

This option is set by the ROM BIOS and need not be changed.

Setting up EMS  There are many steps that need to be completed in order to set

up EMS memory.  From the "Advanced NEAT CHIPSET Register Setup"

screen, execute the following steps:

[C1] Set EMS enable bit (Reg 6BH, bit 4).  This bit enables or

disables the EMS memory function.

[C1] Set EMS memory wait states (Reg 6BH, bits 3,2).  The

default is 2 wait states.  Normally EMS memory wait states

should be more than normal system RAM wait states.

[C1] Set EMS base address (Reg 6DH, bits 7,6,5,4).  The default

address area for standard EMS is D0000H to DFFFFH, however,

other addresses may be specified.  If you have an application

that requires you to change the EMS address, be sure you select

a new address that is not used so that other items in the

system will not conflict.

[C1] Set EMS page I/O address (Reg 6DH, bits 3,2,1,0).  The

default I/0 address is 208H/209H.  However, other I/O addresses

may be specified.  If you have an application that requires you

to change the I/O address, be sure you select a new address

that is not used by other items in the system.

[C1] You do not need to set the EMS page positions (Reg 6EH,

bits 7,6,5,4,3,2,1,0).  You should leave all of these set to

00.  The only time you would be required to change these is

when you have other EMS RAM cards installed in the system.

[C1] Set EMS memory size (Reg 6FH, bits 7,6,5).  This bit

allows you to specify the amount of memory in your system that

you want used as EMS.  That amount you select must be equal to

the (total amount of RAM in your system) MINUS (1Mb and any

Extended memory you want).  The first one meg of RAM must be

used for 640Kb Base and 384Kb Shadow.  This is why you cannot

enable EMS unless you have 1.5Mb of RAM or more installed.  Any

RAM that is left over (not used for EMS, Base or Shadow) will

be set up as Extended memory.

[C1] Press [Key: ].  Highlight the "Write CMOS Registers

and Exit" option and press [Key: ].

[C1] Since the system now finds a new RAM size, the regular

CMOS setup must be run.  Press [Key: ] to enter the set up

choices screen.  Select "CMOS Setup" and make sure all

parameters are correct.  You may not need to change anything;

the system just needs to clarify the new RAM configuration.

[C1] Save and exit from setup.  Boot the system.

[C1] In order to complete the installation of EMS memory, a

CONFIG.SYS file must be created or modified on your boot drive

to include the EMS driver.  The diskette included with the

motherboard contains two files: an EMS driver file and a

document file.  The document file describes the options

available for the EMS driver.  Copy both files to your boot

drive.

An example line in the CONFIG.SYS file would be:

DEVICE=NEATEMM.SYS

NOTE:  When inserting this command in your CONFIG.SYS file,

make sure you insert it AFTER the "FILES=xx" and "BUFFERS=xx"

commands.

[C1] Reboot the system.  The CONFIG.SYS file is read by the

system only during bootup.

Write CMOS      To save all parameters and reboot, highlight this option and

Registers and   press [Key: ].

Exit

Do Not Change   To discard all changes and reboot, highlight this option and

CMOS and Exit   press [Key: ].

Obtaining Assistance

Jameco Electronics carefully selects and tests all of our

products.  However, even the best tested and documented

products can still cause you an occasional problem.  If you

experience difficulty in either assembly or operation of your

equipment, recheck cable connections and configuration settings

against this document and any other addenda that may have been

included.  If you have exhausted all other options and are

still encountering difficulty, refer to the appropriate section

below for assistance.

Technical       Our Technical Support Staff are available between 7am and 5pm,

Support         Pacific Time, Monday through Friday.  When you call, please

have the equipment in question handy, along with the necessary

customer and order numbers.  If appropriate, please write down

settings or other configuration data.  The technicians will

need all of this information to assist you fully.  The phone

number is (415) 592-9990.

Customer        Discrepancies in shipment, returns, exchanges and refunds are

Service         handled by our Customer Service Department.  They are also

available between 7am and 5pm, Pacific Time, Monday through

Friday.  Their number is (415) 592-8121.

Foreign         If you are located outside the United States, you may find it

Customers       more convenient to contact us by either FAX or Telex, both

available 24 hours a day.  Inquiries should be marked to the

attention of either Customer Service or Technical Support, as

appropriate.  Our FAX numbers are (415) 592-2503 and

(415) 595-2664.  Our Telex number is 176043, answerback:

Jameco Blmt.

Corrections     If you run across any errors or omissions in this manual or the

manual does not explain something thoroughly enough, please

write to us and let us know.  Feedback from our customers

provides for consistently high quality now, and in the future.

Please reference the document and revision numbers as well as

the printing date (located on the cover of all Jameco

Electronics documentation) when writing so that corrections can

be made as easily as possible.  Address correspondence to:

JAMECO ELECTRONICS

1355 Shoreway Road

Belmont, CA  94002

ATTN:  Technical Support