Waffle v1.65 / Use of TELEBIT modems with MS DOS Waffle

(C) Copyright 1992 Darkside International, All Rights Reserved.

TELEBIT REGISTER SETTINGS

Telebit makes a number of modems that have explicit support for UUCP.

Many Unix systems use these as they are good for moving large volumes of

data (such as a full newsfeed.)

The important register settings follow. These appear to be available on

all Telebits (we have an antique Trailblazer and a T2500).

S51=5     Interface speed of 19200. Even if you connect at lower speeds,

the modem<->computer connection should be constantly 19200.

S52=1     Actually either =1 or =2 is appropriate. When UUCICO is done, it

turns off DTR to drop the connection. The default Telebit setting

of 0 does not have this behavior.

S58=2     Use full duplex RTS/CTS flow control. The factory setting

for the T2500 is 3, XON/XOFF, which is incredibly bad. XON/XOFF

*must* be disabled in order for UUCP to function.

A high speed connection has a tendency to be 'bursty' and

overrun interrupt drivers. The telebits use RTS/CTS flow control

to solve this problem. Generally RTS/CTS should be used on BOTH

sides of the connection, not just the Waffle side.

S66=1     Lock interface speed and use flow control in low speed modes.

The speed locked at is set with the S51 register.

Registers not available to all Telebits:

S111=30   Turn on UUCP spoofing. This is available on later Telebits or

those with ROM upgrades. UUCP spoofing should be enabled for all

PEP connections, or you'll get horrible throughput.

S180=0    For the T3000 only, turn off MNP.

CHOICE OF UART

One common problem at higher speeds (over 2400) is the UART used. If you

have an 8250 it probably will not support 9600 or 19200 correctly. We

recommend finding a serial card that has a removable UART and replacing it

with a National Semiconductor 16550A UART.

Also, some older async cards do not operate correctly at today's bus speeds.

(We have an old one in our collection that won't reliably function above 6MHZ.)

Preconfigured settings are available for the TB as entries in the

dialers file "TB9600" and "TB19200". These can be used even if you're

connecting to 2400 bps sites.

LOCKING THE BAUD RATE

With the Telebit and other high speed modems (or modems with MNP),

"locking" is generally used. Since the modem sends data in bursts, the speed

at which data is sent from the modem may exceed the connection speed. The

solution to this problem is to set the modem<->computer interface speed at

a higher rate; for example, a 2400b MNP connection might need a 4800b or

9600b modem<->computer setting.

Telebit modems using PEP should generally be locked at 19200b; there is a

static file parameter 'locked: 19200' for this. UUCICO will understand this

setting. If you want to lock UUCICO without locking the speed for other

Waffle programs, use the uu.locked parameter (this should not be necessary.)

USE OF A FOSSIL DRIVER

If you're using a Telebit or other high speed modem, you may want to use

a FOSSIL driver in place of Waffle's. In general, ASYNC drivers are

temperamental at high speeds. If your configuration works fine with Waffle's,

use it -- you may get higher throughput. But use a DRIVER driver if you

need 16550 FIFO's enabled.

Two excellent FOSSIL drivers for high speed use are BNU and X00. In addition

for being well tuned, both can be used with nonstandard IO/IRQ combinations.

For example, we have one 8250 configuration that worked fine with the native

driver using PEP. However, when we wanted to use V.32, we had to upgrade the

UART to a 16550 and switch to BNU FOSSIL driver (for FIFO's). Any other

configuration gave checksum errors when receiving data, which were really

due to dropped bytes (either by the UART or the driver):

Modem           Driver          UART          Result

PEP 19200       ( any )         8250          ok

PEP 19200       ( any )         16650A        ok

V.32 9600       ( any )         8250          dropped chars (hardware)

V.32 9600       Native          16550A        dropped chars (no FIFO's)

V.32 9600       FOSSIL BNU      16550A        ok

This was done on a 16 MHZ 80386 with a Telebit T2500. You shouldn't

rely on this table as gospel, since the data is VERY hardware

dependent. However, when using a high speed modem, it may give you an

idea of which direction to pursue.

--

dell@vox.darkside.com

(C) Copyright 1992 Darkside Int'l