BOOTFIX.SYS

by George A. Stanislav

BootFix is a device driver for Tandy 1000. I was not even aware of the

fact that interrupt 19h (warm boot) works on the Tandy 1000. Whenever I tried

to issue this interrupt, it locked up my system.

Then I read Quick Boot, by Dave Rowell, in the May 1987 issue of 80 MICRO.

He published a device driver called REBOOT.SYS which makes Tandy 1000 to have

a warm boot whenever Ctrl-Alt-Del is pressed. A warm boot as opposed to the

cold boot does not go through the initial checking. Therefore, a warm boot

is immediate, while a cold boot takes 16-20 seconds.

In his article Dave Rowell explained why interrupt 19 fails so often

on the Tandy 1000: If a memory resident program intercepts an interrupt and

changes its address, a warm boot will have that interrupt address point to

a random place in the memory which leads to a lockup of the system. His

driver saves the addresses of all the interrupts before they are changed.

When you hit Ctrl-Alt-Del, the driver restores the original interrupt address

table and issues interrupt 19 for the warm boot. That's why his driver has

to be installed as the first device in config.sys - oh well had to up untill

now.

I decided to take Dave Rowell's idea one step further and wrote a device

driver which too saves the original interrupt table. Then it intercepts inter-

rupt 19. If a program issues int 19, my driver will restore the original

interrupt address table and issue int 19 one more time. This time, as its

address was restored with the other interrupts, int 19 will do what it is

supposed to do: warm-boot the computer.

The main reason for writing this device driver was the fact that I am

running an Opus bulletin board. If any of the support programs of my BBS

(or even Opus itself) want to reboot the computer (e.g. watchdog) by issu-

ing int 19, the computer would lock up and the board would be down.

With the driver installed, the computer will reboot and restart the

bulletin board from autoexec.bat. That way I can go on vacation trusting

that two weeks later I will find my bulletin board up and running.

Another advantage of BOOTFIX.SYS is that during warm boots the

system time remains unchanged. The date is lost (can be reentered),

but the time of the day is preserved. I wished I had it before I got

my battery backed up clock! Bulletin boards are time driven, so preserv-

ing the time at warm reboot maybe the most important reason why you

install this driver.

As I am not running the only Tandy 1000 Opus bulletin board, I am

releasing my driver for public use. I am sure that other Tandy 1000 owners

will find it useful as well. Some programs issue interrupt 19 when they

get into a blind alley. Others do when they think their copy protection

scheme was violated. Some older programs may issue int 19 just for the

fun of it whenever they are finished.

BOOTFIX.SYS is my first step in developing a FOSSIL driver for the

Tandy 1000. A FOSSIL driver is expected to be able to reboot the computer

either in the cold or the warm way. My FOSSIL driver will be able to do

either. BOOTFIX.SYS was my "study" part of developing the larger driver.

How to use BOOTFIX.SYS? It has to be installed as a device in the

config.sys file like this:

device=[drive:][\path\]bootfix.sys

I have mine in my opus\util directory on drive c:. As I boot from

drive c:, I do not need to mention the drive. This is what is in my

config.sys:

device=opus\util\bootfix.sys

Now, the ABSOLUTELY IMPORTANT thing is that this driver M*U*S*T

be installed before any other driver, including Dave Rowell's REBOOT.SYS.

You may be using drivers like ANSI.SYS, KEYCNVRT.SYS, BRK.SYS etc. All these

must be installed after BOOTFIX.SYS. The reason is that most likely these

drivers change the address of an interrupt. If BOOTFIX.SYS were loaded after

any of them, it could not save the original interrupt address table.

If you use Dave Rowell's REBOOT.SYS on your Tandy 1000 together with my

BOOTFIX.SYS (a combination I highly recommend), REBOOT.SYS no longer has to

be the first device installed. As long as BOOTFIX.SYS is the first device

installed, REBOOT.SYS can be anywhere following (before or after any other

drivers, e.g. ANSI.SYS). In that case, REBOOT.SYS will restore a wrong inter-

rupt table. That however does not matter, as REBOOT.SYS ends by calling int 19

which will be fixed with BOOTFIX.SYS.

I tested BOOTFIX.SYS on two Tandy 1000 computers, one was an older one

(the original Tandy 1000). The other one was a Tandy 1000 A. I tested it

with 3 versions of DOS: MS-DOS 2.11 which came from Tandy, PC DOS 3.10 (the

IBM version) and "generic" MS DOS 3.20. With all three versions BOOTFIX.SYS

worked with no difficulties.

I tested it after I loaded several memory resident programs including

the "famous offenders" PRINT and SIDEKICK. Issuing int 19 resulted in an

immediate warm reboot. Issuing the same interrupt under the same circum-

stances but without BOOTFIX.SYS installed resulted in the lock-up of the

system.

One final note to novice users: A device driver is installed at the

boot time. That means after you add device=bootfix.sys into your

config.sys you have to reboot to install it for the first time.

I have enclosed a test program INT19.COM which does nothing but

issuing interrupt 19. You can type INT19 from the DOS prompt. If it

locks up your computer (which is likely), you need BOOTFIX.SYS. If it

reboots the computer and does not lock it up, you can do well without

BOOTFIX.SYS. The program is meant for Tandy 1000 users only. I do not

know if other PC's need it or could use it. You can test it with INT19.COM

if you wish. If you find out that this program helps on other computers,

please let me know. I can be reached on my bulletin board (you need a

modem to call it) at (412) 571-0472. You can probably send me a message

from any Opus bulletin board. Just make sure it is addressed to 129/39.