INSTALLATION PROCEDURE FOR AN IDE DRIVE

1. Install a 40 pin Data Cable ensuring that pin 1, which can be

identified by the striped edge of the cable, is closest to the

power connector on the drive.

2. Install a DC power cable to the back of the drive.

3. Verify the jumper configurations for Master/Slave operation

(Note:  Master will be the bootable drive. The slave will not

be bootable.) Also make sure the existing C: drive is jumped

to be the Master in a two drive system, not the only drive in

the system.

4. Apply power to the computer.

4a. When memory test is complete go into your system's Standard

CMOS set-up.

(Note:  There are various ways to get into CMOS set-up,

please refer to system's manual for instructions.)

4b. If your system's BIOS supports a user programmable drive

type, program the BIOS with the default parameters of your

drive. If your system does not support a user programmable

drive type choose parameters that closely match but do not

exceed the drives MegaByte capacity. Escape from set-up

then choose write to CMOS and exit.

5. Boot from a DOS diskette that has FDISK.EXE and FORMAT.COM on

it. At the A> prompt type in FDISK. At the menu options select

option 1 to create a DOS partition. Another menu will appear

and from those options choose 1 to create a Primary DOS

partition. Select yes to make 1 large partition and it will

automatically become active. Then escape from FDISK.

6. At the A> prompt type in FORMAT C:/S This does a high-level

format on the drive and transfers the system files in order for

the drive to be bootable. (Note: IDE drives are low-level

formatted from the manufacturer and only need a high-level

format).

To configure the drive as a slave drive repeat steps 1-4 and

proceed with steps 5a. and 6a. as follows:

5a. At the C> prompt type in FDISK. When the menu options appear

select option 5 to switch to the second drive. Enter fixed

disk drive number 2. Then choose option 1 to create a DOS

partition, then select option 1 again to create a Primary DOS

partition or option 2 to create an Extended DOS partition.

(Note: C and D drives will always be the Primary partitions

but only the Primary partition on C: will have a status of

active).

6a. Proceed with a high-level format on the drive by typing FORMAT

D: (Note: Make sure the correct drive letter has been selected

for format).

7. The drive is now bootable. As a test remove the DOS diskette

from A and press reset to reboot the computer,  a  C> should be

displayed, the drive is now ready for operation.

Patty Barbee  01/18/94   Rev A