Installing the NetWare OS/2 Client Driver

To install the OS/2 ODI driver, the Novell OS/2 Requester should be

used. Load NetWare OS/2 ODI driver from the OS/2 CONFIG.SYS file in the

sequence below. For more information on installing the OS/2 ODI driver,

refer to the "NetWare Requester for OS/2 installation" documentation.

1. Using the NetWare OS/2 disks provided by Novell, install the NetWare OS/2

Requester on the OS/2 system. Follow the instructions on screen to

configure the NetWare OS/2 Requester for a NetWare-supported LAN adapter

board. (i.e. NE2000)

2. Copy the OS/2 ODI driver, LCS8634.SYS, from the Driver Diskette to the

C:\NETWARE directory.

3. Edit the CONFIG.SYS file and replace the NetWare-supported LAN adapter

driver with the name of the OS/2 ODI driver, LCS8634.SYS. For example:

DEVICE=C:\NETWARE\LCS8634.SYS

4. The default settings (PORT=300,INT=3) may be modified when the driver

loads by creating the file NET.CFG. Also the OS/2 ODI driver may also be

installed to support multiple protocol stacks and Ethernet frame types.