How to add an (S)VGA driver to XFree86

Copyright (c) 1993, 1994 David E. Wexelblat

<dwex@XFree86.org>

Issue 1.3 - May 29, 1994

1.  Introduction

Adding support for a new SVGA chipset to XFree86 is a challenging project for

someone who wants to learn more about hardware-level programming.  It can be

fraught with hazards (in particular, crashing the machine is all too common).

But in the end, when the server comes up and functions, it is immensely satis-

fying.

Adding support for an SVGA chipset does not change any of the basic functioning

of the server.	It is still a dumb 8-bit PseudoColor server or 1-bit StaticGray

server.  Adding support for new hardware (e.g. accelerated chips) is a major

undertaking, and is not anywhere near formalized enough yet that it can be doc-

umented.

Nonetheless, the driver-level programming here is a good introduction.	And can

well be the first step for adding support for an accelerated chipset, as many

are SVGA-supersets.  Writing an SVGA-level driver for the chipset can provide a

stable development platform for making use of new features (in fact, this has

been done for the S3, Cirrus, and WD accelerated chipsets, for internal use as

the accelerated servers are developed for XFree86 2.0).

Now let's get down to it.  In addition to this documentation, a stub driver has

been provided.	This should provide you a complete framework for your new

driver.  Don't let the size of this document persuade you that this is an

overly difficult task.	A lot of work has been put into making this document as

close to complete as possible; hence it should, in theory, be possible to use

this as a cookbook, and come out with a working driver when you reach the end.

I do advise that you read it all the way through before starting.

2.  Getting Started

The first step in developing a new driver is to get the documentation for your

chipset.  I've included a list of vendor contact information that I have col-

lected so far (it's far from complete, so if you have any that isn't on the

list, please send it to me).  You need to obtain the databook for the chipset.

Make sure that the person you speak to is aware that you intend to do register-

level programming (so they don't send you the EE-style datasheet).  Ask for any

example code, or developer's kits, etc.  I've learned that at the SVGA level,

in general, a databook that lists and describes the registers is the most you

can hope to find.

If you are not familiar with VGA register-level programming, you should get

How to add an (S)VGA driver to XFree86

How to add an (S)VGA driver to XFree86

(and read!) a copy of Richard Ferraro's bible (see references below).  The best

way to understand what is happening in the server is to study the workings of

the monochrome server's ``generic'' server, and compare it with the documenta-

tion in Ferraro's book (be aware that there are a few errors in the book).  You

can find the generic-VGA-register handling functions in the file ``vgaHW.c''.

Once you understand what's happening in the generic server, you should study

one or more of the existing SVGA drivers.  Obtain the databook for a supported

SVGA chipset, and study the documentation along with the code.	When you have a

good understanding of what that driver does over and above the generic VGA, you

will know what information you need to obtain from the databook for the new

chipset.  Once you have this information, you are ready to begin work on your

new driver.

3.  Directory Tree Structure

Here is an outline of the directory tree structure for the source tree.  Only

directories/files that are relevant to writing a driver are presented.	The

structure for the Link Kit is presented below.

xc/config/cf/

site.def

Local configuration customization

xf86site.def

XFree86 local configuration customization

xc/programs/Xserver/hw/xfree86/

The server source

common/

Files common to all of the server (XF86Config parser,

I/O device handlers, etc)

xf86.h

Contains the `ScrnInfoRec' data structure

xf86_Option.h

Contains option flags

compiler.h

Contains in-line assembler macros and util-

ity functions

os-support/

OS-support layer

assyntax.h

Contains macro-ized assembler mnemonics

xf86_OSlib.h

OS-support includes, defines, and

How to add an (S)VGA driver to XFree86

prototypes

LinkKit/

site.def.LK

Template for Link Kit site.def

vga256/

256-color VGA server directories

vga/

The generic VGA handling code

vga.h

Contains the `vgaVideoChipRec'

and `vgaHWRec' data structures

vgaHW.c

Contains the generic-VGA-regis-

ter handling functions vgaH-

WInit(), vgaHWSave() and vgaH-

WRestore().

drivers/

Contains the SVGA driver subdirectories.

Each contains an Imakefile, a .c file for

the driver, and a .s file for the bank-

switching functions.

vga2/

The monochrome vga server directories.	Most of the

files are linked from vga256, and the differences han-

dled by conditional compilation.

drivers/

The SVGA driver subdirectories.  The

`generic' VGA driver is also located here.

vga16/

The 16-color vga server directories.  Most of the files

are linked from vga256, and the differences handled by

conditional compilation.

drivers/

The SVGA driver subdirectories.

VGADriverDoc/

This documentation and the stub driver.

The Link Kit is usually installed in /usr/X11R6/lib/Server.  The Link Kit con-

tains everything that is needed to relink the server.  It is possible to write

a new driver and build a new server without having even the server source

installed.

How to add an (S)VGA driver to XFree86

Server/

site.def

Local configuration customization

include/

All of the include files listed under the `common'

directory above

drivers/

All of the SVGA drivers

vga2/

The SVGA driver subdirectories.

vga16/

The SVGA driver subdirectories.

vga256/

The SVGA driver subdirectories.

VGADriverDoc/

The directory with this documentation and the stub

driver.  `vgaHW.c' is also copied here, for reference

(it is not built as part of the Link Kit).

4.  Setting Up The Build Information

This section describes the peripheral configuration and build steps that must

be performed to set up for your new driver.  The steps are the same whether you

are building from the source tree of from the Link Kit; only the locations of

the files is different.  Here are the configuration steps that must be fol-

lowed:

1.	    Choose the name for your driver subdirectory and data structures.

Since the current driver scheme allows (in fact, encourages) putting

drivers for multiple related chipsets in a single driver, it is usually

best to use the vendor name, rather than a chipset version.  The fact

that older XFree86 drivers do not follow this convention should not deter

you from using it now - most of that code was developed before the driver

interface had been made flexible and extensible.

For this documentation, we'll use chips from the SuperDuper Chips vendor.

Hence, we'll use `sdc' for the name of the driver.

2.	    Decide whether your driver will support the color server, the

monochrome server, or both.  For this documentation, we will assume that

both the color and monochrome servers will be supported.	If you intend

to support only the color server, the steps for the monochrome server can

be ignored.  If you intend to support only the monochrome server, the

steps for the color server listed should be performed for the monochrome

server, and the monochrome steps ignored.  Most of the existing drivers

support only the color or both servers; the ``generic'' driver is the

How to add an (S)VGA driver to XFree86

only driver (currently) that supports just the monochrome server.

3.	    Create your driver directories:

o If you are working in the source tree, create the following directo-

ries:

xc/programs/Xserver/hw/xfree86/vga256/drivers/sdc

xc/programs/Xserver/hw/xfree86/vga16/drivers/sdc

xc/programs/Xserver/hw/xfree86/vga2/drivers/sdc

o If you are working in the Link Kit, create the following directo-

ries:

/usr/X11R6/lib/Server/drivers/vga256/sdc

/usr/X11R6/lib/Server/drivers/vga16/sdc

/usr/X11R6/lib/Server/drivers/vga2/sdc

4.	    Set up the Imakefile parameters to cause your driver to be built:

o If you are working in the source tree:

1.  Edit the file xc/config/cf/xfree86.cf, and add `sdc' to the

list for the definitions for `XF86Vga256Drivers',

`XF86Vga16Drivers' and `XF86Vga2Drivers'.  You should put

`sdc' just before `generic' in the list (i.e. second last), to

ensure that none of the other driver's probe functions incor-

rectly detect the `sdc' chipset .

2.  Edit the file xc/config/cf/xf86site.def, and add the same

entries in this file (this is just a comment that shows the

default values).

3.  Edit the site.def.LK file in xc/pro-

grams/Xserver/hw/xfree86/LinkKit/, and add the same entries in

this file.  This is the prototype `site.def' file that will be

installed in the Link Kit.

o If you are working in the Link Kit, edit the file

/usr/X11R6/lib/Server/site.def, and add `sdc' to the

`XF86Vga256Drivers', `XF86Vga16Drivers' and `XF86Vga2Drivers' defi-

nitions as described in (a) above.

5.	    Now copy the prototype files into your new directories:

How to add an (S)VGA driver to XFree86

o If you are working in the source tree, copy the `stub' files as fol-

lows (directories are below xc/programs/Xserver):

Imakefile.stub =>

hw/xfree86/vga256/drivers/sdc/Imakefile

stub_driver.c =>

hw/xfree86/vga256/drivers/sdc/sdc_driver.c

stub_bank.s =>

hw/xfree86/vga256/drivers/sdc/sdc_bank.s

Imakefile.stub =>

hw/xfree86/vga16/drivers/sdc/Imakefile (then edit this

Imakefile and make the changes described in the com-

ments).

Imakefile.stub =>

hw/xfree86/vga2/drivers/sdc/Imakefile (then edit this

Imakefile and make the changes described in the com-

ments).

o If you are working in the Link Kit, copy the `stub' files as fol-

lows:

Imakefile.stub =>

/usr/X11R6/lib/Server/drivers/vga256/sdc/Imakefile

stub_driver.c =>

/usr/X11R6/lib/Server/drivers/vga256/sdc/sdc_driver.c

stub_bank.s =>

/usr/X11R6/lib/Server/drivers/vga256/sdc/sdc_bank.s

Imakefile.stub =>

/usr/X11R6/lib/Server/drivers/vga16/sdc/Imakefile (then

edit this Imakefile and make the changes described in

the comments).

Imakefile.stub =>

/usr/X11R6/lib/Server/drivers/vga2/sdc/Imakefile (then

edit this Imakefile and make the changes described in

the comments).

6.	    Edit each of the files you've just copied, and replace `stub' with

`sdc' and `STUB' with `SDC' wherever they appear.

That's all the prep work needed.  Now it's time to work on the actual driver.

5.  The Bank-Switching Functions

The normal VGA memory map is 64k starting at address 0xA0000.  To access more

than 64k of memory, SuperVGA chipsets implement ``bank switching'' - the high-

How to add an (S)VGA driver to XFree86

order address bits are used to select the bank of memory in which operations

will take place.  The size and number of these banks varies, and will be

spelled out in the chipset documentation.  A chipset will have zero, one or two

bank registers.  Likely the ONLY case of zero bank registers is a generic VGA,

and hence is not a concern.

Note that some of the newer chipsets (e.g. Trident 8900CL, Cirrus) allow for a

linear mapping of the video memory.  While using such a scheme would improve

the performance of the server, it is not currently supported.  Hence there is

no way to use such features for a new chipset.

Most SVGA chipsets have two bank registers.  This is the most desirable struc-

ture (if any banking structure can be called ``desirable''), because data can

be moved from one area of the screen to another with a simple `mov' instruc-

tion.  There are two forms of dual-banking - one where the two bank operations

define a read-only bank and a write-only bank, and one with two read/write win-

dows.  With the first form, the entire SVGA memory window is used for both read

a write operations, and the two bank registers determine which bank is actually

used (e.g. ET3000, ET4000).  With the second form, the SVGA memory window is

split into two read/write banks, with each bank pointer being used to control

one window.  In this case, one window is used for read operations and the other

for write operations (e.g. PVGA1/Western Digital, Cirrus).

A chipset that has a single bank register uses that one bank for both read and

write access.  This is problematic, because copying information from one part

of the screen to another requires that the data be read in, stored, and then

written out.  Fortunately, the server is able to handle both one-bank and two-

bank chipsets; the determination of behavior is defined by an entry in the

driver data structure described below.

A driver requires that three assembly-language functions be written, in the

file `sdc_bank.s'.  These functions set the read bank - SDCSetRead(), the write

bank - SDCSetWrite(), and set both banks - SDCSetReadWrite().  For a chipset

with only one bank, all three will be declared as entry points to the same

function (see the ``tvga8900'' driver for an example).

The functions are fairly simple - the bank number is passed to the function in

register %al.  The function will shift, bitmask, etc - whatever is required to

put the bank number into the correct form - and then write it to the correct

I/O port.  For chipsets where the two banks are read-only HERE and write-only,

the SetReadWrite() function will have to do this twice - once for each bank.

For chipsets with two independent read/write windows, the SetReadWrite() func-

tion should use the same bank as the SetWrite() function.

A special note - these functions MUST be written in the macroized assembler

format defined in the header file ``assyntax.h''.  This will ensure that the

correct assembler code will be generated, regardless of OS.  This macroized

format currently supports USL, GNU, and Intel assembler formats.

That's all there is to the banking functions.  Usually the chipset reference

will give examples of this code; if not, it is not difficult to figure out,

especially using the other drivers as examples.

How to add an (S)VGA driver to XFree86

6.  The Driver Itself

Now it's time to get down to the real work - writing the major driver functions

in the files sdc_driver.c.  First, an overview of what the responsibilities of

the driver are:

1.	      Provide a chipset-descriptor data structure to the server.  This

data structure contains pointers to the driver functions and some data-

structure initialization as well.

2.	      Provide a driver-local data structure to hold the contents of the

chipset registers.  This data structure will contain a generic part and a

driver-specific part.  It is used to save the initial chipset state, and

is initialized by the driver to put the chipset into different modes.

3.	      Provide an identification function that the server will call to

list the chipsets that the driver is capable of supporting.

4.	      Provide a probe function that will identify this chipset as dif-

ferent from all others, and return a positive response if the chipset

this driver supports is installed, and a negative response otherwise.

5.	      Provide a function to select dot-clocks available on the board.

6.	      Provide functions to save, restore, and initialize the driver-

local data structure.

7.	      Provide a function to set the starting address for display in the

video memory.  This implements the virtual-screen for the server.

8.	      Perhaps provide a function for use during VT-switching.

9.	      Perhaps provide a function to check if each mode is suitable for

the chipset being used.

Before stepping through the driver file in detail, here are some important

issues:

1.	      If your driver supports both the color and monochrome servers,

you should take care of both cases in the same file.  Most things are the

same - you can differentiate between the two with the MONOVGA #define.

If the 16 color server is supported, code specific to it can be enabled

with the XF86VGA16 #define.  In most cases  it is sufficient to put the

following near the top of the stub_driver.c file:

#ifdef XF86VGA16

#define MONOVGA

#endif

2.	      The color server uses the SVGA's 8-bit packed-pixel mode.  The

monochrome and vga16 servers uses the VGA's 16-color mode (4 bit-planes).

How to add an (S)VGA driver to XFree86

Only one plane is enabled for the monochrome server.

3.	      It is possible for you to define your monochrome driver so that

no bank-switching is done.  This is not particularly desirable, as it

yields only 64k of viewing area.

Keeping these things in mind, you need to find the registers from your SVGA

chipset that control the desired features.  In particular, registers that con-

trol:

1.	      Clock select bits.  The two low-order bits are part of the stan-

dard Miscellaneous Output Register; most SVGA chipsets will include 1 or

2 more bits, allowing the use of 8 or 16 discrete clocks.

2.	      Bank selection.  The SVGA chipset will have one or two registers

that control read/write bank selection.

3.	      CRTC extensions.	The standard VGA registers don't have enough

bits to address large displays.  So the SVGA chipsets have extension

bits.

4.	      Interlaced mode.	Standard VGA does not support interlaced dis-

plays.  So the SVGA chipset will have a bit somewhere to control inter-

laced mode.  Some chipsets require additional registers to be set up to

control interlaced mode

5.	      Starting address.  The standard VGA only has 16 bits in which to

specify the starting address for the display.  This restricts the screen

size usable by the virtual screen feature.  The SVGA chipset will usually

provide one or more extension bits.

6.	      Lock registers.  Many SVGA chipset prevent modification of

extended registers unless the registers are first ``unlocked''.  You will

need to disable protection of any registers you will need for other pur-

poses.

7.	      Any other facilities.  Some chipset may, for example, require

that certain bits be set before you can access extended VGA memory

(beyond the IBM-standard 256k).  Or other facilities; read through all of

the extended register descriptions and see if anything important leaps

out at you.

If you are fortunate, the chipset vendor will include in the databook some

tables of register settings for various BIOS modes.  You can learn a lot about

what manipulations you must do by looking at the various BIOS modes.

6.1  Multiple Chipsets And Options

It is possible, and in fact desirable, to have a single driver support multiple

chipsets from the same vendor.	If there are multiple supported chipsets, then

you would have a series of #define's for them, and a variable `SDCchipset',

which would be used throughout the driver when distinctions must be made.  See

the Trident and PVGA1/WD drivers for examples (the Tseng ET3000 and ET4000 are

counter-examples - these were implemented before the driver interface allowed

How to add an (S)VGA driver to XFree86

for multiple chipsets, so this example should NOT be followed).  Note that you

should only distinguish versions when your driver needs to do things differ-

ently for them.  For example, suppose the SDC driver supports the SDC-1a,

SDC-1b, and SDC-2 chipsets.  The -1a and -1b are essentially the same, but dif-

ferent from the -2 chipset.  Your driver should support the -1 and -2 chipsets,

and not distinguish between the -1a and -1b.  This will simplify things for the

end user.

In cases where you want to give the user control of driver behavior, or there

are things that cannot be determined without user intervention, you should use

``option'' flags.  Say that board vendors that use the SDC chipsets have the

option of providing 8 or 16 clocks.  There's no way you can determine this from

the chipset probe, so you provide an option flag to let the user select the

behavior from the XF86Config file.  The option flags are defined in the file

``xf86_option.h''.  You should look to see if there is already a flag that can

be reused.  If so, use it in your driver.  If not, add a new #define, and

define the string->symbol mapping in the table in that file.  To see how option

flags are used, look at the ET4000, PVGA1/WD, and Trident drivers.

6.2  Data Structures

Once you have an understanding of what is needed from the above description, it

is time to fill in the driver data structures.	First we will deal with the

`vgaSDCRec' structure.	This data structure is the driver-local structure that

holds the SVGA state information.  The first entry in this data structure is

ALWAYS `vgaHWRec std'.	This piece holds the generic VGA portion of the infor-

mation.  After that, you will have one `unsigned char' field for each register

that will be manipulated by your driver.  That's all there is to this data

structure.

Next you must initialize the `SDC' structure (type `vgaVideoChipRec').	This is

the global structure that identifies your driver to the server.  Its name MUST

be `SDC', in all caps - i.e. it must match the directory name for your driver.

This is required so that the Link Kit reconfiguration can identify all of the

requisite directories and global data structures.

The first section of this structure simply holds pointers to the driver func-

tions.

Next, you must initialize the information about how your chipset does bank

switching.  The following fields must be filled in:

1.	      ChipMapSize - the amount of memory that must be mapped into the

server's address space.  This is almost always 64k (from 0xA0000 to

0xAFFFF).  Some chipsets use a 128k map (from 0xA0000 to 0xBFFFF).  If

your chipset gives an option, use the 64k window, as a 128k window rules

out using a Hercules or Monochrome Display Adapter card with the SVGA.

2.	      ChipSegmentSize - the size of each bank within the ChipMapSize

window.  This is usually also 64k, however, some chipsets split the

mapped window into a read portion and a write portion (for example the

PVGA1/Western Digital chipsets).

How to add an (S)VGA driver to XFree86

3.	      ChipSegmentShift - the number of bits by which an address will be

shifted right to mask of the bank number.  This is log-base-2 of ChipSeg-

mentSize.

4.	      ChipSegmentMask - a bitmask used to mask off the address within a

given bank.  This is (ChipSegmentSize-1).

5.	      ChipReadBottom,ChipReadTop - the addresses within the mapped win-

dow in which read operations can be done.  Usually 0, and 64k, respec-

tively, except for those chipset that have separate read and write win-

dows.

6.	      ChipWriteBottom,ChipWriteTop - same as above, for write opera-

tions.

7.	      ChipUse2Banks - a boolean value for whether this chipset has one

or two bank registers.  This is used to set up the screen-to-screen oper-

ations properly.

There are three more fields that must be filled in:

1.	      ChipInterlaceType - this is either VGA_NO_DIVIDE_VERT or

VGA_DIVIDE_VERT.	Some chipsets require that the vertical timing numbers

be divided in half for interlaced modes.	Setting this flag will take

care of that.

2.	      ChipOptionFlags - this should always be `{0,}' in the data struc-

ture initialization.  This is a bitfield that contains the Option flags

that are valid for this driver.  The appropriate bits are initialized at

the end of the Probe function.

3.	      ChipRounding - this gets set to the multiple by which the virtual

width of the display must be rounded for the 256-color server.  This

value is usually 8, but may be 4 or 16 for some chipsets.

6.3  The Ident() function

The Ident() function is a very simple function.  The server will call this

function repeatedly, until a NULL is returned, when printing out the list of

configured drivers.  The Ident() function should return a chipset name for a

supported chipset.  The function is passed a number which increments from 0 on

each iteration.

6.4  The ClockSelect() function

The ClockSelect() function is used during clock probing (i.e. when no `Clocks'

line is specified in the XF86Config file) to select the dot-clock indicated by

the number passed in the parameter.  The function should set the chipset's

clock-select bits according to the passed-in number.  Two dummy values will be

passed in as well (CLK_REG_SAVE, CLK_SAVE_RESTORE).  When CLK_REG_SAVE is

passed, the function should save away copies of any registers that will be mod-

ified during clock selection.  When CLK_REG_RESTORE is passed, the function

should restore these registers.  This ensure that the clock-probing cannot cor-

rupt registers.

How to add an (S)VGA driver to XFree86

This function should return FALSE if the passed-in index value is invalid or if

the clock can't be set for some reason.

6.5  The Probe() function

The Probe() function is perhaps the most important, and perhaps the least intu-

itive function in the driver.  The Probe function is required to identify the

chipset independent of all other chipsets.  If the user has specified a

`Chipset' line in the XF86Config file, this is a simple string comparison

check.	Otherwise, you must use some other technique to figure out what chipset

is installed.  If you are lucky, the chipset will have an identification mecha-

nism (ident/version registers, etc), and this will be documented in the data-

book.  Otherwise, you will have to determine some scheme, using the reference

materials listed below.

The identification is often done by looking for particular patterns in regis-

ter, or for the existence of certain extended registers.  Or with some

boards/chipsets, the requisite information can be obtained by reading the BIOS

for certain signature strings.	The best advise is to study the existing probe

functions, and use the reference documentation.  You must be certain that your

probe is non-destructive - if you modify a register, it must be saved before,

and restored after.

Once the chipset is successfully identified, the Probe() function must do some

other initializations:

1.	      If the user has not specified the `VideoRam' parameter in the

XF86Config file, the amount of installed memory must be determined.

2.	      If the user has not specified the `Clocks' parameter in the

XF86Config file, the values for the available dot-clocks must be deter-

mined.  This is done by calling the vgaGetClocks() function, and passing

it the number of clocks available and a pointer to the ClockSelect()

function.

3.	      It is recommended that the `maxClock' field of the server's

`vga256InfoRec' structure be filled in with the maximum dot-clock rate

allowed for this chipset (specified in KHz).  If this is not filled in a

probe time, a default (currently 90MHz) will be used.

4.	      The `chipset' field of the server's `vga256InfoRec' structure

must be initialized to the name of the installed chipset.

5.	      If the driver will be used with the monochrome server, the

`bankedMono' field of the server's `vga256InfoRec' structure must be set

to indicate whether the monochrome driver supports banking.

6.	      If any option flags are used by this driver, the `ChipOption-

Flags' structure in the `vgaVideoChipRec' must be initialized with the

allowed option flags using the OFLG_SET() macro.

6.6  The EnterLeave() function

The EnterLeave() function is called whenever the virtual console on which the

How to add an (S)VGA driver to XFree86

server runs is entered or left (for OSs without virtual consoles, the function

is called when the server starts and again when it exits).  The purpose of this

function is to enable and disable I/O permissions (for OSs where such is

required), and to unlock and relock access to ``protected'' registers that the

driver must manipulate.  It is a fairly trivial function, and can be imple-

mented by following the comments in the stub driver.

6.7  The Restore() function

The Restore() function is used for restoring a saved video state.  Note that

`restore' is a bit of a misnomer - this function is used to both restore a

saved state and to install a new one created by the server.  The Restore()

function must complete the following actions:

1.	      Ensure that Bank 0 is selected, and that any other state informa-

tion required prior to writing out a new state has been set up.

2.	      Call vgaHWRestore() to restore the generic VGA portion of the

state information.  This function is in the vgaHW.c file.

3.	      Restore the chipset-specific portion of the state information.

This may be done by simply writing out the register, or by doing a

read/modify/write cycle if only certain bits are to be modified.	Be sure

to note the comment in the sample driver about how to handle clock-select

bits.

6.8  The Save() function

The Save() function is used to extract the initial video state information when

the server starts.  The Save() function must complete the following actions:

1.	      Ensure that Bank 0 is selected.

2.	      Call vgaHWSave() to extract the generic VGA portion of the state

information.  This function is in the vgaHW.c file.

3.	      Extract the chipset-specific portion of the state information.

6.9  The Init() function

The Init() function is the second most important function in the driver (after

the Probe() function).	It is used to initialize a data structure for each of

the defined display modes in the server.  This function is required to initial-

ize the entire `vgaSDCRec' data structure with the information needed to put

the SVGA chipset into the required state.  The generic VGA portion of the

structure is initialized with a call to vgaHWInit() (also located in vgaHW.c).

Once the generic portion is initialized, the Init() function can override any

of the generic register initialization, if necessary.  All of the other fields

are filled in with the correct initialization.	The information about the par-

ticular mode being initialized is passed in the `mode' parameter, a pointer to

a `DisplayModeRec' structure.  This can be dereferenced to determine the needed

parameters.

How to add an (S)VGA driver to XFree86

If you only know how to initialize certain bits of the register, do that here,

and make sure that the Restore() function does a read/modify/write to only

manipulate those bits.	Again, refer to the existing drivers for examples of

what happens in this function.

6.10  The Adjust() function

The Adjust() function is another fairly basic function.  It is called whenever

the server needs to adjust the start of the displayed part of the video memory,

due to scrolling of the virtual screen or when changing the displayed resolu-

tion.  All it does is set the starting address on the chipset to match the

specified coordinate.  Follow the comments in the stub driver for details on

how to implement it.

6.11  The ValidMode() function

The ValidMode() function is required.  It is used to check for any chipset-

dependent reasons why a graphics mode might not be valid.  It gets called by

higher levels of the code after the Probe() stage.  In many cases no special

checking will be required and this function will simply return TRUE always.

6.12  The SaveScreen() function

The SaveScreen() function is not needed by most chipsets.  This function would

only be required if the extended registers that your driver needs will be modi-

fied when a synchronous reset is performed on the SVGA chipset (your databook

should tell you this).	If you do NOT need this function, simply don't define

it, and put `NoopDDA' in its place in the vgaVideoChipRec structure initializa-

tion (NoopDDA is a generic-use empty function).

If you DO need this function, it is fairly simple to do.  It will be called

twice - once before the reset, and again after.  It will be passed a parameter

of SS_START in the former case, and SS_FINISH in the latter.  All that needs to

be done is to save any registers that will be affected by the reset into static

variables on the SS_START call, and then restore them on the SS_FINISH call.

6.13  The GetMode() function

The GetMode() function is not used as of XFree86 1.3; its place in the

vgaVideoChipRec should be initialized to `NoopDDA'.

At some point in the future, this function will be used to enable the server

and/or a standalone program using the server's driver libraries to do interac-

tive video mode adjustments.  This function will read the SVGA registers and

fill in a DisplayModeRec structure with the current video mode.

6.14  The FbInit() function

The FbInit() function is required for drivers with accelerated graphics sup-

port.  It is used to replace default cfb.banked functions with accelerated

chip-specific versions.  vga256LowlevFuncs is a struct containing a list of

functions which can be replaced.  This struct defined in vga256.h.  Examples of

FbInit() functions can be found in the et4000, pvga1 and cirrus drivers.

How to add an (S)VGA driver to XFree86

If you do NOT need this function, simply don't define it, and put `NoopDDA' in

its place in the vgaVideoChipRec structure initialization.

7.  Building The New Server

As in the setup work, the steps for building the server depend whether you are

working in the source tree or in the Link Kit.	Here are the steps for the ini-

tial build after installing your new driver files:

o If you are working in the source tree, follow these steps:

Go to xc/programs/Xserver, and enter `make Makefile', then `make Makefiles

depend all'

o If you are working in the Link Kit, follow these steps:

1.  Go to /usr/X11R6/lib/Server, and enter `./mkmf'

2.  In the same directory, enter `make'

To rebuild the server after the initial build (e.g. after making changes to

your driver):

o	  If you are working in the source tree, follow these steps:

1.	Go to the appropriate drivers/ directory (e.g., xc/pro-

grams/Xserver/hw/xfree86/vga256/drivers), and enter `make'.

2.	Go to xc/programs/Xserver, and enter `make loadXF86_SVGA' (to

link the color server), `make loadXF86_VGA16' (to link the 16 color

server) or `make loadXF86_Mono' (to link the mono server).

o	  If you are working in the Link Kit, follow these steps:

1.	Go to the appropriate driver directory, and enter `make'.

2.	Go to /usr/X11R6/lib/server, and enter `make loadXF86_SVGA' (to

link the color server) or `make loadXF86_VGA16' (to link the 16

color server) or `make loadXF86_Mono' (to link the mono server).

8.  Debugging

Debugging a new driver can be a painful experience, unfortunately.  It is

likely that incorrect programming of the SVGA chipset can lock up your machine.

More likely, however, is that the display will be lost, potentially requiring a

reboot to correct.  It is HIGHLY recommended that the server be run from an

attached terminal or a network login.  This is the only rational way in which a

debugger can be used on the server.  Attempting to use multiple VTs for debug-

ging is basically a waste of time.

How to add an (S)VGA driver to XFree86

Because of the potential for locking up the machine, it is a VERY good idea to

remember to do a `sync' or two before starting the server.  In addition, any

unnecessary filesystems should be unmounted while the debugging session is

going on (to avoid having to run unnecessary fsck's).

By default the server is built without debugging symbols.  The server can grow

VERY large with debugging enabled.  It is very simple to rebuild your driver

for debugging, though.	Do the following:

1.  Go to the driver directory.

2.  Edit the Makefile.  Look for the SECOND definition of `CDEBUGFLAGS'.

Change this definition to

CDEBUGFLAGS = -g -DNO_INLINE

(this will enable debugging symbols and disable inlining of functions,

which can make single-stepping a nightmare).

3.  Remove the `sdc_driver.o' file.

4.  Now follow the steps above for rebuilding the server.

(Alternatively, instead of editing the Makefile, you can simply do `make

CDEBUGFLAGS="-g -DNO_INLINE"' after removing the old .o file, then

rebuild the server as described above).

This will give you a server with which you can set breakpoints in the driver

functions and single-step them.  If you are working in the source tree, and

just learning about SVGA programming, it may be useful to rebuild vgaHW.c with

debugging as well.

9.  Advice

I cannot stress this enough - study all available references, and the existing

code, until you understand what is happening.  Do this BEFORE you begin writing

a driver.  This will save you a massive amount of headache.  Try to find a

driver for a chipset that is similar to yours, if possible.  Use this as an

example, and perhaps derive your driver from it.

Do not let the gloom-and-doom in the debugging section	discourage you.  While

you will probably have problems initially (I still do), careful, deliberate

debugging steps can bear fruit very quickly.  It is likely that, given a good

understanding of the chipset, a driver can be written and debugged in a day or

two.  For someone just learning about this kind of programming, a week is more

reasonable.

How to add an (S)VGA driver to XFree86

10.  Advanced Topics

Newer chipsets are getting into two advanced areas: programmable clock genera-

tors, and accelerated capabilities (BitBlt, line drawing, HW cursor).  These

are new areas, and the formal interfaces to them are not yet defined.  It is

advised that you contact the XFree86 team and get involved with the develop-

ment/beta-testing team if you need to be working in these areas.

11.  References

o Programmer's Guide to the EGA and VGA Cards, 3rd ed.

Richard Ferraro

Addison-Wesley, 1994

ISBN 0-201-62490-7

(This is the bible of SVGA programming - it has a few errors, so watch

out.  The third edition also covers several accelerated video cards.)

o vgadoc4.zip

Finn Thoegersen

(This is a collection of SVGA and other chipset documentation.  It is

available on most MS-DOS/Windows related FTP archives, including

wuarchive.  It is DOS/BIOS oriented, but is still extremely useful, espe-

cially for developing probe functions.)

12.  Vendor Contact Information

ATI Technologies (VGA-Wonder, Mach8, Mach32)

33 Commerce Valley Drive East" Thornhill, Ontario

Canada L3T 7N6

(905) 882-2600 (sales)

(905) 882-2626 (tech support)

(905) 764-9404 (BBS)

(905) 882-0546 (fax)

Chips & Technologies

???

Cirrus Logic (SVGA, Accelerators - CL-GD5426)

3100 West Warren Ave.

Fremont, CA  94538

How to add an (S)VGA driver to XFree86

(510) 623-8300 (N. CA, USA)

(49) 8152-40084 (Germany)

(44) 0727-872424 (UK)

Genoa Systems (GVGA)

75 E. Trimble Road

San Jose, CA 95131

(408) 432-9090 (sales)

(408) 432-8324 (tech support)

Headland Technologies, Inc (Video-7 VGA 1024i, VRAM II)

46221 Landing Parkway

Fremont, CA  94538

(415) 623-7857

Oak Technology, Inc (OTI-067,OTI-077)

139 Kifer Ct.

Sunnyvale, CA 94086

(408) 737-0888

(408) 737-3838 (fax)

S3 (911, 924, 801/805, 928, 864, 868, 964, 968, 764, 765)

(408) 980-5400

Trident Microsystems Inc (8800, 8900, 9000)

205 Ravendale Dr

Mountainside, CA 94043

(415) 691-9211

Tseng Labs Inc,

6 Terry Drive

Newtown, PA  18940

(215) 968-0502

Weitek (Power9000, 5186)

1060 E. Arques Ave,

Sunnyvale, CA  94086

(408) 738-5765

How to add an (S)VGA driver to XFree86

Western Digital

(714) 932-4900

Generated from XFree86: xc/programs/Xserver/hw/xfree86/doc/sgml/VGADriv.sgml,v 3.13 1997/01/25 03:22:16 dawes Exp $

$XConsortium: VGADriv.sgml /main/9 1996/10/28 05:13:22 kaleb $

How to add an (S)VGA driver to XFree86

CONTENTS

1.  Introduction  ........................................................... 1

2.  Getting Started  ........................................................ 1

3.  Directory Tree Structure  ............................................... 2

4.  Setting Up The Build Information  ....................................... 4

5.  The Bank-Switching Functions  ........................................... 6

6.  The Driver Itself  ...................................................... 8

6.1  Multiple Chipsets And Options	..................................... 9

6.2  Data Structures  .................................................. 10

6.3  The Ident() function  ............................................. 11

6.4  The ClockSelect() function  ....................................... 11

6.5  The Probe() function  ............................................. 12

6.6  The EnterLeave() function  ........................................ 12

6.7  The Restore() function  ........................................... 13

6.8  The Save() function  .............................................. 13

6.9  The Init() function  .............................................. 13

6.10 The Adjust() function	............................................ 14

6.11 The ValidMode() function  ......................................... 14

6.12 The SaveScreen() function  ........................................ 14

6.13 The GetMode() function  ........................................... 14

6.14 The FbInit() function	............................................ 14

7.  Building The New Server  ............................................... 15

8.  Debugging  ............................................................. 15

9.  Advice  ................................................................ 16

10. Advanced Topics  ....................................................... 17

11. References	............................................................ 17

12. Vendor Contact Information	............................................ 17

i