--------------------------------------------------------------------------------

RELEASED WITH IMPHOBIA 5 DISK MAGAZINE!

--------------------------------------------------------------------------------

Visual Player

Version 2.0, Barcelona, March '93

Programmed by:

H�ctor Mart�nez, Luis Crespo, Luis Ubeda, David L�pez y Angel Trigo.

Visual Player is a graphic MOD player.

While the music is playing, Visual Player can display the waveform of the

four voices of the MOD, the final waveform, four real volume bars, a

realtime spectrum analyzer and psychedelic lights. This program has been

made to "look" at the music while you listen to it. Everything in Visual

Player is real, everything that is displayed is calculated from the digital

data and not from the score.

How to use VP:

--------------

To run Visual Player, you must enter:

VP

At the DOS command prompt. Optionally, you can use wildcards when specifying

the file name, for example:

VP a*

Will play all the MOD files that start with "a".

Hardware requirements:

----------------------

VP can play music through the following devices:

- The internal PC speaker

- Sound Blaster

- Sound Blaster Pro (stereo)

- Sound Blaster 16 ASP (stereo, 16 bits, 44.1 KHz)

- Adlib

- Stereo In One

- One or two Digital to Analog Converters (DAC) connected to LPT1 and LPT2.

It is very cheap and easy to build a DAC and plug it on the printer port.

A simple circuit design is given below.

VP needs a VGA card, and because of the graphic features of the program, the

faster the computer is, the better VP will run. The sound processing speed

depends also very much on the sound device. If you use a sound device without

DMA, such as the internal PC speaker or the DAC in the LPT, it is better not

to use programs and drivers that leave the 386 in virtual mode, like EMM386,

QEMM, 386MAX, etc, because they slow down the computer and they decrease the

sound quality.

If the display is refreshed very slowly, you can disable some graphic options

or decrease the sampling frequency. If your screen shows some kind of snow,

it can be because of the psychedelic lights option, so to avoid the snow you

should disable that option.

The mouse is completely necessary, because the program works by pushing

buttons, selecting options, moving controls, etc, very much like Windows.

VP takes up quite a bit of memory, so if more memory is available to the

program, it will be possible to load greater MOD files.

The Keys:

---------

Here is a quick keyboard reference for the impatient people:

PgUp     : Decrement Pattern

PgDown   : Increment Pattern

+        : Increment Sampling Frequency

-        : Decrement Sampling Frequency

1..4     : Toggle Voice

S        : OS Shell

Enter    : Next MOD

ESC      : Quit Visual Player

F1       : Help

F3       : File load window

The mixing frequency:

---------------------

The mixing frequency, also called sampling frequency or sampling rate, is

the speed at which the sound is processed. That means that the higher the

sampling frequency, the better the quality of the sound, especially with

the high pitched sounds like charles, bells, etc. A fast machine will allow

a faster sampling frequency, and a sound device with DMA will allow higher

sampling frequencies. If you have a slow machine and run the program with

a high sampling rate, the graphic display will slow down, because the

program will dedicate more time to process the sound.

The graphic options:

--------------------

Each of the graphic element shown on screen can be enabled or disabled

independently.

To do it, you just have to click the mouse button with the mouse pointer

inside the desired element. For example, if you want to disable the central

oscilloscope, you must click the mouse button with the mouse pointer inside

the central oscilloscope frame. If you click the mouse button again inside

the frame, the oscilloscope gets enabled again.

The oscilloscopes:

------------------

The 4 upper oscilloscopes display the waveform of the voices that are being

played, and the central oscilloscope displays the sum of the 4 voices. The

graphs are synchronized with the beginning of the waveform, so that in

most of the cases the waveform will be displayed at the same place.

The disposition of the 4 oscilloscopes has changed from previous versions of

the program. In this version, the oscilloscopes labelled as "voice 1" and

"voice 2" at the left side of the screen will sound through the left channel

and the ones labelled as "voice 3" and "voice 4" will sound through the right

channel.

The volume bars:

----------------

The volume bars are located at each side of the central oscilloscope, and are

placed so that the voices that sound through the left channel, voices 1 and 2,

are at the left side of the oscilloscope, and the voices that sound through

the right channel, voices 3 and 4, are at the right side of the oscilloscope.

Remark that from the point of view of a MOD composer or any other MOD player,

voices 1 and 2 match with 1 and 4, and 3 and 4 with 2 and 3 respectively.

The spectrum analyzer:

----------------------

The spectrum analyzer is the most time-consuming feature of the program. It

displays the power of each frequency in the sound. The spectrum analyzer is

implemented with an FFT algorithm with integer arithmetic. The FFT is

calculated from N samples taken from a buffer that sounds during a 1/50th of

second, so that the first spectrum bar shows the power of F=0 Hz, the second

shows F=50 Hz, the third F=100 Hz, the fourth F=150 Hz, and so on,

independently of the mixing frequency.

There are 5 different spectrum analyzers. The first one analyzes 128 samples

and displays 64 bars (the other 64 are symmetrical). The second method

analyzes 64 samples and displays 32 bars, so it is faster but less precise

than the first method. The third method analyzes 128 samples and displays 32.

The fourth and the fifth display a stereo spectrum analyzer. The fourth

method analyzes 64 samples for each channel and displays 32 bars for each

channel, and the fifth analyzes 128 samples and displays 32 bars for each

channel. The later method is the one that consumes more CPU time.

The psychedelic lights:

-----------------------

The 3 circles at the right side of the central oscilloscope are 3 psychedelic

lights. The red light will shine with the bass frequencies, the green one

with the middle frequencies and the yellow one with the high frequencies. To

know the intensity of each light the FFT has to be calculated, the same way

as the spectrum analyzer. So if you disable only one of both options, the FFT

will still be calculated to display the other one.

The lower button panel:

-----------------------

When VP is loaded, you can only see 6 buttons on screen: 4 buttons to toggle

the voices, one to get help and another one to quit the program. But the

screen does not end at the lower part of the spectrum analyzer. If you move

the mouse down, the whole screen will scroll up, letting you see the lower

part of the program panel. Three different parts can be distinguished there:

- at the left side, the name of each instrument of the current MOD file is

shown. This zone will change depending of the selected function on the right

button panel.

- at the center-right part, there is a button panel with the look of a CD or

cassette player panel, with very similar functions to those devices. The

lower buttons on this panel allow you to increment or decrement the current

pattern, and to load the next or previous MOD file.

- at the lower-right part there is a group of buttons which cover all kind of

functions, that are described further:

Load MOD:

---------

Opens a window with a listing of MOD files, and lets you select one to

load it and play it.

Load FLI:

---------

Opens a window with a listing of FLI files, and lets you load one to XMS

and animate it while the music sounds. This option is only available on

the registered version. With the registered version, you can also load a

FLI file from the command line, by writing something like:

VP modfile /FLI=flifile

In such a case the FLI will be animated while the MOD sounds, until a key

is pressed, going back to the command line.

SB Mixer:

---------

Opens a window with the SB Pro mixer controls. The changes that you make

to the mixer never get saved to the configuration file because it is

supposed that every user has the mixer already configured externally.

ADC Mode:

---------

This feature is independent with all relating to MOD files. When you push

the ADC mode button, the signal that is analyzed and displayed on the

oscilloscopes, spectrum analyzer, etc, is the input read from the SB, SB

Pro or SB 16 ASP.

So you can use Visual Player to listen to any kind of music and "see" it

on your screen. If the sound card is an SB, or an SB Pro, the 4 voices will

contain the same signal, and if the sound card is an SB 16 ASP, voices 1

and 2 will show the left channel and voices 3 and 4 will show the right

channel (in the next version, the SB Pro will also analyze in stereo mode).

Play Device:

------------

This window lets you select your sound hardware and the mixing frequency.

Config:

-------

Opens a window from which you can configure a lot of options of the

program. Four different sections can be configured:

- Palette: The program's color palette

- Spectrum: Lets you select the type of spectrum analyzer, the speed of

its bars, etc.

- Psycho: Lets you calibrate the sensibility of each psychedelic light

to the power of its frequency range.

- Misc: miscellaneous options.

- Loop MOD: if this option is activated, the MOD will be played

again when it finishes.

- Allow backward jumps: if activated, the program will allow backward

jumps on the music score. This means that maybe the MOD will never

end.

- Stereo mixing: If you are listening to the music with a stereo device,

the left and right channels can be mixed to achieve a less artificial

sensation. If this option is enabled, the program will mix a 75% of

the left channel and a 25% of the right channel to the left channel,

and vice-versa. This option is especially recommended if you use

headphones.

- Defective MOD loading: specifies the behavior in front of a

defective MOD. The file can be loaded always (ignoring the errors),

skipped if an error is found, or the program can show an error

message and ask the user if the file should be loaded or not.

At the bottom right part of the configuration window, there is a button

that lets you save the configuration, if you have the registered version

of the program. Remark that the "save" button will save the options of the

configuration windows and also the whole state of the program, including

the sound device, the mixing frequency, the graphic options, etc.

Big Psycho:

-----------

Shows a full-screen 256 color psychedelic. This feature is very useful for

parties, if combined with de ADC mode option ;-). All the usual keys work

the same way as in the normal mode, including ESC, that exits the program.

To go back to the normal mode you must just push the a mouse button.

OS Shell:

---------

Loads the command interpreter while the MOD file still sounds. To go back

to VP, type EXIT on the command line.

New in Version 2.0:

-------------------

- New graphic shell, with mouse support, 3d-buttons, windows-like controls,

vertical scroll, etc.

- SB 16 ASP support, at 16 bits, stereo and 44.1 KHz.

- The SB and SB Pro do not reverberate any more if an expanded memory

controller is loaded.

- The spectrum analyzer is more precise in the logarithmic scale.

- New graphic options: stereo spectrum analyzer, psychedelic lights.

- Load window, allowing to select a MOD from a directory.

- SB Pro mixer control.

- FLI animation while playing a MOD file.

- Input signal analyses, with the SB, SB Pro or SB 16 ASP (ADC Mode).

- Help window.

- Configuration window.

Next versions:

--------------

We have tested the program in a great variety of configurations, but due

to the size of the program, some bugs may still exist in this version. The

same way as with version 1.0, we would release version 2.1 to correct the

possible bugs.

Also, in the next versions, our program will have:

- Juke Box mode, allowing to play various MOD files from a list.

- Total SB 16 ASP mixer control.

- Resident OS Shell mode, allowing to control the state and options of the

program through a hotkey.

- Efficient XMS usage, to play several MOD files from an OS Shell.

- Packed MOD file format, to save disk space.

The latest version of the program can be found in:

- Dracker BBS, FidoNet 2:343/116, SBCNet 14:4500/100, CDNet 94:62/0.

Tel +34-3-3853393, HST D/S V32bis, 16800 bps.

- ST-Telecos, FidoNet 2:343/108

Tel +34-3-4017068, V32bis.

Credits:

--------

Greetings go to:

Juan Carlos Ar�valo, Rob de Luna, Alex Miquel, Joffre Heredia, Victor Neira,

Manolo Ruiz Moscoso, Juan Pedro Teruel, David L�pez Vinacua, Hakan Gustavsson

(The CodeBlasters, Sweeden), James Chow (SBNet, Canada), Steve McKnelly, Jos�

Mar�a Alvarez, David S�nchez, Xavier Mund� Balcells, Carlos L�pez Marco and

all the registered users of VP 1.1.

Shareware:

----------

Visual Player is distributed as shareware. Registering VP for personal use is

optional, but if you use the program for commercial purposes (for example, to

sell sound cards), you must register inconditionally.

If you have obtained VP from a friend, a BBS or similar source, you have an

unregistered version. The unregistered version sounds through the internal PC

speaker from the second MOD file, does not have some features of the program,

the ADC mode only lasts a while, and the program waits for a few seconds

before exiting.

When you register, you get:

- A 3'5", diskette with a copy of the latest version of VP registered

to you, with all the features fully working.

- Free next version registration.

- Due notice of every new version.

- Some MOD files that fit into the program diskette.

To register, send a letter to:

Luis Crespo

P.O. Box 93142

08080 Barcelona, SPAIN

With your full name and address and 3.000 Ptas if you live in Spain, or $30

if you live outside Spain. You can also register of only one version of

Visual Player, by sending 2.000 Ptas or $20.

Copyright/License/Warranty:

---------------------------

Visual Player must be distributed unmodified and with its full

documentation and files. The registered version must not be distributed.

Visual Player is copyright of the authors. The authors allow to: use

software, make copies of it, give copies to anybody and distribute it

through electronic media.

It is not allowed to ask for money or donations for any copy or copies of

the program, neither distribute the software and/or documentation with

commercial products, without previous written acknowledgement of the

authors.

There is not warranty of any kind, and the authors are not responsible for

any kind of damage that the use of the software may cause. When using this

software, you agree with everything written above.

How to contact the authors:

---------------------------

We are waiting for your comments and opinions about our program. Our

electronic addresses are:

Luis Crespo: FidoNet 2:343/108.21

David L�pez: FidoNet 2:343/107.989

The circuit:

------------

And here it is the circuit: a Digital to Analog Converter that plugs into

the printer port and to the amplifier. To build it, you will just need 18 1%

resistors, 2 capacitors, a male D-25 connector, and a jack. It is very

important that the resistors have 1% tolerance, because if they have 5% or

more, the thing will sound like hell.

Printer Port

signal   pin

20k   20k

D0       2      >��İ����İ����0v (GND, pin 20)

20k � 10k

D1       3      >��İ��Ĵ

20k � 10k

D2       4      >��İ��Ĵ

20k � 10k

D3       5      >��İ��Ĵ

20k � 10k

D4       6      >��İ��Ĵ

20k � 10k

D5       7      >��İ��Ĵ

20k � 10k

D6       8      >��İ��Ĵ

20k � 10k

D7       9      >��İ��Ĵ

20k � 10k  100nF

������Ĵ�����> To Amplifier

�         �

� 10k     � 10nF

�         �

GND      20     >��������������������>

0v

This circuit is an enhancement from Luis Ubeda from the one that came with

Mark J.Cox's ModPlay, so we thank Harry Stox for his original design.