STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows NT v3.51 NDIS v3.1 Driver

For Ethernet LAN Network Cards

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1996 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Supported network cards

o  An overview

o  Supplied files

o  Driver installation

- Primary installation (network card not presently installed)

- Network card already installed or older driver previously loaded

SUPPORTED NETWORK CARDS

o  EtherEZ Ethernet network cards

(SMC8416 board types)

o  Elite16 Ethernet network cards

(SMC8013 board types)

o  Ultra16 Ethernet network cards

(SMC8216 board type)

o  Elite network cards

(SMC8003 board type)

o  PLUS Ethernet network cards

(SMC/WD8003 board type)

o  EtherCard PLUS Micro Channel network cards

(SMC/WD8003xx/A or 8013xx/A board type)

OVERVIEW

This file explains how to install the Windows NT driver for x86-based

platforms.

SUPPLIED FILES

General files

o  NT_351.DOC       This file

o  RELEASE.DOC      Release notes for this version of the driver

Files to install the driver on Windows NT

o  SMC8000N.SY_     The driver

o  SMC8000N.HL_     The driver's associated help file

o  OEMSETUP.INF     The driver's setup file

DRIVER INSTALLATION

o Primary installation (network card not presently installed)

If you've already installed another card or loaded another driver,

refer to the procedure titled 'Network card already installed or

older driver previously loaded'.

NOTE:  If you experience any problems during driver installation,

you may choose to run EZStart and use its 'Automatic' feature

to configure the EtherEZ PnP SMC8416 network card. Refer to

the README.DOC file on the SuperDisk, or for further detail,

refer to the EZSTART.DOC file found within EZStart. EZStart

will not run in a Windows NT DOS window. You must boot from

DOS to setup and test the card.

1.  Complete the Windows NT installation without the SMC network card

installed in your PC. Have the SMC SuperDisk and Windows NT

installation files (on CD or hard drive) available. When prompted

with a window asking about 'Participating on a network', change

the default to 'Do not connect this computer to a network at

this time'. Click 'Next' and follow the prompts to reboot.

2.  Install the SMC network card. Start Windows NT and double-click

the 'Control Panel' and 'Network' icons.

3.  You will be prompted to install networking functionality. Click

'Yes'.

4.  Follow the screen prompts. When prompted, insert the Windows NT

CD or specify the path to those installation files on your hard

drive.

5.  When prompted with the 'Network Adapter Card Detection' dialog

box, select 'Do Not Detect'. Click on 'Continue'.

6.  When prompted with the 'Add Network Adapter' window, scroll to

and select '<Other> Requires disk from manufacturer'.

7.  Select 'Continue'.

8.  When prompted, insert the SuperDisk, specify the path to the root

directory (e.g., A:\), and click 'OK'.

9.  Select 'SMC 8416 EtherEZ' (or appropriate) and click 'OK'.

10. You will be prompted with the 'Adapter Card Setup' window.

Verify the correct network card has been detected. Click on 'OK'

to accept the default settings.  In most cases Windows NT will

detect the correct card settings and you will not have to change

anything.

NOTE:  If you select the IPX/SPX protocol, the 'NWLINK IPX/SPX

Protocol Configuration' window appears, permitting

customization of the protocol. Click 'OK'.

11. At this point, you can install additional protocols and network

components via the 'Windows NT Setup' window. Click 'Coninue'

when finished.

12. When presented with the 'Network Settings' window, click on 'OK'.

13. Follow the screen prompts as directed. Enter the 'Domain' or

'Workgroup' names when prompted (optional) and click 'OK'.

14. When prompted. select 'Restart Computer'.

NOTE:  In the unlikely event you encounter any device driver errors

after Windows NT boots, check the Event Log (under

'Administrative Tools' icon) to see whether the error is

related to your SMC network card.

o Network card already installed or older driver previously loaded

Follow this procedure to replace Windows NT driver files with the

latest SMC driver files. By removing driver files, you will lose

network settings. Make sure you have a record of settings such as

IP address, workgroup name, etc. Check with your network

administrator for complete information.

NOTE:  If you experience any problems during driver installation,

you may choose to run EZStart and use its 'Automatic' feature

to configure the EtherEZ PnP SMC8416 network card. Refer to

the README.DOC file on the SuperDisk, or for further detail,

refer to the EZSTART.DOC file found within EZStart. EZStart

will not run in a Windows NT DOS window. You must boot from

DOS to setup and test card.

1.  Install the SMC network card, attach network cable, and boot

Windows NT.

2.  Double-click the 'Control Panel' and 'Network' icons.

3.  When prompted with the 'Network Settings' window, highlight the

network card you wish to remove, click on 'Remove', and click on

'OK'.

4.  Select 'Restart Now' when prompted.

5.  Start Windows NT and double-click the 'Control Panel' and

'Network' icons. The 'Network Settings' window appears. Select

'Add Adapter'.

6.  When prompted with the 'Add Network Adapter' window, scroll to

and select '<Other> Requires disk from manufacturer'.

7.  Select 'Continue'.

8.  When prompted, insert the SuperDisk, specify the path to the root

directory (e.g., A:\), and click 'OK'.

9.  Select 'SMC 8416 EtherEZ' (or appropriate) and click 'OK'.

10. You will be prompted with the 'Adapter Card Setup' window.

Verify the correct network card has been detected. Click on 'OK'

to accept the default settings.  In most cases Windows NT will

detect the correct card settings and you will not have to change

anything.

11. At this point, you can install additional protocols and network

components via the 'Windows NT Setup' window. Click 'OK'

when finished.

NOTE:  If you select the IPX/SPX protocol, the 'NWLINK IPX/SPX

Protocol Configuration' window appears, permitting customization

of the protocol. Click 'OK'.

12. When prompted, select 'Restart Now'.

NOTE:  In the unlikely event you encounter any device driver errors

after Windows NT boots, check the Event Log (under

'Administrative Tools' icon) to see whether the error is

related to your SMC network card.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation, 80 Arkay Drive,

Hauppauge, NY 11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, PLUS, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.