* PERFECT BOX *

Loudspeaker Enclosure Simulation

using the

Neville Thiele - Richard Small Model

Version 4.5

January 1, 1990

Warren A. Merkel

2851 Newcomb Ct.

Orlando, FL  32826

Program registration is required.

See final page of this document for details.

***** PERFECT BOX ***** Version 4.5 ******

*  Program and documentation written by  *

*            Warren A. Merkel            *

******************************************

I.   INTRODUCTION:

PERFECT BOX is a Computer Aided Design (CAD) program for the

simulation of loudspeaker performance.  PERFECT BOX is a very

powerful tool for the amateur or professional speaker builder.

Loudspeaker design is a balance of compromises involving

enclosure size, low frequency cutoff, and transient response.

This program will allow you to model the loudspeaker-enclosure

combination entirely on your PC.  With proper use, you can be

assured of the best (lowest in frequency) bass response in the

smallest  enclosure.  You can then build the enclosure with full

confidence that the final result will not become the new dog

house for Fido, but an accurate, natural sounding system.

PERFECT BOX does not deal with midrange or tweeter drivers. It

only considers the low frequency driver in a multi-way system (or

the only driver in a full-range system.)  Since enclosure

alignments, such as box size, box type, and box frequency (now on

referred to as Alignments), effect the woofer response, PERFECT

BOX models systems covering low frequencies from 10 to 200 Hertz.

The mathematical model will hold for  higher  frequencies, but I

have optimized the program for the most useful range.

II.  REQUIREMENTS:

PERFECT BOX requires certain parameters that uniquely define the

driver's characteristics.  Some of these parameters have been

coined as the Thiele/Small (or T/S) parameters.  If you are new

to loudspeaker design, it isn't important that you understand

what each value represents, only that you are able to obtain

these values from the driver manufacturer, or measure them

yourself (the preferred method.)

PARAMETER                             UNITS

Free air resonance    (Fs)                   Hertz

Electrical   (Qes)

Q Mechanical (Qms)

Q Total   (Qts)

Volume   Equivalent   Compliance(Vas)         Ft^3

D.C.  Voice  Coil Resistance (Re)             Ohms

Peak  Cone Displacement  (Xmax)               Inch

Effective Piston Diameter                     Inch

Thermal Power (P)                         Watts, RMS

PERFECT BOX allows you to store these driver parameters in a data

file ("BOXDATA4").  This file can store characteristics for

thousands of different drivers.  A few sample drivers have been

included so that you can experiment with PERFECT BOX.  These

samples represent "real life" units which are currently available

at this time.

III. DEFINITIONS:

***SYSTEM ORDER***

PERFECT BOX uses a classical high-pass filter model to simulate a

loudspeaker mounted  in an enclosure.  The order of a loudspeaker

system represents the  same characteristic roll off as that of an

electrical filter.  The roll off rate (or slope) is:

roll off rate = (6  * system order) dB/frequency octave

or     roll off rate = (20 * system order) dB/frequency decade.

PERFECT BOX can simulate both infinite baffle (sealed box), and

vented (ported) systems.  Each of these systems have a natural

order in which they resemble an electrical high pass circuit.

For infinite baffle, order=2, or that is to say that they are of

2nd order.  And for vented systems, order=4. It seems that each

kind of enclosure has pros and cons associated with them.

In general, both  systems, if properly aligned, perform with the

same low frequency response.  In practice, an infinite baffle

system is easier to build, and possibly has better transient

response.   The vented system offers slightly higher efficiency

(2.9 dB), and good cone dampening near the vent frequency (but

less below Fb.)  My preference is for 4th and 6th order systems.

The sixth order system involves the use of a 4th order system

tuned slightly lower than the 4th order ideal, and an 2nd order

active equalizer circuit in the amplifier path.

***SYSTEM CUTOFF FREQUENCY***

This is the frequency at which the system amplitude response is

exactly 3.0 dB less than at a mid-band reference frequency (where

response is basically flat at 0 dB.)  This is a standard point of

interest that can be used to compare with another system's low

frequency response data.  This point on the frequency plot

represents the "half-power" amplitude, as compared to the

amplitude at a mid-band reference frequency.

***OTHER SYSTEM ORDERS***

As you are probably aware, a Graphic Equalizer can make a

tremendous effect on the way your speaker system sounds.  The

bad news is that an equalizer (EQ) usually doesn't have a control

exactly centered on the proper frequency to properly extend low

frequency cutoff, and thus will typically make matters worst.

PERFECT  BOX can incorporate the effects of either a 1st or

2nd order, ACTIVE EQ (that you must build) into the loudspeaker

model.  The affects of which are easily depicted in the PB Graphic

Plots.

This mini EQ consists of a simple active high-pass filter, having

a low turn-over (cutoff) frequency (typically  20-60  Hz),  which

actually has a slight boost (peak) in its amplitude response

above its cutoff frequency.  If the amount of boost and cutoff

frequency are suitably chosen, the total system response

typically is extended a half-octave or more, while

providing  infrasonic (rumble) filtering to boot.  In

2nd-order EQ's, the boost is controlled by the "D" parameter

(D=1/filter Q, for you filter buffs.)  Typical values  of

"D" range from 0.3 to 1.414, giving most to least amount of

amplitude boost.  A "D" of 0.52 gives approximately 6 dB of

boost, a typical starting value.  The EQ frequency and the

"D" factor should be juggled for the flattest, most extended low

frequency response.  Use the "EQ2.EXE" program to calculate

components to implement the 2nd order EQ.

A word of caution.  Using large amounts of boost may tax small

power amplifiers  (less than 50 Watts,RMS) beyond their

limits.  A properly designed 3, 3.5,  5, or 6th order system will

not over tax your amplifier unless you listen to music either :

1.  at extremely high SPL levels,

2.  with heavy (double fortissimo) organ pedal notes, or

3.  use other signal modification equipment such as an

extremely low-end boosted Graphic Equalizer

(quite stupid!), or Sub-Sonic Synthesizer (Harmonizer.)

The EQ can be inserted into the signal path of your system at

either a "Preamp-out/Amp-in" loop, or "tape monitor" connection.

A simple design for a 2nd order EQ is provided  with PERFECT BOX.

Type EQ2 at the DOS prompt to execute this option. You  must

have (at least) a CGA graphics card in order to run EQ2 because

the electrical schematic is drawn on the screen after all of the

circuit values are calculated.

IV.  RUNNING PERFECT BOX  -  PROGRAM EXECUTION:

If you  have  an  IBM, Epson or HP LaserJet compatible printer,

you can print the graphic plots generated by PERFECT BOX.  Perfect

Box 4.5 has built-in support for Graphic printing for these

printers in CGA and EGA modes.  You no longer have to use

the DOS GRAPHICS.COM or PERFGRAF.COM included with previous

releases of Perfect Box.  You might have to set your printer

switches to DEFEAT "automatic" Linefeed [LF] after a Carriage

Return [CR], (see your printer manual.)

I have included a simple  batch file called GO.BAT,

which will execute Perfect Box for you.  You may change this

batch file to suit your particular needs.  The actual program

file is BOX4_5.EXE.  You will not have to change the program

file name until you receive a future upgrade of PERFECT BOX.

Insert the PERFECT BOX program disk into your default disk

drive,  and type GO <Return>.  You should see the PERFECT BOX

opening screen.  Take the time to read this screen and remember

that many, many hundreds of hours of effort have been put into

this program.  If you use this program  more than five times, you

must be understanding the purpose so don't forget your obligation

to this author!

If  you  see  an  error  message  stating  "Bad  Command  or

Filename", check to see if you are in the  same  directory as the

PERFECT BOX files.  After about 5 seconds, you may  press  RETURN

to continue.

If all is well, you should see the following screen.

PERFECT BOX - Version 4.5            Current Time:

�͹ Boxdata Choices ������������ͻ   �����������������������������ͻ

�                                �   � Drivers on file :  1573     �

� (B) Browse the database        �   � Boxdata path  : G:\perfect\ �

� (L) Load a driver by #         �   � Graphics mode : EGA         �

� (S) Store New Driver data      �   �����������������������������ͼ

� (E) Edit the database          �                       Quick Mode

� (B) Boxdata Reports       ����������������������͹ Utility �ͻ

�                           �                                  �

���������������������������͹ (C) Change Boxdata path          �

� (P) Sort/Pack Boxdata file       �

��������������������ͻ   � (A) Append new records           �

� REGISTERED TO:     �   � (W) Write defaults               �

� Trial copy.        �   � (D) DOS Shell                    �

� Your name or       �   � (Q) Quit to DOS                  �

� company name should�   ����������������������������������ͼ

� be listed here.    �

�                    �

��������������������ͼ

Use PgUp - PgDn - Home - End -  -  - <RETURN>= Select - or HotKey

Output Menu

Each  of the menu choices may be executed by either pressing the

"key letter"  or  by using the cursor arrow keys to highlight the

choice and then pressing RETURN.  If you have a mouse, you can

choose by clicking the left button on any of the options

BOXDATA CHOICES:

(B)  Browse.  This option will allow you to view the drivers

currently in the BOXDATA file.  You can move through the list

using the arrow, Home, End, Page Up, and Page Down Keys.  The end

key takes you to the bottom of the list, and Home will return you

to the first listing.  Use these keys to point the "flashing

arrows" at your driver choice.  If you have sorted your Boxdata

file using the Sort/Pack option, you can press any of the Alpha

keys (A-Z) to jump to that corresponding Manufacturer name.  If

you have a mouse installed, you can move the highlight arrows

straight to the driver of interest with the left button.  The

right button will load the currently selected line.

Browse-Choose  the driver with the arrow keys and  press  RETURN.

�106  �PR 38 XM 150  �POLYDAX        �15",PRO,MONITOR,96dB     �

107   PR 38 XT 150   POLYDAX         15",PRO,FOAM EDGE,MONITOR

108   PR 38 XT 250   POLYDAX         15",PRO,99dB

109   PR 38 XT 350   POLYDAX         15",PRO,HI-POWER,100dB

110   TA-305 FSW     PRECISION       DUAL V.C. SUBWOOFER

111   TA-305F        PRECISION       OMEGA

112   TD-125F        PRECISION       ALPHA

113   TD-255F        PRECISION       EPSILON

114   TD-255F-SW     PRECISION       10",Alum Dual VC,90.7dB

115   TO-205F        PRECISION       INFINITE BAFFLE 8"

(** Only ten drivers are listed here for clarity. **)

<Esc>=Quit - <F2>=Edit - <F3>=Load defaults.

PgUp-PgDn-Home-End -   <INS>=Get #   < <� >= Get Record 106

Perfect Box Browse function.

After pointing to a particular driver, you may LOAD the driver

parameters by pressing RETURN.  You can EDIT the driver

parameters by pressing the <F2> function key.  The <F3> function

key will load any previous Box Volume, Vent Frequency, Order, D

factor,  and EQ frequency information   saved  in a previous

execution of PERFECT BOX, and initiate the calculation loop.  You

still can bypass the current selected driver and  load any other

driver by pressing <INSERT> and entering the record  number.  The

<F3> function key will take you back out to the opening menu.

When you return to the BROWSE function, the BOXDATA list will be

on the same page as the last time you were there.

(L)  LOAD:

This option allows you to  LOAD  driver  data  directly  from the

BOXDATA  file  using the record number.  On Floppy based systems,

this option might save you time by skipping the Browse feature.

(S)  STORE:

This option  allows  you  to  enter new driver data into the

BOXDATA file.  ALL parameters listed are required, so don't skip

any unless you are told you may enter a Zero.  If you don't have

all of the correct T/S parameters, you should try to obtain them

from the manufacturer.  In a pinch, you can approximate  values

from other drivers in the BOXDATA file.  Don't forget to add the

correct values when you  get them.  Of course, the accuracy of

this loudspeaker model is DIRECTLY proportional to the T/S

parameter accuracy!  Which brings me to say this:

I claim no responsibility  for  the completeness or accuracy

of the T/S parameters included with  your  copy of PERFECT

BOX.  Many (very many!) of the current drivers listed have

been appended from files sent to me by Registered users of

PERFECT BOX.  If you are unsure of  any  data,  it is your

responsibility to verify  the T/S parameters with

the manufacturer or test them yourself!

(Soapbox off.)

The new entries are placed  at  the  bottom  of the BOXDATA file

and once entered, can be LOADED immediately.

(E)  EDIT:

You can edit any of the  T/S parameters in the BOXDATA file. Use

the current driver record number  to  call  up the driver and

follow the screen prompts for instructions.  You can also edit a

driver directly from the BROWSE function.

(B)  REPORTS:

You can print the full BOXDATA file on your  printer  in two

forms.  The first form contains  ALL of the driver parameters

including  comments.  The second form will print the driver's

basic T/S parameters, Fs, Vas, and Qts, and use them to calculate

the "Ideal" 4th order Box volume, Vent frequency, and F3 cutoff

point.  This form is very handy to compare driver performance.

UTILITY

(C)  Change Boxdata Path:

The BOXDATA file containing all  driver  parameters,  can be

located  on  a  different  drive/directory  than  the current DOS

default.  Choose this option if you want to change  the  path  to

the data file.  Note:  The path is saved for future executions of

PERFECT BOX using the Write Defaults option.  You could use this

feature to maintain more than one BOXDATA4 file.

(P)  Sort/Pack

Each new driver that you enter into the BOXDATA file is placed

at the bottom of the file.  You can use the Sort/Pack option to

sort  the file alphabetically by Manufacturer.  If you have

marked a driver for deletion in the EDIT driver menu, the Sort /

Pack routine will purge it from the BOXDATA file.  This routine

is the an updated version of the sort program that was included

with earlier versions of PERFECT BOX, and thus (if you  have

them) you should delete the old BSORT.BAT and BOXSORT3.EXE

program files.

(A)  Append new records

This option is used to combine other BOXDATA files into the

current, active BOXDATA file.  If you are upgrading from Version

3.xx, rename the OLD data file to "DATAUP.DAT".  Start PERFECT

BOX and choose the Append option.  If you are upgrading from

Version 2.xx, you must run the 2to4.exe program first.  Version 3

uses a slightly different file structure, and thus the 2to4.exe

program will convert you old BOXDATA file into the new structure,

leaving a new file called "BOXDATA4".

Since this is the current name of the Version 4.0 data file, YOU

MUST DO THIS CONVERSION IN ANOTHER DIRECTORY THAN THE ONE

CONTAINING THE NEW BOXDATA4 FILE INCLUDED IN THE VERSION 4.5

ARCHIVE!  Either use a different directory or copy the NEW,

original BOXDATA4 to another directory for safe keeping.  I

suggest that you make backup copies of EVERYTHING, OLD AND NEW,

before attempting the conversion  process.   After running the

2to4.exe  program, rename  the resulting file from BOXDATA4  to

"DATAUP.DAT".  Copy "DATAUP.DAT" file into the PERFECT  BOX

directory, choose the APPEND function, and watch it fly!

(W)  Write defaults.

Each time PERFECT BOX is loaded, a file named PERFECT.CTL is read

for startup defaults.  There are many default values kept in

this file including:  1. Boxdata4 data file directory:   2.

Graphics mode:  CGA, EGA, or HERC;  3. "Quick mode" toggle; 4.

Printer Port assignment, mouse calibration values and other

variables.

The first two values have obvious purposes.  The third parameter

is used to hold the default mode for the calculation loop end

point.  In normal mode, the loop counts up to 200 Hz. In Quick

mode, the loop counts up to 100 Hz.  This is useful  for making

quick estimates on the slower "XT" type machines.  You can toggle

this mode within PERFECT BOX with the <F9> function key. The

write defaults option will over-write the configuration file with

current  values  for  BOXDATA  file  path, Graphics and Quick

Modes.  PERFECT BOX 4.5 uses a different format for this control

file than earlier releases.  You should delete you old

perfect.ctl file and restart PERFECT BOX.

(D)  DOS Shell

You can spawn another copy of COMMAND.COM from within PERFECT BOX

using this option.  Use this option to check directories or

file space.  Type EXIT at the DOS prompt to return to PERFECT

BOX. This option actually executes a batch file called "SH.BAT".

You could modify this batch file to run another program instead

of loading Command.com.

(Q)  Quit to DOS

Just like it says!  This choice will halt execution of PERFECT

BOX and immediately return you to the DOS.

OK, OK!, enough talk.  Let's see some action!

Now that you know what all of that stuff does on the opening

menu, choose a driver from the BOXDATA file.  Use the BROWSE

option, point at an interesting driver, and press RETURN. The

parameters listed are the minimum that you will need  to know

about  any  driver simulated with PERFECT BOX.  For this example,

answer "Y" to the question "Do you want this driver?"  Now choose

"1" for a single driver simulation.

Record # ���������>  106

Manufacturer �����> POLYDAX

Model Number �����> PR 38 XM 150

Comment ����������> 15",PRO,MONITOR,96dB

Free air resonance=  19.5 hz.

Vas =  24.37 ft^3.

Qts =  .27

Qes =  .3

Qms =  2.7

Re =   5.8 �

Xmax=  .1377953

Piston Diameter=  14 in.

Power= 150 Watts.

DO YOU WANT TO USE THIS DRIVER (Y)?

Now, PERFECT BOX will calculate the "Ideal" 4th Order Box volume,

the port frequency for that ideal box volume, and the cutoff

frequency (-3dB point.)  The ideal box volume is the smallest

enclosure that  provides the flattest, deepest low frequency

response. The cutoff point will give you a general idea of how

low the driver will perform in any box design (best case.) The

port frequency (box frequency) must be set as calculated to

obtain the cutoff predicted.

In a 4th order system:       Ideal vented volume is  8.53  Ft^3.

System cut-off (-3dB) is  32.54  Hz.

If the Vent frequency is  26.61  Hz.

�������������������������������������������������������ͻ

� ORDER �             SYSTEM DESCRIPTION                �

�������������������������������������������������������͹

�   2.  �  CLOSED BOX���without Active Equalizer        �

�   3.  �             ��with 1st Order Active Equalizer �

�  3.5  �             ��with 2nd Order Active Equalizer �

� � 4.  �  VENTED BOX���without Active Equalizer        �

�   5.  �             ��with 1st Order Active Equalizer �

�   6.  �             ��with 2nd Order Active Equalizer �

�������������������������������������������������������ͼ

Efficiency Bandwidth Product..(Fs/Qes).. is 64 , which suggests a

4th  order system.

Enter System Order [ 4  =default] ?

Enter BOX VOLUME (Ft^3) [ 8.53 =default] ?

Enter BOX VENT FR. (Hz.) [ 26.61 =default] ?

Now you must decide what type of enclosure to use.  If the driver

has a Qts of 0.5 or greater,  I would try infinite baffle

(order=2) systems first, otherwise, use Order=4,  with  the ideal

box size and frequency.  Save the other Order options for later.

Since PERFECT BOX will recommend the Order for you, use it as a

default.  Press RETURN to accept the default Order.

Enter  the Box volume in cubic feet.  I suggest that you use the

ideal box  size as a starting point.  Then, on the next pass,

change one of the  box  alignments and check how the response was

affected.  Again, press RETURN  to accept the default Box volume,

or change it if you prefer to do so.

Enter the Vent Frequency in Hertz.  Again, since this is your

first time through, press RETURN to accept the default value.

PERFECT BOX will now calculate :

1.  Amplitude response in decibels;

2.  Maximum Power in Watts;

and  3.  Maximum Sound Pressure Level, at each frequency cycle

between 10 and 200 Hertz.  (100  Hz  in  Quick mode.)

Please wait ... Calculating data for the frequency of  xx  Hz.

Be patient here. PERFECT BOX is running through a healthy chunk

of floating point calculations, just sit back, watch the counter

change and reminisce about the good old slide rule days.  When the

calculations are complete, an Output/Changes Menu will be

presented.  A math coprocessor chip makes this loop fly by in

about 2 seconds (if you have one installed!)

PERFECT    BOX    -   Version   4.5

�ͺ Output Choices ������������ͻ ��������������������������������ͻ

�                               � � Record # :  106                �

� (F) Frequency vs. Amplitude   � � Driver   : PR 38 XM 150        �

� (L) Excursion Limit plot      � � Manufacturer: POLYDAX          �

� (V) View data points          � ��������������������������������ͼ

� (U) QUAD-Plot Set-Up [ OFF ]  �

� (D) Print Data points         �                    Quick mode

� (P) Calculate Ports  �����������͹ Changes ��������������ͻ

� (W) Driver Worksheet �                                    �

�                      � (B) Change Box Volume or Vent freq.� �=2.857

����������������������͹ (E) Change EQ. Frequency or D      � h=1.365

� (A) Change All parameters          �

� (T) Toggle graphics mode (EGA)     �

� (C) Change QLoss    ���������������ʹ Done̻

�       (Qloss = 7 )  �                      �

���������������������͹ (S) Load/Save Align. �

� (R) Retrieve another �

� (Q) Quit to DOS      �

�                      �

����������������������ͼ

Use PgUp-PgDn-Home-End-  -  - <RETURN>= Select - or HotKey

Output/Changes Menu

OUTPUT CHOICES:

(F)  Amplitude vs. Frequency Plot

This choice will plot the classical Bode (pronounced BO-DEE) or

Amplitude Response plot.  If the Quick mode was on during

calculations, the plot will stop a 100 Hz. You may print a copy

of the plot on your graphics printer by pressing a "P".  This

pops up a nice time-date stamp, dumps the plot to your printer.

After the printer is finished, you can send a Form Feed (FF) to

your printer by pressing the <F10> function key.

(L)  Excursion Limit Plot

This graph of SPL & Power vs. Frequency, will give you an picture

of the system driven  at  maximum power.  This plot gives the

Sound Pressure Level (in dB) and  the input power required to

reach the excursion limit (Xmax) of the driver.  See the Design

Notes chapter of this document for more discussion of this

important graph.

(U)  QUAD-Plot Setup  (New to Version 4.5)

������������Ŀ

�Current Data�

�   in RAM.  �

��������������

�Trace toggle  Model#: Filename:  Load Data:

���> F1 On  ��Ŀ

�

F2 Off ����< --- �Empty      <� F6Ŀ

�                        �

Press F1-F4   F3 Off ����< --- �Empty      <� F7Ĵ

to enable               �                        �

that trace #. F4 Off ����< --- �Empty      <� F8Ĵ

�                        �

�                        �

����������������

�From Disk file.�

�����������������

Press F10 to Exit

Save driver data in the Output Menu with the F7 key.

You may load up to three data files for display

in the QUAD Plot mode.

QUAD-Plot Setup Menu

For EGA users, the QUAD-Plot function will allow you to compare

Amplitude Plots from the current run with up to three other runs

previously saved from the Output Menu with the F7 option.

Once in the QUAD-Plot Setup menu, you load the data files with

function keys F6, F7 and F8.  The function keys F1-F4 will toggle

each trace off and on, allowing printout of fewer traces than the

loaded number.  Once you have loaded your comparison files, press

F10 to return to the Output Menu.  You should notice that the

prompt beside the QUAD-Plot Setup function should say

"[ ON ]" This is automatically set to ON after leaving the QUAD-

Plot Setup Menu.  You can toggle the QUAD-Plot OFF/ON with the F4

key in the Output Menu.  Turning OFF the QUAD-Plot will not erase

the currently loaded data, and will allow you to bounce back and

forth between the usual plot modes and QUAD-Plot.

The QUAD-Plot function is available only in the EGA graphics

mode.  If your machine cannot support EGA 43 line mode, you

cannot use QUAD-Plot.  Also, your mouse becomes useful tool

during the QUAD-Plot display.  Without a mouse, some features

will be unavailable to you, but you still can view up to four

graphs at once.

Perfect Box (4.5) � QUAD-Plot

|                                    Fb

����������������������������������������������������������������Ŀ

�Registered to:�+6 �                                             �

�              � 5 �                                             �

�Your name or  � 4 �                                             �

�Company should� 3 �                                             �

�be listed     � 2 �                                             �

�here.         � 1 �                                             �

�              � 0��������������������������������������         �

�              �-1 �                                             �

�              �-2 �                                             �

�              �-3 �                                             �

�              �-4 �                                             �

�              �-5 �                                             �

�              �-6 �                                             �

�              �-7 �                                             �

�              �-8 �                                             �

�              �-9 �                                             �

�              �-10�                                             �

�  PERFECT BOX �-11�                                             �

�              �-12�                                             �

�    by        �-13�                                             �

�  Warren      �-14�                                             �

�     A.       �-15�������������������������������������������   �

�    Merkel    �                                                 �

�              �   20      30   40   50 60 7 8 9 100    200 Hz   �

����������������������������������������������������������������Ĵ

�              �               �          �          �           �

�              �Trace 1        � Trace 2  �  Trace 3 �  Trace 4  �

�At Magnifier: � 3124          � Mod:     � Mod:     � Mod:      �

�Amp =         � ALTEC         � Man:     � Man:     � Man:      �

�Freq=         � From RAM      � File:    � File:    � File:     �

�At Ruler:     � Vb  : 2.79 CF � Vb  :    � Vb  :    � Vb  :     �

�Amp =         � Fb  : 33.44 Hz� Fb  :    � Fb  :    � Fb  :     �

�Freq=         � Ord : 4       � Ord :    � Ord :    � Ord :     �

�              � Feq :         � Feq :    � Feq :    � Feq :     �

�              � D   :         � D   :    � D   :    � D   :     �

�              �       Single  �          �          �           �

������������������������������������������������������������������

Press P to Print � <�� to continue

The QUAD-Plot Screen (not to scale)

Each trace is plotted in a different color and can be temporally

erased with function keys F1-F4 or with a left click in the data

area for that particular trace.  If you have a mouse, you can use

the cursor as a magnifier to determine which Trace is where.  As

you pass over a particular trace, a asterisk will flash beside

that Trace's number.

You can also set a ruler line at any point on the graph with the

left mouse button.  This will allow quick measurements of any

point on the plot.  The ruler will be drawn from the center of

the magnifier to each axis, and the values for the ruler will be

locked at the "At Ruler" area.  To erase the ruler, click the

right mouse button.  The "At Magnifier" values are constantly

updated to the cursor magnifier position.

You may have to calibrate your mouse to read the plot correctly.

If the magnifier values do not reflect the actual point on the

plot, move the magnifier to the words "At Magnifier" and click

both buttons.  You will be asked to move the magnifier to the

20Hz-0dB point an click the left button.  Then you will be asked

to move the magnifier to adjust the "At Magnifier" values to

reflect exactly 0dB at 20 Hz.  Then click left again.  Then, if

you are satisfied that you correctly positioned the mouse, press

the right button to save these new calibrated values in the

Perfect.ctl file.  You can abort by pressing the right button.

As with the other Plot screens in PERFECT BOX, you can press a

"P" to print the screen on your graphics printer.  The F10 key

will cause a Form Feed command to be sent to your printer.

At this time, only the Amplitude Plots can be compared.  If

sufficent user interest dictates the addition of the SPL-Power

plots into the QUAD-Plot function, I will work that mode into the

next release.  (Registered users first though...)

(V)  View data points

This option  will  list  on the screen, all calculated data,

between the frequency limits of your choice.

(D)  Print data points

This option will print on your printer, all calculated data,

between the frequency limits of your choice.

(W)  Driver Worksheet

This  option produces on your printer, a worksheet which

gives you a handy reference table to use as you change the

different alignments about your system.  Each block can be filled

from data printed on the Frequency vs. Amplitude plot option.  I

suggest that you do this option first (only if you have a printer

on-line.) and log each repetition of your progress.

(B)  Change Box Volume or Vent Frequency

(E)  Change EQ. Frequency or D

(A)  Change ALL parameters

At this point you may choose what parameter to change in an

attempt to optimize the frequency plot, or what ever else you are

trying to achieve (maximum power for instance.)  If you are

currently working on a 2nd or 4th order system, the "Change

EQUALIZER Frequency or D." choice will not be allowed.  Thus if

you want to change to an "equalized" order, you must use the

"Change ALL Parameters" option.  Remember that the previous value

for each prompt in the CHANGE menu, there will be the default

value (just press RETURN.)

(T)  Toggle graphics mode

This choice toggles you between EGA and CGA modes.  This is

really not needed anymore.  I left it in for those of you whom

can't get the build-in graphics dump program to function properly

with your printer.  You could for example, have the DOS

GRAPHICS.COM loaded before PB, and switch to CGA for printing.

(C)  Change Qloss

This option allows you to account for box leakage losses. Normal

vented cabinets  are modeled at Ql=7.  If you do not have the

means to accurately measure Qloss, then leave Ql=7.

(S)  Load/Save Alignment

This choice allows you to save your current design parameters to

the BOXDATA file for future use.  This becomes very handy if you

want to keep track of your best designs  for  each driver.

(R)  Retrieve another driver

This choice will dump all calculated data, and return you to the

opening menu.

(P)  Calculate Ports

After you are satisfied with the current design parameters,

PERFECT BOX will calculate the port lengths required to tune the

cabinet to the frequency currently chosen.  A column for two

identical length ports is also given.  Dual ports are useful when

the box volume or the MACH speed is too large for a single port.

The MACH number is the percent of the speed of sound that will

occur at the largest port output.

POLYDAX             PR 38 XM 150

BOX VOLUME=  8.53  Ft^3   BOX FREQUENCY=  26.61 Hz.  ORDER :  4

HEIGHT=  39.72  in.   WIDTH=  24.52  in.   DEPTH=  15.2  in.

VENT LENGTHS FOR  TYPICAL  INTERNAL  DIAMETERS  OF  PVC  PIPE

(Inches)

DIAMETER      LENGTH       MACH       DUAL VENTS    MACH

-------       ------       ----       ----------    ----

1           -.38        1.243 NO !    |   -.03       .311  NO!

1.5         -.31        .553  NO !    |    .48       .138

2           -.06        .311  NO !    |   1.35       .078  Best

2.5          .37        .199  Use Dual|   2.57       .05   Best

3            .97        .138  O.K.    |   4.13       .035  Best

3.5         1.75        .101  O.K.    |   6.05       .025  Best

4           2.7         .078  Best    |   8.32       .019  Best

4.5         3.83        .061  Best    |  10.95       .015  Best

5           5.13        .05   Best    |  13.92       .012

Vent Calculations.

Stay away from large (speed> 0.2) port MACH speeds as they

will cause unmusical whistle noises from the port.  Ports that

are longer than the box depth, can be curved inside the enclosure

with PVC swept elbows.

V.  DESIGN HINTS:

If  the ideal box volume is too large to be realistically built,

PERFECT BOX will allow you to see just how the loudspeaker

performance will suffer, as you down size box volume.  It is do to

this iterative design process that PERFECT BOX accels other

modeling programs.  By changing either box volume, port

frequency, or in EQ cases, EQ frequency and boost, a "perfect"

frequency curve should be obtainable.  At this point you can tell

how good a driver really is because sometimes one driver will

out perform others in half the enclosure volume.

The port frequency is set by the area and length of the vent.

PERFECT BOX will calculate the port for you, but the length is

typically 10% off, so add an half-inch to the calculated value,

and measure the box frequency for yourself.  The system will not

perform as calculated if you miss the port frequency by as little

as 1 Hz, so try to hit it as close at possible.  Also, after you

have reached a final box size, increase the box volume by as much

as required to account for volume used by the drivers,  bracing,

and crossover network.  It is much better to error on the high

side of Box Volume.  I typically add 10% to the final size as a

fudge factor.  You can always add braces to account for any

overage.

To find box dimensions from the box volume, take the volume

in cubic feet, multiply by 1728 cubic in/ft^3, to get in^3.  Then

take the cube root (or raise it to the 0.3333334 th power.)  Now

this number is the width in inches.  Multiply the width by 1.62

for the height, and multiply the width by 0.62 for the depth.

volume (in^3)  =  volume (ft^3) * 1728

width (in.)    =  (volume (in^3) )^0.3333334

height (in.)   =   width * 1.62

depth  (in.)   =   width * 0.62

NOTE: All dimensions are internal, not external!

This will give the box a typical speaker "look" and also set the

dimensions up for minimum internal reflections.  PERFECT BOX

calculates these dimensions for you and prints them in the port

length table.

VI.  COMMAND LINE SWITCHES

There are a few command switches which will override the settings

in the PERFECT.CTL file.

/CGA   = Sets graphics mode to CGA.

/EGA   = Sets graphics mode to EGA.

/HERC  = Sets graphics mode to Hercules.

/LPT1  = Sets the active printer port to LPT1:

/LPT2  = Sets the active printer port to LPT2:

/LPT3  = Sets the active printer port to LPT3:

/QUICK = Runs PB in the fast mode (10-100 Hz).

/RESETMOUSE = Clears previous mouse calibrations.

/NOMOUSE = Defeats all mouse functions.

/PTF   = Reroutes all text printing to a file called

"Printer.out".  All text output is APPENDED to this file.

/NOPAPER = Defeats the printer error checking.

This might be useful if you are using a "Print-TO-File" TSR to

grab output.  As it stands now, the Novell "SPOOL" command is

supported for LPT1: if your workstation has an active local

printer attached.  The /NOPAPER option bypasses local printer

checking and allows SPOOL to capture LPT1 correctly without a

local printer attached to LPT1:.

All values affected by the Command line switches except /PTF can

be made the current default by the "W" Write Defaults command in

the opening menu.

NOTE:  The /NOTEST switch is no longer supported (or required.)

If you have a 8087/80287 coprocessor installed, PB will

automatically use it for all Floating Point calculations.  If for

some reason, you would like to defeat the coprocessor from

functioning, set an Environment variable in your CONFIG.SYS file

NO87=Coprocessor is Defeated

and reboot, before executing Perfect Box.  You may also use the

DOS "SET" command to create the NO87 variable.

VII.  REFERENCES:

This document and the program PERFECT BOX are only tools for

the speaker designer.  One must know how to use these tools

properly before judging the usefulness of PERFECT BOX.  I suggest

that you read or subscribe to any of the following publications

on the finer art of speaker building.

SPEAKER BUILDER MAGAZINE (6 times a year)

Edward T. Dell, Jr., Publisher

PO Box 494 Peterborough, NH 03458

An excellent publication.  Articles vary from the most basic

"cookbook" projects, to the exotic mathematical theories used in

PERFECT BOX.  Electronic projects and crossover designs are

covered in depth.  Many of the systems discussed are years before

the designs hit the market.  Articles are written by

knowledgeable authors well known in the field, ideas of which

have been mimicked by the major Speaker Manufacturers.  Now with

full color layouts, software and book reviews, Speaker Builder is

an absolute must.

"The Loudspeaker Design Cookbook"

by Vance Dickason

Distributed by:

Old Colony Sound Lab

PO Box 243

Peterborough, NH 03458-0243

Hats Off! to Dickason for handy reference to all design formulas

required by the speaker designer.  Covers explanations of many

types of crossover designs and enclosure systems, including Push-

Pull, and Isobaric.

"How to Design, Build, & Test Complete Speaker Systems"

by David B. Weems

Library of Congress #TK9968.W43

Published by:

TAB Books

Blue Ridge Summit, PA  17214

No. 1064

The perfect starter book in speaker design.  Written in a down to

earth style, Weems covers the T/S theories with just enough math

to get the job done correctly without being overbearing to the

beginner.  Many complete projects and testing methods are

covered.  A beginners standard for the past decade!

VIII.  ERRATA:

Your support helps! PERFECT BOX is supported by YOU, the user.

Without user support, Shareware such as PERFECT BOX cannot

continue to exist!  Use this program!, give a copy of PERFECT BOX

to your friends  (the BBS Archive file is the best way to make

sure all the PERFECT BOX files are present.)  Above all, don't

forget your part in the deal!  If you are using PERFECT BOX on a

trial basis, that's fine.  Determine its usefulness in accurately

designing your new speaker system, then REGISTER!  The money that

you save by building your own system can easily cover the

registration fee.

If you are using PERFECT BOX in a business or commercial

environment, you MUST Register, PERIOD.  IF YOU ARE MAKING A

COMMERCIAL PROFIT USING PERFECT BOX WITHOUT REGISTRATION, YOU ARE

IN VIOLATION OF YOUR SHAREWARE AGREEMENT WITH THIS AUTHOR.  Your

job is easy with PERFECT BOX in your tool kit.  Don't forget who

bought you that tool!

Many registered users are using PERFECT BOX for professional

sound reinforcement or automotive sound applications.  If this is

true for you, just don't forget the concepts under which

shareware functions.  Perfect Box can easily pay for itself with

ONE SALE.

PERFECT BOX is under constant scrutiny for improvement.  If you

have any ideas for new features/functions, or problems in program

operation, send them to me at the address on the next page.

Your suggestion could be included in the next revision of PERFECT

BOX.  By registering your copy, you will be sent (if requested) a

disk with the current Version installed in your name.  You will

also receive a copy of the next MAJOR PERFECT BOX revision, and

new BOXDATA file parameters.  Registered users can have their

current copy updated at any time.  Also, Registered users can

download the future releases of PERFECT BOX directly from my 24

hour BBS at FIDONET 363/507 (407-281-5522 : 2400-N-8-1),

including unlimited "Online  support" through BBS E-Mail.

If you want  to others to share in your BOXDATA resources, please

send a COPY of your BOXDATA file on disk (5.25" or 3.5") to me

in a U.S. Post Office approved disk envelope (or safe

equivalent.)  Or, upload your BOXDATA file to my BBS.

TO REGISTER:

Send $40.00 in U.S. funds (personal  checks accepted)  to cover

registration and return postage.  IT'S THAT SIMPLE.

The un-registered copy is NOT "crippled" in any way.  All PERFECT

BOX functions are available on the un-registered program.

Please make checks payable to:

>>>    WARREN A. MERKEL     <<

and send your contribution to:

Warren A. Merkel

2851 Newcomb Ct.

Orlando, FL  32826

(PERFECT BOX 4.5)      *** End of Document ***     (PERFECT.DOC)