PRESS [arrow keys] to scroll through this document:

PENTAX PCS-2s:

Initial settings of the instrument should be checked.

Install instrument on stand now and proceed per below:

"At Instrument: PRESS [ON/OFF]+[left arrow]."

"At Instrument: Use arrow keys to select various options."

"               Press [up arrow] or [dn arrow] to proceed to the next Item."

"              MODE A"

"              Specifically insure each of the below Items:"

"              1) INT MEAS = ANG for angle measurements only ."

"                          = DIST for angle & distance (ETTS) ."

"              2) PRSM CONST = prism constant ."

"              3) V ANG MODE = Z ."

"              4) ANG DISPLAY = FINE ."

"              5) ANG UNIT = GRD {preferred} or DEG ."

"              6) DIST UNIT = FT {preferred} or M ."

"              7) SHOT MEAS = 1 ."

"              Press [OK] ."

"At Instrument: PRESS [ON/OFF]+[right arrow]."

"At Instrument: Use arrow keys to select various options."

"               Press [up arrow] or [dn arrow] to proceed to the next Item."

"              MODE B"

"              Specifically insure each of the below Items:"

"              1) POWER OFF = NO ."

"                 !!!!Caution!!!!: Powering OFF Instrument may"

"                 cause absolute H Angle loss and thus require"

"                 re-orientation."

"              Press [OK] ."

"At Instrument: PRESS [ON/OFF]+[dn arrow]."

"At Instrument: Use arrow keys to select various options."

"               Press [up arrow] or [dn arrow] to proceed to the next Item."

"              MODE C"

"              Specifically insure each of the below Items:"

"              1) BAUD RATE = 12 {1200 Baud}."

"              2) DATA BITS = 8 ."

"              3) PARITY = NON ."

"              4) STOP BITS = 1 ."

"              Press [OK] ."

"General Notes:"

"1) The main scope must be plunged through the horizon {level} to initialize."

"2) To initialize the Horizontal circle to zero: Press [0 SET] twice."

"3) To change the temp,pressure,or prism settings {affects distance"

"   measurements}:"

"   a) Press [MODE]; b) Press [PPM.PSM]"

"   c) Press [left arrow] or [right arrow] to select temperature digit."

"      Press [up arrow] or [dn arrow] to increase or decrease number."

"   d) Press [OK] to store the new value."

"   e) Repeat above (c & d) for pressure."

"   The error is .036 inch (.003 ft) @ 300 feet per +2 deg.F."

"   To change the prism constant:"

"   f) Repeat above (c & d) for the prism constant."

"END."

Typical RS232 Cable Configurations (Ref. Only):

1:

Inst          Inst Cord          Std RS232 Cable      Computer

[PCS-2s] [Limo Conn]-----[DB-9] [DB-9]----------[DB-9] [PC]

2a:

Inst          Inst Cord               Long-Range Cable        Std RS232 Cable   Computer

[PCS-2s] [Limo Conn]-----[DB-9] [DB-25/1000$/DTE]--[DB-9/1009$] [DB-9]----[DB-9] [PC]

2b:

Inst          Inst Cord                              Long-Range Cable            Std RS232 Cable   Computer

[PCS-2s] [Limo Conn]-----[DB-9] [3PMF power] [DB-25/1000$/DTE]--[DB-25/1000$/DCE] [DB-25]----[DB-9] [PC]

^--+V

3a:                                       Cable       Cable

Inst          Inst Cord        Special Null Modem  Power     Long-Range Cable                    Std RS232 Cable   Computer

[PCS-2s] [Limo Conn]-----[DB-9] [p2/p3 switch cable] [V] [DB-9/1009$]--[DB-25/1000$/DCE] [MP Box] [DB-25]----[DB-9] [PC]

^--+9V                              ^--+V

3b:

Inst          Inst Cord                              Long-Range Cable                                  Std RS232 Cable   Computer

[PCS-2s] [Limo Conn]-----[DB-9] [3PMF power] [DB-25/1000$/DTE]--[DB-25/1000$/DCE] [3PMF power] [MP Box] [DB-25]----[DB-9] [PC]

^--+V                                             ^--+V        ^--+V

4:

Inst          Inst Cord          Std RS232 Cable                  Std RS232 Cable     Computer

[PCS-2s] [Limo Conn]-----[DB-9] [DB-9]----------[DB-25] [MP Box] [DB-25]--------[DB-9] [PC]

^--+V

Pentax PigTail Cord Circuit (06-01-95):

Pin #        Pin #

LIMO CONN    DB-9 CONN

(1)----------(2)

(2)----------(3)

(3)--�       (4)--�

(4)--�            �

(5)----------(5)  �

(6)               �

(6)--�

Cable Power test @ 1009$ Conn (ref only):

note: current draw of 1009$ is 10mw @ 9V.

+9v dc ------(pin 4/DB-9)

-9v dc ------(pin 5/DB-9)

TO EXIT:

If using MS-DOS Editor--> PRESS [Alt],[ENTER],[X]

For Other Editors--> Follow specific editors menus.