+------------------------------------------------------------+

|     Trackball Driver Release Notes (version 10.10)         |

|     Copyright (C) KYE Systems Corp. 1992                   |

+------------------------------------------------------------+

This document contains information that supplements the

"Trackball User's Guide" and accompanying software.

CONTENTS

(1). Notes on Setup for DOS and Windows

PATH Modifications

CONFIG Modifications

Operation from Floppy Disk Drive

(2) Modifying POINT.INI

Acceleration Profiles

(3). Miscellaneous Items

Loading POINT.INI Settings

=========================================================================

(1). Notes on Setup for DOS and Windows

**********************

* PATH Modifications *

**********************

If you have an older version of the mouse driver in your system,

delete it from your system and use a text editor to remove

the old directory from the PATH statement in your AUTOEXEC.BAT file.

************************

* CONFIG Modifications *

************************

If you have an older version of the Trackball driver - POINT.SYS

in your CONFIG.SYS file, the Setup program will eliminate it

and save the original CONFIG.SYS file as CONFIG.BAK.

************************************

* Operation from Floppy Disk Drive *

************************************

If you don't have a hard disk, you can load the DOS Trackball driver and

DOS Trackball Control Panel into the memory from the Floppy Disk Drive

and operate from there. A minimum of 256K RAM is necessary.

=========================================================================

(2). Modifying POINT.INI

The POINT.INI file contains the program information and the default

settings of the trackball driver program. Any changes to it will affect

the operation of your trackball. It is recommended that only advanced users

make changes directly to this file. Before making any changes to the file,

always first save it as a backup file in case of mistakes, i.e. POINT.BAK.

*************************

* Acceleration Profiles *

*************************

The acceleration parameters determine the relative speed of cursor movement

for set distances of trackball movement within a standard amount of time.

On the existing four acceleration profiles, speed is slowed for shorter

distances and increased for longer distances. Thus, when operating your

trackball, you get more precision in short distances, but are still able to

move the cursor across the whole acreen with a quick, short movement.

Acceleration curves work by establishing multipliers for different distances

covered in a single unit of time. The multiplier adjusts the speed set under

the Sensitivity setting. Thus the multiplier may be 0.5 for a short distance

to slow the movement speed by one-half, but for a long distance the

multiplier may be 2.0 thus doubling the movement speed. Here is a sample

profile.

[AccelerationProfile2]

Label=Cruising

Movement=1    26    42     61    80

Factor=1.00  1.25  1.50   1.75  1.50

You may make changes directly to the existing four acceleration curves. In

each set of figures the upper line contains the number of "mickeys", which

for input devices is unit of distance per unit of time. The number of mickeys

cannot exceed 127. Below each mickey designation is the multiplier. Be sure

to leave at least one space between each number. Each line must have less

than 80 characters. Do not use tabs, use the space key.

=========================================================================

(3). Miscellaneous Items

******************************

* Loading POINT.INI settings *

******************************

The settings in the POINT.INI file are read by both the DOS and Windows

trackball drivers only when they load. Similarly, the Windows Trackball

Control Panel reads the settings in POINT.INI only when it is started.

Saving changes permanently from any of the trackball utilities updates

both the settings in driver memory and the settings in the POINT.INI file.

However, if you edit the POINT.INI file manually, your new settings

will not be recognized by either trackball driver until it is reloaded.

To make your changes to POINT.INI settings take effect in DOS:

1. Save the edited POINT.INI file.

2. Restart your computer.

Or,

Remove the trackball driver from memory by typing POINT U;

then reload the trackball driver by typing POINT.

To make your changes to POINT.INI settings take effect in Windows:

1. Save the edited POINT.INI file.

2. Exit Windows.

3. Restart Windows.

=========================================================================